IEEE P802.11
Wireless LANs

| LB97 PHY Comment Resolutions: 40 MHz Operations |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-06-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bjorn A. Bjerke | Qualcomm, Inc. | 9 Damonmill Sq. Suite 2A Concord, MA 01742, USA | +1 781-276-0912 | bbjerke@qualcomm.com |

Abstract

This submission suggests resolutions of LB97 PHY comments related to the sub-topic 40 MHz Operations. The following CIDs are addressed: 800, 1582, 2649, 2650, 2677, 2685, 3185.

The changes marked in this document are based on TGn Draft 2.04.

CIDs:

| 800 | 223.00 | 20.2.2 | Definition of CH_OFFSET parameter does not account for transmission of NON_HT PPDUs in 40 MHz, which can happen with non-HT duplicate transmission or with transmission of 20 MHz non-HT PPDU in a 40 MHz mask. | Delete text "FORMAT is HT_MF or HT_GF" in the "Condition" column in the first sub-row under "CH_OFFSET". Delete the second sub row under "CH_OFFSET" with "Otherwise" in the "Condition" column. | Counter. Accept in principle. See resolution of CID 2900, already implemented in D2.04 (see Table 178, the row labeled “CH_OFFSET”). |
| --- | --- | --- | --- | --- | --- |
| 1582 | 223.29 | 20.3.2 | Here is a description of "NON_HT_CBW40 for non-HT duplicate, NON_HT_CBW20 for all other non-HT". However, according to the definition of CH_BANDWIDTH parameter and CH__OFFSET parameter in Figure n55, CH_OFF_20U and CH_OFF_20L also use NON_HT_CBW40. | Please be consistent. | Counter. The resolution of CID 2907 adopted and implemented in D2.04 resolves the inconsistency. CH_OFF_20U/L is now defined for NON_HT_CBW20 and not defined for NON_HT_CBW40 |
| 2649 | 223.29 | 20.3.2 | "NON_HT_CBW40 for non-HT duplicate" This is incomplete, because NON_HT_CBW40 also generates 40 MHz Non-HT upper and lower formats. | Replace with: "NON_HT_CBW40 for non-HT duplicate, 40 MHz non-HT upper and 40 MHz non-HT lower formats. | Reject. After the adoption and implementation of CID 2907 in D2.04 (see Table 179), NON_HT_CBW40 only generates non-HT duplicate, whereas NON_HT_CBW20 generates the 40 MHz non-HT upper and lower formats (in addition to 20 MHz non-HT format) |
| 2650 | 223.58 | 20.3.2 | The "otherwise, not present" entry for CH_OFFSET ignores the existence of NON_HT_CBW40 entries. | Add a new row before the "otherwise row": condition=FORMAT is NON_HT and CH_BANDWIDTH is NON_HT_CBW40. value="The CH_OFFSET parameter indicates which portion of the channel is used for transmission. Enumerated type: CH_OFF_40 indicates non-HT duplicate format CH_OFF_20U indicates non-HT upper format CH_OFF_20L indicates non-HT lower format" txvector=yes rxvector=no | Counter. The resolution of CID 2900 adopted and implemented in D2.04 resolves the issue by adding NON_HT with modulation NON-HT-DUP-OFDM or UPPER-20-NON-HT-OFDM, or LOWER-20-NON-HT-OFDM to the existing FORMAT condition. |
| 2677 | 236.38 | 20.3.4 | "Determine whether 20 MHz or 40 MHz operation is to be used from the CH_BANDWIDTH, CH_OFFSET field in the TXVECTOR." There's enough confusion about what's 40MHz and what isn't, that this description should call out the specific combination(s) of these parameters that make this determination. | Add an explicit description of the parameter values that determine "20MHz operation". (i.e. I want to know whether 40MHz upper/lower is viewed as "20MHz operation" for the purposes of this paragraph). | Accept. Modify text in step o) of 20.3.4 Overview of the PPDU encoding process accordingly. 20 or 40 MHz operation is now uniquely determined from CH_BANDWIDTH. Refer to 07/2046r0. |
| 2685 | 240.40 | 20.3.7 | "two adjacent 20 MHz channels are used" I disagree. I think we have a single 40MHz channel. There is nothing special about 20MHz channels. Channel numbers relate to a 5MHz grid, and we don't constantly describe a 20MHz channel as "four adjacent 5MHz channels". | Replace with: "a 40MHz channel is used" | Accept. |
| 3185 | 256.40 | 20.3.9.4.6 | In the sentence "In a 40 MHz transmission the sequence to be transmitted is based on:" it is not necessary to write "based on". | Change the sentence to: "In a 40 MHz transmission the sequence to be transmitted is:". | EDITOR: 2007-05-01 14:23:57Z - transferred to PHY. EDITOR: 2007-05-01 14:21:35Z - Please see resolution of CIDs 3180 and 3181. I would like to make the same change here, if applicable. However in the previous cases, the change was a rearrangement of existing text. Here there is no reference to the phase rotation. If correct, for consistency I would like to make similar changes to those in 3180 and 3181. Accept. Modify the sentence as proposed by the commenter. The additional changes suggested by the Editor (similar to resolutions of CIDs 3180 and 3181) are not correct, strictly speaking, as they confuse “OFDM symbol” with “sequence”. There is no need to mention the 90 degree rotation when defining the sequence. This is taken care of by the equation describing the waveform. For consistency with this resolution, make the following change to the text in 20.3.9.4.5 describing the HT-STF for 40 MHz: replace the sentence just prior to Eq. (65) with “For 40 MHz:” |

TGn Editor: Modify the first part of step o) of 20.3.4 Overview of PPDU encoding process as follows:

Determine whether 20 MHz or 40 MHz operation is to be used from the CH_BANDWIDTH, CH_OFFSET field in the TXVECTOR. Specifically, when CH_BANDWIDTH is HT_CBW20 or NON_HT_CBW20, 20 MHz operation is to be used. When CH_BANDWIDTH is HT_CBW40 or NON_HT_CBW40, 40 MHz operation is to be used.  For 20 MHz operation (with the exception of non-HT formats), insert four subcarriers as pilots into positions –21, –7, 7, and 21. The total number of the subcarriers, NST, is 56. For 40 MHz operation (with the exception of MCS 32 and non-HT duplicate format),  insert six subcarriers as pilots into positions -53, -25, -11, 11, 25, and 53, resulting in a total of NST=114 subcarriers…

TGn Editor: Modify the third paragraph of 20.3.7 Mathematical description of signals as follows:

In the case of the 40 MHz HT Format, two adjacent a 240 MHz channels areis used. The channel is divided into 128 sub-carriers. The signal is transmitted on sub-carriers -58 to -2 and 2 to 58.

TGn Editor: Modify the sentence just prior to Equation (65) as follows:

The HT-STF in 40 MHz is based onFor 40 MHz:

TGn Editor: Modify the sentence just prior to Equation (69) as follows:

In 40 MHz transmissions, including HT duplicate format frames, the sequence to be transmitted is based on (#3186):