IEEE P802.11
Wireless LANs

| Comment Resolutions for Measurement |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-09-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Yongho SEOK | LG Electronics | 16 Woomyeon-Dong, Seocho-Gu, Seoul 137-724, Korea | +8225264225 | yhseok@lge.com |

Abstract

This document is a normative text proposal to provide comment resolution for TGv Letter Ballot 108 comment 66, 202, and 263. Changes are shown with respect to TGv draft D1.0.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

        - Change 7.3.2.21.8 as shown

- 7.3.2.21.8 STA Statistics Request

|  | Randomization Interval | Measurement Duration | Group Identity | Triggered Reporting (optional) |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 1 | variable |

Figure 79e—Measurement Request field format for a STA Statistics Request

Table 29c—Group Identity for a STA Statistics Request

| Statistics Group Name | Group Identity |
| --- | --- |
| RSNA Counters | 11 16 |
| Reserved | 12 17 – 255 |

The Measurement Duration field is set to the duration of the requested measurement in TUs. except for the triggered reporting. When the triggered reporting is used, the measurement duration is set to 0. When requesting a triggered STA Statistics report, the Measurement Duration is not used and is set to 0.

The Triggered Reporting field is used to specify trigger conditions and thresholds for triggered STA Statistics measurements. It is present only when setting up triggered reporting from STA Counters, QoS STA Counters, or RSNA Counters, see .

The format of the Triggered Reporting field for STA Counters is shown in Figure 79e1. The fields marked as optional are the fields that are only present if the appropriate bit in the STA Counter Trigger Condition is set to 1.

|  | STA Counter Trigger Condition | dot11Failed Threshold (optional) | dot11FCS Error Threshold (optional) | dot11Multiple Retry Threshold (optional) | dot11Frame Duplicate Threshold (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 2 | 4 | 4 | 4 | 4 |
|  | dot11RTS Failure Threshold (optional) | dot11ACK Failure Threshold (optional) | dot11Retry Threshold (optional) | Measurement Count | Trigger Timeout |
| Octets: | 4 | 4 | 4 | 4 | 2 |

- Figure 79e1— Triggered Reporting Field for STA Counters

The STA Counter Trigger Condition field is a bit-field that specifies reporting triggers when requesting triggered STA Statistics reporting. The format of the STA Counter Trigger Condition field is shown in Figure79e2.

|  | B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7-B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dot11Failed | dot11FCS Error | dot11Multiple Retry | dot11Frame Duplicate | dot11RTS Failure | dot11ACK Failure | dot11Retry | Reserved |
| Bits | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 9 |

- Figure 79e2— STA Counter Trigger Condition field

- Each bit in the STA Counter Trigger Condition field is set to 1 to request that a STA Statistics Report frame be generated when the corresponding STA counter defined in Figure 85h and Figure 85i (in Error: Reference source not found) increases by a value greater than or equal to the value of the corresponding STA counter threshold in Figure v1 79e1, within the total number of MSDUs MPDUs given in Measurement Count. One or more trigger conditions may be set with specified thresholds. When the group identity of the triggered STA Statistics request is set to 0, B2 – B6 in the STA Counter Trigger Condition field shall be set to 0. And, When the group identity of the triggered STA Statistics request is set to 1, B0 and B1 in the STA Counter Trigger Condition field shall be set to 0.

The Measurement Count field contains a number of MSDUs used in determining trigger thresholds as described above.

The dot11Failed Threshold field contains a value representing the number of "dot11Failed" occurrences within "Measurement Count" MPDUs that results in a triggered report sent by the measuring STA. MSDUs to be used as the threshold value for the dot11Failed condition. This field is only present when the dot11Failed bit-field bit in the STA Counter Trigger Condition field is set to 1.

The dot11Retry Threshold field contains a value representing the number of "dot11Retry" occurrences within "Measurement Count" MPDUs that results in a triggered report sent by the measuring STA. MSDUs to be used as the threshold value for the dot11Retry condition. This field is only present when the dot11Retry bit-field bit in the STA Counter Trigger Condition field is set to 1.

The dot11MultipleRetry Threshold field contains a value representing the number of "dot11MultipleRetry" occurrences within "Measurement Count" MPDUs that results in a triggered report sent by the measuring STA. MSDUs to be used as the threshold value for the dot11MultipleRetry condition. This field is only present when the dot11MultipleRetry bit-field bit in the STA Counter Trigger Condition field is set to 1.

The dot11FrameDuplicate Threshold field contains a value representing the number of "dot11FrameDuplicate" occurrences within "Measurement Count" MPDUs that results in a triggered report sent by the measuring STA. MSDUs to be used as the threshold value for the dot11FrameDuplicate condition. This field is only present when the dot11FrameDuplicate bit-field bit in the STA Counter Trigger Condition field is set to 1.

The dot11RTSFailure Threshold field contains a value representing the number of "dot11RTSFailure" occurrences within "Measurement Count" MPDUs that results in a triggered report sent by the measuring STA. MSDUs to be used as the threshold value for the dot11RTSFailure condition. This field is only present when the dot11RTSFailure bit-field bit in the STA Counter Trigger Condition field is set to 1.

The dot11ACKFailure Threshold field contains a value representing the number of "dot11ACKFailure" occurrences within "Measurement Count" MPDUs that results in a triggered report sent by the measuring STA. MSDUs to be used as the threshold value for the dot11ACKFailure condition. This field is only present when the dot11ACKFailure bit-field bit in the STA Counter Trigger Condition field is set to 1.

The dot11FCSError Threshold field contains a value representing the number of "dot11FCSError" occurrences within "Measurement Count" MPDUs that results in a triggered report sent by the measuring STA. MSDUs to be used as the threshold value for the dot11FCSError condition. This field is only present when the dot11FCSError bit-field bit in the STA Counter Trigger Condition field is set to 1.

The Measurement Count field contains the number of continuous MPDUs that shall be used to count the number of the occurrence of a specific event (e.g., dot11Failed, dot11Retry, etc.). When the number of occurrence meets the corresponding threshold, a triggered report shall be generated subject to meeting the requirement of triggered Timeout. If Measurement Count MPDUs are received without any of the occurrences meeting the corresponding threshold then the measuring STA shall reset all the accumulated counts of the occurrences and shall restart measuring for another Measurement Count window.

The Trigger Timeout field contains a value in units of 100 TU 1 second during which a measuring STA does not generate further triggered STA Statistics Reports after a trigger condition has been met. The minimal value of Trigger Timeout shall be 10 second.

The format of the Triggered Reporting field for QoS STA Counters is shown in Figure 79e3. The fields marked as optional are the fields that are only present if the appropriate bit in the QoS STA Counter Trigger Condition is set to 1.

|  | QoS STA Counter Trigger Condition | dot11QoSFailed Threshold (optional) | dot11QoSRetry Threshold (optional) | dot11QoSMultiple Retry Threshold (optional) | dot11QoSFrame Duplicate Threshold (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 2 | 4 | 4 | 4 | 4 |
|  | dot11QoSRTS Failure Threshold (optional) | dot11QoSACK Failure Threshold (optional) | dot11QoSDiscarded Threshold (optional) | Measurement Count | Trigger Timeout |
|  | 4 | 4 | 4 | 4 | 2 |

- Figure 79e3— Triggered Reporting Field for QoS STA Counters

The Qos STA Counter Trigger Condition field is a bit-field that specifies reporting triggers when requesting triggered STA Statistics reporting. The format of the QoS STA Counter Trigger Condition field is shown in Figure 79e4.

|  | B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7- B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dot11QoS Failed | dot11QoS Retry | dot11QoSMultiple Retry | dot11QoSFrame Duplicate | dot11QoSRTS Failure | dot11QoSACK Failure | dot11QoS Discarded | Reserved |
| Octets Bits | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 9 |

- Figure 79e4— QoS STA Counter Trigger Condition field

- Each bit in the QoS STA Counter Trigger Condition field is set to 1 to request that a STA Statistics Report frame be generated when the corresponding QoS STA counter defined in Figure 85j (in Error: Reference source not found) increases by a value greater than or equal to the value of the corresponding QoS STA counter threshold in Figure v3 79e3, within the total number of MSDUs MPDUs given in Measurement Count. One or more trigger conditions may be set with specified thresholds.

The Measurement Count field contains a number of MSDUs used in determining trigger thresholds as described above.

The dot11QoSFailed Threshold field contains a value representing the number of "dot11QoSFailed" occurrences within "Measurement Count" MPDUs that results in a triggered report sent by the measuring STA. MSDUs to be used as the threshold value for the dot11QoSFailed condition. This field is only present when the dot11QoSFailed bit-field bit in the QoS STA Counter Trigger Condition field is set to 1.

The dot11QoSRetry Threshold field contains a value representing the number of "dot11QoSRetry" occurrences within "Measurement Count" MPDUs that results in a triggered report sent by the measuring STA. MSDUs to be used as the threshold value for the dot11QoSRetry condition. This field is only present when the dot11QoSRetry bit-field bit in the QoS STA Counter Trigger Condition field is set to 1.

The dot11QoSMultipleRetry Threshold field contains a value representing the number of "dot11QoSMultipleRetry" occurrences within "Measurement Count" MPDUs that results in a triggered report sent by the measuring STA. MSDUs to be used as the threshold value for the dot11QoSMultipleRetry condition. This field is only present when the dot11QoSMultipleRetry bit-field bit in the QoS STA Counter Trigger Condition field is set to 1.

The dot11QoSFrameDuplicate Threshold field contains a value representing the number of "dot11QoSFrameDuplicate" occurrences within "Measurement Count" MPDUs that results in a triggered report sent by the measuring STA. MSDUs to be used as the threshold value for the dot11QoSFrameDuplicate condition. This field is only present when the dot11QoSFrameDuplicate bit-field bit in the QoS STA Counter Trigger Condition field is set to 1.

The dot11QoSRTSFailure Threshold field contains a value representing the number of "dot11QoSRTSFailure" occurrences within "Measurement Count" MPDUs that results in a triggered report sent by the measuring STA. MSDUs to be used as the threshold value for the dot11QoSRTSFailure condition. This field is only present when the dot11QoSRTSFailure bit-field bit in the QoS STA Counter Trigger Condition field is set to 1.

The dot11QoSACKFailure Threshold field contains a value representing the number of "dot11QoSACKFailure" occurrences within "Measurement Count" MPDUs that results in a triggered report sent by the measuring STA. MSDUs to be used as the threshold value for the dot11QoSACKFailure condition. This field is only present when the dot11QoSACKFailure bit-field bit in the QoS STA Counter Trigger Condition field is set to 1.

The dot11QoSDiscarded Threshold field contains a value representing the number of "dot11QoSDiscarded" occurrences within "Measurement Count" MPDUs that results in a triggered report sent by the measuring STA. MSDUs to be used as the threshold value for the dot11QoSDiscarded condition. This field is only present when the dot11QoSDiscarded bit-field bit in the QoS STA Counter Trigger Condition field is set to 1.

The Measurement Count field contains the number of continuous MPDUs that shall be used to count the number of the occurrence of a specific event (e.g., dot11QoSFailed, dot11QoSRetry, etc.). When the number of occurrence meets the corresponding threshold, a triggered report shall be generated subject to meeting the requirement of triggered Timeout. If Measurement Count MPDUs are received without any of the occurrences meeting the corresponding threshold then the measuring STA shall reset all the accumulated counts of the occurrences and shall restart measuring for another Measurement Count window.

The Trigger Timeout field contains a value in units of 100 TU 1 second during which a measuring STA does not generate further triggered STA Statistics Reports after a trigger condition has been met. The minimal value of Trigger Timeout shall be 10 second.

The format of the Triggered Reporting field for RSNA Counters is shown in Figure v5 79e5. The fields marked as optional are the fields that are only present if the appropriate bit in the RSNA Trigger Condition is set to 1.

|  | RSNA Counter Trigger Condition | dot11RSNAStats CMACICVErrors Threshold (optional) | dot11RSNAStats CMACReplays Threshold (optional) | dot11RSNAStats BIPReplays Threshold (optional) | dot11RSNA StatsRobustMgmt CCMPReplays Threshold (optional) | dot11RSNAStats TKIPICVErrors Threshold (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 2 | 4 | 4 | 4 | 4 | 4 |
|  | dot11RSNAStats TKIPReplays Threshold (optional) | dot11RSNAStats CCMPDecryptErrors Threshold (optional) | dot11RSNAStats CCMPReplays Threshold (optional) | Measurement Count | Trigger Timeout |  |
| Octets: | 4 | 4 | 2 4 | 4 | 2 |  |

- Figure 79e5—Triggered Reporting Field for Triggered RSNA Counters

The RSNA Counter Trigger Condition field is a bit-field that specifies reporting triggers when requesting triggered STA Statistics reporting. The format of the RSNA Trigger Condition field is shown in Figure v6 79e6.

|  | B0 | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- | --- |
|  | dot11RSNAStats CMACICVErrors | dot11RSNAStats CMACReplays | dot11RSNAStats BIPReplays | Dot11RSNAStats RobustMgmt CCMPReplays | dot11RSNAStats TKIPICVErrors |
| Bits: | 1 | 1 | 1 | 1 | 1 |
|  | B5 | B6 | B7 | B8 – B15 |  |
|  | dot11RSNAStats TKIPReplays | dot11RSNAStats CCMPDecryptErrors | dot11RSNAStats CCMPReplays | Reserved |  |
| Bits: | 1 | 1 | 1 | 8 |  |

- Figure 79e6—RSNA Trigger Condition field

- Each bit in the Trigger Condition field is set to 1 to request that a STA Statistics Report frame be generated when the corresponding RSNA Error or Failure counter defined in Figure 86k1 (see Error: Reference source not found) increases by a value grater than or equal to the value of the corresponding RSNA Failure Threshold in Figure v5 79e5 within the total number of MPDUs given in Measurement Count. One or more trigger conditions may be set with specified thresholds.

The dot11RSNAStatsCMACICVErrors Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsCMACICVErrors condition. This field is only present when the dot11RSNAStatsCMACICVErrors bit-field bit in the RSNA Trigger Condition field is set to 1.

The dot11RSNAStatsCMACReplays Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsCMACReplays condition. This field is only present when the dot11RSNAStatsCMACReplays bit-field bit in the RSNA Trigger Condition field is set to 1.

The dot11RSNAStatsBIPReplays Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsBIPReplays condition. This field is only present when the dot11RSNAStatsBIPReplays bit-field bit in the RSNA Trigger Condition field is set to 1.

The dot11RSNAStatsRobustMgmtCCMPReplays Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsRobustMgmtCCMPReplays condition. This field is only present when the dot11RSNAStatsRobustMgmtCCMPReplays bit-field bit in the RSNA Trigger Condition field is set to 1.

The dot11RSNAStatsTKIPICVErrors Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsTKIPICVErrors condition. This field is only present when the dot11RSNAStatsTKIPICVErrors bit-field bit in the RSNA Trigger Condition field is set to 1.

The dot11RSNAStatsTKIPReplays Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsTKIPReplays condition. This field is only present when the dot11RSNAStatsTKIPReplays bit-field bit in the RSNA Trigger Condition field is set to 1.

The dot11RSNAStatsCCMPDecryptErrors Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsCCMPDecryptErrors condition. This field is only present when the dot11RSNAStatsCCMPDecryptErrors bit-field bit in the RSNA Trigger Condition field is set to 1.

The dot11RSNAStatsCCMPReplays Threshold field contains a value representing the number of discarded MPDUs to be used as the threshold value for the dot11RSNAStatsCCMPReplays condition. This field is only present when the dot11RSNAStatsCCMPReplays bit-field bit in the RSNA Trigger Condition field is set to 1.

The Measurement Count field contains a number of MSDUs to be used in determining trigger thresholds as described above.

The Measurement Count field contains the number of continuous MPDUs that shall be used to count the number of the occurrence of a specific event (e.g., dot11RSNAStatsCMACICVErrors, dot11RSNAStatsCMACReplays, etc.). When the number of occurrence meets the corresponding threshold, a triggered report shall be generated subject to meeting the requirement of triggered Timeout. If Measurement Count MPDUs are received without any of the occurrences meeting the corresponding threshold then the measuring STA shall reset all the accumulated counts of the occurrences and shall restart measuring for another Measurement Count window..

The Trigger Timeout field contains a value in units of 100 TU 1 second during which a measuring STA does not generate further triggered STA Statistics Reports after a trigger condition has been met. The minimal value of Trigger Timeout shall be 10 second.

- Change 7.3.2.22.8 as shown

- 7.3.2.22.8 STA Statistics Report

|  | Measurement Duration | Group Identity | Statistics Group Data | Reporting Reason |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 1 | variable | variable |

Figure 85g—Measurement Report field format for a STA Statistics Report

Table 31b—Group Identity for a STA Statistics Report

| Group Identity Requested | Statistics Returned |
| --- | --- |
| 11 16 | dot11RSNAStatsCMACICVErrors (Counter32) dot11RSNAStatsCMACReplays (Counter32) dot11RSNAStatsTKIPHdrErrors (Counter32) dot11RSNAStatsBIPReplays (Counter32) dot11RSNAStatsRobustMgmtCCMPReplays (counter32) dot11RSNAStatsTKIPICVErrors (counter32) dot11RSNAStatsTKIPReplays (counter32) dot11RSNAStatsCCMPDecryptErrors (counter32) dot11RSNAStatsCCMPReplays (counter32) |
| 12 17– 255 | None |

The Measurement Duration is set to the duration over which the change in Statistics Group Data was measured and reported, expressed in units of TUs. A Measurement Duration value of 0 indicates a report of the current values of the Statistics Group Data except for a triggered report.

The format of the Measurement Report field for RSNA Counters Group is shown in Figure v7.

|  | dot11RSNAStats CMACICVErrors | dot11RSNAStats CMACICVErrors CMACReplays | dot11RSNAStats BIPReplays | dot11RSNAStats RobustMgmt CCMPReplays | dot11RSNAStats TKIPICVErrors |
| --- | --- | --- | --- | --- | --- |
| Octets: | 4 | 4 | 4 | 4 | 4 |
|  | dot11RSNAStats TKIPReplays | dot11RSNAStats CCMPDecryptErrors | dot11RSNAStats CCMPReplays |  |  |
| Octets: | 4 | 4 | 4 |  |  |

- Figure 85k1—Measurement Report field format for RSNA Counters Group

The Reporting Reason field is a bit-field indicating the reason that the measuring STA sent the STA Statistics report. It is only present if Statistics Group Name is from STA Counters, QoS STA Counters, or RSNA Counters

The Reporting Reason field for STA Counters is shown in Figure v8.

|  | B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 – B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dot11Failed | dot11FCS Error | dot11Multiple Retry | dot11Frame Duplicate | dot11RTS Failure | dot11ACK Failure | dot11Retry | Reserved |
| Bits | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 9 |

- Figure 85k2—Reporting Reason field for STA Counters

The Reporting Reason field for QoS STA Counters is shown in Figure v9.

|  | B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 – B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | dot11QoS Failed | Dot11QoS Retry | dot11QoSMultiple Retry | dot11QoSFrame Duplicate | dot11QoSRTS Failure | dot11QoSACK Failure | dot11QoS Discarded | Reserved |
| Bits | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 9 |

- Figure 85k3—Reporting Reason field for QoS STA Counters

The Reporting Reason field for RSNA Counters is shown in Figure v10.

|  | B0 | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- | --- |
|  | dot11RSNAStats CMACICVErrors | dot11RSNA StatsCMACReplays | dot11RSNAStats BIPReplays | dot11RSNAStats RobustMgmt CCMPReplays | dot11RSNAStats TKIPICVErrors |
| Bits: | 1 | 1 | 1 | 1 | 1 |
|  | B5 | B6 | B7 | B8 – B15 |  |
|  | dot11RSNAStats TKIPReplays | dot11RSNAStats CCMPDecryptErrors | dot11RSNAStats CCMPReplays | Reserved |  |
| Bits: | 1 | 1 | 1 | 8 |  |

- Figure 85k4—Reporting Reason field for RSNA Counters

In a requested STA Statistics Report, all bit fields in the Reporting Reason field are set to 0.

- Change 11.10.8.5 as shown

- 11.10.8.5 STA Statistics Report

A STA may request that a STA Statistics report be sent when statistics of interest reach a defined threshold defined in Measurement Request element for STA Statistics Request. This is termed a triggered STA Statistics measurement and shall be requested is requested by setting the Enable and Report bits to 1 within a Measurement Request Element containing the STA Statistics Measurement Type. The Measurement Request field shall contain a STA Statistics Request with the trigger conditions specified in the Triggered Reporting field. One or more trigger conditions may be set with specified thresholds. See 11.10.7.

A STA accepting a triggered STA Statistics measurement shall measure the requested statistics. If a trigger condition occurs, the measuring STA shall send a STA Statistics measurement report to the requesting STA. The measuring STA shall not send further triggered STA Statistics reports until the Trigger Timeout period specified in the request has expired, or new trigger conditions have been requested.

If a STA receives a requested STA Statistics measurement STA Statistics measurement request when triggered STA Statistics measurement is in progress, the triggered measurement shall be suspended for the duration of the requested measurement. When triggered measurement resumes, the counted statistics shall be reset.

STA Statistics reported in a triggered STA Statistics report shall be the values accumulated over the number of transmitted MSDUs MPDUs prior to the trigger event given in the Measurement Count field of the measurement request that established the trigger condition. Measurement duration shall not be used in triggered STA statistics measurement and the Measurement Duration field in both the Measurement Request and any Measurement Report shall be set to 0.

Once accepted by a measuring STA, a triggered STA Statistics measurement continues to be active until:

- The measuring STA disassociates or successfully reassociates

- The measuring STA receives a disassociates or successfully reassociates a STA Statistics measurement request with the Enable bit set to 1 and the Report bit set to 0.

- All triggered STA Statistics measurements shall be terminated at a measuring STA by receiving a STA Statistics measurement request with the Enable bit set to 1 and the Report bit set to 0. A STA requesting a triggered STA Statistics measurement may update the trigger conditions by sending a STA Statistics measurement request specifying the new trigger conditions.