IEEE P802.11
Wireless LANs

| LB97 Coex Protection Mechanisms – Part 1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-08-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bjorn A. Bjerke | Qualcomm, Inc. | 9 Damonmill Square, Suite 2A Concord, MA 01742, USA | +1 781-276-0912 | bbjerke@qualcomm.com |
| Ali Raissinia | Qualcomm, Inc. |  |  | alirezar@qualcomm.com |

Abstract

This submission suggests resolutions of LB97 Coex comments related to the sub-topic Protection Mechanisms. The following CIDs are addressed: 3321, 24, 26, 105, 106, 108, 109, 322, 563, 1734, 2221, 2222, 2809, 2815.

The changes marked in this document are based on TGn Draft 2.05

CIDs related to 9.13.1

| 3321 | 126.19 | 19 | 9.13.1 | Indefinite reference. | Change "a STA" to "a receiving STA" | Reject. It is sufficiently clear from the context that “a STA” refers to a STA that might interfere with ERP and/or HT transmissions. |
| --- | --- | --- | --- | --- | --- | --- |

CIDs related to 9.13.3.1

| 24 | 126.45 | 45 | 9.13.3.1 | Do the cases for operating modes 0 and 2 only applied to infrastructure BSSs? | Change "… since all associated HT stations …" to "… since all HT stations …". | Accept. This applies to infrastructure BSSs and IBSSs alike. |
| --- | --- | --- | --- | --- | --- | --- |
| 26 | 127.56 | 56 | 9.13.3.1 | "… that requires a response that is sent as a non-HT frame or non-HT duplicate frame." Reading 9.6.7, there is no strict rule that a STA can expect the response frame to be sent in a non-HT or a non-HT duplicate frame. Needless to say, there is no specification that a STA can request in its transmission frame the response frame to be sent in a non-HT or a non-HT duplicate frame. | Add a description how a PPDU can require a response to be sent in a non-HT frame or in a non-HT duplicate frame. | Counter. Modify the text in row four of Table 9-7 as per 07/2316, also taking into account the proposed resolution of CID 2222). |
| 105 | 126.37 | 37 | 9.13.3.1 | "HT transmissions" need to be defined. Those should be what are defined in clause 20 (including RIFS sequences). | Add a description that the HT transmissions are those defined in clause 20. | Counter. Use the term “HT frame” defined in 3.n23. Modify the subclause heading for 9.13.3 to read “Protection mechanisms for HT frame transmissions”. Modify the first sentence of 9.13.3.1 to read “Transmissions of HT frames, referred to as HT transmissions, shall be protected…” |
| 106 | 126.37 | 37 | 9.13.3.1 | "… that cannot interpret HT transmissions correctly." This sentence should be more specific. | Change it to "… that cannot decode the Non-HT SIGNAL field of the OFDM preamble of HT transmissions." The same expression also exists in l.5, p.129. | Counter. PHY specific terms should be avoided in the MAC section. Change the sentence in question to “...shall be protected if there are other STAs present that do not support HT mixed format or HT greenfield format.” (delete “(HT or non-HT)”.) |
| 108 | 126.52 | 52 | 9.13.3.1 | What is "enough of the preamble"? | Change it to "Non-HT SIGNAL field of the OFDM preamble". | Counter. Delete Note 1 as per the proposed resolution of CID 1734. |
| 109 | 126.56 | 56 | 9.13.3.1 | BSA can be an area also covered by other BSSs but operating modes 0 and 2 don't care about other BSSs. | Change BSA to BSS. | Reject. The note is correct. BSA refers to other OBSSs that the STA can hear. |
| 322 | 126.45 | 45 | 9.13.3.1 | Text refers to BSS yet on line 55, BSA is referred to and in the introductory sentence it just says "shall be protected" without being limited to just BSS protection. Therefore the intent seems to be BSA protection. | Harmonize the paragraph by referring to BSA throughout. | Counter. In this particular case, the text refers to BSS/IBSS (see proposed resolution of CID 24). In other cases the text deliberately refers to BSA. |
| 563 | 126.37 | 37 | 9.13.3.1 | The sentence says that there are HT STAs which cannot interpret HT transmissions, which sounds odd. | Replace the first sentence with: "HT transmissions shall be protected if non-HT STAs are present. HT transmissions using certain optional features shall be protected if HT STAs that cannot interpret those transmissions are present." | Counter. See resolution of CID 106. |
| 1734 | 126.51 | 51 | 9.13.3.1 | I'm not sure what is point of the note. The preamble fields are the same for 20 MHz and 40 MHz, so why call special attention to 40 MHz in the note? | remove Note 1 | Accept. Delete NOTE 1. |
| 2221 | 127.51 | 51 | 9.13.3.1 | "Using a non-HT preamble, transmit first a PPDU that requires a response frame." - there is no definition of non-HT preamble that the MAC knows about. | Reword: "Transmit an initial frame within a non-HT PPDU that requires a response frame." | Accept. Change the sentence (which now appears in Table 9-7, line 51) as proposed. |
| 2222 | 127.55 | 55 | 9.13.3.1 | "Using a mixed format preamble, transmit first a PPDU that requires a response that is sent as a non- HT frame or non-HT duplicate frame." The MAC doesn't know about mixed format preamble. | Reword thus: "Using a PPDU with the TXVECTOR FORMAT parameter set to HT_MM, transmit an initial frame that requires a response that is sent as a non-HT frame or non-HT duplicate frame. | Accept. Change the sentence (which now appears in Table 9-7, line 20) as proposed. |
| 2809 | 127.51 | 51 | 9.13.3.1 | In case b, the first frames needs to be transmitted at a BSS basic rate so that all the STAs in the network set their NAV | Modify as suggested | Counter. Move the note that appears in the second row of Table 9-7 to just below the table, and modify it to make it applicable to all HT protection mechanisms listed in the table. |
| 2815 | 127.15 | 15 | 9.13.3.1 | All HT STAs are required to decode HT-SIG part of the GF preamble/header and defer. If the HT-SIG is missed they will defer based on ED. So the restriction of requiring protection for all GF transmissions is too restrictive | Remove this restriction | Withdrawn by the commenter. |

TGn Editor: make changes as shown below:

9.13.3 Protection mechanisms for HT frame transmissions

9.13.3.1 General

HT tTransmissions of HT frames, referred to as HT transmissions, shall be protected if there are other STAs (HT or non-HT) present that do not support HT mixed format or HT Greenfield formatcannot interpret HT transmissions

correctly. The fields HT Protection (#548) and Non-greenfield HT (#20) STAs Present in the HT Information element within the Beacon and Probe Response frames are used to determine if protection is required for transmission.

When the HT Protection (#548) field is set to 0 or 2 and Non-greenfield HT (#20) STAs Present field is set

to 0, no protection is required since all associated HT stations in the BSS are capable of decoding HT mixed

format and HT greenfield format transmissions.

NOTE 1—When the HT Protection (#548) field is set to 0 or 2 and Non-greenfield HT (#20) STAs Present field is set to

0, no protection is required for 40 MHz transmissions since enough of the preamble is sent on both 20 MHz halves of the

40 MHz transmission to ensure that a 20 MHz STA is able to decode the preamble and infer the duration of the transmission.

(#107)

When the HT Protection (#548) field is set to 0 or 2 and the Non-greenfield HT (#20) STAs Present field is

set to 1, HT transmissions that use the HT greenfield format (#23) shall be protected.

NOTE 2—When the BSS is operating in the 2.4 GHz band and when the HT Protection (#548) field is set to 0 or 2, there

are no non-HT STAs in the BSA and thus the Use_Protection (#33) field of the ERP Information element is set to 0.

When the HT Protection (#548) field is set to 1 and the Use_Protection (#33) field in the ERP Information

element is set to 0, HT transmissions may be protected according (#1285) to implementation dependent criteria.

When the HT Protection (#548) field is set to 1, the Use_Protection (#33) field in the ERP Information

element is set to 0 and the Non-greenfield HT (#20) STAs Present field is set to 1, HT transmissions using

HT greenfield format shall be protected. When the HT Protection (#548) field is set to 1 and the

Use_Protection (#33) field in the ERP Information element is set to 1, HT transmissions shall be protected

using the mechanisms described in Table 9-6 (Protection requirements for HT Protection (#548) field values

1 and 3)

When the HT Protection (#548) field is set to 3, HT transmissions shall be protected. The type of protection

that is required depends on the type of transmission as well as the type of the non-HT STAs that are present in the BSS. The specific mechanisms that are to be used for protection for the HT Protection (#548) field isset to 3 are described in Table 9-6 (Protection requirements for HT Protection (#548) field values 1 and 3).

| Table 9-6-Protection requirements for HT Protection (#548) field values 1 and 3 |  |  |
| --- | --- | --- |
| Type of Transmission | Use_Protection (#33) = 0 or ERP IE is not present (HT Protection (#548) field set to 3) | Use_Protection (#33) = 1 (HT Protection (#548) field set to 1 or 3) |
| 20 MHz transmission: | HT transmissions using HT greenfield format and/or using RIFS within the HT transmission burst shall be protected using the sequences defined in Table 9-7 (Table 9-7-Applicable HT protection mechanisms). (#2220) | All HT transmissions shall be protected using mechanisms such as RTS/CTS or CTS-to-Self prior to the HT transmissions. The frames that are used for providing the protection shall be sent at a Clause 15 or Clause 18 rate. |
| 40 MHz transmission: | HT transmissions using HT greenfield format and/or using RIFS within the HT transmission burst shall be protected using the sequences defined in Table 9-7 (Table 9-7-Applicable HT protection mechanisms). (#2220) | All HT transmissions shall be protected using mechanisms such as RTS/CTS or CTS-to-Self prior to the HT transmissions. The frames that are used for providing the protection shall be sent at a Clause 15 or Clause 18 rate. |

If the transmission requires protection and Use_Protection (#33) field within the ERP Information element is

set to 0 or the ERP Information element is not present in the Beacon, HT transmissions shall be protected

using one of the mechanisms identified in Table 9-7 (Applicable HT protection mechanisms). (#2220)

Table 9-7-Applicable HT protection mechanisms

HT protection mechanism

Control frames such as RTS/CTS or CTS-to-self prior to the HT transmissions: - For 20 MHz transmissions, use the rates defined in Clause 17 or Clause 19 - For 40 MHz transmissions, use the non-HT duplicate frames defined in Clause 20. NOTE—The rate of a protection frame is (#565) selected based on the rules described in 9.6.0c.1 (Rate selection for control frames that initialize a TXOP) and 9.6.0c.2 (Rate selection for control frames that are not control response frames).

Transmit an initial frame within a non-HT Using a non-HT preamble, transmit first a PPDU that requires a response frame. The remaining TXOP following the first PPDU exchange may contain PPDUs using HT greenfield format and/or separated by RIFS (#23).

L-SIG TXOP protection

Using a PPDU with the TXVECTOR FORMAT parameter set to HT_MM HT mixed format (#35), transmit first a PPDU that requires a response frame. that is sent as a non-HT frame or non-HT duplicate frame. The remaining TXOP following the first PPDU exchange may contain HT greenfield format and/or RIFS sequences only when the response frame is a non-HT or non-HT duplicate frame.

NOTE—Rules for rate selection for the HT protection mechanisms listed in Table 9-7 (Applicable HT protection mechanisms) are described in 9.6 (Multirate support).