IEEE P802.11
Wireless LANs

| LB97 Coex Protection Mechanisms – Part 3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-09-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bjorn A. Bjerke | Qualcomm, Inc. | 9 Damonmill Square, Suite 2A Concord, MA 01742, USA | +1 781-276-0912 | bbjerke@qualcomm.com |
| Ali Raissinia | Qualcomm, Inc. |  |  | alirezar@qualcomm.com |
| Srini Kandala | Qualcomm, Inc. |  |  | kandala@qualcomm.com |

Abstract

This submission suggests resolutions of LB97 Coex comments related to the sub-topic Protection Mechanisms. The following CIDs are addressed: 1651,  1650,  1939,  2971,  21,  553,  848,  554,  88,  849,  31,  3107,  3096,  1851, 1946,  30,  851,  837,  32,  850,  1661,  1662,  2855,  3281,  1947,  2256,  46.

The changes marked in this document are based on TGn Draft 2.06

CIDs related to 9.13.3.4

| 30 | 128.36 | 36 | 9.13.3.4 | It says "… the duration of a transmit burst *should* be limited." but "the STA *shall* ensure that it limits the duration of a transmit burst to the value" in l.43. | Change "… the duration of a transmit burst should be limited." to "… the duration of a transmit burst shall be limited." | Counter. Delete all text associated with Transmit burst limit. |
| --- | --- | --- | --- | --- | --- | --- |
| 31 | 128.00 |  | 9.13.3.4 | Is the Transmit Burst Limit field set in an IBSS? | Add a description that HT STAs in an IBSS shall set this field to 1 in their Beacon and Probe Response frames. | Counter. Delete all text associated with Transmit burst limit.. |
| 1851 | 128.00 |  | 9.13.3.4 | STAs that don't support APSD but beacon PS could also benefit from Transmit Burst limit requirements. | Add and "and/or beacon power save scheme" after "APSD". | Counter. Delete all text associated with Transmit burst limit. |
| 1946 | 128.32 | 32 | 9.13.3.4 | A transmit burst is no worse, in terms of fairness, than an A-MPDU occupying the same time on air. In addition, bursting is already limited by TXOP limits for the access categories in which bursting is permitted. | Change "Transmit burst limit" to "Transmission duration limit", and broaden the definition to include any HT transmissions, as the intent is to protect legacy STAs. | Counter. Delete all text associated with Transmit burst limit. |
| 3096 | 128.00 |  | 9.13.3.4 | When the operating mode is 1 or 3 (clause 9.13.3.5), you don't know if there are STAs using the secondary channel or in the overlapping BSS scenario are non-HT APSD STAs. Mandate the use the Transmit burst limit when the operating mode = 1 or 3 | Add the following text to the end of line 50: "and/or if the operating mode is 1 or 3" | Counter. Delete all text associated with Transmit burst limit. |
| 3107 | 128.00 |  | 9.13.3.4 | Transmission burst limit is redundant: existing mecanism ( EDCA Parameter IE) covers the use case described in the clause 9.13.3.4 | Use TxOP Limit per AC in the EDCA Parameter IE to indicate the max transmit burst length. This definition is sufficient to cover the RIFS transmissions as well. | Counter. Delete all text associated with Transmit burst limit. |

CIDs related to 9.13.3.5

| 32 | 129.03 | 3 | 9.13.3.5 | "The OBSS Non-HT STAs Present field allows HT devices to report the presence of other non-HT STAs that cannot interpret HT transmissions correctly." Isn't the reporting HT device an HT AP? Or is this intended to include the IBSS case, too? | Clarify how this field is set and used in an IBSS. If this is only used in an infrastructure BSS, then change the cited sentence to "The OBSS Non-HT STAs Present field allows HT APs to report to other BSSs the presence of other non-HT STAs that cannot decode the Non-HT SIGNAL field of the OFDM preamble of HT transmissions." | Counter. Accept in principle. The field is only used in an infrastructure BSS. |
| --- | --- | --- | --- | --- | --- | --- |
| 837 | 129.01 | 1 | 9.13.3.5 | OBSS is not defined in clause 4 | Define OBSS in clause 4 | Counter. The term and acronym is defined in clause 3. |
| 850 | 129.07 | 7 | 9.13.3.5 | On line 7, the statement "When an HT AP detects a first HT AP's beacon with the OBSS Non-HT STAs Present field set to 1 may, etc..." contradicts with the requirement stated in clause 9.13.3.1, which said that "when the operating mode field is set to 3, HT transmission shall be protected". | Change "When an HT AP detects a first HT AP's beacon with the OBSS Non-HT STAs Present field set to 1 may" to "When an HT AP detects a first HT AP's beacon with the OBSS Non-HT STAs Present field set to 1 shall". | Reject. There is not necessarily a contradiction here. HT Protection could be set to 1, which does not mandate protection. |
| 851 | 129.00 |  | 9.13.3.5 | Since Non-HT devices are not able to decode HT greenfield format or RIFS, HT devices must use protection when transmitting greenfield or RIFS when Non-HT devices are present. | Change lines 29-30 to "When non-HT devices are detected, the STA shall enable protection of its HT greenfield format and RIFS sequence transmissions" | Reject. It is the AP’s prerogative to set HT Protection to either 1 or 3. Only the latter mandates protection. |
| 1661 | 129.07 | 7 | 9.13.3.5 | Change "may" to "shall" | Change "may" to "shall" | Reject. See proposed resolution of CID 851. |
| 1662 | 129.11 | 11 | 9.13.3.5 | Change "may" to "shall" | Change "may" to "shall" | Reject. See proposed resolution of CID 851. |
| 1947 | 129.30 | 30 | 9.13.3.5 | When non-HT devices are detected in an OBSS, the use of the greenfield preamble may cause deleterious effects to the non-HT STAs in the OBSS, such as false radar detection. MAC-level protection of greenfield transmissions is not sufficient to prevent these problems, since the non-HT STAs will still receive undecodable greenfield transmissions following the protection frames. | When OBSS non-HT STAs are detected and an HT AP sets the corresponding bit in the HT information element, require that greenfield transmission is forbidden in the HT BSS. | Reject. Refer to the resolution of CID 1660 (in 07/2380r1) |
| 2855 | 129.19 | 19 | 9.13.3.5 | If one or more non-HT STA are associated the operating mode is 3 and not 1 | remove the case "— one or more non-HT STAs are associated , or" | Accept. |
| 3281 | 129.26 | 26 | 9.13.3.5 | It is probably not a good idea to encourage implementers to include the following as a possible condition for setting the OBSS non HT STA present bit: "reception of a Beacon containing an HT Information element with the OBSS Non-HT STAs Present field set to 1." Including this condition encourages an infinite propagation situation. I recognize that eliminating this condition does not eliminate the possibility of setting the bit for this very condition, but that is ok - the presence of the condition encourages the behavior - I am not asking for an outright restriction against using that condition. | Delete the condition "reception of a Beacon containing an HT Information element with the OBSS Non-HT STAs Present field set to 1." from the list of possible conditions that an AP might use to determine when to set the OBSS Non-HT STA present bit. | Reject. The text in question gives examples of how non-HT STAs may be detected. Once detected, a STA may enable protection of GF and RIFS transmissions. |

CIDs related to 7.3.2.50 (D2.06: 7.3.2.53)

| 21 | 73.50 | 50 | 7.3.2.50 | From the definition, the Non-greenfield STAs Present field is sent by only an AP and cannot be sent by STAs in an IBSS. | Clarify in 9.13.3.3 that for the IBSS case, when the HT STAs transmit in HT greenfield format and/or use RIFS within the HT transmission burst, it shall protect those transmissions (even though the Non-greenfield STAs Present field is not present and regardless of the operating mode in Beacon and Probe Response frames sent in the IBSS). | Counter. Add IBSS protection requirements to 9.13.3.2 and 9.13.3.3. |
| --- | --- | --- | --- | --- | --- | --- |
| 88 | 74.14 | 14 | 7.3.2.50 | Setting the OBSS Non-HT STAs Present bit to 1 is not strictly limited. Hence the case of setting it to 0 becomes vague and the word "otherwise" for it is not appropriate. | Change "Set to 0 otherwise." to "Set to 0 when the protection for non-HT STAs by overlapping BSSs is determined not to be desirable." | Counter. Accept in principle. |
| 553 | 73.50 | 50 | 7.3.2.50 | The "Non-greenfield STAs Present" field expresses something more specific than its name implies. The field is used to signal the protection requirements for Greenfield transmissions. | Rename the field to "Greenfield Protection". | Reject. This field indicates whether there are non-greenfield STAs present. Protection requirements for this case are communicated using the HT Protection field. The field name is therefore appropriate. |
| 554 | 74.14 | 14 | 7.3.2.50 | The "OBSS Non-HT STAs Present" field expresses something more specific than its name implies. The field is used to signal the protection requirements for HT transmissions when a non-HT OBSS may be present. | Rename the field to "OBSS HT Protection". | Reject. This field indicates whether there are non-HT STAs present in an OBSS. Protection requirements for this case are communicated using the HT Protection field. The field name is therefore appropriate. |
| 848 | 74.14 | 14 | 7.3.2.50 | Protection for non-HT STAs operating in overlapping BSSs is an important aspect in ensuring coexistence of 802.11n and 802.11 a/b/g devices. The use of protection should be made mandatory for all 802.11n devices when one or more OBSS non-HT STAs are present. | In the definition and Encoding columns of Table n30, lines 16 and 15 respectively, change " determined to be desirable" to "required" | Reject. This field indicates whether there are non-HT STAs present in an OBSS. Protection requirements for this case are communicated using the HT Protection field. HT STAs treat the presence of non-HT STAs in OBSS as information that may be used in deciding what, if any, protection to use. |
| 849 | 74.16 | 16 | 7.3.2.50 | This comment is similar to the previous one but the recommended change has a different implication. Protection for non-HT STAs operating in overlapping BSSs is an important aspect in ensuring coexistence of 802.11n and 802.11 a/b/g devices. The use of protection should be made mandatory for all 802.11n devices when one or more OBSS non-HT STAs are present. | In the Encoding column of Table n30, line 16, change "Examples of when this bit may be set to 1 include" to "Examples of when this bit shall be set to 1 include" | Reject. This field indicates whether there are non-HT STAs present in an OBSS. Protection requirements for this case are communicated using the HT Protection field. HT STAs treat the presence of non-HT STAs in OBSS as information that may be used in deciding what, if any, protection to use. |
| 1650 | 73.30 | 30 | 7.3.2.50 | At 2.4 GHz, I can think of 12 different coexistence conditions: non_40_MHz_client_in_BSS, non_40_MHZ_client_in_OBSS_on_same_channel, non_40 MHz_client_in_OBSS_on_nearby_channel, non_GF_client_in_BSS, non_GF_client_in_OBSS_on_same_channel, non_GF_client_in_nearby_channel, non_HT_client_in_BSS, non_HT_client_in_OBSS_on_same_channel, non_HT_client_in_nearby_channel,non_ERP_client_in_BSS, non_ERP_client_in_OBSS_on_same_channel, non_ERP_client_in_nearby_channel and 2 radically different PHY modes: 40 MHz and Greenfield. That leads to 2^14 or 16384 differenct scenarios. The four "Operating Modes" defined in Table N30 and section 9.13 do not sufficiently address this problem. Operating Modes 0 & 2 don't even have names, and there doesn't appear to be any difference between the two. |  | Counter. See also proposed resolution of CID 1658 (functional duplicate) |
| 1651 | 73.30 | 30 | 7.3.2.50 | At 5 GHz, there are many different coexistence conditions: non_40_MHz_client_in_BSS, non_40_MHZ_client_in_OBSS_on_same_channel, non_40 MHz_client_in_OBSS_on_nearby_channel, non_GF_client_in_BSS, non_GF_client_in_OBSS_on_same_channel, non_GF_client_in_nearby_channel, non_HT_client_in_BSS, and whether DFS applies to that channel. There are 2 radically different operating modes: 40 MHz and GF, leaving 2^12 possible scenarios. The four operating modes do not cover all these cases and do not address the most catastrophic case: GF transmission in a channel where DFS is required. | We will need a detailed submission to address this problem. | Counter. See also proposed resolution of CID 1659 (functional duplicate) |
| 1939 | 73.50 | 50 | 7.3.2.50 | Non-greenfield STAs present bit should not be specific to HT STAs. Legacy STAs cannot receive greenfield transmissions. If these STAs are associated, this bit should be set. | Remove the HT-specific language to allow this bit to indicate when any non-greenfield STAs are associated in the BSS. | Reject. When non-HT STAs are associated in the BSS, HT Protection is set to 3, which mandates the use of MAC protection for GF transmissions. Keep the HT specific aspect of this bit. |
| 2971 | 73.50 | 50 | 7.3.2.50 | Green Field protection: In Table n30, the "Non-greenfield STA's present bit" of the "HT Information Element" is set by the AP (or beaconing STA in an IBSS) to signal the usage of a protection mechanism before GF packets are transmitted. The text in 9.3.3.3 also sheds light on its usage. However, the case of a legacy overlapping BSS is not discussed. | Describe the cases where protection modes are required for packets transmitted with GF preamble. | Reject. The field is used to indicate the presence of non-greenfield STAs. Protection requirements are communicated using the HT Protection field as described in clause 9.13.3.3. |

CIDs related to A.14.7.1

| 46 | 342.53 | 53 | A.14.7.1 | The description here for HTM6.1 does not distinguish the infrastructure BSS case and the IBSS case. This is related to the clarification of RIFS protection in IBSS. From 9.13.3.2, it is quite clear that the protection under presence of non-HT STAs is mandatory for the infrastructure BSS. But how about for the IBSS case? From what can be read from 9.13.3.2 RIFS protection, the IBSS case will be the otherwise case and the RIFS mode field will be set to 1 but the STAs' behaviour in IBSS is not described. | Correct the description written for protocol capability of HTM6.1 to reflect the concern in the comment. Also add a new status if necessary. | Counter. Insert item HTM6.1a concerned with IBSS. For consistency, also insert HTM6.2a concerned with GF protection in IBSS. |
| --- | --- | --- | --- | --- | --- | --- |

CIDs related to 9.13.6

| 2256 | 133.30 | 30 | 9.13.6 | "9.13.6 Protection mechanisms for A-MPDU exchange sequences" The clause title implies that the protection mechanisms are specific to A-MPDU. This is not the case. Most of the content has now gone anyway. The content needs to be distributed to appropriate homes and this subclause title removed. 9.13.6.1: "The Duration/ID field in all frames within an A-MPDU shall contain the remaining duration of TXOP (referencedto the end of the PPDU carrying the frame). The Duration/ID field of all frames sent in the TXOP by the TXOP holder or TXOP responder shall contain the time remaining in the TXOP." These statements are more constraining that the rules (i.e. conflict with) for the Duration/ID field in 7.1.4. 9.13.6.2: This subclause is not specific to A-MPDU | I believe the rules we have in 7.1.4 and 7.1.3.2 are adequate without 9.13.6.1 Therefore, Delete subclause 9.13.6.1. Delete subclause header 9.13.6 Promote subclause 9.13.6.2 up a level | Accept. Already implemented in D2.06. |
| --- | --- | --- | --- | --- | --- | --- |

TGn Editor: in subclause 7.3.2.53, Figure 7-95aj (HT Information element format), delete the “Transmit Burst Limit” field; in Table 7-43i (HT Information element), delete the row corresponding to “Transmit Burst Limit”. Delete subclause 9.13.3.4 “Transmit burst limit” including the subclause header, contents (text and Table 9-8) and any instructions to the Editor. Also delete any other occurrences of “Transmit Burst Limit” throughout the document.

9.13.3.5 Use of OBSS Non-HT STAs Present

The OBSS Non-HT STAs Present field allows HT APs devices to report the presence of other non-HT STAs in either the primary or the secondary channel or in both the primary and secondary channels, that are not members of its BSSthat cannot interpret HT transmissions correctly. See 9.13.3 (Protection mechanisms for HT transmissions).

A second HT AP that detects a first HT AP’s Beacon (#1191) frame with the OBSS Non-HT STAs Present

field set to 1 may, in conjunction with radio resource measurements and/or heuristics, cause HT greenfield format and RIFS sequence transmissions of the second AP’s BSS to be protected by setting the HT Protection (#548) field of its HT Information element to 3. If the NonERP_Present bit is set to 1 in the first AP’s Beacon (#1191) frame, then the Use_Protection bit may also be set to 1 by the second AP. See also (#1175) R.2 (Summary of the use of HT Protection signaling (#1283)) of Annex RT.

An HT STA (#2225) may also scan for the presence of non-HT devices either autonomously or, for example, after the STA’s AP transmits an HT Information element with the HT Protection (#548) field set to 1. Non-HT devices may be detected as follows :

— one or more non-HT STAs are associated , or

— a non-HT BSS is overlapping (a non-HT BSS may be detected by the reception of a Beacon where

the supported rates only contain Clause 15, 17, 18 or 19 rates), or

— reception of a management frame (excluding a Probe Request) where the supported rate set includes

only Clause 15, 17, 18 and 19 rates, or

— reception of a Beacon containing an HT Information element with the OBSS Non-HT STAs Present

field set to 1.

When non-HT devices are detected, the STA may enable protection of its HT greenfield format and RIFS

sequence transmissions.

TGn Editor: in subclause 7.3.2.53, Table 7-43i (HT Information element), the row labelled “OBSS Non-HT STAs Present”, insert the sentence “Set to 0 otherwise” immediately after the sentence that ends with “desirable”. Then delete the last sentence in the cell (“Set to 0 otherwise”).

TGn Editor: in A.14.19.1, insert item HTM6.1a as follows:

| HTM6.1a | Protection of RIFS PPDUs in an IBSS | 9.13.3.2 RIFS protection | CF16:M | Yes_ No_ N/A_ |
| --- | --- | --- | --- | --- |

TGn Editor: in A.14.19.1, insert item HTM6.2a as follows:

| HTM6.2a | Protection of HT greenfield PPDUs in an IBSS | 9.13.3.3 Greenfield protection | CF16:M | Yes_ No_ N/A_ |
| --- | --- | --- | --- | --- |

References: