IEEE P802.11
Wireless LANs

| LB97 BF explict quant comment resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-07-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Assaf Kasher | Intel Corporation | Matam Industrial Park Haifa Israel 31015 | +972-4-8651547 | Asssaf.kasher@intel.com |

Abstract

This document address several BF explict quantizaiton related comments.  The base draft is D2.07

The addressed CIDs are:  343, 1606, 1608, 1797, 1955, 2731

This document suggests resolution for some BF explict quantization related comments

| 343 | The quantization scheme presented here does not cover the full range of two's complement encoding with N_b bits. Specifically, the smallest value with this scheme is -(2^{N_b-1}-1), but the smallest value in a N_b-bit two'complement is -2^{N_b-1}. As a result we would lose precision in the negative values. | Redine Eq. 20-77 and 20-78 to correct this problem. |
| --- | --- | --- |

Suggested resolution: Reject

Reason:  equations 20-77 and 20-78 are only examples of the encoding process and are not normative text.  Implementors may choose different normalization scheme.

| 1606 | Here is a description of "The CSI matrix, H_eff, shall be determined from the transmitter spatial mapper input to the receiver FFT output (the beamformee removes the CSD in table n67 from the measured channel matrix)." Yes, we all know that the CSD shall be removed by beamformee. but how about P_HTLTF specified in equation (20-27) ? I expect it shall be removed by the beamformee also, and it is worth to explicitly state this in the spec. | Add "and P_HTLTF defined at (20-27)", so this sentence would be; "The CSI matrix, H_eff, shall be determined from the transmitter spatial mapper input to the receiver FFT output (the beamformee removes the CSD in table n67 and P_HTLTF defined at (20-27) from the measured channel matrix)." |
| --- | --- | --- |

Suggested resolution: Reject

Reason: P_{HTLTF} is removed by the process of estimation of the CSI matrix H_{eff} at the receiver, so there is no need to define who removes it.

| 1608 | In these equation, maximum functions for -N_SR<k<N_SR are used. When the grouping is not used, i.e., Ng has non-zero value, should we take into account the amplitude of unused subcarriers for these equation ? | Please clarify whether all subcarriers shall be used for calculating these maximum functions or only selected subcarriers for grouping shall be used. |
| --- | --- | --- |

Suggested Resolution: Reject

Equations 20-77 and 20-78 are only exmples of the encoding process and are not normative text.  Implementors can make their choice of what to maximize without impairing interoperability.

| 1797 | Eq. 20-76 doesn't make any sense to me. Line 26 says that Eq. 20-76 is the linear counterpart of 20-75. However, the linear counterpart of 20-75 is just the denominator of 20-76 | please clarify |
| --- | --- | --- |

Suggested Resolution: Counter:

TGn Editor: change lines 51-52 of page 292 of draft D2.07 as follows:

b) The scaling ratio is calculated and quantized to 3 bits as defined by Equation (20-78). and its linear

counterpart A linear scaler is given by Equation (20-79).

| 1955 | This clause already gives the rule how to decode received CSI matrices. The encoding (quanization) scheme is up to the implementation of vendors. The example of encoding processing is unncessarily complicated. The long and confusing encoding example does not serve the purpose of clarification. In fact, the old example provided in draft 1.0 is far better than the new one in draft 2.0. Comment resolution is supposed to bring clarification, not confusion. | Remove the example of the encoding process and replace it by a brief explanation of the meaning of the carrier matrix amplitude field (Mh(k)). |
| --- | --- | --- |

Suggestion: Reject

Reason: The group thinks that the example is necessary to indicate how an efficient encoding can work.

| 2731 | "The beamformee shall encode the matrices so a beamformer applying the procedure below will optimally reconstruct the matrix." This is a topsy turvy way of specifying anything. I don't see any need for this obfuscation. | Please indicate how to encode the values, not how to decode them. |
| --- | --- | --- |

Suggestion: Reject

Reason: coding (compression) standars (JPEG, MPEG, etc) are specified by showing the decoding process.  This allows the encoder, which is doing the more complicated part, control the complexity by compromising performance vs. complexity.