IEEE P802.11
Wireless LANs

| TGn LB97 Submission for Legacy Duplicate |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-09-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yuichi Morioka | Sony Corporation | 5-1-12 Kita-Shinagawa, Shinagawa-ku, Tokyo, 141-0001 Japan | +81-3-5448-4017 | Yuichi.Morioka@jp.sony.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB97 comments:

777, 778, 779, 781, 782, 783, 784, 785, 786, 787, 788, 2186

The changes marked in this document are based on TGn Draft version P802 11n D2.07.pdf.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Proposed Resolution

| CID | Page | Line | Subclause | Comment | Proposed Change | Proposed Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 777 | 107 | 1-5 | 9.6.3.3 | HT PPDU cannot have a CH_BANDWIDTH parameter value of NON_HT_CBW40 | change "...a non-HT PPDU or in an HT PPDU that has a value of NON_HT_CBW40 for the TXVECTOR parameter CH_BANDWIDTH,…" to "...a non-HT PPDU,…" | Accept |
| 778 | 107 | 12-15 | 9.6.3.3 | HT PPDU cannot have a CH_BANDWIDTH parameter value of NON_HT_CBW40 | change "If the Control Response frame is carried in an HT PPDU that does not have a value of NON_HT_CBW40 for the TXVECTOR parameter CH_BANDWIDTH,…" to "If the Control Response frame is carried in an HT PPDU,…" | Accept |
| 779 | 107 | 33-34 | 9.6.3.3 | HT PPDU cannot have a CH_BANDWIDTH parameter value of NON_HT_CBW40 | change "For a non-HT PPDU control response frame or a control response frame that is transmitted in an HT PPDU with the TXVECTOR parameter CH_BANDWIDTH equal to NON_HT_CBW40,…" to "For a non-HT PPDU control response frame,…" | Accept |
| 781 | 107 | 43-44 | 9.6.3.3 | HT PPDU cannot have a CH_BANDWIDTH parameter value of NON_HT_CBW40 | change "For an HT PPDU control response frame that is transmitted in a PPDU with the TXVECTOR parameter CH_BANDWIDTH not equal to NON_HT_CBW40,…" to "For an HT PPDU control response frame,…" | Accept |
| 782 | 108 | 14-16 | 9.6.6 | HT PPDU cannot have a CH_BANDWIDTH parameter value of NON_HT_CBW40 | change "If the control response frame format is to be transmitted within an HT PPDU with the TXVECTOR parameter CH_BANDWIDTH not equal to NON_HT_CBW40 in accordance…" to "If the control response frame format is to be transmitted within an HT PPDU in accor | Accept |
| 783 | 108 | 51-53 | 9.6.6 | HT PPDU cannot have a CH_BANDWIDTH parameter value of NON_HT_CBW40 | change "If the frame eliciting the response is within a non-HT PPDU or an HT-PPDU with TXVECTOR parameter CH_BANDWIDTH equal to NON_HT_CBW40," to "If the frame eliciting the response is within a non-HT PPDU," | Accept |
| 784 | 108 | 61-62 | 9.6.6 | HT PPDU cannot have a CH_BANDWIDTH parameter value of NON_HT_CBW40 | change "If the frame eliciting the response is within an HT-PPDU with the TXVECTOR parameter CH_BANDWIDTH not equal to NON_HT_CBW40," to "If the frame eliciting the response is within an HT-PPDU," | Accept |
| 785 | 109 | 21-23 | 9.6.7 | HT PPDU cannot have a CH_BANDWIDTH parameter value of NON_HT_CBW40 | change "A control frame shall be carried in a non-HT PPDU or an HT PPDU with the TXVECTOR parameter CH_BANDWIDTH equal to NON_HT_CBW40…" to "A control frame shall be carried in a non-HT PPDU…" | Accept |
| 786 | 109 | 25-26 | 9.6.7 | HT PPDU cannot have a CH_BANDWIDTH parameter value of NON_HT_CBW40 | change "A control frame shall be carried in an HT PPDU with the TXVECTOR parameter CH_BANDWIDTH not equal to NON_HT_CBW40…" to "A control frame shall be carried in an HT PPDU…" | Accept |
| 787 | 109 | 34-35 | 9.6.7 | HT PPDU cannot have a CH_BANDWIDTH parameter value of NON_HT_CBW40 | change "A control frame shall be carried in HT PPDU with the TXVECTOR parameter CH_BANDWIDTH not equal to NON_HT_CBW40…" to "A control frame shall bbe carried in HT PPDU…" | Accept |
| 788 | 109 | 47-48 | 9.6.7 | HT PPDU cannot have a CH_BANDWIDTH parameter value of NON_HT_CBW40 | change "A control frame may be carried in HT PPDU with the TXVECTOR parameter CH_BANDWIDTH not equal to NON_HT_CBW40..." to "A control frame may be carried in HT PPDU…" | Accept |
| 2186 | 109 | 42 | 9.6.7 | "the frame eliciting the response was an RTS frame carried in HT PPDU" I believe that classifying a non-HT dupicate frame as an HT PPDU is likely to cause confusion, as well as a number of missed corner conditions (e.g. receiving a non-HT 20MHz in 40 mask satisfies this condition). For example, the quoted phrase probably needs at least "that is not a non-HT duplicate" added to it. | I think we need to crisp up our terminology in the MAC, and modify definitions accordingly. I propose we use the following mutually exclusive terms: non-HT - add non-HT 20 in 40MHz mask non-HT duplicate HT - changed to exclude non-HT duplicate and non-HT upper and lower modes Also consider adding a guide to this terminology somewhere (5.2.7 / Annex T ??). Then check all uses of these terms and select the appropriate one. Hopfully we'll find the text in 9.6 simplifies substantially, and we'll also catch the logical errors like the above. | Counter – the comment is accepted in principle. see document doc2526r01 and doc2513r1 which addresses a related comment CID2899. |

# Editor’s Instruction

TGn Editor: In D2.07, modify subclause 9.6.0e.3.1, lines 22 through 26 as follows

9.6.0e.3.1 Selection of a rate or MCS

b) If the Control Response frame is carried in an HT PPDU, then it shall be transmitted at the MCS as determined by the procedure defined in 9.6.0e.3.3 (Control response frame MCS computation (#1532, moved from 9.6.6)).

TGn Editor: In D2.07, modify subclause 9.6.0e.3.2, lines 59 through 62 as follows

9.6.0e.3.2 Selection of an alternate rate or MCS

For acontrol response frame that is transmitted in an HT PPDU, an alternative MCS to that determined by following the procedure of 9.6.0e.3.3 (Control response frame MCS computation (#1532, moved from 9.6.6)) may be used provided that the following conditions are met:

TGn Editor: In D2.07, modify subclause 9.6.0e.3.3, lines 4 through 11 as follows

9.6.0e.3.3 Control response frame MCS computation (#1532, moved from 9.6.6)

If a control response frame (#2183) is to be transmitted within an HT PPDU in accordance with rules in 9.6.2 (HT PPDU/non-HT PPDU selection for control frames), the responding STA shall choose an MCS from a set of MCS called the CandidateMCSSet. The CandidateMCSSet is determined using the following rules:

TGn Editor: In D2.07, modify subclause 9.6.0e.3.3, lines 53 through 54 as follows

9.6.0e.3.3 Control response frame MCS computation (#1532, moved from 9.6.6)

b) If the frame eliciting the response is within an HT-PPDU

TGn Editor: In D2.07, modify subclause 9.6.2

9.6.2 HT PPDU/non-HT PPDU selection for control frames

A control frame shall be carried in a non-HT PPDU or a non-HT duplicate PPDU (#2175) except under the conditions listed below: (#1694)

- A control frame shall be carried in an HT PPDU when the control frame meets any of the following conditions:

  - the PPDU containing the control frame has a TXVECTOR SOUNDING parameter set to SOUNDING, or

  - the control frame contains an L-SIG Duration value (see 9.13.5 (L-SIG TXOP protection)), or

  - the control frame is sent using an STBC frame.

- A control frame shall be carried in an HT PPDU (#116) when the control frame is a response to a frame that meets the following conditions:

  - the frame eliciting the response included an HT Control field with the TRQ field set to 1 and the NDP announcement field was set to 0 and this responder sets the Implicit TxBF Receiving Capable field to 1, or

  - the frame eliciting the response was an RTS frame carried an HT PPDU (#3299), or

  - the frame eliciting the response was an STBC frame and the Dual CTS Protection field was set to 1 (see 9.2.5.5a (Dual CTS protection)).

- A control frame may be carried an HT PPDU (#116) when the control frame meets any of the following conditions:

  - the control frame contains an HT Control field with the MRQ field set to 1, or

  - the control frame contains an HT Control field with the TRQ field set to 1.

- A BlockAck may be aggregated with data (using A-MPDU) within an HT PPDU. (#1694)

NOTE.If improved protection from non-HT STAs is desired, aggregation of BlockAck should not be used.