IEEE P802.11
Wireless LANs

| 20/40 MHz Coexistence for 5 GHz |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-09-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Douglas Chan | Cisco Systems, Inc. | 170 W. Tasman Dr San Jose, CA USA 95134 | +1 408 527 9344 | dougchan@cisco.com |
| Brian Hart | Cisco Systems, Inc. | 170 W. Tasman Dr San Jose, CA USA 95134 | +1 408 525 3346 | brianh@cisco.com |
| Tomoya Yamaura | Sony Corporation | 5-1-12 Kitashinagawa, Shinagawa-ku, Tokyo, 1410001, Japan | +81-3-5448-4018 | yamauratomoya@yahoo.co.jp |

Abstract

This submission proposes constructive 20/40 MHz coexistence measures for operations in 5 GHz.  One of the main objectives is we try to protect 20 MHz BSSs that are on the seoncdary of a 20/40 MHz BSS.  Discussion of the rationales behind these changes is provided in submission 07/2564.

Changes are based on D2.07.  There are no comment resolved here.

11.9.8.3 Scanning requirements for a 20/40 MHz BSS

TGn Editor: Change the second paragraph of subclause “11.9.8.3 Scanning prior to starting a BSS” that starts with the words “If the AP chooses to start a 20/40 MHz BSS in 5 GHz” found at page 202 line 57 of TGn draft D2.07 as shown by the underlined text:

If the AP chooses to start a 20/40 MHz BSS in 5 GHz that occupies the same two channels as an existing 20/40 MHz BSS, then the AP shall ensure that the primary channel of the new BSS is identical to the primary channel of the existing 20/40 MHz BSS and that the secondary channel of the new 20/40 MHz BSS is identical to the secondary channel of the existing 20/40 MHz BSS, unless the AP discovers that on these two channels are existing 20/40 MHz BSSs with different primary and secondary channels.

TGn Editor: Insert after the second paragraph of subclause “11.9.8.3 Scanning prior to starting a BSS” that starts with the words “If the AP chooses to start a 20/40 MHz BSS in 5 GHz” found at page 202 line 57 of TGn draft D2.07 with the following underlined paragraphs:

If an AP chooses to start a 20/40 MHz BSS in 5 GHz on a pair of channels, the selected secondary channel shall correspond to a channel on which no beacons are detected during the dot11BSSWidthChannelTransitionDelayFactor Overlapping BSS scan time performed by the AP, unless there are beacons detected on both channels.

NOTE – The 20/40 MHz channel sets and their corresponding behavior limits (i.e., choice of primary and secondary channels) permissible in each regulatory class are respectively defined in Annex J and I.

An HT AP should not start a 20 MHz BSS in 5 GHz that is a secondary channel of any 20/40 MHz BSS.

- (informative) Regulatory Classes

TGn Editor: Add the note (underlined in red) to Table I.3 in Annex I in TGn draft D2.07 as indicated below:

| - Behavior limits sets |  |  |  |
| --- | --- | --- | --- |
| Behavior limits set | USA | Europe | Japan |
| 13 40 MHz Primary channel with Secondary = 1 and also 20 MHz operation on primary channel | Reserved | Reserved | Reserved |
| 14 40 MHz Primary channel with Secondary = -1 and also 20 MHz operation on primary channel | Reserved | Reserved | Reserved |
| 1511 – 255 | Reserved | Reserved | Reserved |
| NOTE – For 20 MHz operation where the regulatory class signifies 40 MHz channel spacing, the 20 MHz channel corresponds to the channel number indicated. |  |  |  |
| NOTE – The fields in Behavior limits set 13 and 14 to specify the 40 MHz channels are described in Clause 20.3.14.3. |  |  |  |