IEEE P802.11
Wireless LANs

| TGp WAVE Optional Synchronization in Clause 11 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-10-23 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Vinuth Rai | VSC-2 | 4009 Miranda Avenue, Palo Alto, CA, 94304 | 650-251-0530 | Vinuth@us.toyota-itc.com |
| John Kenney | VSC-2 | 1555 Woodridge Ave, Ann Arbor, MI 48105 | 574-272-1403 | johnkenney@alumni.nd.edu |
| Susan Dickey | Caltrans | 1357 S. 46th Street, Richmond, CA 94804 | 510-665-3664 | dickey@path.berkeley.edu |

Abstract

Synchronization is optional for a STA that belongs to a WBSS.  D3.0 attempts to document that option with selective references and additions to clause 11.1, which was written to standardize the mandatory synchronization operation for infrastructure and independent BSS types.  There are numerous problems with this approach, which are explained herein.  This submission retains the substance of the optional WBSS synchronization, but shows how it can be documented more effectively within clause 11.18.

The submission documents nine separate places where clause 11.1 text in 802.11-2007 needs to be altered if it is to reflect not only mandatory infrastructure and independent BSS synchronization, but optional WBSS synchronization as well.  Only two of these are addressed in D3.0, and even these are insufficient.  The submission shows how a single qualifying statement can be added to 11.1, restricting its scope to infrastructure and independent BSS types, and how  a few sentences can be copied or moved from 11.1 to 11.18 to document the optional WAVE BSS synchronization procedures.  We believe that this approach not only corrects errors in D3.0, but is easier for a reader to follow.

We have studied the following:

- baseline text for clause 11.1 (including sub-clauses),

- changes for 11.1 defined in D3.0, and

- clause 11.14 ( now 11.18) as defined in D3.0.

We have also looked at the comments directed toward these sections.

The purpose of this submission is to discuss how the optional synchronization function should be standardized for a STA in a WBSS (there are other issues with the 11.18 text that we do not address here).  Currently (D3.0) when we mention optional synchronization in 5.2.2a we refer to 11.1.3.4.  This in turn references 11.1.2.4.  There are other sub-clauses within 11.1 that pertain to the WBSS case as well, some of which are documented and others of which are not.  Below we analyze clause 11.1, as amended by D3.0.  We find nine distinct places where text needs to be altered to accommodate the WBSS case, only two of which are attempted in D3.0, and those two are shown to be inadequate.  We also show that large portions of 11.1 do not apply for the WAVE BSS case, so a reader interested in optional WBSS synchronization will have a very difficult time discerning what is required.  Our conclusion is that rather than trying to document optional synchronization for the WAVE BSS case within 11.1, it is better to document it within 11.18.  We should do two things:

- redefine all of 11.1 to apply only to the cases of infrastructure BSSs and IBSSs,

- put all WAVE BSS synchronization language in 11.18.

Step 1 can be achieved with a simple qualifier added to the introductory text in 11.1.  Step 2 will take some additional text (approximately 6 sentences that are new to 11.18), most of which will be copied or moved from appropriate parts of 11.1.

What follows in Section 1 of this submission is an analysis of 11.1, including the modifications proposed in D3.0.  We use marginal comments to show what the problems are.  Section 2 shows how 11.1 can be qualified and how 11.18 could be augmented to cover optional synchronization for the WBSS case.

# 1.  Analysis of 11.1

## 1.1 Table of Contents of 11.1

Here is the table of contents for 11.1, from the baseline 802.11-2007 (minus the page numbers):

11.1 Synchronization

11.1.1 Basic approach

11.1.1.1 TSF for infrastructure networks

11.1.1.2 TSF for an IBSS

11.1.2 Maintaining synchronization

11.1.2.1 Beacon generation in infrastructure networks

11.1.2.2 Beacon generation in an IBSS

11.1.2.3 Beacon reception

11.1.2.4 TSF timer accuracy

11.1.3 Acquiring synchronization, scanning

11.1.3.1 Passive scanning

11.1.3.2 Active scanning

11.1.3.3 Initializing a BSS

11.1.3.4 Synchronizing with a BSS

11.1.4 Adjusting STA timers

11.1.5 Timing synchronization for FH PHYs

## 1.2 Annotated text of 11.1

D3.0 proposes three changes to 11.1:

- a modification to the first sentence in 11.1 to say the sentence does not apply for a WBSS;

- a new one-paragraph sub-clause 11.1.1.3 called “TSF in WAVE mode”, and

- a modification to the first sentence in 11.1.3 to say “Except in WAVE mode”.

In the following, we show how 11.1 would look, with the D3.0 modifications shown underlined, using a blue font so it is easy to tell where the insertion stops.  We show our analysis of the problems with this text, vis a vis WAVE mode, in margin comments.  Note that many sentences, paragraphs, and sub-clauses are specific to infrastructure BSSs, IBSSs, and/or FH PHYs, and so are not applicable for our consideration.  These are not problematic per se, but moving the optional synchronization discussion for WBSS to 11.18 also has the additional benefit of being more concise for the reader interested specifically in WAVE.  These inapplicable sections are noted in the margins as “DNA to WAVE.”

11. MLME

11.1 Synchronization

All STAs within a single BSS shall be synchronized to a common clock using the mechanisms defined herein, unless the BSS is a WAVE BSS.

11.1.1 Basic approach

A Timing Synchronization Function (TSF) keeps the timers for all STAs in the same BSS synchronized. All STAs shall maintain a local TSF timer.

11.1.1.1 TSF for infrastructure networks

In an infrastructure BSS, the AP shall be the timing master for the TSF. The AP shall initialize its TSF timer independently of any simultaneously started APs in an effort to minimize the synchronization of the TSF timers of multiple APs. The AP shall periodically transmit special frames called Beacon frames that contain a copy of its TSF timer to synchronize the TSF timers of other STAs in a BSS. A receiving STA shall always accept the timing information in Beacon frames sent from the AP servicing its BSS. If a STA’s TSF timer is different from the timestamp in the received Beacon frame, the receiving STA shall set its local TSF timer to the received timestamp value.

Beacon frames shall be generated for transmission by the AP once every dot11BeaconPeriod TUs.

11.1.1.2 TSF for an IBSS

The TSF in an IBSS shall be implemented via a distributed algorithm that shall be performed by all of the members of the BSS. Each STA in the IBSS shall transmit Beacon frames according to the algorithm described in this clause. Each STA in an IBSS shall adopt the TSF value received from any Beacon frame or probe response from the IBSS of which it is a member and which has a TSF value later than its own TSF timer.

11.1.1.3 TSF in WAVE mode

Synchronization is not required for STAs operating in WAVE mode.  The local TSFtimer is maintained as described in 11.2, and may be set by higher layers using the MLME-SETTSFTIME and MLME-INCTSFTIME requests described in 10.3.25.

11.1.2 Maintaining synchronization

Each STA shall maintain a TSF timer with modulus 264 counting in increments of microseconds. STAs expect to receive Beacon frames at a nominal rate. The interval between Beacon frames is defined by the dot11BeaconPeriod parameter of the STA. A STA sending a Beacon frame shall set the value of the Beacon frame’s timestamp so that it equals the value of the STA’s TSF timer at the time that the data symbol containing the first bit of the timestamp is transmitted to the PHY plus the transmitting STA’s delays through its local PHY from the MAC-PHY interface to its interface with the WM [e.g., antenna, lightemitting diode (LED) emission surface].

11.1.2.1 Beacon generation in infrastructure networks

The AP shall define the timing for the entire BSS by transmitting Beacon frames according to the dot11BeaconPeriod attribute within the AP. This defines a series of TBTTs exactly dot11BeaconPeriod TUs apart. Time zero is defined to be a TBTT with the Beacon frame being a DTIM and transmitted at the beginning of a CFP. At each TBTT, the AP shall schedule a Beacon frame as the next frame for transmission according to the medium access rules specified in Clause 9. The beacon period is included in Beacon and Probe Response frames, and STAs shall adopt that beacon period when joining the BSS.

NOTE—Though the transmission of a Beacon frame may be delayed because of CSMA deferrals, subsequent Beacon frames shall be scheduled at the undelayed nominal beacon interval. This is shown in Figure 11-1.

Figure 11-1—Beacon transmission on a busy network

11.1.2.2 Beacon generation in an IBSS

Beacon generation in an IBSS is distributed. The beacon period is included in Beacon and Probe Response frames, and STAs shall adopt that beacon period when joining the IBSS. All members of the IBSS participate in beacon generation. Each STA shall maintain its own TSF timer that is used for dot11BeaconPeriod timing. The beacon interval within an IBSS is established by the STA at which the MLME-START.request is performed to create the IBSS. This defines a series of TBTTs exactly dot11BeaconPeriod TUs apart. Time zero is defined to be a TBTT. At each TBTT the STA shall

a) Suspend the decrementing of the backoff timer for any pending nonbeacon or non-ATIM transmission,

b) Calculate a random delay uniformly distributed in the range between zero and twice aCWmin aSlotTime,

c) Wait for the period of the random delay, decrementing the random delay timer using the same algorithm as for backoff,

d) Cancel the remaining random delay and the pending beacon transmission, if a Beacon frame arrives from the IBSS of which the STA is a member before the random delay timer has expired, at which time the ATIM backoff timer shall resume decrementing.

e) Send a Beacon frame if the random delay has expired and no Beacon frame has arrived from the IBSS of which the STA is a member during the delay period.

(See Figure 11-2.)

Figure 11-2—Beacon transmission in an IBSS

Copyright © 2007 IEEE. All rights reserved. 421

The beacon transmission shall always occur during the Awake Period of STAs that are operating in a lowpower mode. This is described in more detail in 11.2.

11.1.2.3 Beacon reception

STAs shall use information from the CF Parameter Set element of all received Beacon frames, without regard for the BSSID, to update their NAV as specified in 9.3.2.2.

STAs in an infrastructure network shall only use other information in received Beacon frames, if the BSSID field is equal to the MAC address currently in use by the STA contained in the AP of the BSS.

STAs in an IBSS shall use other information in any received Beacon frame for which the IBSS subfield of the Capability field is set to 1, the content of the SSID element is equal to the SSID of the IBSS, and the TSF value is later than the receiving STA’s TSF timer. Use of this information is specified in 11.1.4.

11.1.2.4 TSF timer accuracy

Upon receiving a Beacon frame with a valid FCS and BSSID or SSID, as described in 11.1.2.3, a STA shall update its TSF timer according to the following algorithm: The received timestamp value shall be adjusted by adding an amount equal to the receiving STA’s delay through its local PHY components plus the time since the first bit of the timestamp was received at the MAC/PHY interface. In the case of an infrastructure BSS, the STA’s TSF timer shall then be set to the adjusted value of the timestamp. In the case of an IBSS, the STA’s TSF timer shall be set to the adjusted value of the received timestamp, if the adjusted value of the timestamp is later than the value of the STA’s TSF timer. The accuracy of the TSF timer shall be no worse than ±0.01%.

11.1.3 Acquiring synchronization, scanning

Except in WAVE mode (see 11.1 and 11.14), a STA shall operate in either a Passive Scanning mode or an Active Scanning mode depending on the current value of the ScanMode parameter of the MLME-SCAN.request primitive.

Active scanning is prohibited in some frequency bands and regulatory domains. The MAC of a STA receiving an MLME-SCAN.request shall use the regulatory domain information it has to process the request and shall return a result code of NOT_SUPPORTED to a request for an active scan if regulatory domain information indicates an active scan is illegal.

Upon receipt of the MLME-SCAN.request primitive, a STA shall perform scanning. The SSID parameter indicates the SSID for which to scan. To become a member of a particular ESS using passive scanning, a STA shall scan for Beacon frames containing that ESS’s SSID, returning all Beacon frames matching the desired SSID in the BSSDescriptionSet parameter of the corresponding MLME-SCAN.confirm primitive with the appropriate bits in the Capabilities Information field indicating whether the Beacon frame came from an infrastructure BSS or IBSS. To actively scan, the STA shall transmit Probe request frames containing the desired SSID. Upon completion of scanning, an MLME-SCAN.confirm is issued by the MLME indicating all of the BSS information received.

Upon receipt of an MLME-JOIN.request, the STA shall use the syncrhonization procedure described in 11.1.3.4.

Upon receipt of an MLME-SCAN.request with the SSID parameter set to the wildcard SSID, the STA shall passively scan for any Beacon frames, or actively transmit Probe request frames containing the wildcard SSID, as appropriate depending upon the value of ScanMode. Upon completion of scanning, an MLMESCAN. confirm is issued by the MLME indicating all of the BSS information received.

If a STA’s scanning does not result in finding a BSS with the desired SSID and of the desired type, or does not result in finding any BSS, the STA may start an IBSS upon receipt of the MLME-START.request. The MAC of a STA receiving an MLME-START.request shall use the regulatory domain information it has to process the request and shall return a result code of NOT_SUPPORTED to the request if regulatory domain information indicates starting the IBSS is illegal.

When a STA starts a BSS, that STA shall determine the BSSID of the BSS. If the BSSType indicates an infrastructure BSS, then the STA shall start an infrastructure BSS and the BSSID shall be equal to the STA’s dot11StationID. The value of the BSSID shall remain unchanged, even if the value of dot11StationID is changed after the completion of the MLME-START.request. If the BSSType indicates an IBSS, the STA shall start an IBSS, and the BSSID shall be an individual locally administered IEEE MAC address as defined in 5.2 of IEEE Std 802-1990. The remaining 46 bits of that MAC address shall be a number selected in a manner that minimizes the probability of STAs generating the same number, even when those STAs are subjected to the same initial conditions. The value SSID parameter shall be used as the SSID of the new BSS. It is important that designers recognize the need for statistical independence among the random number streams among STAs.

11.1.3.1 Passive scanning

If the ScanType parameter indicates a passive scan, the STA shall listen to each channel scanned for no longer than a maximum duration defined by the MaxChannelTime parameter.

11.1.3.2 Active scanning

Active scanning involves the generation of Probe request frames and the subsequent processing of received Probe Response frames. The details of the active scanning procedures are as specified in the following subclauses.

11.1.3.2.1 Sending a probe response

STAs, subject to criteria below, receiving Probe Request frames shall respond with a probe response only if

a) The SSID in the probe request is the wildcard SSID or the specific SSID of the STA,

b) The BSSID field in the probe request is the wildcard BSSID or the BSSID of the STA, and

c) The DA field in the probe request is the broadcast address or the specific MAC address of the STA.

Probe Response frames shall be sent as directed frames to the address of the STA that generated the probe request. The probe response shall be sent using normal frame transmission rules. An AP shall respond to all probe requests meeting the above criteria. In an IBSS, the STA that generated the last Beacon frame shall be the STA that responds to a probe request.

Only APs and STAs in an IBSS respond to probe requests. The procedures defined in this subclause ensure that in each BSS there is at least one STA that is awake at any given time to receive and respond to probe requests. A STA that sent a Beacon frame shall remain in the Awake state and shall respond to probe requests, subject to criteria in the next paragraph, until a Beacon frame with the current BSSID is received. If the STA is an AP, it shall always remain in the Awake state and always respond to probe requests, subject to criteria in the next paragraph. There may be more than one STA in an IBSS that responds to any given probe request, particularly in cases where more than one STA transmitted a Beacon frame following the most recent TBTT, either due to not receiving successfully a previous Beacon frame or due to collisions

between beacon transmissions. STAs receiving Probe Request frames shall respond with a probe response when the SSID in the probe request is the wildcard SSID or matches the specific SSID of the STA. Probe Response frames shall be sent as directed frames to the address of the STA that generated the probe request. The probe response shall be sent using normal frame transmission rules. An AP shall respond to all probe requests meeting the above criteria. In an IBSS, the STA that generated the last Beacon frame shall be the STA that responds to a probe request.

11.1.3.2.2 Active scanning procedure

Upon receipt of the MLME-SCAN.request with ScanType indicating an active scan, a STA shall use the following procedure:

For each channel to be scanned,

a) Wait until the ProbeDelay time has expired or a PHYRxStart.indication has been received;

b) Perform the Basic Access procedure as defined in 9.2.5.1;

c) Send a probe request to the broadcast destination address, with the SSID and BSSID from the

MLME-SCAN.request primitive;

d) Clear and start a ProbeTimer;

e) If PHY-CCA.indication (busy) has not been detected before the ProbeTimer reaches

MinChannelTime, then clear NAV and scan the next channel, else when ProbeTimer reaches

MaxChannelTime, process all received probe responses;

f) Clear NAV and scan the next channel.

See Figure 11-3.

Figure 11-3—Probe response

When all channels in the ChannelList have been scanned, the MLME shall issue an MLME-SCAN.confirm with the BSSDescriptionSet containing all of the information gathered during the scan.

11.1.3.3 Initializing a BSS

Upon receipt of an MLME-START.request, a STA shall determine the BSS’s BSSID (as described in 11.1.3), select channel synchronization information, select a beacon period, initialize and start its TSF timer, and begin transmitting Beacon frames. If the dot11MultiDomainCapabilityEnabled attribute is true, a STA shall not start a BSS, neither an infrastructure BSS nor an IBSS, unless a properly formed Beacon frame including a Country information element can be constructed, and the dot11CountryString attribute has been set.

11.1.3.4 Synchronizing with a BSS

Upon receipt of an MLME-JOIN.request, a STA shall adopt the BSSID in the request. Upon receipt of a Beacon frame from the BSS, a STA shall adopt the channel synchronization information (applicable only if the STA contains an FH PHY), and TSF timer value of the parameters in the Beacon frame using the algorithm described in 11.1.2.4, and the MLME shall issue an MLME-JOIN.confirm indicating the operation was successful.

In addition to these synchronization parameters, a STA joining an infrastructure BSS will adopt each of the parameters found in the BssDescription of the MLME-JOIN.request except Local time, Capability Information, and BSSBasicRateSet parameters. Local time is not adopted but is used as a local variable in adopting the TSF as described in 11.1.2.4. The Capability Information reflects the capabilities of the sender and is not adopted but may be used to determine local configuration or behavior. The BSSBasicRateSet parameter is not adopted but may determine if the STA can join the BSS. A STA joining an IBSS will adopt the same parameters except the CF parameter set (since contention free period is not permitted in an IBSS).

If the JoinFailureTimeout timer expires prior to the receipt of a Beacon frame from the BSS, the MLME shall issue an MLME-JOIN.confirm indicating the operation was unsuccessful.

If the dot11MultiDomainCapabilityEnabled attribute is true, a STA receiving a Beacon or Probe Response frame containing a Country information element shall adopt the all parameters included in that Country information element when joining a BSS and the dot11RegDomainsSupportEntry shall be set to Other.

If a Hopping Pattern Parameters element is present in the Beacon or Probe Response frame, and if the dot11MultiDomainCapability-Enabled attribute is true, a STA shall adopt the pattern parameters in the element and calculate the hopping patterns using one of the algorithms defined in 7.3.2.10 or 7.3.2.11. Using the appropriate pattern, set, and index values from the FH Parameter Set element, the STA shall adopt the values in use by the BSS when joining. The dot11RegDomainsSupportedValue shall be set to Other when the STA is operating using Country information element settings

11.1.4 Adjusting STA timers

In an infrastructure BSS, STAs shall always adopt the TSF timer value in a Beacon frame or probe response coming from the AP in their BSS by using the algorithm in 11.1.2.4.

In response to an MLME-JOIN.request, a STA joining an IBSS shall initialize its TSF timer to 0 and shall not transmit a Beacon frame or probe response until it hears a Beacon frame or probe response from a member of the IBSS with a matching SSID. This ensures that such a STA will adopt the timer from the next Beacon frame or probe response from its IBSS.

All Beacon and Probe Response frames carry a Timestamp field. A STA receiving such a frame from another STA in an IBSS with the same SSID shall compare the Timestamp field with its own TSF time. If the Timestamp field of the received frame is later than its own TSF timer, the STA in the IBSS shall adopt all parameters contained in the Beacon frame, except the Capability bits, Supported Rates information element, and Extended Supported Rates information element.

11.1.5 Timing synchronization for FH PHYs

NOTE—This subclause pertains only to STAs using an FH PHY.

The TSF described here provides a mechanism for STAs in an FH system to synchronize their transitions from one channel to another (their “hops”). Every STA shall maintain a table of all of the hopping sequences that are used in the system. All of the STAs in a BSS shall use the same hopping sequence. Each Beacon frame and probe response includes the channel synchronization information necessary to determine the hop pattern and timing for the BSS.

STAs shall use their TSF timer to time the aCurrentDwellTime. The aCurrentDwellTime is the length of time that STAs shall stay on each frequency in their hopping sequence. Once STAs are synchronized, they have the same TSF timer value.

STAs in the BSS shall issue an appropriate PLME service primitive for the PHY in use to tune to the next frequency in the hopping sequence when

TSF timer MOD aCurrentDwellTime = 0.

END OF ANNOTATED 11.1 TEXT

Conclusion:

Nine marginal comments (1, 2, 5, 7, 13, 15, 16, 19, and 21) indicate places where clause 11.1 would need to be altered to make its statements accurate for the WAVE mode case.  Those alterations need to be precise, indicating whether for a STA in WAVE mode a given statement constitutes a requirement, an option, or is excluded from consideration.  Simply saying “except in WAVE mode” is not adequate in most of these cases.   In addition, comment 22 above indicates that a sentence in 11.1.3.4 concerning the Country information element also requires alteration.  This is not related to the synchronization issue per se; it should probably be copied to 11.18.2 regardless.

We conclude that rather than attempting these extensive alterations, it is advisable to move WBSS synchronization out of 11.1 and into 11.14.  This is advisable on three grounds:

- it will improve the likelihood of passing a future letter ballot,

- it will be easier for a reader to follow, and

- it is a relatively modest editing task, as seen in the next section.

# 2. Modifications to D3.0 to document optional synchronization for the WAVE case.

This section shows a way to modify D3.0 so that the optional synchronization is documented in clause 11.18.  Clause 11.1 is qualified to say that it applies only for the infrastructure and independent BSS types, and to point to 11.18 for WAVE BSS synchronization.

## 2.1 Modification to 11.1 if we put WBSS synchronization in 11.18

Our proposal to modify Clause 11.1:

- remove all three of the changes proposed in D3.0

- Modify the initial paragraph as follows:

Start text covered by motion here:

All STAs within a single infrastructure BSS or IBSS shall be synchronized to a common clock using the mechanisms defined herein; refer to Clause 11.18 for optional synchronization in WAVE mode.

Rewrite 11.18.1, 11.18.2 and 11.18.3 as shown below:

11.18.1 Initializing a WAVE BSS

A STA in WAVE mode initializes a WAVE BSS by sending a first WAVE beacon containing all the information necessary to join the WAVE BSS (including the BSSID as described in 7.1.3.3.3). A STA may send subsequent WAVE beacons to continue to advertise the presence of the WAVE BSS.

A STA sending a WAVE beacon shall set the value of the timestamp so that it equals the value of the STA’s TSF timer (see 11.18.3) at the time that the data symbol containing the first bit of the timestamp is transmitted to the PHY plus the transmitting STA’s delays through its local PHY from the MAC-PHY interface to its interface with the WM [e.g., antenna, light emitting diode (LED) emission surface].

A WAVE beacon is generated by a STA’s MLME in response to receiving an MLME-ONDEMANDBEACON.request from the STA’s SME.  After generating the WAVE beacon, the MLME returns an MLME-ONDEMANDBEACON.confirm to the SME.

11.18.2 Joining a WAVE BSS

The following steps are taken by a STA in WAVE mode that receives a WAVE beacon.

1. The MLME issues an MLME-ONDEMANDBEACON.indication to the SME.

2. If the STA elects to join the WAVE BSS, the SME issues an MLME-SET.request (see 10.x.y.1) to the MLME. The MLME issues an MLME-SET.confirm (see 10..x.y.2) to the SME to convey the results of the set request.

A STA not in WAVE mode shall not join a WAVE BSS.

The process of initiating and joining a WAVE BSS is shown in Figure 11-23. After joining a WAVE BSS a STA may transmit data to another STA or to a DS.

[Editor’s note: The MLME-SET.request and MLME-SET.confirm primitives do not yet exist in Clause 10.]

11.18.3 TSF in WAVE mode

Each STA shall maintain a TSF timer with modulus 264 counting in increments of microseconds.   The local TSF timer may be set by higher layers using the MLME-SETTSFTIME and MLME-INCTSFTIME requests described in 10.3.25a-10.3.25c.  MAC sublayer synchronization is not required for a STA operating in a WAVE BSS.  If a STA in a WAVE BSS chooses to synchronize with the WAVE BSS at the MAC sublayer, it shall update its TSF timer according to the following algorithm: the received timestamp value is adjusted by adding an amount equal to the receiving STA’s delay through its local PHY components plus the time since the first bit of the timestamp was received at the MAC/PHY interface.

Motion (if technical and/or significant):

Move to accept recommended rewrites from section 2. 1 in this document and instruct the editor to make necessary changes to the current draft.

Motion by: John Kenney

Date: January 17th, 2008

Second:  Wayne Fisher

| Approve: | Disapprove: | Abstain:0 |
| --- | --- | --- |

## NOTE TO EDITOR

## 2.3 Additional Changes Required

In addition to the changes suggested above for 11.18.1 and 11.18.3, three additional changes should be made:

- The reference in 5.2.2a to 11.1.3.4 should be changed to 11.18.3

- as noted in margin comment #8 above, in clause 7.3.1.10 of the baseline the reference to TSF should be changed from “see 11.1” to “see 11.1 and 11.18