IEEE P802.11
Wireless LANs

| Summary of 16m Requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-10-26 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Bruce Kraemer | Marvell | 5488 Marvell Lane, Santa Clara, CA 95054 | +1-321-751-3988 | bkraemer@marvell.com |

Abstract

The following is a brief summary of 16m requirements deemed relevant for discussion in 802.11 IMT-Advanced.

Refernce source is the 26 page document 802.16m-07_0024r4

16m Requirements Summary

Extracted from 802.116m-07_002r4 (26 pages)

Note: Only selected pieces are included but all extracts are verbatim.

5.0 General requirements

5.1 Legacy support

IEEE 802.16m shall provide continuing support and interoperability for legacy WirelessMAN-OFDMA equipment, including MSs and BSs. Specifically, the features, functions and protocols enabled in IEEE 802.16m shall support the features, functions and protocols employed by WirelessMAN-OFDMA legacy equipment.

5.2 Complexity

IEEE 802.16m shall provide the ability to disable legacy support.

complexity of MSs and BSs shall be minimized by adhering to the following:

a) The functional and performance requirements shall be met with mandatory features only.

b) Optional features shall be considered only if they provide significant functional and performance improvements over mandatory features.

c) Support of multiple mandatory features which are functionally similar and/or have similar impact on performance shall be avoided.

d) The number of states of protocols and procedures should be minimized.

5.5 Operating bandwidths

IEEE 802.16m shall support scalable bandwidths from 5 to 20 MHz. Other bandwidths shall be considered as necessary to meet operator and ITU-R requirements.

5.6 Duplex schemes

IEEE 802.16m shall support both Time Division Duplex (TDD) and Frequency Division Duplex (FDD) operational modes. The FDD mode shall support both full-duplex and half-duplex MS operation. Specifically, a half-duplex FDD (H-FDD) MS is defined as an FDD MS that is not required to transmit and receive simultaneously.

In TDD mode, the DL/UL ratio should be adjustable. In the extreme, the IEEE 802.16m system should be capable of supporting downlink-only configurations on a given carrier.

In FDD mode, the UL and DL channel bandwidths may be different and should be configurable (e.g.10MHz downlink, 5MHz uplink).

6.0 Functional requirements

6.1 Peak data rate

This section defines the peak data rate achievable between a BS and an MS under ideal conditions.

IEEE 802.16m shall support the peak, expressed as a normalized peak rate (i.e. absolute maximum supported data rate divided by the occupied channel bandwidth) as specified in Table 1.

6.6 Handover

IEEE 802.16m shall support handover within and between all cell types in an IEEE802.16m system.

IEEE 802.16m shall provide handover with WirelessMAN-OFDMA Reference Systems in accordance with Section 5.1.

IEEE 802.16m shall provide support for handover with other RATs. However, an IEEE 802.16m MS is not required to be multi-mode.

IEEE 802.16m shall provide service continuity during handover for both inter-RAT and intra-RAT handover.

IEEE 802.16m should support IEEE 802.21 Media Independent Handover (MIH) Services.

Mobility procedures should be fully compatible with the IEEE 802.16 Network Control and Management Services (NCMS).

6.12 Multi-RAT operation

IEEE 802.16m shall support interworking functionality to allow efficient handover to other radio access technologies. Those of interest include:

• IEEE 802.11

• 3GPP GSM/EDGE, UTRA (FDD and TDD) and E-UTRA (FDD and TDD)

• 3GPP2 CDMA2000

7.0 Performance requirements

Annex A: Usage models (informative)

IEEE 802.16m should support a wide range of deployment scenarios and usage models including

a) those considered during formulation of the existing standards and

b) as envisioned by IMT-Advanced requirements.

The examples provided in this section are informative only.

IEEE 802.16m should support different usage models. More specifically, it should cover (but not be restricted to):

1) Higher data rates and improved performance (compared to WirelessMAN-OFDMA Reference System) in legacy cell sizes (of several kilometers radius).

2) Very high data rates in smaller cells

3) High mobility optimized scenarios

4) Deployment with Multi-hop Relay Networks

5) Co-Deployment with Other Networks

6) Provision for PAN/LAN/WAN Collocation / Coexistence

802.116m-07_002r4 Complete Table of Contents

1.0 Overview........................................................................................................................................ 4

2.0 References...................................................................................................................................... 5

3.0 Definitions...................................................................................................................................... 6

4.0 Abbreviations and acronyms........................................................................................................... 7

5.0 General requirements ...................................................................................................................... 8

5.1 Legacy support........................................................................................................................... 8

5.2 Complexity................................................................................................................................. 8

5.3 Services...................................................................................................................................... 9

5.4 Operating frequencies ................................................................................................................. 9

5.5 Operating bandwidths ................................................................................................................. 9

5.6 Duplex schemes .......................................................................................................................... 9

5.7 Support of advanced antenna techniques.................................................................................. 10

5.8 Support for government mandates and public safety................................................................ 10

6.0 Functional requirements................................................................................................................ 11

6.1 Peak data rate ............................................................................................................................ 11

6.2 Latency..................................................................................................................................... 12

6.2.1 Data latency ...................................................................................................................... 12

6.2.2 State transition latency...................................................................................................... 12

6.2.3 Handover interruption time............................................................................................... 13

6.3 QoS .......................................................................................................................................... 13

6.4 Radio resource management ..................................................................................................... 13

6.4.1 Reporting.......................................................................................................................... 13

6.4.2 Interference management.................................................................................................. 14

6.5 Security .................................................................................................................................... 14

6.6 Handover.................................................................................................................................. 14

6.7 Enhanced multicast broadcast service ...................................................................................... 14

6.7.1 MBS channel reselection delay and interruption times .................................................... 15

6.8 Location based services (LBS) ................................................................................................. 15

6.9 Reduction of user overhead ...................................................................................................... 15

6.10 System overhead ....................................................................................................................... 15

6.11 Enhanced power saving ............................................................................................................ 15

6.12 Multi-RAT operation................................................................................................................. 15

7.0 Performance requirements ............................................................................................................ 16

7.1 User throughput ........................................................................................................................ 16

7.1.1 Relative performance........................................................................................................ 16

7.1.2 Absolute performance....................................................................................................... 16

7.2 Sector throughput and VoIP capacity........................................................................................ 17

7.2.1 Relative sector throughput and VoIP capacity .................................................................. 17

7.2.2 Absolute sector throughput and VoIP capacity ................................................................. 17

7.3 Mobility.................................................................................................................................... 18

7.4 Cell coverage ............................................................................................................................ 18

7.5 Enhanced multicast-broadcast service ...................................................................................... 19

7.6 Location-based services performance....................................................................................... 20

8.0 Operational requirements.............................................................................................................. 20

8.1 Support for multi-hop relay ...................................................................................................... 20

8.2 Synchronization ........................................................................................................................ 20

8.3 Co-deployment with other networks......................................................................................... 20

8.3.1 Co-deployment requirements............................................................................................ 20

8.3.2 Coexistence scenarios ....................................................................................................... 21

8.4 Support of self organizing mechanisms.................................................................................... 21

Annex A: Usage models (informative).................................................................................................. 22

A.1 Service and application scenarios ............................................................................................. 22

A.2 Deployment scenarios............................................................................................................... 23

A.2.1 Frequency reuse ................................................................................................................ 23

A.2.2 Deployment with multi-hop relay networks ..................................................................... 23

A.2.3 High mobility optimized scenario..................................................................................... 26

A.2.4 Provision for PAN/LAN/WAN collocation / coexistence................................................. 26

A.2.5 Very high data rates in smaller cells ................................................................................. 26

References: