IEEE P802.11
Wireless LANs

| LB115 Coex Comment Resolutions: Protection Mechanisms |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-06 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bjorn A. Bjerke | Qualcomm, Inc. | 9 Damonmill Sq. Suite 2A Concord, MA 01742, USA | +1 781-276-0912 | bbjerke@qualcomm.com |

Abstract

This submission suggests resolutions of LB115 Coex comments related to the sub-topic Protection Mechanisms. The following CIDs are addressed: 5005, 5006, 5796, 5822, 5625, 5886, 5626, 5627, 5535 5100, 5099, 5101, 5102, 5728.

The changes marked in this document are based on TGn Draft 3.0.

| 5005 | 80.07 | 7.3.2.53 | The encoding of HT Protection when it is 0 is not clear. | Change it to "Set to 0 if: - All STAs in the primary or the secondary channel if detected are HT STAs and all STAs that are known by the transmitting STA to be a member of this BSS are either, - 20/40 MHz HT in a 20/40 MHz BSS, or - 20 MHz HT in a 20 MHz BSS | Counter. Accept with minor change. |
| --- | --- | --- | --- | --- | --- |
| 5006 | 80.23 | 7.3.2.53 | In the encoding of HT Protection when it is 1, the part "or in both the primary and secondary channels" is redundant. "At least" at the beginning already covers this case. | Delete the cited part. | Counter. Deleting the cited text would eliminate the case where there are non-HT STAs in BOTH the primary and secondary channels. Instead modify text to make it more clear. |
| 5796 | 80.30 | 7.3.2.53 | "Set to 2 if:— All STAs detected in the primary or the secondary channel or that are known by the transmitting STA to be a member of this BSS are HT STA, …" | Not clear what is the BSS member in the secondary channel. Make it clear | Reject. HT STAs detected only in the secondary channel are not members of the BSS. No clarification needed. |
| 5822 | 135.27 | 9.13 | Please clarify what the stuck-out phrase "for non-ERP receivers" is doing here. If the phrase is needed, remove the strike-out. If the phrase is not needed, then delete the phrase. | Please clarify what the struck-out phrase "for non-ERP receivers" is doing here. Either delete the phrase or delete the strike-out. | Reject. The heading appears in the base standard and is modified as explained in the Editorial Notes appearing on the same page. |
| 5625 | 136.65 | 9.13.3.1 | "then a STA shall not transmit a 40 MHz HT PPDU (TXVECTOR parameter CH_BANDWIDTH set to HT_CBW40) to initiate a TXOP provided that the restrictions specified in 9.6 (Multirate support) are obeyed" This is very odd. Clearly a cut and paste. But whereas in the previous para the "...provided restrictions..." makes sense, here it doesn't make any sense at all. | Remove "provided that ... are obeyed" | Accept |
| 5886 | 136.65 | 9.13.3.1 | "then a STA shall not transmit a 40 MHz HT PPDU (TXVECTOR parameter CH_BANDWIDTH set to HT_CBW40) to initiate a TXOP provided that the restrictions specified in 9.6 (Multirate support) are obeyed" Something isn't quite right here; I suspect a cut and paste error here. The portion of the sentence that talks about the restrictions just doesn't make sense in the context. | "then a STA shall not transmit a 40 MHz HT PPDU (TXVECTOR parameter CH_BANDWIDTH set to HT_CBW40) to initiate a TXOP" | Accept. See resolution of CID 5625, |
| 5626 | 137.35 | 9.13.3.2 | Line 35 "In IBSS, the RIFS Mode field of the HT Information element is reserved but the HT STAs shall operate as though this field were set to 1." and line 46: "A STA that is a member of an IBSS shall protect RIFS sequences, adhering to the same requirements as described in the column of Table 9-6 (Protection requirements for HT Protection field values 1 and 3) labeled "Use_Protection = 0 or ERP IE is not present (HT Protection field set to 3)"." Also page 135.63: "In an IBSS, the HT protection field is reserved, but the HT STAs shall protect HT transmissions as though the HT protection field were set to 3. A STA that is a member of an IBSS shall protect HT greenfield format ...." There appears to be redundancy here. | In 9.13.3.1 call out for IBSS: 1. Act as though HT Protection was 3 2. Act as though RIFS Mode was 1 3. Act as though "Use-Protection=0 or ERP IE is not present) This requires moving the first cited sentence into 9.13.3.1. The second cited sentence can then be deleted. | Counter. Accept in principle |
| 5627 | 138.08 | 9.13.3.3 | "Beacon where the supported rates only contain Clause 15, 17, 18 or 19 rates" The supported rate set can only contain these rates (except IR and FHSS). An HT STA will always see this condition as true. | Replace with "Beacon that does not contain an HT Capabilities element" Make a similar change in the following bullet (other management frame). | Counter. Accept the change in the first bullet and modify the second bullet. |
| 5535 | 139.01 | 9.13.4 | Note for resolving CID 332 suggests that reducing the L_LENGTH is the appropriate means of solving legacy interop issues, but using RTS/CTS protection is a perfectly reasonable means of solving the problem. | Change "should" in the note to a "may" and/or suggest using protection as a valid fallback mechanism. | Counter. Leave “should” and add RTS/CTS and CTS-to-self protection as suggestions. |
| 5100 | 212.38 | 11.15.6 | If DSSS/CCK Mode is set to 1, the appropriate protection mechanisms associated with protecting DSSS/CCK STAs shall also be employed. | Devise these protection mechanisms if not exist and mention that they shall be enabled if DSSS/CCK Mode in 40 MHz is set to 1. | Counter. The protection mechanism is already in place (ERP Information element) |
| 5099 | 212.38 | 11.15.6 | The DSSS/CCK field is also specified in MP's, as per 7.3.2.52.2, yet it's not mentioned here. | Add mention of Measurement Pilot frames as well. | Accept. |
| 5101 | 212.50 | 11.15.6 | The description of when a STA "has a 40 MHz operating channel" is vague; should say instead that a STA supports 40 MHz operations. | Clarify "has a 40 MHz operating channel" and say instead that a STA supports 40 MHz operations. | Reject. The text in question talks about operating bandwidth, not the capability of supporting 40 MHz.The proposed change fails to convey this. |
| 5102 | 212.55 | 11.15.6 | When DSSS/CCK Mode is set to 1, according to the second paragraph, HT STA shall not generate DSSS/CCK transmissions, then naturally the DSSS/CCK rates are not allowed. However, here it says the HT STA may use these rates, which contradicts what's allowed. | Change "may" to "shall not". | Counter. See resolution of CID 5728. |
| 5728 | 212.55 | 11.15.6 | "An HT AP declares whether DSSS/CCK rates may be used in a 20/40 MHz BSS through the DSSS/CCK Mode in 40 MHz field of the HT Capabilities element in its Beacons. If this field is set to 0, associated HT STAs shall not use DSSS/CCK rates. If it is set to 1, associated HT STA may use DSSS/CCK rates." This adds nothing to the para and bulleted list starting at 212.38. | Delete the cited text. | Accept. |

Changes to 7.3.2.53 HT Information element

TGn Editor: in Table 7-42m (HT Information element) on page 80, in the row labelled HT Protection, make the following changes:

a) in the third column, change the encoding description for mode 0 as follows:

Set to 0   if:

- All STAs detected in the primary or the sec-

ondary channel or that are a member of this

BSS are HT STAs, and

- All STAs that are known by the transmitting STA

to be a member of this BSS are either

-- 20/40 MHz HT in a 20/40 MHz BSS, or

-- 20 MHz HT in a 20 MHz BSS

—    And either,

— all STAs that are known by the transmitting

STA to be a member of this BSS are

20/40 MHz HT in a 20/40 MHz BSS, or

— this BSS is a 20 MHz BSS

b) in the third column, change the encoding description for mode 1 as follows:

Set to 1 (HT non-member protection mode) if:

— there is at least one a non-HT STA is detected in

either the primary or the secondary channel

or in both the primary and secondary channels,

that is not known by the transmitting

STA to be a member of this BSS, and

— all STAs that are known by the transmitting

STA to be a member of this BSS are HT STA

Changes to 9.13 Protection mechanisms

TGn Editor: in 9.13.3.1, page 137, line 24, delete the last part of the sentence, beginning with “provided that the restrictions…”

TGn Editor: in 9.13.3.1, page 137, change the paragraph that begins on line 63, and insert a new paragraph immediately after the first paragraph as follows:

In an IBSS, the HT protection field is reserved, but the HT STAs shall protect HT transmissions as though

the HT pProtection field were set to 3. A STA that is a member of an IBSS shall protect HT greenfield format

PPDUs and RIFS sequences, adhering to the same requirements as described in the column of Table 9-6 (Protection requirements for HT Protection field values 1 and 3) labeled "Use_Protection = 0 or ERP IE is not present (HT Protection field set to 3)".

In an IBSS, the RIFS Mode field of the HT Information element is also reserved but the HT STAs shall operate as though this field were set to 1.

TGn Editor: in 9.13.3.2, page 137, delete the paragraph that begins with “In IBSS…” on line 35.

TGn Editor: in 9.13.3.2, page 137, delete the paragraph that begins with “ A STA that is a member…” on line 46.

TGn Editor: in 9.13.3.3, page 138, change the paragraph that begins on line 3 as follows:

An HT STA may also scan for the presence of non-HT devices either autonomously or, for example, after

the STA’s AP transmits an HT Information element with the HT Protection field set to 1. Non-HT devices can

may be detected as follows :

— a non-HT BSS is overlapping (a non-HT BSS canmay be detected by the reception of a Beacon that does not contain an HT Capabilities elementwhere the supported rates only contain Clause 15, 17, 18 or 19 rates), or

— reception of any management frame (excluding a Probe Request) required to carry an HT Capabilities element during HT operation that does not carry such an elementwhere the supported rate set includes only Clause 15, 17, 18 and 19 rates, or

— reception of a Beacon containing an HT Information element with the OBSS Non-HT STAs Present

field set to 1.

TGn Editor: in 9.13.4, page 139, line 1, modify NOTE 3 as follows:

NOTE 3—The transmission of frames with L_LENGTH above 2340 octets should be accompanied by a protection mechanism fall back mechanism using a reduced L_LENGTH (e.g., RTS/CTS or CTS-to-self protection),  if it is determined that the use of L_LENGTH mechanism fails to effectively suppress non-HT transmissions. How this is determined is outside the scope of this standard.

TGn Editor: in 11.15.6, page 212, lines 38 and 42, insert “, measurement pilot,” immediately after “Beacon”.

TGn Editor: in 11.15.6, page 212, delete the paragraph that begins on line 55 and ends on line 57.