IEEE P802.11
Wireless LANs

| LB115 CID 5283 MAC BA Management |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-11-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 USA | +1 408 543 3370 | mfischer@broadcom.com |
| Adrian Stephens | Intel |  |  | Adrian.p.stephens@intel.com |

Abstract

Addressing TGn LB115 MAC comment group BA Management, one comment CID 5283.

R1:

Added a resolution for CID 5283 in the table.

CID 5283

| 5283 | Fischer, Matthew | 204.11 | 11.5.1.3 |  | I thought that we had parallel language describing when exactly an HT-immediate BA agreement has been established, but I cannot find such language. | Add language, similar to what is found in 11.15.1.3, to describe just exactly what has to happen in order to establish an HT-immediate BA agreement. | Counter – accept in principle – editor shall execute instructions provided in document 11-07/2746r1 under the heading CID 5283. |
| --- | --- | --- | --- | --- | --- | --- | --- |

11.5.1.3 Procedure common to both originator and recipient

TGn Editor: Change 11.5.1.3 as follows:

Once a Block Ack agreement has been successfully established between two STAs,  the type of agreement thus established is dependent on the capabilities of the STAs and the contents of the ADDBA frames used to establish this agreement as defined in table ??.

Table ?? – Types of Block Ack agreement based on capabilities and ADDBA conditions

| Capabilities condition | ADDBA condition | Type of Block Ack agreement |
| --- | --- | --- |
| One or both of the STA are non-HT | Block Ack Policy subfield set to 1 | Immediate |
|  | Block Ack Policy subfield set to 0 | Delayed |
| Both STA are HT STA | Block Ack Policy subfield set to 1 | HT-Immediate |
| Both STA are HT STA and both of the STA set the HT-Delayed Block Ack subfield of the HT Capabilities element to 1 | Block Ack Policy subfield set to 0 | HT-Delayed |
| Both STA are HT STA and at least one of the STA sets the HT-Delayed Block Ack subfield of the HT Capabilities element to 0 | Block Ack Policy subfield set to 0 | Delayed |

References: