IEEE P802.11
Wireless LANs

| Proposed Resolutions to LB115 Multirate Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-12-27 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yuichi Morioka | Sony Corporation | 5-1-12 Kitashinagawa Shinagawa-ku, Tokyo 141-0001, Japan | +81-3-5448-4017 | Yuichi.Morioka@jp.sony.com |

Abstract

This submission proposes resolutions to CIDs 5007, 5019, 5024, 5300, 5301, 5303, 5368, 5369, 5370, 5461, 5462, 5463, 5464, 5465, 5486, 5519, 5520, 5521, 5533, 5608, 5610, 5611, 5612, 5613, 5614, 5615, 5799, 5800, 5801, 5802 ,5803, 5826, and 5878 related to Multirate support..

The changes proposed in this document are based on 802.11n D3.02.

The highlighted text indicates the actual changes required to made by the editor.

# CID 5486

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5486 | 109.44 | 9.6.0a | Clause 17, 18, 19, and 20 are mentioned in discussing TxTime. What about clause 15? | Add clause 15. | Accept |

# CID 5608

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5608 | 109.57 | 9.6.0b | "The BasicMCSSet field of the HT Information element shall indicate which MCS values are supported by all HT STAs in a BSS." This is a normative requirement across multiple STAs - which is not possible. | Replace with: "The BasicMCSSet field of the HT Information element indicates which MCS values are supported by all HT STAs in a BSS." | Accept |

# CID 5533

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5533 | 109.62 | 9.6.0b | The following paragraph is confusingly-worded: "In 9.6.0d (Rate selection for Data and Management frames (#2164)) and 9.6.0e (Rate selection for control frames) when a MCS from the BSSBasicMCSSet is required and the BSSBasicMCSSet is empty, the STA shall select a rate from the BSSBasicRateSet. (#2179)" | It would read better to just put a "(see 9.6.0d and 9.6.0e)" at the end and strike the "In 9.6.0d (Rate selection for Data and Management frames (#2164)) and 9.6.0e (Rate selection for control frames)" from the lead-in. | Counter – deletion of the reference may change the context of the sentence. To clarify, reword thus; “When a MCS from the BSSBasicMCSSet is required in 9.6.0d and 9.6.0e but the BSSBasicMCSSet is empty, the STA shall select …” See also resolution to CID 5300 and 5461. |

# CID 5300

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5300 | 109.64 | 9.6.0b | Isn't the following expressed in the subsequent subclauses: "when a MCS from the BSSBasicMCSSet is required and the BSSBasicMCSSet is empty, the STA shall select a rate from the BSSBasicRateSet." plus - in those subclauses the question of what to do when both sets are empty is answered. | Change "shall select" in the cited text to "selects" - maybe consider adding declarative (non-normative) text to mention the double-empty case falling to mandatory rates as well. | Counter – to clarify and simplify the subclauses related to multirate support, the rules when Basic Rate, Basic MCS, and Basic STBC MCS are needed but are empty should be unified into one place. See resolution in doc2992. |

Proposed resolution:  Counter

TGn Editor: Add the following subclause before 9.6.0b, in page 110, line 11.

9.6.0a Basic Rate Set (Editor’s note: Insert this subclause before the 9.6.0a in D3.02 and re-number the following subclauses)

The BSSBasicRateSet parameter indicates the rates that are supported by all STAs in a BSS. An AP shall refuse an association or reassociation request from a STA that does not support all the Rates in the BSSBasicRateSet parameter

When a rate from the BSSBasicRateSet is required in 9.6.0d and 9.6.0e but the BSSBasicRateSet is empty, the STA shall select a rate from the mandatory rate set.

TGn Editor: Add the following at the end of subclause 9.6.0c Basic STBC MCS, page 110, line 42.

When an MCS from the Basic STBC MCS is required in 9.6.0d and 9.6.0e but the Basic STBC MCS is not defined, the STA shall select an MCS from the mandatory MCS set.

TGn Editor: Delete the following sentence in page 111, line 8.

If the Basic STBC MCS is not defined (see 9.6.0c (Basic STBC MCS (#2806, 2179))), any MCS within the

Mandatory MCS set for the attached PHY shall be used to transmit these frames.

TGn Editor: Change the sentence in page 111, line 59 to the following.

In the case where the supported rate set of the receiving STA or STAs is not known, the transmitting STA shall transmit using a rate in the BSSBasicRateSet or BSSBasicMCS Set parameter.

TGn Editor: Change the two paragraphs in page 112, line 23 through 35 to the following.

9.6.0e.2 Rate selection for control frames that initialize a TXOP

If a control frame other than a BlockAckReq or BlockAck that initiates a TXOP is carried in a non-HT PPDU, the transmitting STA shall transmit the frame using one of the rates in the BSSBasicRate set parameter.

If a BlockAckReq or BlockAck that initiates a TXOP is carried in a non-HT PPDU, the transmitting STA may transmit the frame using a rate supported by the receiver STA, as reported in the Supported Rates element and/or Extended Supported Rates element in frames transmitted by that STA. In the case where the supported rate set of the receiving STA or STAs is not known, the transmitting STA shall transmit using a rate from the BSSBasicRateSet parameter.

TGn Editor: Change subclause 9.6.0e.3s in page 112, line 43 to the following.

9.6.0e.3 Rate selection for CF_End control frames

If not operating during the 40MHz phase of PCO, then CF_End control frame shall be sent at any rate in BSSBasicRateSet if the BSSBasicRateSet.

If operating during the 40MHz phase of PCO, then the CF_End control frame shall be sent at any rate within the BSSBasicMCSSet parameter if the BSSBasicMCSSet parameter.

TGn Editor: Change sentence in page 113, line 5 to the following.

If a control frame that is a BlockAckReq or BlockAck and that is not a control response frame is not the first frame transmitted within a TXOP and the control frame is carried in a non-HT PPDU, the transmitting STA shall transmit the control frame using a rate supported by the receiver STA, as reported in the Supported Rates element and/or Extended Supported Rates element in frames transmitted by that STA. In the case where the supported rate set of the receiving STA or STAs is not known, the transmitting STA shall transmit using a rate from the BSSBasicRateSet parameter.

TGn Editor: Change sentence in page 113, line 14 to the following.

If a control frame that is not a control response frame is not the first frame transmitted within a TXOP, is not the frame that terminates a TXOP (CF_End), and is carried in an HT PPDU, the transmitting STA shall transmit the frame using an MCS supported by the receiver STA, as reported in the Supported MCS field in the HT capabilities element in management frames transmitted by that STA. In the case where the supported rate set of the receiving STA or STAs is not known, the transmitting STA shall transmit using an MCS in the BSSBasicMCSSet parameter.

# CID 5461

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5461 | 109.65 | 9.6.0b | When a frame is to be sent in a MCS from a BasicMCS, the HT/non-HT PPDU selection is already concluded. Therefore forcing it to be sent in BasicRate when BasicMCS is empty will contradict with the PPDU selection rules defined in subclause 9.6.0e.6. Therefore change "rate" to "MCS" and "BSSBasicRateSet" to "Mandatory MCS Set" | As suggested. | Accept. See also CID 5533 and 5300. |

# CID 5301

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5301 | 110.07 | 9.6.0c | Should have the word "term" in this sentence for clarity when reading the next sentence. | Add "term" as appropriate to allow an explicit connection between the nouns of the sentences in the two paragraphs of this subclause. | Counter – add the word “term” after the first “The” in 110.36 in D3.02 |

# CID 5610

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5610 | 110.15 | 9.6.0d.1 | I think the rules for Beacon frames are mutually contradictory: we say "send using a basic rate or mandatory phy rate" in one place and "When a STA has the MIB attribute dot11TxSTBCOptionEnabled set to true, it shall use the basic STBC MCS when it transmits an STBC Beacon frame" in another. Also 9.6.0d.1 is complicated by the exclusions for Beacon/PSMP. | Create a new subclause "Rate selection for non-STBC Beacon and PSMP frames" and move the bits specific to non-STBC Beacon/PSMP there. Create a new subclause for "Rate selection for STBC group addressed data and management frames" and put the STBC rules there. Then in 9.6.0d.1 change title to "Rate selection for other group addressed data and management frames" and call out that it excludes frames mentioned in the new subclauses. | Accept |

# CID 5611

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5611 | 110.40 | 9.6.0d.1 | "If the Basic STBC MCS is not defined (see 9.6.0c (Basic STBC MCS)), any MCS within the Mandatory MCS set for the attached PHY shall be used to transmit these frames." The "these" is not clear. Is it any frame mentioned in this subclause, or only STBC beacon frames? If the latter, it makes no sense and there is no need to say what to do when the Basic STBC MCS is not defined, because according to 9.6.0b, it will be defined. | Remove the cited sentence. | Counter – the cited sentence is removed. See resolution for CID 5300 |

# CID 5799

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5799 | 110.40 | 9.6.0d.1 | If the Basic STBC MCS is not defined (see 9.6.0c (Basic STBC MCS)), any MCS within the Mandatory MCS set for the attached PHY shall be used to transmit these frames | This sentence is not clear and if it is related to the paragraph above it is not true. Make it clear | Counter – the cited sentence is removed. See resolution for CID 5300 |

# CID 5612

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5612 | 110.41 | 9.6.0d.1 | "NOTE--If a data or management frame other than a Beacon, with a group address in the Address 1 field is sent in a non-HT PPDU, the TXVECTOR parameter CH_BANDWIDTH is set to either NON_HT_CBW20 or NON_HT_CBW40 for that frame." I really don't see what value this note adds | Remove cited note. | Reject – the note is necessary because it clarifies the channel bandwidth allowed for these frames. |

# CID 5613

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5613 | 110.45 | 9.6.0d.1 | "NOTE--If a data or management frame with a group address in the Address 1 field is sent in an HT PPDU, the TXVECTOR parameter CH_BANDWIDTH is set to either HT_CBW20 or HT_CBW40." I don't see what value this note adds. | Remove cited note. | Reject – the note is necessary because it clarifies the channel bandwidth allowed for these frames. |

# CID 5800

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5800 | 111.10 | 9.6.0d.4 | "A STA shall not transmit a frame using a rate or MCS that is not supported by the receiver STA or STAs, as reported in any Supported Rates element, Extended Supported Rates element or Supported MCS field and in the management frames transmitted by the receiver STA." The mentioned elements and filed can be sent only in the management frames so no need for "and" | A STA shall not transmit a frame using a rate or MCS that is not supported by the receiver STA or STAs, as reported in any Supported Rates element, Extended Supported Rates element or Supported MCS field in the management frames transmitted by the receiver STA. | Accept - change the quoted sentence as suggested in the proposed change column. |

# CID 5614

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5614 | 111.19 | 9.6.0d.4 | "In the case where the supported rate set of the receiving STA or STAs is not known, the transmitting STA shall transmit using a rate in the BSSBasicRateSet parameter, or using a mandatory rate or MCS when the BSSBasicRateSet and BSSBasicMCSSet are not available." The relevance of the BSSBasicMCSSet is not clear, because it is not used in this normative statement. | Replace with: "In the case where the supported rate set of the receiving STA or STAs is not known, the transmitting STA shall transmit using a rate in the BSSBasicRateSet parameter or using an MCS from the BSSBasicMCSSet or using a mandatory rate or MCS when the BSSBasicRateSet and BSSBasicMCSSet are not available." | Counter - the cited sentence is clarified in resolution to CID 5300. |

# CID 5462

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5462 | 111.49 | 9.6.0e.2 | In the "before 11n" specification, BAR is sent at any rate supported by the receiver and the BA is sent at the same rate as the BAR. We found this to be a problem because we can not assume symmetrical tx/rx capability. Understanding that it makes non HT devices non-conformant, we have decided to limit the BA to be sent at basic rate or MCS. Following the same logic, although it may make non-HT device non-conformant, because a TXOP initiating control frame should be heard by all members of the BSS to provide protection, suggest to remove the exception for BAR. | Remove the exception of BAR and BA in line 49 of page 111. Remove lines 54 through line 60 in the same page. Also remove the exception of BA and BA in line 16 of page 112. Remove lines 28 through line35 in the same page. | Accept |

Need Discussion

# CID 5615

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5615 | 111.59 | 9.6.0e.2 | "the mandatory rate set" - there is no such beast | say "using a mandatory rate of the attached PHY" Check for other uses of "the mandatory rate set" and make similar changes. Same comment/change for "mandatory MCS set". | Accept – although the quoted sentence is removed, make the suggested changes in throughout the text. |

# CID 5619

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5519 | 111.63 | 9.6.0e.2 | Is there a valid reason to have a "shall" when L-SIG TXOP Protection is not used and a "should" when it is used? | Use "shall" for both cases, in order to maximize the probability that Control frames can be understood by as much stations as possible. | Reject – in case when L-SIG TXOP is used, the control frame need not be heard by third party because L-SIG portion can virtually convey the duration information. |

# CID 5801

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5801 | 111.63 | 9.6.0e.2 | "When L-SIG TXOP protection is not used, an HT STA shall select an MCS from the BasicMCSset for HT PPDU control frames that initiate a TXOP" There is no need for such strong rule. TxOP initialization is not the same as TxOP protection. And if the intent to use the control frames is not to use it for protection but to use is for training for example there is no need to limit the control frame to BasicMCSset. | Replace the Basic MCS by "Supported MCS of the intended receiver" in this sentence and add sentence that if the HT PPDU control frame that initializes the TxOP is used for protection it shall be chosen from BasicMCSset | Reject – After long and extensive discussion, the TG have decided to mandate protection when control frames are used to initiate a TXOP to maximize interoperability, which is also in alignment with the baseline spec (e.g RTS/CTS). If training is needed with the initial handshake, implementer may choose to use a Data/ACK sequence which may not provide protection or may utilize L-SIG TXOP. |

# CID 5368

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5368 | 112.05 | 9.6.0e.3 | what is MandatoryRateSet? |  | Counter – see resolution in CID 5615 |

# CID 5368

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5024 | 112.08 | 9.6.0e.3 | "If operating during the 40MHz phase of PCO, then the CF_End control frame shall be sent at any rate within the BSSBasicMCSSet if the BSSBasicMCSSet is non-empty, and at one of rates in the MandatoryMCSSet if the BSSBasicMCSSet is empty." This is misleading because it looks as though if the BSSBasicMCSSet is not empty, you can select any rate from it. But if there is no MCS with the TXVECTOR parameter CH_BANDWIDTH set to HT_CBW40, then you cannot select from the BSSBasicMCSSet. The same thing applies to the MandatoryMCSSet. | Change it to "If operating during the 40 MHz phase of PCO, then the CF-End shall be sent at any rate within the BSSBasicMCSSet if there is any MCS in the BSSBasicMCSSet that meets the condition described in 11.16 (Phased Coexistence Operation), or else at one of the rates in the MandatoryMCSSet that meets the condition." | Counter – accept in principle. Add “or if none of the MCS meets the condition” after “BSSBasicMCSSet is empty” in subclause 9.6.0b. Add a similar text in 9.6.0a BSSBasicRateSet and 9.6.0c BSSSTBCBasicMCSSet. See also resolution to CID 5300. |

# CID 5369

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5369 | 112.10 | 9.6.0e.3 | what is MandatoryMCSSet? |  | Counter – see resolution in CID 5615 |

# CID 5520

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5520 | 112.13 | 9.6.0e.4 | I guess the real issue is not whether the Control frame initializes the TXOP or is in the middle, the issue is always (i.e. regardless of the position within the TXOP) to maximize the probability that a Control frame (that is not a control response) can be understood by as much stations as possible and that the NAV is properly set everywhere in the BSS in order to minimize collisions. Always using a rate from the BSSBasicRateSet would significantly simplify the protocol with no losses in terms of performance (there is no significant gain in terms of transmission time in using HT format and modulation for so short frames) and higher robustness. | Remove the text in clause 9.6.0e.4 and use a common rule for all Control frames that are not Control response frames as suggested in the comment. | Reject – although the intent of the commenter is understood, the TG feels that there is difference between whether of not the control frame is a TXOP initiating frame. If the frame initiates a TXOP it needs to play an important role of protecting against third party, where if it is not, it only needs to re-enforce the protection provided by the initiating frame. Therefore, we feel that there is a need for different set of rules. |

# CID 5521

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5521 | 112.47 | 9.6.0e.5 | Same comment as CID 1885 of LB97. Too many rules/combinations. It is difficult to understand which Rate/MCS can be used for Control response frames during any frame exchange. The submission 07/2442r2 accepted during LB97 didn't solve the problem. Control frames are usually so short that allowing too many MCS/rates for their transmissions is useless since it doesn't add any benefit in terms of performance but it just increases the complexity of the protocol: limiting the choice to fewer mandatory rates/MCSs would solve the problem. | Suggest to define a BSSBasicControlMCSset including the MCSs that all 11n devices must support. Rates/MCSs for protection mechanisms and Control response frames should be chosen from this BSSBasicControlMCSset in case only 11n devices are associated to the BSS, from the BSSBasicRateSet in case there are legacy stations associated. As usual in the standard, in the case of control responses, the rule should be to transmit at the highest rate/MCS in the BSSBasicRateSet /BSSBasicControlMCSset parameter that is less than or equal to the rate/MCS of the previous frame. | Reject – the intent of the commentor is reflected by requiring control frames be sent in basic rate or MCS, under the conditions described by the commenter. The exceptions to this rule described in 9.6.0e.5.1 allows for TGn enhancements such as rate adaptation, STBC, and TxBF. |

# CID 5019

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5019 | 112.52 | 9.6.0e.5.1 | Here, the selection rules of a rate or MCS for a control response frame are described. However, it is described under the assumption that the HT PPDU/non-HT PPDU selection is already done for the control response frame, where the HT PPDU/non-HT PPDU selection is described in 9.6.0e.6. | Add a reference to see 9.6.0e.6 at the beginning. | Reject – the quoted subclause is adequately referenced in 9.6.0e.1 General rules for rate selection for control frames. |

# CID 5802

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5802 | 113.01 | 9.6.0e.5.1 | "The modulation class of the control response frame shall be selected according to the following rules:" There is not rule for modulation class of HT-PPDU control response | Add c) If the Control Response frame is carried in HT-PPDU the modulation class shall be HT PHY | Accept |

# CID 5463

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5463 | 113.34 | 9.6.0e.5.2 | Channel selection rules should be common to HT/non-HT PPDU, so this sentence should be moved up one subclause. | Move the sentence starting in line 34 of page 113 to line 12 of page 113. | Accept - Move the sentence starting in line 15 of page 114 to line 57 of page 113 in D3.02. |

# CID 5303

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5303 | 114.01 | 9.6.0e.5.2 | I believe that the very last item c) in this subclause should be either a step 4) for the last set of steps, or should somehow be adjusted to reference only item b) and not item a) - maybe a subclause split is in order. I.e. note that it refers to step 3), but there are at least two steps 3) in the subclause. | Clarify that the very last item c) in the subclause 9.6.0e.5.2 Control response frame MCS computation actually only applies to the last three numbered steps shown, and not to the numbered set of three steps shown above the last three | Accept – change the labelling of last step “c)” to “4)” |

# CID 5878

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5878 | 114.01 | 9.6.0e.5.2 | "If there is no MCS that meets the condition in step 3), then repeat step 3) including the set of MCS with the NSS value that is one less than the NSS value found in step 2) from the CandidateMCSSet and continue likewise until step 3) produces a non-empty result." This, as written, may not converge. The problem is that you want to decrement the NSS value each time through the loop, and yet this sentence doesn't do that. | Fix the algorithm description. | Reject – by including MCS with lower Nss values into the selection choices, the responder can eventually find a MCS that meet the condition in b) 1) through b) 3). See also resolution to CID 5303. |

# CID 5370

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5370 | 114.06 | 9.6.0e.5.3 | This alternative rate section adds more computation/complexity for rate selection. I can see there are some benefits for using the alternative rates, but if so why have this section in the standard with a very weak "may be used". | either change all "may" to "shall" or at least "should" in this section or just simply delete this section. | Reject – the section existed with a weak “may” from the baseline spec. The TG feels there is not a strong need for the section to change. |

# CID 5878

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5370 | 114.06 | 9.6.0e.5.3 | This alternative rate section adds more computation/complexity for rate selection. I can see there are some benefits for using the alternative rates, but if so why have this section in the standard with a very weak "may be used". | either change all "may" to "shall" or at least "should" in this section or just simply delete this section. | Reject – the section existed with a weak “may” from the baseline spec. The TG feels there is not a strong need for the section to change. |

# CID 5464

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5464 | 114.59 | 9.6.0e.6 | There is ambiguity what to do when both NDP and staggered sounding formats are supported by the responder. | Add statement in an appropriate place that when both NDP and staggered sounding formats are supported at both rx and tx, staggered shall be used. This would result in a change in line 59 of page 114, deleting the mention of NDP announcement field. |  |

Need Discussion

# CID 5803

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5803 | 115.17 | 9.6.0e.7 | "the responding STA generates a response with the TXVECTOR parameter CH_BANDWIDTH set to either NON_HT_CBW20 or HT_CBW20" The terminology is not consistent with definitions of the table 9-2 Modulation classes | Change either direction to make it consistent | Reject – the terminology used here (NON_HT_CBW20 and HT_CBW20) are valid terms found in Table 20-1 |

# CID 5826

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5826 | 115.20 | 9.6.0e.7 | The phrase "...rules in 9.6.0e.6…" is a poor reference. I only find one rule in 9.6.0e.6, and multiple exceptions. If "rules" in this phrase refers to all of 9.6.0.0e.6, then it should reference the entire clause. If it is referring to the rule with some subset of the exceptions, then it should specify which exceptions apply. If I am wrong and there is more than one rule in 9.6.0e.6, then this phrase should be clarified to "...all rules..." or "the following rules..." or "...any rule...". | Please clarify the "…rules in 9.6.0e.6…" reference. | Counter – accept in principle change “rules” to “the rule” found in page 116, line 6. |

# CID 5007

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5007 | 117.35 | 9.6.2 | Not all the cases are covered in Table 9-2a. When MCS is 34, 35, 41-45, or 56-64, the modulation of stream 1 is 64-QAM, R=1/2. | Add a row for 64-QAM, R-1/2 in Table 9-2a. How about 48 Mbps for the non-HT reference rate for it? | Counter – see resolution for CID 5465 |

# CID 5465

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5465 | 117.36 | 9.6.2 | Add a row for 64-QAM code rate = 1/2 as this combination is allowed for unequal modulation. | Add; 64-QAM, 1/2, 36(Mb/s) After line 34 page 117. | Accept |

References:

Draft P802.11n_D3.02.pdf