IEEE P802.11
Wireless LANs

| Proposed Resolutions to LB115 L-SIG TXOP MISC Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-01-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Yuichi Morioka | Sony Corporation | 5-1-12 Kitashinagawa Shinagawa-ku, Tokyo 141-0001, Japan | +81-3-5448-4017 | Yuichi.Morioka@jp.sony.com |

Abstract

This submission proposes resolutions to CIDs 5470 and 5630 related to L-SIG TXOP.

The changes proposed in this document are based on 802.11n D3.02.

# CID 5470

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5470 | 139.38 | 9.13.5.1 | "L-SIG TXOP Protection may be used even when not all HT STA in the BSS support the feature, provided that the frames using L-SIG TXOP Protection are not directed to a recipient that does not support L-SIG TXOP Protection." This overlaps with what is written in the following paragraph. When the Full Support field indicates that not all HTS STAs support the feature, TXOP initiating frames should be sent at BasicMCS. | Change the quoted text to; "When the L-SIG TXOP Protection Full Support field indicates that not all HT STAs support the feature, L-SIG TXOP may still be used but BasicMCS should be used for TXOP initiating frames to ensure protection against non supporting HT STAs. " | Counter – accept in principle, see doc0105r0 for details. |

# CID 5630

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 5630 | 139.39 | 9.13.5.1 | "provided that the frames using L-SIG TXOP Protection are not directed to a recipient that does not support L-SIG TXOP Protection." There is redundancy with the start of the following para. | We probably need the ", provided" dependency. So remove the following sentence: "A STA shall not transmit a frame using L-SIG TXOP Protection directed to a recipient that does not support L-SIG TXOP Protection." | Counter – accept in principle, see doc0105r0 for details |

# Proposed Resolution

TGn Editor: Change the two paragraphs starting on line 43 in page141 of D3.02 to the following;

A STA shall not transmit a frame using L-SIG TXOP Protection directed to a recipient that does not support L-SIG TXOP Protection.

A STA that transmits an L-SIG TXOP Protected frame that initiates a TXOP in an IBSS or when the L-SIG TXOP Protection Full Support field is set to zero by an AP, should use BasicMCS for the transmission of that frame.

References:

Draft P802.11n_D3.02.pdf