IEEE P802.11
Wireless LANs

| WAVE BSS Related Issues in 802.11p D 3.03 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-02-22 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| John Kenney | VSC-2 | 4009 Miranda Avenue, Palo Alto, CA, 94304 | 574-272-1403 | john@jbkenney.com |
| Daniel Jiang | VSC-2 | 850 Hansen Way, Palo Alto, CA, 94304 | 650-845-2548 | daniel.jiang@daimler.com |
| Vinuth Rai | VSC-2 | 4009 Miranda Avenue, Palo Alto, CA, 94304 | 650-251-0530 | vinuth@us.toyota-itc.com |

Abstract

In this submission the authors propose a revision to the existing definition of WAVE BSS. The authors also propose editing clause 11.18 so as to better explain WAVE mode management

- WAVE BSS Definition: Please note revised WAVE BSS definition

  - 3.168a WAVE basic service set (WAVE BSS):  a type of BSS consisting of a set of one or more cooperating stations in WAVE mode that communicate using a common BSSID.  A WAVE BSS is initialized when a STA in WAVE mode sends a WAVE beacon.

- Clause 11.18 deals with WAVE mode management. We believe that the clause should serve as an introduction to the following sections which describe how to initiate a WAVE BSS, how to join a WAVE BSS and how to use the TSF Timer in WAVE mode. Here is the revised version of clause 11.18. We also propose changes to 11.18.2 and 11.18.2. Finally we propose adding a new sub clause 11.18.2a which talks aobut leaving and terminating a WAVE BSS. Only the edited text is listed below. The unedited sections are not included and should remain the same.

11.18 WAVE mode Management

A STA in WAVE mode may communicate as part of a WAVE BSS or outside of the context of a BSS. This clause provides the rules for operations associated with a WAVE BSS and maintaining a TSF timer in WAVE mode. A STA shall not be a member of more than one WAVE BSS at one time. A STA in WAVE mode shall not join an infrastructure BSS or IBSS, and it shall not use active or passive scanning, and it shall not use MAC sublayer authentication or association procedures.

11.18.1 Initializing a WAVE BSS

A STA in WAVE mode initializes a WAVE BSS by sending a first WAVE beacon, including the BSSID (described in 7.1.3.3.3). A STA may send subsequent WAVE beacons to continue to advertise the presence of the WAVE BSS.

11.18.2 Joining a WAVE BSS

2. If the STA elects to join the WAVE BSS, the SME issues an MLME-

JOIN.request (see 10.x.y.1) to the MLME. The MLME issues an MLME-

JOIN.confirm (see 10..x.y.2) to the SME to convey the results of the set request.

Insert the following subclause after 11.18.2

11.18.2a Leaving a WAVE BSS and Terminating a WAVE BSS

A STA ceases to belong to a WAVE BSS when its MAC sublayer entity stops sending and receiving frames that use the BSSID of the WAVE BSS, e.g. when it joins another WAVE BSS or leaves WAVE mode.  A WAVE BSS ceases to exist when it has no members.  A WAVE BSS can continue to exist if the initiating STA ceases to be a member.

- Motion (if technical and/or significant):

Move to accept the proposed text from above and instruct the TGp editor to incorporate the proposed changes in text noted above

Motion by: Vinuth Rai

Date:

Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |