IEEE P802.11
Wireless LANs

| Normative Text for Proxy Frame Format |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-03-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Yongho SEOK | LG Electronics | 16 Woomyeon-Dong, Seocho-Gu, Seoul 137-724, Korea | +8225264225 | yhseok@lge.com |
| Alexander Safonov | IITP RAS | 19 Bolshoy Karetny per, Moscow, 127994, Russia | +74956503617 | safa@iitp.ru |
| Guenael Strutt | Motorola | 1064 Greenwood Blvd – Lake Mary, FL 32771 -- USA | +1-407-562-4050 | Guenael.Strutt@motorola.com |

Abstract

The Proxy Update IE is transmitted by a source MP to a destination MP for updating its proxy information. And, the Proxy Update Confirmation IE is transmitted by a destination MP to a source MP in response to the Proxy Update.

But, current TGs draft is missing those frame formats for delivering a Proxy Update IE and Proxy Update Confirmation IE.

This document is a normative text proposal to update two missing frame formats for Proxy Update IE and Proxy Update Confirmation IE. For not distributing proxy information within the mesh network, the proposed proxy frame formats uses a Multihop Action frames.

Changes are shown with respect to TGs draft D1.09.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Change 7.3.2.99 as as shown:

7.3.2.99 Proxy Update (PU) information element

The Proxy Update information element is transmitted by a source MP to a destination MP to update its proxy information. This frame element is transmitted using individual addresses. The frame format of PU element is shown in Figure s37.

Change 7.3.2.100 as as shown:

7.3.2.100 Proxy Update Confirmation (PUC) information element

The Proxy Update Confirmation element is transmitted by an MP in response to a PU. This frame element is transmitted using individual addresses. The frame format of PUC element is shown in Figure s38.

Change 7.3.1.11 as as shown:

7.3.1.11 Action field

Insert the following rows into Table 7-24 - Category values and change the last row (Reserved) as shown.

| Table 7-24 - Category values |  |  |
| --- | --- | --- |
| Value | Meaning | See subclause |
|  | Mesh Peer Link Management | 7.4.12 |
|  | Mesh Link Metric | 7.4.13 |
|  | Mesh Path Selection | 7.4.14 |
|  | Mesh Interworking | 7.4.15 |
|  | Mesh Resource Coordination | 7.4.16 |
|  | Mesh Security Architecture (MSA) | 7.4b.1 |
|  | Mesh Proxy Forwarding | 7.4b.2 |
| 97-126 | Reserved | --- |

Insert the following after 7.4b.2 as shown:

7.4b.2 Mesh Proxy Forwarding action details

Two Multihop Action frame formats are defined for forwarding proxy information to a destination MP. An Action Value field, in the octet field immediately after the Category field, differentiates the formats. The Action Value field values associated with each frame formats are defined in Table sXX.

| Table sXX – Mesh Proxy Forwarding Action field values |  |
| --- | --- |
| Action Field Value | Description |
| 0 | Proxy Update |
| 1 | Proxy Update Confirmation |
| 2-255 | Reserved |

7.4b.2.1 Proxy Update frame format

The Proxy Update frame uses the Multihop Action frame body format and is transmitted using individual addresses. The format of the Proxy Update frame body is shown in Table sYY.

| Table sYY – Proxy Update frame body format |  |
| --- | --- |
| Order | Information |
| 1 | Mesh Header |
| 2 | Category |
| 3 | Action Value |
| 4 | Proxy Update elements (see 7.3.2.72) |

The Category field is one octet and is set to the value in Table 7-24 for category Mesh Proxy Forwarding.

The Action Value field is one octet and is set to 1 (representing a Proxy Update frame).

The Proxy Update element is described in 7.3.2.99.

7.4b.2.2 Proxy Update Confirmation frame format

The Proxy Update Confirmation frame uses the Multihop Action frame body format and is transmitted using individual addresses. The format of the Proxy Update Confirmation frame body is shown in Table sZZ.

| Table sZZ – Proxy Update Confirmation frame body format |  |
| --- | --- |
| Order | Information |
| 1 | Mesh Header |
| 2 | Category |
| 3 | Action Value |
| 4 | Proxy Update Confirmation element (see 7.3.2.73) |

The Category field is one octet and is set to the value in Table 7-24 for category Mesh Proxy Forwarding.

The Action Value field is one octet and is set to 1 (representing a Proxy Update Confirmation frame).

The Proxy Update Confirmation element is described in 7.3.2.100.

Change 11B.6.5.1.3 as as shown:

11B.6.5.1.3 PU processing

A PU element in a PU frame is individually addressed from an MP to a destination MP.  The destination MP shall update the proxy address in its proxy information with the list of proxied addresses reported in the PU.

Change 11B.6.5.2.3 as as shown:

11B.6.5.2.3 PUC processing

On receiving a PUC IE element in a PUC frame, the MP shall no longer send any PU with the same sequence number.

Add the following to end of table A.4.4.4:

| Item | Protocol capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| FR40 | Proxy Update Frame | 7.4b.2.1 | MWM1:M | Yes No N/A |
| FR41 | Proxy Update Confirmation Frame | 7.4b.2.2 | MWM1:M | Yes No N/A |