IEEE P802.11
Wireless LANs

| Access Point Collaboration |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Lusheng Ji | AT&T | AT&T Labs - Research 180 Park Avenue, Florham Park, NJ 07932, USA | +1 973 360 7241 | lji@research.att.com |
| Robert Miller | AT&T | AT&T Labs - Research 180 Park Avenue, Bldg. 103, Room B251 Florham Park, NJ 07932, USA | +1 973 236 6920 | rrm@research.att.com |
| Yongho Seok | LG Electronics | 16 Woomyeon-Dong, Seocho-Gu, Seoul 137-724, Korea | +8225264225 | yhseok@lge.com |
| Bill Marshall | AT&T | AT&T Labs - Research 180 Park Avenue, Florham Park, NJ 07932, USA | +1 973 360 8718 | wtm@research.att.com |
| David Hunter | Panasonic | 550 South Winchester, San Jose, CA 95128 | +1 805 259 8564 | hunter@timefactor.com |
| Roger Durand | RIM | 18 Williamsburg Dr, Amhurst, NH 08031 | +1 603-275-4101 | rdurand18@comcast.net |
| Joe Kwak | Interdigital | 482 Degas, Bollingbrook, IL 60440 | +1 630-739-4159 | joekwak@sbcglobal.net |

Abstract

This document describes a mechanism to allow multiple APs on the same channel to cooperatively share time on the wireless medium. This enables enhanced QoS by allowing collaborating APs to avoid allocating overlapping contention free periods and reducing back off periods due to collisions.

Updated for MIB table numbering in TGv D15.0.

Background:

The spectrum assignments in which IEEE 802.11 can be used are a finite resource that is becoming more congested as 802.11 becomes more popular. The use of this spectrum can be made more efficient by techniques such as intelligent channel selection, transmit power control and time division. This document describes a method to allow access points to collaborate by sharing time on the wireless medium.

The proposal is designed to fulfill requirement 2050 “Access Point Coordination” in the TGV objectives document [1].

This document is based upon TGv draft D14.0 and the IEEE P802.11-2007 baseline.

11.9 DFS procedures

Insert the following new clause after 11.9.7:

11.9.8 AP collaboration

11.9.8.1 Suppressing channel access for AP collaboration

An AP in a BSS may schedule suppressed intervals, which are time intervals during which channel accesses by the member STAs of the BSS are suppressed.  During a suppressed interval scheduled by the AP of a BSS, all member STAs of the BSS yield their channel access attempts to channel access attempts by member STAs of BSSes that are not suppressed.  These suppressed intervals can be used to provide time-period based collaboration between APs operating on the same channel.

The AP that is suppressing its BSS is referred to as a “suppressed AP” and the AP that is able to use this period of suppressed channel access in other BSSes is referred to as the “recipient AP”. Correspondingly the BSS of a suppressed AP is referred to as a “suppressed BSS” and the BSS of a recipient AP is referred to as a “recipient BSS”.  There can be multiple suppressed APs and multiple recipient APs as there may be more than two BSS operating on the same channel within radio range.

Co-channel BSSes can collaborate by scheduling concurrent suppressed intervals in certain BSSes to provide channel access precedence to other BSSes.  Figure v98 illustrates how this can be done.  In this example, BSS#1 has a suppressed interval scheduled within its Beacon Interval; during this suppressed interval channel accesses by member STAs of BSS#1 are suppressed, thus effectively member STAs of BSS#2 receive precedence in their rights to access the channel. Likewise BSS#2 has a suppressed inhterval scheduled within its Beacon Interval; during this suppressed interval channel access by member STAs of BSS#2 are suppressed, thus effectively member STAs of BSS#1 receive precedence in their rights to access the channel.

Figure 11-17a—Example timing of silence and grant periods

11.9.8.2 AP collaboration using suppressed intervals

Time-period based collaboration between APs operating on the same channel can be controlled by appropriate configuration of the dot11APCEntryBeaconOffset, dot11APCEntryGrantPeriodOffset, dot11APCEntryGrantPeriodLength, dot11APCEntrySuppressedPeriodOffset and dot11APCEntrySuppressedPeriodLength MIB variables.

The MIB variable dot11APCEntryBeaconOffset establishes a reference time for scheduling suppressed intervals by specifying the offset (in TUs) from the TBTT of the BSS.

The MIB variables dot11APCEntrySuppressedPeriodOffset and dot11APCEntrySuppressedPeriodLength specifies the offset and duration, both in TUs, of a suppressed time window relative to the reference time.  The period of suppressed medium access starts at reference AP’s TBTT + dot11APCEntryBeaconOffset + dot11APCEntrySuppressedPeriodOffset and ends at

TBTT + dot11APCEntryBeaconOffset + dot11APCEntrySuppressedPeriodOffset + dot11APCEntrySuppressedPeriodLength.

The MIB variables dot11APCEntryGrantPeriodOffset and dot11APCEntryGrantPeriodLength specifies the offset and duration, both in TUs, of a grant time window relative to the reference time.  The period of granted medium access starts at reference AP’s TBTT + dot11APCEntryBeaconOffset + dot11APCEntryGrantPeriodOffset and ends at

TBTT + dot11APCEntryBeaconOffset + dot11APCEntryGrantPeriodOffset + dot11APCEntryGrantPeriodLength.

An AP that (1) has dot11MgmtOptionAPCollaborationImplemented set TRUE, (2) has MIB variable dot11MgmtOptionAPCollaborationActivated set TRUE, (3) has a non-zero value of dot11APCEntrySuppressedPeriodLength, (4) is operating in a regulatory domain for which suppression is allowed (e.g. adequate support for Emergency Services in non-suppressed intervals), and (5) has no prior agreements with any associated STA for transmissions during this period shall schedule a suppressed interval, using the mechanism of Quiet elements described in 11.9.2.

The scheduling mechanism used by the AP to avoid future use of the suppressed intervals, and the renegotiation mechanism (if any) to reschedule current use of the suppressed intervals are beyond the scope of this standard.

Insert the following new rows into A.4.18:

### A.4.18 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RME13 | AP Collaboration | 11.9.8 | (CFv&SM7):O | Yes, No, N/A |

Annex D

Insert the following after dot11MgmtOptionUTCTSFOffsetActivated TruthValue:

dot11MgmtOptionAPCollaborationImplemented		TruthValue,

dot11MgmtOptionAPCollaborationActivated		TruthValue }

Insert the following after dot11MgmtOptionChannelUsageEnabled OBJECT-TYPE:

dot11MgmtOptionAPCollaborationImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This is a capability variable.

Its value is determined by device capabilities.

This attribute, when TRUE, indicates the capability of the AP to support AP collaboration is implemented.  The capability is not implemented otherwise.  The default value of this attribute is FALSE.”

::= { dot11WirelessMgmtOptionsEntry 46 }

dot11MgmtOptionAPCollaborationEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This is a control variable.

It is written by an external management entity or the SME.

Changes take effect as soon as practical in the implementation.

This attribute, when TRUE, indicates the capability of the AP to support AP collaboration is enabled.  The capability is not enabled otherwise.  The default value of this attribute is FALSE.”

::= { dot11WirelessMgmtOptionsEntry 47 }

-- ********************************************************************

-- * dot11APC TABLE

-- ********************************************************************

dot11APCTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11APCEntry

MAX-ACCESS READ-WRITE

STATUS current

DESCRIPTION

"Group contains conceptual table of attributes for MIB based APC."

::= { dot11smt 23 }

dot11APCEntry OBJECT-TYPE

SYNTAX Dot11APCEntry

MAX-ACCESS READ-WRITE

STATUS current

DESCRIPTION

"An entry in the dot11APCTable, Indexed by dot11APCIndex."

INDEX { dot11APCIndex }

::= { dot11APCTable 1 }

Dot11APCEntry ::=

SEQUENCE {

dot11APCEntryBeaconOffset			INTEGER,

dot11APCEntryGrantPeriodOffset			INTEGER,

dot11APCEntryGrantPeriodLength			INTEGER,

dot11APCEntrySuppressedPeriodOffset		INTEGER,

dot11APCEntrySuppressedPeriodLength		INTEGER}

dot11APCEntryBeaconOffset OBJECT-TYPE

SYNTAX INTEGER (-1..32767)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This is a control variable.

It is written by an external management entity or the SME.

Changes take effect as soon as practical in the implementation.

This attribute contains the offset, in TUs, from the common time reference until the start of the Beacon transmission. A value of -1 indicates no common time reference has been established for this AP.”

::= { dot11APCTableEntry 1  }

dot11APCEntryGrantPeriodOffset OBJECT-TYPE

SYNTAX INTEGER (0..131071)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This is a control variable.

It is written by an external management entity or the SME.

Changes take effect as soon as practical in the implementation.

This attribute contains the offset, in TUs, from the scheduled start of the Beacon transmission until the start of the time period granted exclusively for use by this AP.”

::= { dot11APCTableEntry 2 }

dot11APCEntryGrantPeriodLength OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This is a control variable.

It is written by an external management entity or the SME.

Changes take effect as soon as practical in the implementation.

This attribute contains the duration, in TUs, of the period granted exclusively for use by this AP. When zero, no period has been granted for exclusive use by this AP.”

::= { dot11APCTableEntry 3 }

dot11APCEntrySuppressedPeriodOffset OBJECT-TYPE

SYNTAX INTEGER (0..131071)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This is a control variable.

It is written by an external management entity or the SME.

Changes take effect as soon as practical in the implementation.

This attribute contains the offset, in TUs, from the scheduled start of the Beacon transmission until the start of the time period during which channel access attempts by member STAs of this BSS is to remain suppressed.”

::= { dot11APCTableEntry 4 }

dot11APCEntrySuppressedPeriodLength OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This is a control variable.

It is written by an external management entity or the SME.

Changes take effect as soon as practical in the implementation.

This attribute contains the duration, in TUs, of the suppressed period for this AP. When zero, no suppressed period has been set for this AP.”

::= { dot11APCTableEntry 5 }

-- ********************************************************************

-- * End of dot11APC TABLE

-- ******************************************************************** --

References:

[1]	11-05-0827-13-000v-tgv-objectives.doc