IEEE P802.11
Wireless LANs

Normative Text for D2.0 Location Services and Time Synchronization

Date: 2008-03-05

| Name | Company | Address | Phone | email |
| --- | --- | --- | --- | --- |
| Allan Thomson | Cisco Systems | 170 W. Tasman Drive, San Jose, CA95134 | +1-408-853-5570 | allant@cisco.com |
| Brian Hart | Cisco Systems | 170 W. Tasman Drive, San Jose, CA95134 | +1-408-525-3346 | brianh@cisco.com |
| Kevin Stanton | Intel Corp | 2111 NE 25th Ave, Hillsboro, OR | +1-503-712-1888 | Kevin.Stanton@intel.com |
| Emily Qi | Intel Corp | 2111 NE 25th Ave, Hillsboro, OR | +1-503-888-7644 | Emily.h.qi@intel.com |
| Ganesh Venkatesan | Intel Corp | 2111 NE 25th Ave, Hillsboro, OR | +1-503-334-6720 | Ganesh.Venkatesan@intel.com |

Abstract

This submission contains the normative text for changes to D2.0 Location Services that addresses ALL accepted and countered comments in spreadsheet 08/0440r0.

.

This proposal text is aligned with P802.11v-D2.0.

- Wireless LAN Network Management

- Overview

WLAN Network Management enables STAs to exchange information for the purpose of improving the overall performance of the wireless network. STAs use Wireless Network Management protocols to exchange operational data so that each STA is aware of the network conditions, allowing STAs to be more cognizant of the topology of the network. Wireless Network Management protocols provide a means for STAs to be aware of presence of co-located interference, and allow STAs to manage RF parameters based on network conditions.

In addition to providing information on network conditions, Wireless Network Management also provides a means to exchange location information, provide support for multiple BSSID's on the same wireless infrastructure, more efficient delivery of group addressed frames, and enable a sleep mode in which a STA can sleep for long periods of time without receiving buffered frames from the AP.

Wireless Network Management service includes:

- BSS Transition Management

- Co-located interference Reporting

- Diagnostic Reporting

- Event reporting

- Location Services

- Timing Measurement

- Maximum Multicast Rate processing

- Multicast Diagnostic reporting

- Multiple BSSIDs

- Proxy ARP

- Sleep Mode

- STA Diagnostics

- TIM Broadcast

- Traffic Filtering Service

- Traffic Generation

- Location Services

Location Configuration Request and Response frames enable STAs to configure a collection of location related parameters for Location Track Notification frames. The AP can indicate that it can provide E-911 Civic or GEO Location data for its location, to support emergency services applications.

      - Timing Measurement

Time Measurement request/report pair returns accurate time information for a frame received at the responding STA.

- Information Elements

Insert Element IDs <ANA> to <ANA> +18 into Table 26 and change the Reserved row accordingly:

| - Element IDs |  |  |
| --- | --- | --- |
| Information Element | Element ID | Length (in octets) |
| Event Request (see Error: Reference source not found) | <ANA> | 4 to 256 |
| Event Report (see Error: Reference source not found) | <ANA>+1 | 5 to 256 |
| Diagnostic Request (see Error: Reference source not found) | <ANA>+2 | 4 to 256 |
| Diagnostic Report (see Error: Reference source not found) | <ANA>+3 | 5 to 256 |
| Location Parameters (see Error: Reference source not found) | <ANA>+4 | 2 to 256 |
| Multiple BSSID (see Error: Reference source not found) | <ANA>+5 | 3 to 256 |
| SSID List (see Error: Reference source not found) | <ANA>+6 | 2 to 256 |
| Multiple BSSID-Index (see Error: Reference source not found) | <ANA>+7 | 3 to 5 |
| FBMS Descriptor (see Error: Reference source not found) | <ANA>+8 | 3 to 256 |
| FBMS Request (see Error: Reference source not found) | <ANA>+9 | 4 to 256 |
| FBMS Response (see Error: Reference source not found) | <ANA>+10 | 3 to128 |
| Traffic Generation (see Error: Reference source not found) | <ANA>+11 | 3-5 |
| BSS Max Idle Period (see Error: Reference source not found) | <ANA>+12 | 3 |
| TFS Request (see Error: Reference source not found) | <ANA>+13 | 5 to 256 |
| TFS Response (see Error: Reference source not found) | <ANA>+14 | 2 to 256 |
| Sleep Mode (see Error: Reference source not found) | <ANA>+15 | 5 |
| TIM Broadcast Request (see Error: Reference source not found) | <ANA>+16 | 3 |
| TIM Broadcast Response (see Error: Reference source not found) | <ANA>+17 | 10 |
| Reserved | <ANA>+18, 220 |  |

- Measurement Request element

Change Table 29 in 7.3.2.21 and split the "Measurement Use" column into four separate entries as follows:

| - Measurement Type definitions for measurement requests |  |  |
| --- | --- | --- |
| Name | Measurement Type | Measurement Use |
| Basic Request | 0 |  |
| Clear Channel Assessment (CCA) request | 1 | Spectrum Management |
| Receive power indication (RPI) histogram request | 2 |  |
| Channel load request | 3 |  |
| Noise histogram request | 4 | Radio Resource Measurement |
| Beacon request | 5 |  |
| Frame request | 6 |  |
| STA statistics request | 7 | Radio Resource Measurement and Wireless Network Management |
| LCI request | 8 | Radio Resource Measurement |
| Transmit stream/category measurement request | 9 | Radio Resource Measurement and Wireless Network Management |
| Multicast diagnostics request | 10 | Wireless Network Management |
| Location Civic request | 11 | Radio Resource Measurement |
| Location Identifier request | 12 | Radio Resource Measurement |
| Reserved | 1310-254 | N/A |
| Measurement Pause request | 255 | Radio Resource Measurement and Wireless Network Management |

        - Location Civic Request

The Measurement Request field corresponding to a Location Civic Request is shown in Figure vXXX. The response to a Location Civic Request is a Location Civic Report.

|  | Location Subject | Location Service Interval Units | Location Service Interval |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |

Figure vXXX: Location Civic Request Format

The Location Subject field is a single octet and is defined in Table 7-29I.

The Location Service Interval Units field contains the units for the Location Service Interval field, as indicated in Table v24.

| - Location Service Interval Units |  |
| --- | --- |
| Location Service Interval Units Value | Description |
| 0 | Seconds |
| 1 | Minutes |
| 2 | Hours |
| 3-255 | Reserved |

The Location Service Interval field is the time interval, expressed in the units indicated in the Location Service Interval Units field, at which the STA requests to receive Location Civic Reports. A Location Service Interval of 0 indicates that only a single Location Civic Report is requested.

        - Location Identifier Request

The Measurement Request field corresponding to a Location Identifier Request is shown in Figure vXXX. The response to a Location Identifier Request is a Location Identifier Report.

|  | Location Subject | Location Service Interval Units | Location Service Interval |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |

Figure vXXX: Location Identifier Request

The Location Subject field is a single octet and is defined in Table 7-29I.

The Location Service Interval Units field is defined in Table v24.

The Location Service Interval field is the time interval, expressed in the units indicated in the Location Service Interval Units field, at which the STA requests to receive Location Identifier Reports. A Location Service Interval of 0 indicates that only a single Location Identifier Report is requested.

- Measurement Report element

Change Table 30 in 7.3.2.22 and split the "Measurement Use" column into four separate entries as follows:

| - Measurement Type definitions for measurement reports |  |  |
| --- | --- | --- |
| Name | Measurement Type | Measurement Use |
| Basic report | 0 |  |
| Clear Channel Assessment (CCA) report | 1 | Spectrum Management |
| Receive power indication (RPI) histogram report | 2 |  |
| Channel load report | 3 |  |
| Noise histogram report | 4 | Radio Resource Measurement |
| Beacon report | 5 |  |
| Frame report | 6 |  |
| STA statistics report | 7 | Radio Resource Measurement and Wireless Network Management |
| LCI report | 8 | Radio Resource Measurement |
| Transmit stream/category measurement report | 9 | Radio Resource Measurement and Wireless Network Management |
| Multicast diagnostics report | 10 | Wireless Network Management |
| Location Civic report | 11 | Radio Resource Measurement |
| Location Identifier report | 12 | Radio Resource Measurement |
| Reserved | 1310-255 | N/A |

        - Location Civic Report

The Location Civic Report includes the location information defined in CIVIC format for the location subject provided in the Location Civic measurement request.

|  | Location Accuracy X-estimate | Location Accuracy Y-estimate | Location Accuracy Z-estimate | Civic Location |
| --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 2 | variable |

The Location Accuracy X-estimate field is an estimated accuracy in the X-dimension in 0.1 meter increments, defined by a little endian 16 bit unsigned integer. For example, an accuracy estimate of +/- 5 meters is represented by the number X'32'. If the location accuracy estimate in the X-dimension is unknown the field is set to 65535.

The Location Accuracy Y-estimate field is an estimated accuracy in the Y-dimension in 0.1 meter increments, defined by a little endian 16 bit unsigned integer. For example, an accuracy estimate of +/- 5 meters is represented by the number X'32'. If the location accuracy estimate in the Y-dimension is unknown the field is set to 65535.

The Location Accuracy Z-estimate field is an estimated accuracy in the Z-dimension in 0.1 meter increments, defined by a little endian 16 bit unsigned integer. For example, an accuracy estimate of +/- 5 meters is represented by the number X'32'. If the location accuracy estimate in the Z-dimension is unknown the field is set to 65535.

The Civic Location field is a variable octet field and the format of the data is defined in RFC 4776.

        - Location Identifier Report

The Location Identifier Report includes an indirect reference to where the location information for the location subject provided in the Location Identifier measurement request can be retrieved.

|  | Public Identifier Link |
| --- | --- |
| Octets: | variable |

The Public Identifier Link field is a string value, provided to comply with RFC 3693. The Public Identifier Link confirms the validity of the location estimate to an external agent when a STA forwards a location estimate to that agent. The protocol used to query the infrastructure for a location report based on the Public Identifier Link is beyond the scope of this standard.

- Extended Capabilities information element

Change the last paragraph of 7.3.2.27 as follows:

The Capabilities field is a bit field indicating the capabilities being advertised by the STA transmitting the information element. One octet of extended information has been defined. The format of that octet appears in Figure 7-76a (Capabilities fieldfirst octet). The length of the Capabilities field is a variable n. The Capabilities field is shown in Capabilities field

Delete Figure 7-67a, and insert the following table at the end of 7.3.2.27 as follows:

| - Capabilities field |  |  |
| --- | --- | --- |
| Bit | Information | Notes |
| <ANA> | Event | When dot11MgmtOptionEventsEnabled is set to 1, the Event bit is set to 1 to indicate the STA supports Event processing as described in 11.20.3. When dot11MgmtOptionEventsEnabled is set to 0, the Event bit is set to 0 to indicate that the STA does not support this capability. |
| <ANA>+1 | Diagnostics | When dot11MgmtOptionDiagnosticsEnabled is set to 1, the Diagnostics bit is set to 1 to indicate the STA supports Diagnostics as described in 11.20.4. When dot11MgmtOptionDiagnosticsEnabled is set to 0, the Diagnostics bit is set to 0 to indicate that the STA does not support this capability. |
| <ANA>+2 | Multicast Diagnostics | When dot11MgmtOptionMulticastDiagnosticsEnabled is set to 1, the Multicast Diagnostics bit is set to 1 to indicate the STA supports Multicast diagnostics as described in 11.20.2. When dot11MgmtOptionMulticastDiagnosticsEnabled is set to 0, the Multicast Diagnostics bit is set to 0 to indicate that the STA does not support this capability. |
| <ANA>+3 | Location | When dot11MgmtOptionLocationEnabled is set to 1, the Location field is set to 1 to indicate that the STA supports Location as described in 11.20.5. When dot11MgmtOptionLocationEnabled is set to 0, the Location field is set to 0 to indicate that the STA does not support this capability. |
| <ANA>+4 | FBMS | When dot11MgmtOptionFBMSEnabled is set to 1, the FBMS bit is set to 1 to indicate the STA supports FBMS as described in 11.2.1.4a and 11.20.8. When dot11MgmtOptionFBMSEnabled is set to 0, the FBMS bit is set to 0 to indicate the STA does not support FBMS. |
| <ANA>+5 | Proxy ARP Service | When dot11MgmtOptionProxyARPEnabled is set to 1, the Proxy ARP Service bit is set to 1 to indicate the AP is providing proxy ARP service as described in 11.2.1.4b. If Proxy ARP service is enabled, then the AP responds to broadcast ARP request on behalf of the STA. When dot11MgmtOptionProxyARPEnabled is set to 0, the Proxy ARP Service bit is set to 0 to indicate the AP is not providing proxy ARP service for any associated STA. The Proxy ARP Service bit is set to zero by a non-AP STA. |
| <ANA>+6 | Co-located Interference Reporting | When dot11MgmtOptionCLIReportingEnabled is set to 1, the Co-located Interference Reporting bit is set to 1 to indicate the STA supports Co-located Interference Reporting as described in 11.20.10. When dot11MgmtOptionCLIReportingEnabled is set to 0, the Co-located Interference Reporting bit is set to 0 to indicate that the STA does not support this capability. |
| <ANA>+7 | E911 CIVIC Location | The E911 CIVIC Location field is set to 0 and reserved when the Location field is set to 0. When dot11MgmtOptionE911CIVICLocationEnabled is set to 1, the E911 CIVIC Location field is set to 1 to indicate that the STA has its location defined in CIVIC format, and provides it upon request via the Location Civic Request described in 7.3.2.21.12. When dot11MgmtOptionE911CIVICLocationEnabled is set to 0, the E911 CIVIC Location field is set to 0 to indicate that the STA does not have its location defined in CIVIC format or is not capable of providing CIVIC format location upon requestLocation Descriptor sub-element |
| <ANA>+8 | E911 Geo Location | The E911 Geo Location field is set to 0 and reserved when the Location field is set to 0. When dot11MgmtOptionE911GeoLocationEnabled is set to 1, the E911 Geo Location field is set to 1 to indicate that the STA has its location defined in Geo format and provides it upon request via the LCI Request described in 7.3.2.21.9. When dot11MgmtOptionE911GeoLocationEnabled is set to 0, the E911 Geo Location field is set to 0 to indicate the STA does not have its location defined in Geo format or is not capable of providing Geo format location upon requestLocation Descriptor sub-element |
| <ANA>+9 | TFS | When dot11MgmtOptionTFSEnabled is set to 1, the TFS bit is set to 1 to indicate that the STA supports TFS as described in 11.20.11. When dot11MgmtOptionTFSEnabled is set to 0, the TFS bit is set to 0 to indicate that the STA does not support TFS. |
| <ANA>+10 | Sleep Mode | When dot11MgmtOptionSleepModeImplemented is set to 1, the Sleep Mode bit is set to 1 to indicate that the STA supports sleep mode as described in 11.20.12. When dot11MgmtOptionSleepModeImplemented is set to 0, the Sleep Mode bit is set to 0 to indicate that the STA does not support sleep mode. |
| <ANA>+11 | TIM Broadcast | When dot11MgmtOptionTIMBroadcastEnabled is set to 1, the TIM Broadcast bit is set to 1 to indicate that the STA supports TIM Broadcast as described in 11.2.1.11a. When dot11MgmtOptionTIMBroadcastEnabled is set to 0, the TIM Broadcast bit is set to 0 to indicate that the STA does not support TIM Broadcast. |
| <ANA>+12 | BSS Transition | When dot11MgmtOptionBSSTransitionEnabled is set to 1, the BSS Transition bit is set to 1 to indicate that the STA supports BSS Transition Management as described in 11.20.8. When dot11MgmtOptionBSSTransitionEnabled is set to 0, the BSS Transition bit is set to 0 to indicates that the STA does not support BSS Transition Management. |
| <ANA>+13 | Traffic Generation | When dot11MgmtOptionTrafficGenerationEnabled is set to 1, the Traffic Generation bit is set to 1 to indicate the STA supports Traffic Generation as described in 11.20.11. When dot11MgmtOptionTrafficGenerationEnabled is set to 0, the Traffic Generation bit is set to 0 to indicate that the STA does not support this capability. |
| <ANA>+14 | AC Station Count | When dot11MgmtOptionACStationCountEnabled is set to1, the AC Station Count bit is set to 1 to indicate the STA supports AC Station Count as described in 11.20.12. When dot11MgmtOptionACStationCountEnabled is set to 0, the AC Station Count bit is set to 0 to indicate that the STA does not support this capability. |
| <ANA>+15 | Time Measurement | When dot11MgmtOptionTimingMsmtEnabled is set to 1, the Time Sync field is set to 1 to indicate the STA supports Time Measurement as described in XXXX. When dot11MgmtOptionTimingMsmtEnabled is set to 0, the Time Measurement field is set to 0 to indicate that the STA does not support this capability |
| <ANA>+16-n*8 | Reserved | All other bits are reserved, and are set to 0 on transmission and ignored on reception. |

- Location Parameters element

- Location Parameters definition

The Location Parameters information element is used for location services. The format of this information element is shown in Location Parameters Information element format.

|  | Element ID | Length | Location Sub-elements |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| - Location Parameters Information element format |  |  |  |

The Element ID field is equal to the Location Parameters value in Table 7-26.

The value of the Length field is variable and depends on the length of the Location Sub-elements field.

The Location Sub-elements field contains one or more Location sub-elements described in Location Sub-elements.

| - Location Sub-elements |  |  |
| --- | --- | --- |
| Identifier | Sub-element Name | Sub-element Length (in octets) |
| 1 | Location Indication Parameters (see Error: Reference source not found) | 10 |
| 2 | Location Indication Channels (see Location Indication Channels sub-element) | 4 to 254 |
|  | Location Request Options sub-element |  |
| 3 | Location Status (see7.3.2.66.4 Location Status sub-element [correct ref]) | 4 |
|  | Location Service Parameters sub-element |  |
| 4 | Radio Information (seeRadio Information sub-element [correct ref]) | 7 |
|  | Timing Measurements sub-element |  |
| 5 | Motion (seeMotion sub-element [correct ref]) | 5 |
|  | Location Descriptor sub-element |  |
|  | Error: Reference source not found |  |
|  | Error: Reference source not found |  |
| 6 | Location Indication Broadcast Data Rate (seeLocation Indication Broadcast Data Rate sub-element [correct ref]) | 4 |
| 7 | Time Of Departure (see [add ref]) | 7 |
| 8-220 | Reserved |  |
| 221 | Vendor Specific (see [correct ref]) | 5 to 254 |
| 222-255 | Reserved |  |

The Location Parameters element is included in Location ation Configuration Request frames, as described in Location Configuration Request frame format , Location Configuration Response frames, as described in Location Configuration Response frame format, Location Track Notification frames, as described in 7.4.XX.. The use of the Location Parameters element and frames is described in 11.20.5.

- Location Indication Parameters sub-element

The Location Indication Parameters sub-element contains STA Location reporting characteristics. The format of the Location Indication Parameters sub-element is shown in Location Indication Parameters sub-element.

|  | Sub-element ID | Length | Indication Multicast Address | Report Interval Units |  | Normal Report Interval |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 1 |  | 2 |  |  |  |
|  | Normal Number of Frames per Channel | In-Motion Report Interval | In-Motion Number of Frames per Channel | Burst Inter-frame Interval |  |  |  |  |  |
| Octets: | 1 | 2 | 1 | 1 |  |  |  |  |  |
| - Location Indication Parameters sub-element |  |  |  |  |  |  |  |  |  |

The Sub-element ID field contains the value for Location Indication Parameters as defined in Location Sub-elements.

The Length field is set to 16.

The Indication Multicast Address field specifies the destination address the Location Track Notification frames are sent to in non-IBSS network. The field is reserved when Location Track Notifications are transmitted in an IBSS.

The Report Interval Units field contains the units used for the Normal Report Interval field and In-Motion Report Interval field, as indicated in Report Interval Units Field.

| - Report Interval Units Field |  |
| --- | --- |
| Report Interval Units | Description |
| 0 | Milliseconds |
| 1 | Seconds |
| 2 | Minutes |
| 3 | Hours |
| 4 - 255 | Reserved |

The Normal Report Interval is the time interval, expressed in the units indicated in the Report Interval Units field at which the STA is expected to transmit one or more Location Track Notification frames. The STA will not transmit Location Track Notifications when the Normal Report Interval is set to 0.

The Normal Number of Frames per Channel is the number of Location Track Notification frames per channel sent or expected to be sent by the STA at each Normal Report Interval.

The In-Motion Report Interval is the time interval, expressed in the units indicated in the Report Interval Units field at which the STA reports its location by sending a Location Track Notification frame when the STA is in motion. If motion detection is not supported, this field is set to 0. The definition of motion and the means to determine motion are outside the scope of this standard.

The In-Motion Number of Frames per Channel is the number of Location Track Notification frames per channel sent or expected to be sent by the STA at each In-Motion Report Interval. If dot11MgmtOptionMotionDetectionEnabled is false, this field is set to 0.

The Burst Inter-frame Interval is the target time interval, expressed in milliseconds between the transmissions of each of the Normal or In-Motion frames on the same channel. The Burst Inter-frame interval value is set to 0 to indicate that frames will be transmitted with no target inter-frame delay.

- Location Indication Channels sub-element

The Location Indication Channels sub-element contains location reporting channel information. The format of the Location Indication Channels sub-element format is shown in Location Reporting Channels sub-element.

|  | Sub-element ID | Length | Regulatory Class 1 | Channel 1 | …Regulatory Class n | Channel n |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 |
| - Location Reporting Channels sub-element |  |  |  |  |  |  |

The Sub-element ID field contains the value for Location Indication Channels as defined in Location Sub-elements.

The Length field is set to twice the number of channels.

The Regulatory Class 1 through Regulatory Class n fields each indicates the frequency band on which a STA transmits Location Track Notification frames. Valid values of the Regulatory Class field are defined in Annex J.

The Channel 1 through Channel n fields each includes the channel numbers on which a STA sends or an ESS expects to receive Location Track Notification frames. Valid values of the Channel field are defined in Annex J.

Location Request Options sub-element.

7.3.2.66.4 Location Status sub-element

The Location Status sub-element provides the result of a Location Configuration Request frame. The format of the Location Status sub-element is shown in Location Status sub-element.

|  | Sub-element ID | Length | Config Sub-Element ID | Status |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 |  |  |  |
| - Location Status sub-element |  |  |  |  |  |  |  |

The Sub-element ID field contains the value for Location Status as defined in Location Sub-elements.

The Length field is set to 2.

The Config Sub-Element ID field is set to a specific Location Parameters sub-element ID transmitted in a Location Configuration Request frame as defined in Table v19. If the Status field value applies to more than one sub-elements then the Config Sub-Element ID is set to 0.

The Status field identifies the result of the Location Request frame and is one of the values in Error: Reference source not found.

Location Service Parameters sub-element .

7.3.2.66.5

Radio Information sub-element

The Radio Information sub-element contains radio information. The format of the Radio Information sub-element is shown in Radio Information sub-element.

|  | Sub-element ID | Length | Transmit Power | Antenna ID | Antenna Gain | RSNI | RCPI |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| - Radio Information sub-element |  |  |  |  |  |  |  |

The Sub-element ID field contains the value for Radio Information as defined in Location Sub-elements.

The Length field is set to 5.

The Transmit Power field is set to the transmit power used to transmit the current Location Track Notification frame containing the Location Parameters element with the Radio Information sub-element and is a signed integer, one octet in length, reported in dBm. A value of -128 indicates that the transmit power is unknown. The maximum tolerance for the transmit power value reported in the Radio Information sub-element is ± 5 dB. This tolerance is defined as the difference, in decibels, between the reported power value and the total transmitted power across all antennas of the STA which are measured when transmitting Location Request frames.

The Antenna ID field is the identifying number for the antenna used to transmit the Location Request frame. The Antenna ID is defined in 7.3.2.40.

- Antenna ID is defined in 7.3.2.40 of TGk D9.0.

The Antenna Gain field is the antenna gain of the antenna (or group of antennas) over which the Location Track Notification frame is transmitted and is a signed integer, one octet in length reported in dB. A value of -128 indicates that the antenna gain is unknown.

The RSNI field contains the RSNI value (dB) measured against the most recently received Location Configuration Request frame requesting that a Radio Information sub-element be included in the Location Track Notification frame. The RSNI value is defined in 7.3.2.41. A value of 255 indicates that the RSNI value is unknown or is not used.

The RCPI field contains the RCPI value (dBm) measured against the most recently received Location Configuration Request frame requesting that a Radio Information sub-element be included in the Location Track Notification frame. The RCPI value is defined 7.3.2.39. A value of 255 indicates that the RCPI value is unknown or is not used.

Timing Measurements sub-element.

        - Motion sub-element

The Motion sub-element contains motion information. The format of the Motion sub-element is shown in Motion sub-element.

|  | Sub-element ID | Length | Motion Indicator | Bearing | Speed Units | Horizontal Speed | Vertical Speed |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2 | 1 | 2 | 2 |  |  |  |  |  |  |  |
| - Motion sub-element |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The Sub-element ID field contains the value for Motion as defined in Location Sub-elements.

The Length field is set to 8.

The Motion Indicator field is defined Error: Reference source not foundin Table v28. The mechanism that a STA uses to determine the value of the Motion Indicator field is beyond the scope of the standard.

| Table v30 – Motion Indicator Field |  |
| --- | --- |
| Motion Indicator Value | Description |
| 0 | Stationary |
| 1 | Start of motion |
| 2 | In motion |
| 3 | End of motion |
| 4 | Unknown |
| 5-255 | Reserved |

The Bearing field, defined by a 2 octet unsigned integer, specifies the direction that the STA is traveling with relation to true north, increasing clockwise, measured in degrees from 0 degree to 359 degrees. If the Bearing value is unknown, the field is set to 65535.

The Speed Units field contains the units for both Horizontal and Vertical Speed field, as defined in Speed Units.

| - Speed Units |  |
| --- | --- |
| Speed Units Value | Description |
| 0 | centimeters per second |
| 1 | meters per second |
| 2-255 | Reserved |

The Horizontal Speed field contains the horizontal speed of the STA expressed in the units indicated in the Speed Units field. If the Horizontal Speed value is unknown, the field is set to 65535.

The Vertical Speed field contains the vertical speed of the STA expressed in the units indicated in the Speed Units field. If the Vertical Speed value is unknown, the field is set to 65535.

Location Descriptor sub-element .

Location Data sub-element.

Location Source Identifier sub-element.

        - Location Indication Broadcast Data Rate sub-element

The Location Indication Broadcast Data Rate sub-element contains location reporting transmission rate information. The format of the Location Indication Broadcast Data Rate sub-element format is shown in Location Indication Broadcast Data Rate sub-element.

|  | Sub-element ID | Length | Broadcast Target Data Rate |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |
| - Location Indication Broadcast Data Rate sub-element |  |  |  |

The Sub-element ID field contains the value for Location Indication Broadcast Data Rate as defined in Location Sub-elements.

The value of the Length field is set to 2.

The Broadcast Target Data Rate field specifies the target data rate, in 0.5Mb/s units, at which the STA transmits broadcast Location Track Notification frames.

        - Time Of Departure sub-element

The Time Of Departure sub-element contains time of departure information for the Location Track Notification frame including the sub-element. The format of the Time Of Departure sub-element format is shown in Figure vXXXXX.

|  | Sub-element ID | Length | TOD Timestamp | TOD Tolerance |  | TOD Units |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 4 | 2 |  | 1 |  |  |  |  |  |
| - Time Of Departiure sub-element |  |  |  |  |  |  |  | Figure vXXXX – Time Of Departure sub-element format |  |  |  |

The Sub-element ID field contains the value for Time Of Departure as defined in Location Sub-elements.

The value of the Length field is set to 7.

The TOD Timestamp field specifies the value of the TOD timestamp counter at a constant interval before the frame carrying the TOD timestamp is transmitted in units specified by the TOD Units field.

The TOD Tolerance field specifies 95% tolerance of the TOD Timestamp field value.

The TOD Units field specifies the units for the TOD Timestamp field and may be one of the following values defined in Table XXX below:

| - TOD Units |  |
| --- | --- |
| TOD Units Value | Description |
| 0 | TODU 22: 1 / 1408 MHz |
| 1 | TODU 20: 1 / 1280 MHz |
| 2 | TODU40: 1 / 2560 MHz |
| 3-255 | Reserved |

octets.

- Action frame format details

7.4.7 Public Action frame details

7.4.7.1 Public Action frames

Insert the following row to Table 7-57e:

| Table 7-57e – Public Action field values |  |
| --- | --- |
| Action field value | Description |
| 9 | Location Track Notification |
| 10 | Timing Measurement |
| 11-255 | Reserved |

Insert the following section after 7.4.7.9:

      - Location Track Notification frame format

The Location Track Notification frame uses the Action frame body format and is transmitted by a STA to allow remote location determination to occur by another STA. The format of the Location Track Notification frame body is shown in Figure vXX.

|  | Category | Action | Location Parameters Element |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |  |  |  |  |
|  |  |  |  |  |  | Figure vXX: Location Track Notification frame format |  |

The Category field is set to the value for Public Action defined Table 7-24.

The Action field is set to the value indicating Location Track Notification, as specified in  Table 7-57e.

The Location Parameters Element field contains the Location Parameters sub-elements, described in Error: Reference source not found. Table vXX defines the allowed Location Parameters sub-elements for a Location Parameters element that is included in the frame.

| Table vXX - Location Parameters Element field for Location Track Notification frame |  |  |
| --- | --- | --- |
| Allowed Sub-elements | Sub-element ID | Notes |
| Radio Information | 6 | The Radio Information sub-element is included in the Location Track Notification frame. |
| Motion | 8 | The Motion sub-element is included in the Location Track Notification frame if dot11MgmtOptionMotionEnabled is true. |
| Time Of Departure | 7 | The Time Of Departure sub-element is included in the Location Track Notification frame and dot11MgmtOptionTODEnabled is true. |
| Vendor Specific | 221 | The Vendor Specific sub-element may be included in the Location Track Notification frame. |

7.4.7.11 Timing Measurement frame format

The Timing Measurement frame uses the Action frame body format and is used to exchange timing measurement related data between two STAs. The format of the frame is shown in Figure vXXX.

|  | Category | Action |  | Dialog Token |  | Follow On to Dialog Token |  | Timestamp Difference |  | Frame Use Mode/Timestamp Difference Units |  | Timestamp Difference Accuracy |  | Timestamp |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 |  | 1 |  | 1 |  | 1 |  | 4 |  | 1 |  | 1 |  | 10 |

Figure vXX: Timing Measurement public action frame format

The Category field is set to the value indicating Public Action category as specified in Table 7-24.

The Action field is set to the value indicating Timing Measurement as specified in Table 7-57e.

The Dialog Token field is set to a non-zero value chosen by the STA sending the request to identify the transaction.

The Follow On to Dialog Token field is set to the value of the Dialog Token used in a Timing Measurement frame that to which the current frame is a follow on. This field is set to zero if the current frame is not a follow on.

The Timestamp Difference field contains the time difference between the time that an individually addressed Timing Measurement public action frame was received from a STA, defined to occur at the PHY-RXSTART.indication of the received Timing Measurement public action frame, and the time that the corresponding ACK frame was sent to the STA, defined to occur at the PHY-TXSTART.confirm of the ACK frame transmission.

The Frame Use Mode/Timestamp Difference Units field is divided into two sub-fields.

- Bit 0 indicates Frame Use Mode identifying the role played by the STA that is transmitting the action frame as described in Table v25a and

- Bits 1 through 7 indicate the units for the timestamp difference field, as indicated in Timestamp Difference Units.

| Table v25a Bit0 of Frame Use Mode/ Timestamp Difference Units field |  |
| --- | --- |
| Frame Use Mode | Description |
| 0 | The Timing Measurement is - an Initial (Follow On to Dialog Token is zero) or - a Follow On frame (the frame is transmitted by the STA that transmitted the Initial frame. |
| 1 | The Timing Measurement public action frame is a Response frame. This means that the frame is transmitted by the peer STA to which the corresponding Initial Timing Measurement public action frame was sent. |
| - Bits 1-7 Frame Use Mode/Timestamp Difference Units |  |
| Timestamp Difference Units | Description |
| 0 | Microseconds |
| 1 | Hundreds of Nanoseconds |
| 2 | Tens of Nanoseconds |
| 3 | Nanoseconds |
| 4 – 126 | Reserved |
| 127 | TimestampDifference Unit unspecified |

The Timestamp Difference Accuracy field contains the standard deviation of the value in the timestamp difference field in the units indicated in the Timestamp Difference Units field.

The Timestamp Difference and TimeStamp Difference Accuracy fields are ignored if the Timestamp Difference Units is set to 255.

The Timestamp field contains the time at which an individually addressed Timiming Measurement public action frame was transmitted by a STA, defined to occur at the PHY-TXSTART.confirm of the Timing Measurement public action frame. The Timestamp field is composed of the time fields defined in Ingress Timestamp Field..

| - Timestamp Field |  |  |
| --- | --- | --- |
| Octets | Type | Description |
| 6 | Unsigned Integer | Seconds |
| 4 | Unsigned Integer | Nanoseconds |

The Timestamp field is captured from a monotonically increasing free-running counter and is provided to higher layers in the STA which can compute its offset to the time of another STA.

Insert the following after 7.4.10:

- 802.11-2007 ends with 7.4.5. 11k adds 6. 11r adds 7. 11y adds none 11n adds 9 (8). 11w adds none. 11p adds 8 (9). 11s adds 6 (10).

- Wireless Network Management action details

Several Action frame formats are defined for Wireless Network Management purposes. An Action field, in the octet field immediately after the Category field, differentiates the formats. The Action field values associated with each frame format are defined in Wireless Network Management Action field values.

| - Wireless Network Management Action field values |  |
| --- | --- |
| Action field value | Description |
| 0 | Event Request |
| 1 | Event Report |
| 2 | Diagnostic Request |
| 3 | Diagnostic Report |
| 4 | Location Configuration Request |
| 5 | Location Configuration Response |
| 6 | BSS Transition Management Query |
| 7 | BSS Transition Management Request |
| 8 | BSS Transition Management Response |
| 9 | FBMS Request |
| 10 | FBMS Response |
| 11 | Co-located Interference Request |
| 12 | Co-located Interference Response |
| 13 | TFS Request |
| 14 | TFS Response |
| 15 | TFS Notify |
| 16 | Sleep Mode Request |
| 17 | Sleep Mode Response |
| 18 | TIM Broadcast Request |
| 19 | TIM Broadcast Response |
| 20 | TIM |
| 21 | Traffic Generation Update |
| 22-255 | Reserved |

Location Request frame body format.

Error: Reference source not found.

- Location Configuration Request frame format

The Location Configuration Request frame uses the Action frame body format and is transmitted by a STA to configure another STA to send a Location Track Notification frame on a set of channels periodically for the purposes of determining location of the STA. The format of the Location Configuration Request frame body is shown in Location Configuration Request frame body format.

|  | Category | Action | Dialog Token | Location Parameters Element |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |
| - Location Configuration Request frame body format |  |  |  |  |

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Location Configuration Request, as specified in Wireless Network Management Action field values in Error: Reference source not found.

The Dialog Token field is set to a nonzero value that is unique among the Location Configuration Request frames sent to each destination MAC address for which a corresponding Location Configuration Response frame has not been received.

The Location Parameters Element field contains the location parameters sub-elements, described in Error: Reference source not found. Location Parameters Element field for Location Configuration Request frame defines the allowed Location Parameters sub-elements for a Location Parameters element that is included in the Location Configuration Request frame.

| - Location Parameters Element field for Location Configuration Request frame |  |  |
| --- | --- | --- |
| Allowed Sub-element Fields | Sub-element ID | Notes |
| Location Indication Parameters | 1 | The Location Reporting Parameters sub-element is included in the Location Configuration Request frame. |
| Location Indication Channels | 2 | The Location Reporting Channels sub-element is included in the Location Configuration Request frame. |
| Location Indication Broadcast Data Rate | 6 | The Location Indication Broadcast Data Rate sub-element is included in the Location Configuration Request frame. |
| Vendor Specific Information | 221 | The Vendor Specific Information sub-element may be included in the Location Configuration Request frame. |

- Location Configuration Response frame format

The Location Configuration Response frame uses the Action frame body format and is transmitted by a STA in response to the receipt of a Location Configuration Request frame. The format of the Location Configuration Response frame body is shown in Location Configuration Response frame body format.

|  | Category | Action | Dialog Token | Location Parameters Element |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |
| - Location Configuration Response frame body format |  |  |  |  |

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Location Configuration Response, as specified in Wireless Network Management Action field values in Error: Reference source not found.

The Dialog Token field is set to the nonzero value received in the Location Configuration Request frame to identify the request/response transaction.

The Location Parameters Element field contains the location parameters sub-elements, described in Error: Reference source not found. Location Parameters Element field for Location Configuration Response frame defines the allowed Location Parameters sub-elements for a Location Parameters element that is included in the Location Configuration Response frame.

| - Location Parameters Element field for Location Configuration Response frame |  |  |
| --- | --- | --- |
| Allowed Sub-element Fields | Sub-element ID | Notes |
| Location Indication Parameters | 1 | The Location Reporting Parameters sub-element may be included in the Location Configuration Response frame. |
| Location Indication Channels | 2 | The Location Reporting Channels sub-element may be included in the Location Configuration Response frame. |
| Location Indication Broadcast Data Rate | 6 | The Location Indication Broadcast Data Rate sub-element may be included in the Location Configuration Response frame. |
| Location Status | 3 | The Location Status sub-element is included in the Location Configuration Response frame. If all configuration of the sub-elements contained in a Location Configuration Request frame was successful then a single Location Status sub-element is included in the Location Configuration Response frame. For each sub-element contained in the Location Configuration Request frame that is not successful a Location Status sub-element is included in the Location Configuration Response frame that indicates the sub-element ID and the unsuccessful status code for that sub-element ID. |
| Vendor Specific Information | 221 | The Vendor Specific Information sub-element may be included in the Location Configuration Response frame. |

- Location Configuration request

This set of primitives supports the exchange of location configuration parameter information between peer SMEs. Location Configuration Request and Response Protocol Exchange depicts the location configuration request and response procese, and is not meant to be exhaustive of all possible protocol uses.

- Location Configuration Request and Response Protocol Exchange

- MLME-LOCATIONCFG.request

- Function

This primitive requests the transmission of location configuration request action frame to a peer entity.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-LOCATIONCFG.request	(
Peer MAC Address,
Dialog Token,
Location Parameters)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the location configuration request shall be sent. |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the location transaction. |
| Location Parameters | Location element | Location element | A location element containing one or more sub-elements describing the STA location information. See 7.3.2.66. |

- When generated

This primitive is generated by the SME to request that a Location Configuration Request frame be sent to a peer entity to convey location information.

- Effect of receipt

On receipt of this primitive, the MLME constructs a Location Configuration Request frame containing the set of location elements specified. This frame is then scheduled for transmission.

- MLME-LOCATIONCFG.confirm

- Function

This primitive reports the result of a location configuration request.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-LOCATIONCFG.confirm	(
Dialog Token,
ResultCode,
Peer MAC Address,
Location Parameters)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the location transaction. |
| ResultCode | Enumeration | SUCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a Location Configuration Request frame. |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the location configuration response shall be sent. |
| Location Parameters | Location element | Location element | A location element containing one or more sub-elements describing the STA location information. See 7.3.2.66. |

- When generated

This primitive is generated by the MLME as a result of the receipt of a Location Configuration Response action management frame. This primitive is also generated when the MLME-LOCATIONCFG.request contains invalid parameters and when a timeout or failure occurs.

- Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

- MLME-LOCATIONCFG.indication

- Function

This primitive indicates that a Location Configuration Request frame has been received requesting a location transaction.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-LOCATIONCFG.indication	(
Peer MAC Address,
Dialog Token,
Location Parameters)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the location configuration request shall be sent. |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the location transaction. |
| Location Parameters | Location element | Location element | A location element containing one or more sub-elements describing the STA location information. See 7.3.2.66. |

- When generated

This primitive is generated by the MLME when a valid Location Configuration Request frame is received.

- Effect of receipt

On receipt of this primitive, the SME either rejects the request or commences the location transaction.

- MLME-LOCATIONCFG.response

- Function

This primitive requests the transmission of location information to a peer entity, in response to a received Location Configuration Request frame.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-LOCATIONCFG.response (
Peer MAC Address,
Dialog Token,
Location Parameters)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the location request shall be sent. |
| Dialog Token | Integer | 1 – 255 | The dialog token to identify the location transaction. |
| Location Parameters | Location element | Location element | A location element containing one or more sub-elements describing the STA location information. See 7.3.2.66. |

- When generated

This primitive is generated by the SME to request that a Location Configuration Response frame be sent to a peer entity to convey location information.

- Effect of receipt

On receipt of this primitive, the MLME constructs a Location Configuration Response frame containing the set of location elements specified. This frame is then scheduled for transmission.

    - Location Track Notification

This set of primitives supports the location track notification from one SME to one or more receiving SMEs. Location Configuration Request and Response Protocol Exchange depicts the location track notification process, is not meant to be exhaustive of all possible protocol uses.

- Location Track Notification Protocol Exchange

- MLME-LOCATIONTRACKNOTIF.request

- Function

This primitive requests the transmission of location configuration request action frame to a peer entity.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-LOCATIONTRACKNOTIF.request	(
Peer MAC Address,
Location Parameters)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual or group addressed MAC Address | The address of the peer MAC entity to which the location track notification shall be sent. |
| Location Parameters | Location Parameters element | Location Parameters element | A location parameters element containing one or more sub-elements describing the STA location information. See 7.3.2.66. |

- When generated

This primitive is generated by the SME to request that a Location Track Notification frame be sent to a peer entity to help convey location information.

- Effect of receipt

On receipt of this primitive, the MLME constructs a Location Track Notification frame containing the set of location elements specified. This frame is then scheduled for transmission.

- MLME-LOCATIONTRACKNOTIF.indication

- Function

This primitive indicates that a Location Track Notification frame has been received.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-LOCATIONTRACKNOTIF.indication	(
Peer MAC Address,
Location Parameters)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual or group addressed MAC Address | The address of the peer MAC entity to which the location track notification shall be sent. |
| Location Parameters | Location Parameters element | Location Parameters element | A location element containing one or more sub-elements describing the STA location information. See 7.3.2.66. |

- When generated

This primitive is generated by the MLME when a valid Location Track Notification frame is received.

- Effect of receipt

On receipt of this primitive, the SME uses the information contained within the notification.

    - Timing Measurement

The following set of primitives supports exchange of timing measurement information from one SME to another. Location Configuration Request and Response Protocol Exchange depicts the time sync information exchange. Multiple exchange of time sync information may be required in order for a STA to synchronize itself with respect to another STA.

Figure v107 -- Time Sync Information Exchange

10.3.54.1 MLME-TIMINGMSMT.request

10.3.54.1.1 Function

This primitive requests the transmission of Timing Measurement public action frame to a peer entity.

10.3.54.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-TIMINGMSMT.request	(
Peer MAC Address,

Dialog Token,

Follow On to Dialog Token,

Timestamp Difference,

Timestamp Difference Units,

Timestamp Difference Accuracy,

Timestamp)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual or group addressed MAC Address | The address of the peer MAC entity to which the Timimg Measurement public action frame shall be sent. |
| Dialog Token | Integer | 1-255 | The dialog token to identify the Timing Measurement transaction. |
| Follow On to Dialog Token | Interger | 0-255 | The dialog token of a Timing Measurement public action frame to which the current frame is a follow on. See 11.20.5.5. |
| Timestamp Difference | Location Parameters element | Location Parameters element | See 7.4.7.11. |
| Timestamp Difference Units | As defined in Timing Measurement public action frame | As defined in Timing Measurement public action frame | See 7.4.7.11. |
| Timestamp Difference Accuracy | As defined in Timing Measurement public action frame | As defined in Timing Measurement public action frame | See 7.4.7.11. |
| Timestamp | As defined in Timing Measurement public action frame | As defined in Timing Measurement public action frame | See 7.4.7.11. |

10.3.54.1.2 When generated

This primitive is generated by the SME to request that a Timing Measurement public action frame be sent to a peer entity.

10.3.54.1.3 Effect of receipt

On receipt of this primitive, the MLME constructs a Timing Measurement public action frame containing the time measurement parameters specified. This frame is then scheduled for transmission.

10.3.54.2 MLME-TIMINGMSMT.confirm

10.3.54.2.1 Function

This primitive indicates that a Timing Measurement public action frame has been successfully received by the peer STA to which it was sent.

10.3.54.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-TIMINGMSMT.confirm	(

Peer MAC Address,

Dialog Token,

Timestamp,

Ingress Timestamp)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual or group addressed MAC Address | The address of the peer MAC entity to which acknowledges the receipt of the Timing Measurement public action frame |
| Dialog Token | Integer | 1-255 | The dialog token to identify the Timing Measurement transaction. |
| Timestamp | As defined in Timing Measurement public action frame | As defined in Timing Measurement public action frame | See 7.3.2.21.14 for format of Timestamp. The time at which the Timing Measurement public action frame was sent. This corresponds to PHY-TXSTART.confirm. |
| Ingress TimeStamp | As defined in Timing Measurement public action frame | As defined in Timing Measurement public action frame | See 7.3.2.21.14 for format of Timestamp. The Ingress Timestamp contains the time at which the ACK for Timing Measurement public action frame was received by the STA, defined to occur at the PHY-RXSTART.Indication of the received frame. (t4 in Figure v107) |

10.3.54.2.3 When generated

This primitive is generated by the MLME when an ACK corresponding to the Timing Measurement public action frame is received from the peer STA.

10.3.54.2.4 Effect of receipt

On receipt of this primitive, the SME uses the information contained within the notification.

10.3.54.3 MLME-TIMINGMSMT.indication

10.3.54.3.1 Function

This primitive indicates that a Timing Measurement public action frame has been received.

10.3.54.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-TIMINGMSMT.indication	(

Peer MAC Address,

Dialog Token,

Follow On to Dialog Token,

Timestamp Difference,

Timestamp Difference Units,

Timestamp Difference Accuracy,

Timestamp,

Ingress Timestamp)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual or group addressed MAC Address | The address of the peer MAC entity to which the Timing Measurement public action frame shall be sent. |
| Dialog Token | Integer | 1-255 | The dialog token to identify the Timing Measurement transaction. |
| Follow On to Dialog Token | Interger | 0-255 | The dialog token of a Timing Measurement public action frame to which the current frame is a follow on. See 11.20.5.5. |
| Timestamp Difference | Location Parameters element | Location Parameters element | See 7.3.2.21.14. |
| Timestamp Difference Units | As defined in Timing Measurement public action frame | As defined in Timing Measurement public action frame | See 7.3.2.21.14. |
| Timestamp Difference Accuracy | As defined in Timing Measurement public action frame | As defined in Timing Measurement public action frame | See 7.3.2.21.14. |
| Timestamp | As defined in Timing Measurement public action frame | As defined in Timing Measurement public action frame | See 7.3.2.21.14. |
| Ingress TimeStamp | As defined in Timing Measurement public action frame | As defined in Timing Measurement public action frame | See 7.3.2.21.14 for format of TimeStamp. The Ingress Timestamp contains the time at which the Timing Measurement public action frame was received by the STA, defined to occur at the PHY-RXSTART.Indication of the received frame. (t2 in Figure-v107) |

10.3.54.3.3 When generated

This primitive is generated by the MLME when a valid Timing Measurement public action frame is received.

10.3.54.3.4 Effect of receipt

On receipt of this primitive, the SME uses the information contained within the notification.

10.3.54.4 MLME-TIMINGMSMTACK.confirm

10.3.54.4.1 Function

This primitive indicates that an ACK has been sent to the peer STA acknowledging the receipt of a Timing Measurement public action frame from the peer STA.

10.3.54.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-TIMINGMSMTACK.confirm	(

Peer MAC Address,

Dialog Token,

Timestamp)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual or group addressed MAC Address | The address of the peer MAC entity to which the ACK was sent |
| Dialog Token | Integer | 1-255 | The dialog token to identify the Timing Measurement transaction. |
| Timestamp | As defined in Timing Measurement public action frame | As defined in Timing Measurement public action frame | See 7.3.2.21.14 for format of Timestamp. The time at which the ACK to the Timing Measurement public action frame was sent to the peer. This corresponds to PHY-TXSTART.confirm for the ACK frame. (t3 in Figure v107) |

10.3.54.2.3 When generated

This primitive is generated by the MLME when an ACK corresponding to the Timing Measurement public action frame is sent to  the peer STA.

10.3.54.2.4 Effect of receipt

On receipt of this primitive, the SME uses the information contained within the notification.

Insert the following sentence at the end of 11.10.8.6

11.10.8.6 Location Configuration Information Report

If dot11RRMLCIMeasurementEnabled is true and an AP has its own location configured in LCI format it shall set the E911 Geo bit to 1 in the Extended Capabilities element.

Insert the following paragraphs

11.10.8.9 Location Civic Report

If dot11RRMCivicMeasurementEnabled is true, a STA shall reject any Location Civic Request for location information which is not available and shall respond with a Measurement Report frame including a Measurement Report element with the incapable bit set to 1. If dot11RRMCivicMeasurementEnabled is true and a STA accepts a Location Civic Request it shall respond with a Measurement Report frame including one Location Civic Report element.

A Location Civic Request shall indicate a location request for the requesting STA or the reporting STA by setting the Location Civic request Location Subject field to indicate Local or Remote request respectively. Local Location Civic Measurement Request is used by requesting STA to obtain its own location by asking “Where am I?”. Remote Location Civic Measurement Request is used by requesting STA to obtain location of the reporting STA by asking “Where are you?”.

If dot11RRMCivicMeasurementEnabled is false, a STA shall reject the received Location Civic Measurement Request and shall respond with a Location Civic Report where the Incapable bit is set in the Measurement Report Mode field.

If dot11RRMCivicMeasurementEnabled is true and an AP has its own location configured in CIVIC format it shall set the E911 CIVIC bit to 1 in the Extended Capabilities element.

11.10.8.10 Location Identifier Report

If dot11RRMIdentifierMeasurementEnabled is true, a STA shall reject any Location Identifier Request for location information which is not available and shall respond with a Measurement Report frame including a Measurement Report element with the incapable bit set to 1. If dot11RRMIdentifierMeasurementEnabled is true and a STA accepts a Location Identifier Request it shall respond with a Measurement Report frame including one Location Identifier Report element.

A Location Identifier Request shall indicate a location request for the requesting STA or the reporting STA by setting the Location Identifier request Location Subject field to indicate Local or Remote request respectively. Local Location Identifier Measurement Request is used by requesting STA to obtain its own location by asking “Where am I?”. Remote Location Civic Measurement Request is used by requesting STA to obtain location of the reporting STA by asking “Where are you?”.

If dot11RRMIdentifierMeasurementEnabled is false, a STA shall reject the received Location Identifier Measurement Request and shall respond with a Location Identifier Report where the Incapable bit is set in the Measurement Report Mode field.

- Location procedures

If dot11MgmtOptionLocationEnabled is true a STA shall set the Location bit to 1 in the Extended Capabilities element.

If dot11MgmtOptionLocationEnabled is true and a STA receives a Location Configuration Request frame, it shall respond with a Location Configuration Response frame.

In an infrastructure BSS, a non-AP STA shall not transmit Location Configuration Request frames.

A STA may configure another STA to transmit Location Track Notification frames for the purpose of tracking that STA’s location by sending Location Indication Channels, Location Indication Interval and Location Indication Broadcast Data Rate sub-elements in a Location Parameters information element in a Location Configuration Request frame.

A Location Configuration Request frame may be a broadcast or individually addressed frame. A STA receiving a broadcast Location Configuration Request frame shall not send a Location Configuration Response frame.

A STA that receives an individually addressed Location Configuration Request shall respond with a Location Configuration Response frame. If all Location Parameter sub-elements included in the Location Configuration Request are successfully configured on the receiving STA, then the STA shall include in the Location Configuration Response frame a single Location Status sub-element indicating success. If one or more Location Parameter sub-elements are unsuccessfully configured, then the STA shall include in the Location Configuration Response frame a Location Status sub-element for each failed sub-element indicating the sub-element ID and status value and if applicable the Location Parameters sub-element with the current values configured on the STA.

The location configuration methods, from highest to lowest precedence, are as follows: 1) an individually addressed Location Configuration Request frame, 2) broadcast Location Configuration Request frame. When a STA receives a new Location Configuration frame at the same or higher precedence than the previous it shall cancel the previous configuration and begin using the newest configuration.

The Location Indication Broadcast Data Rate sub-element included in Location Configuration Request frames indicates the target data rate at which the STA shall transmit Location Track Notification frames. The Location Indication Broadcast Data Rate included in the Location Configuration Request frame shall be a data rate defined in the operational data rate set. ..

A STA shall terminate the transmission of Location Track Notification frames on another STA by sending a Location Configuration Request frame including a Location Parameters information element with the Location Indication Interval sub-element specifying an interval of zero.  If a STA configured to transmit Location Track Notification frames leaves or can no longer detect the ESS that configured it, the STA shall terminate the transmission of Location Track Notification frames.

The Location Status sub-element has four possible status values: Success, Fail, Refused and Incapable. When the STA that sent a Location Configuration Request frame receives a Location Configuration Response frame with Location Status indicating anything other than Success, the STA shall assume the original request was not processed and the STA should take appropriate action based on the status value returned. For Location Status Fail, the STA may either retry the original request or send an alternate request. For Location Status Incapable, the STA shall not send another configuration request matching the previous configuration request while associated to the same BSS. For Location Status Refuse, the STA may send an alternate request.

        - Location Track Notification procedures

A STA configured by another STA as described in the previous section shall transmit Location Track Notification frames as shown in the informative diagram in Figure vXXX below. The diagram shows the STA was configured to transmit on 3 channels, 3 frames per channel.

Figure vXXX: STA transmission on 3 channels, 3 frames per channel.

The STA transmits Location Track Notification frames based on the following parameters:

- Location Indication Channels

  - This sub-element indicates the channels the STA shall transmit on during normal or motion intervals.

- Indication Multicast Address

  - For non-IBSS networks, the STA shall transmit the Location Track Notification frames to the Indication Multicast Address field in the Location Indication Parameters sub-element configured by the Location Configuration Request frame.

  - For IBSS networks, the STA shall transmit the Location Track Notification frames to the destination address of the STA that configured the STA using Location Configuration Request frames.

- Location Indication Interval

  - When the STA is stationary or dot11MgmtOptionMotionDetectionEnabled is false, the following procedures are to be followed:

    - For each normal interval, the STA shall transmit on each channel, the number Location Track Notification frames specified by the Normal number of frames per channel field.

  - If dot11MgmtOptionMotionDetectionEnabled is true and the STA is in motion

    - For each motion interval the STA shall transmit on each channel, the number Location Track Notification frames specified by the Motion number of frames per channel field.

- Location Indication Broadcast Data Rate

  - The STA shall transmit Location Track Notification frames at the data rate specified in this sub-element.

- Time Of Departure

  - If dot11MgmtTODEnabled is true, the STA shall transmit this sub-element in the Location Track Notification frame.

- If there are multiple antennas and/or TXBF capability then the STA shall transmit using the most omni-directional mode possible.

11.20.5.5 Timing Measurement Procedure

If dot11MgmtOptionTimingMsmtEnabled is true, the STA supports the Timing Measurement procedure.  A STA provides timing measurement values to a peer STA using the Timing Measurement public action frame. A receiving STA uses the timing values in the Timing Measurement public action frame to:

- estimate the location of the STA transmitting the action frame and

- estimate the offset of the location clock with respect to the clock of the STA transmitting the action frame.

Two Timing Measurement public action frames are required to be received by a STA from a peer STA in order for the receiving STA to determine the location and the clock-offset relative to the sender. The Follow On to Dialog Token field in the action frame is used in the second action frame to identify the first action frame and shall be set to the value of the Dialog Token from the first action frame.

If dot11MgmtOptionTimingMsmtEnabled is false the STA does not support the Timing Measurement procedure or support for the Timing Measurement procedure is disabled. If dot11RRMTimingMsmtEnabled is false the STA receiving a Timing Measurement public action frame shall ignore the frame.

12.3.4.3 PHY-SAP service primitives parameters

TGV editor: Insert the following entries into Table 12-3:

| TXSTATUS | PHY-TXSTART.confirm | A set of parameters |
| --- | --- | --- |

12.3.5.5 PHY-TXSTART.confirm

12.3.5.5.1 Function

TGV editor: Change

This primitive is issued by the PHY to the local MAC entity to confirm the start of a transmission and to indicate parameters related to the start. The PHY will issue this primitive in response to every PHY-TXSTART.request primitive issued by the MAC sublayer.

12.3.5.5.2 Semantics of the service primitive

TGV editor: Change

The semantics of the primitive are as follows:

PHY-TXSTART.confirm(TXSTATUS)

The TXSTATUS represents a list of parameters that the local PHY entity provides to the MAC sublayer related to the transmission an MPDU. This vector contains both PLCP and PHY management parameters. The required PHY parameters are listed in 12.3.4.4 (presently none).

This primitive has no parameters.

12.3.5.5.3 When generated

TGV editor: Change

This primitive will be issued by the PHY to the MAC entity when the PHY has received a PHYTXSTART.

request from the MAC entity, frame transmission has begun, and the PHY is ready to begin accepting outgoing data octets from the MAC.

12.3.5.5.4 Effect of receipt

TGV editor: Change

The receipt of this primitive by the MAC entity will cause the MAC to start the transfer of data octets. Parameters in the TXSTATUS vector may be used locally or remotely after being incorporated into a transmitted frame in order to determine time-of-flight and hence range and/or location.

15.2.6 Transmit PLCP

TGV editor: Change, noting changes in diagrams

The transmit PLCP is shown in Figure 15-6.

Figure 15-6—Transmit PLCP

In order to transmit data, PHY-TXSTART.request shall be enabled so that the PHY entity shall be in the

transmit state. Further, the PHY shall be set to operate at the appropriate channel through STA management

via the PLME. Other transmit parameters such as DATARATE, TX antenna, and TX power are set via the

PHY-SAP with the PHY-TXSTART.request(TXVECTOR) as described in 15.4.4.2.

Based on the status of CCA indicated by PHY-CCA.indicate, the MAC will assess that the channel is clear. A clear channel shall be indicated by PHY-CCA.indicate(IDLE). If the channel is clear, transmission of the

PPDU shall be initiated by issuing the PHY-TXSTART.request(TXVECTOR) primitive. The TXVECTOR

elements for the PHY-TXSTART.request are the PLCP header parameters SIGNAL (DATARATE),

SERVICE, and LENGTH, and the PMD parameters of TX_ANTENNA, and TXPWR_LEVEL and TIME_OF_DEPARTURE_REQUESTED. The PLCP header parameter LENGTH is calculated from the TXVECTOR element by multiplying by 8 for 1 Mb/s and by 4 for 2 Mb/s.

The PLCP shall issue PMD_ANTSEL, PMD_RATE, and PMD_TXPWRLVL primitives to configure the PHY. The PLCP shall then issue a PMD_TXSTART.request and the PHY entity shall immediately initiate data scrambling and transmission of the PLCP preamble based on the parameters passed in the PHYTXSTART.request primitive. The time required for transmit power-on ramp described in 15.4.7.7 shall be included in the PLCP SYNC field. If the MIB variables dot11MgmtOptionLocationTODImplemented and dot11MgmtOptionLocationTODEnabled exist and are set to TRUE and the TXVECTOR parameter TIME_OF_DEPARTURE_REQUESTED is TRUE then the PLCP shall issue a PHY_TXSTART.confirm(TXSTATUS) primitive to the MAC, forwarding the TIME_OF_DEPARTURE, TIME_OF_DEPARTURE _TOLERANCE and TIME_OF_DEPARTURE_UNITS parameters for the PMD_TXSTART.request within the TXSTATUS vector.

Once the PLCP preamble transmission is complete, data shall be exchanged between the MAC and the PHY by a series of PHY-DATA.request(DATA) primitives issued by the MAC and PHY-DATA.confirm primitives issued by the PHY. The modulation rate change, if any, shall be initiated with the first data symbol of the MPDU as described in 15.2.5. The PHY proceeds with MPDU transmission through a series of data octet transfers from the MAC. At the PMD layer, the data octets are sent in LSB-to-MSB order and presented to the PHY through PMD_DATA.request primitives. Transmission can be prematurely terminated by the MAC through the primitive PHY-TXEND.request. PHY-TXSTART shall be disabled by the issuance of the PHY-TXEND.request. Normal termination occurs after the transmission of the final bit of the last MPDU octet according to the number supplied in the TXVECTOR LENGTH field. The packet transmission shall be completed and the PHY entity shall enter the receive state (i.e., PHY-TXSTART

shall be disabled). It is recommended that chipping continue during power-down. Each PHY-TXEND.request is acknowledged with a PHY-TXEND.confirm primitive from the PHY.

A typical state machine implementation of the transmit PLCP is provided in Figure 15-7.

Figure 15-7—PLCP transmit state machine

15.4.4.2 PMD_SAP peer-to-peer service primitive parameters

TGv Editor: Insert the following new rows at the end of Table 15-4—DSSS PMD_SAP peer-to-peer service primitives

| TIME_OF_DEPARTURE_REQUESTED | TXVECTOR | FALSE, TRUE. When TRUE, the MAC entity requests that the PHY PLCP entity measure and report time of departure parameters; when FALSE, the MAC entity requests that the PHY PLCP entity neither measure nor report time of departure parameters |
| --- | --- | --- |
| TIME_OF_DEPARTURE | TXSTATUS | 0 to 232-1. The locally-measured time of the PMD_TXSTART.request, in units of TIME_OF_DEPARTURE_UNITS. |
| TIME_OF_DEPARTURE_TOLERANCE | TXSTATUS | 1 to 216-1. The 95% tolerance of TIME_OF_DEPARTURE, in units of TIME_OF_DEPARTURE_UNITS. |
| TIME_OF_DEPARTURE_UNITS | TXSTATUS | Enumerated type: TODU22: 1 / 1408 MHz TODU20: 1 / 1280 MHz TODU40: 1 / 2560 MHz |

15.4.7 PMD transmit specifications

TGv Editor: Insert a new section after 15.4.7.9

15.4.7.10 Time of Departure accuracy test

The Time of Departure accuracy test shall be performed by instrumentation capable of converting signals transmitted on one or more channels into a stream of complex samples at fs sample/s or more, with sufficient accuracy in terms of I/Q arm amplitude and phase balance, dc offsets, phase noise, etc. The minimum sampling rate fs is 22×1061+fH−fL22MHz sample/s where fH is the nominal center frequency in Hz of the highest channel, fL is the nominal center frequency in Hz of the lowest channel, and x equals the smallest integer equal to or larger than x. A possible embodiment of such a setup is converting the signal to a low IF frequency with a microwave synthesizer, sampling the signal with a digital oscilloscope and decomposing it digitally into quadrature components. The sampled signal shall be processed in a manner similar to an actual time of arrival processor, according to the following steps, or an equivalent procedure:

a) Start of frame shall be detected.

b) Channel number, coarse and fine frequency offsets shall be estimated.

c) The packet shall be derotated according to estimated frequency offsets.

d) Transition from the SYNC field to the SFD field shall be detected, and fine timing

(with one sample resolution) shall be established.

e) The SYNC and SFD portions of the derotated signal shall be up-sampled to above 1 GHz.

f) The up-sampled signal shall be cross-correlated with a reference waveform of the SYNC and SFD fields. The reference waveform shall use as its chip pulse a rectangular pulse of duration 1/ 11 MHz convolved with a brick-wall low pass filter of bandwidth 11 MHz

g) The measured time of departure shall equal the time of the peak of the magnitude of the cross-correlation.

h) The reported and measured times of departure shall be recorded, and the reported time of departure tolerance.

i) Steps a) to h) shall be repeated for at least 4 packets within 1 second. Time of departure errors shall be calculated as the RMS deviation of the reported values from estimated values. The estimated values shall be determined using the weighted least squares line of best fit, where the X, Y and weight parameters shall be the measured times of departure, the reported times of departure and half the reported tolerances respectively.

j) Step i) shall be repeated 500 times.

k) The Time of Departure accuracy test is passed if 95% or more of the time of departure errors are less than their reported time of departure tolerances, and 95% of the reported time of departure tolerances are less than 2 ns.

Note: One implementation of a time of departure measurement system is a free-running oscillator clocking (a) the digital-to-analog converter(s) used to transmit the packet, (b) a 32-bit continuously-counting counter and (c) a hardware finite state machine such that PMD_TXSTART.request causes a transition within the FSM which in turn causes frame transmission at the DACs a fixed number of cycles later; where the time of departure is recorded as the value of the counter at that transition. In this implementation, the principal source of time of departure error is only short term oscillator imperfection (e.g. phase noise).

17.2.2 TXVECTOR parameters

TGV editor: Insert one new row at the end of Table 17-1—TXVECTOR parameters

| TIME_OF_DEPARTURE_REQUESTED | PHY-TXSTART.request(TXVECTOR) | FALSE, TRUE. When TRUE, the MAC entity requests that the PHY PLCP entity measure and report time of departure parameters; when FALSE, the MAC entity requests that the PHY PLCP entity neither measure nor report time of departure parameters |
| --- | --- | --- |

TGV editor: Insert new section after 17.2.2.4

17.2.2.5 TIME_OF_DEPARTURE_REQUESTED

The allowed values are FALSE or TRUE. A parameter value of TRUE indicates that the MAC sublayer is requesting that the PLCP entity provide time of departure measurement and reporting in the PHY-TXSTART.confirm(TXSTATUS) primitive. A parameter value of FALSE indicates that the MAC sublayer is requesting that the PLCP entity not provide time of departure measurement nor reporting in the PHY-TXSTART.confirm(TXSTATUS) primitive.

TGV editor: Insert new section after 17.2.3

17.2.4 TXSTATUS parameters

The parameters listed in Table 17-2a are defined as part of the TXSTATUS parameter list in the

PHY-TXSTART.confirm service primitive.

Table 17-2a—TXSTATUS parameters

| Parameter | Associate Primitive | Value |
| --- | --- | --- |
| TIME_OF_DEPARTURE | TXSTART.confirm(TXSTATUS) | 0 to 232-1. The locally-measured time of the PMD_TXSTART.request, in units of TIME_OF_DEPARTURE_UNITS. |
| TIME_OF_DEPARTURE_TOLERANCE | TXSTART.confirm(TXSTATUS) | 1 to 216-1. The 95% tolerance of TIME_OF_DEPARTURE, in units of TIME_OF_DEPARTURE_UNITS. |
| TIME_OF_DEPARTURE_UNITS | TXSTART.confirm(TXSTATUS) | Enumerated type: TODU22: 1 / 1408 MHz TODU20: 1 / 1280 MHz TODU40: 1 / 2560 MHz |

17.2.4.1 TXSTATUS TIME_OF_DEPARTURE

The allowed values for the TIME_OF_DEPARTURE parameter are integers in the range of 0 to 232-1 This parameter is used to indicate the time of departure of the last transmitted frame in units given by TIME_OF_DEPARTURE_UNITS. TIME_OF_DEPARTURE may be used locally or remotely after being incorporated into a transmitted frame in order to determine time-of-flight and hence range and/or location.

17.2.4.2 TXSTATUS TIME_OF_DEPARTURE_TOLERANCE

The allowed values for the TIME_OF_DEPARTURE_TOLERANCE parameter are integers in the range of 1 to 216-1 This parameter is used to indicate the 95% tolerance of the TIME_OF_DEPARTURE in units given by TIME_OF_DEPARTURE_UNITS. TIME_OF_DEPARTURE_TOLERANCE may be used locally or remotely after being incorporated into a transmitted frame in order to determine time-of-flight and hence range and/or location.

17.2.4.3 TXSTATUS TIME_OF_DEPARTURE_UNITS

TIME_OF_DEPARTURE_UNITS indicates the units used for TIME_OF_DEPARTURE and TIME_OF_TOLERANCE. The allowed values for TIME_OF_DEPARTURE_UNITS are TODU22 (1 / 1408 MHz), TODU20 (1 / 1280 MHz) and TODU40 (1 / 2560 MHz).

17.3.9 PMD transmit specifications

TGv Editor: Insert a new section after 17.3.9.7

17.3.9.8 Time of Departure accuracy test

The Time of Departure accuracy test shall be performed by instrumentation capable of converting signals transmitted on one or more channels into a stream of complex samples at fs sample/s or more, with sufficient accuracy in terms of I/Q arm amplitude and phase balance, dc offsets, phase noise, etc. For 20 MHz channel spacing, the minimum sampling rate fs is 20×1061+fH−fL20MHz sample/s where fH is the nominal center frequency in Hz of the highest channel, fL is the nominal center frequency in Hz of the lowest channel, and x equals the smallest integer equal to or larger than x. A possible embodiment of such a setup is converting the signal to a low IF frequency with a microwave synthesizer, sampling the signal with a digital oscilloscope and decomposing it digitally into quadrature components. The sampled signal shall be processed in a manner similar to an actual time of arrival processor, according to the following steps, or an equivalent procedure:

a) Start of frame shall be detected.

b) Channel number, coarse and fine frequency offsets shall be estimated.

c) The packet shall be derotated according to estimated frequency offsets.

d) Transition from the short symbols to the long symbols shall be detected, and fine timing (with one sample resolution) shall be established.

e) The long symbols of the derotated signal shall be up-sampled to above 1 GHz.

f) The up-sampled signal shall be cross-correlated with a reference waveform of the long symbols. The reference waveform shall use as its windowing the windowing described in 17.3.2.4 with TTR = 100 ns for 20 MHz channel spacing, TTR = 200 ns for 10 MHz channel spacing and TTR = 400 ns for 5 MHz channel spacing.

g) The measured time of departure shall equal the time of the peak of the magnitude of the cross-correlation.

h) The reported and measured times of departure shall be recorded, and the reported time of departure tolerance.

i) Steps a) to h) shall be repeated for at least 4 packets within 1 second. Time of departure errors shall be calculated as the RMS deviation of the reported values from estimated values. The estimated values shall be determined using the weighted least squares line of best fit, where the X, Y and weight parameters shall be the measured times of departure, the reported times of departure and half the reported tolerances respectively.

j) Step i) shall be repeated 500 times.

k) The Time of Departure accuracy test is passed if 95% or more of the time of departure errors are less than their reported time of departure tolerances, and 95% of the reported time of departure tolerances are less than 2 ns.

Note: One implementation of a time of departure measurement system is a free-running oscillator clocking (a) the digital-to-analog converter(s) used to transmit the packet, (b) a 32-bit continuously-counting counter and (c) a hardware finite state machine such that PMD_TXSTART.request causes a transition within the FSM which in turn causes frame transmission at the DACs a fixed number of cycles later; where the time of departure is recorded as the value of the counter at that transition. In this implementation, the principal source of time of departure error is only short term oscillator imperfection (e.g. phase noise).

17.3.11 Transmit PLCP

TGV editor: Change, noting changes in diagrams

The transmit PLCP is shown in Figure 17-14. In order to transmit data, PHY-TXSTART.request shall be

enabled so that the PHY entity shall be in the transmit state. Further, the PHY shall be set to operate at the

appropriate frequency through STA management via the PLME. Other transmit parameters, such as

DATARATE and TX power, are set via the PHY-SAP with the PHY-TXSTART.request(TXVECTOR), as

described in 17.2.2.

A clear channel shall be indicated by PHY-CCA.indicate(IDLE). The MAC considers this indication before

issuing the PHY-TXSTART.request. Transmission of the PPDU shall be initiated after receiving the PHYTXSTART.

request(TXVECTOR) primitive. The TXVECTOR elements for the PHY-TXSTART.request

are the PLCP header parameters DATARATE, SERVICE, and LENGTH, and the PMD parameters

TXPWR_LEVEL and TIME_OF_DEPARTURE_REQUESTED.

The PLCP shall issue PMD_TXPWRLVL and PMD_RATE primitives to configure the PHY. The PLCP

shall then issue a PMD_TXSTART.request, and transmission of the PLCP preamble and PLCP header,

based on the parameters passed in the PHY-TXSTART.request primitive, shall be immediately initiated. If the MIB variables dot11MgmtOptionLocationTODImplemented and dot11MgmtOptionLocationTODEnabled exist and are set to TRUE and the TXVECTOR parameter TIME_OF_DEPARTURE_REQUESTED is TRUE then the PLCP shall issue a PHY_TXSTART.confirm(TXSTATUS) primitive to the MAC, forwarding the TIME_OF_DEPARTURE, TIME_OF_DEPARTURE _TOLERANCE and TIME_OF_DEPARTURE_UNITS parameters for the PMD_TXSTART.request within the TXSTATUS vector.

Once PLCP preamble

transmission is started, the PHY entity shall immediately initiate data scrambling and data encoding.

The

scrambled and encoded data shall then be exchanged between the MAC and the PHY through a series of PHY-DATA.request(DATA) primitives issued by the MAC, and PHY-DATA.confirm primitives issued by

the PHY. The modulation rate change, if any, shall be initiated from the SERVICE field data of the PLCP

header, as described in 17.3.2.

The PHY proceeds with PSDU transmission through a series of data octet transfers from the MAC. The

PLCP header parameter, SERVICE, and PSDU are encoded by the convolutional encoder with the

bit-stealing function described in 17.3.5.5. At the PMD layer, the data octets are sent in bit 0–7 order and

presented to the PHY through PMD_DATA.request primitives. Transmission can be prematurely terminated

by the MAC through the primitive PHY-TXEND.request. PHY-TXSTART shall be disabled by the issuance

of the PHY-TXEND.request. Normal termination occurs after the transmission of the final bit of the last

PSDU octet, according to the number supplied in the OFDM PHY preamble LENGTH field.

Figure 17-14—Transmit PLCP

The packet transmission shall be completed and the PHY entity shall enter the receive state (i.e., PHYTXSTART

shall be disabled). Each PHY-TXEND.request is acknowledged with a PHY-TXEND.confirm

primitive from the PHY. If the coded PSDU (C-PSDU) is not multiples of the OFDM symbol, bits shall be

stuffed to make the C-PSDU length multiples of the OFDM symbol.

In the PMD, the GI shall be inserted in every OFDM symbol as a countermeasure against severe delay

spread.

A typical state machine implementation of the transmit PLCP is provided in Figure 17-15. Requests (.req)

and confirmations(.confirm) are issued once with designated states.

Figure 17-15—PLCP transmit state machine

18.2.5 Transmit PLCP

TGV editor: Change, noting changes in diagrams

The transmit procedures for a High Rate PHY using the long PLCP preamble and header are the same as the

transmit procedures described in 15.2.6 and 15.2.7 and do not change apart from the ability to transmit

5.5 Mb/s and 11 Mb/s.

The procedures for a transmitter employing HR/DSSS/short and HR/DSSS/PBCC/short are the same except

for length and rate changes. The decision to use a long or short PLCP is beyond the scope of this standard.

The transmit PLCP is shown in Figure 18-7.

A PHY-TXSTART.request(TXVECTOR) primitive will be issued by the MAC to start the transmission

of a PPDU. In addition to parameters DATARATE and LENGTH, other transmit parameters

such as PREAMBLE_TYPE and MODULATION are set via the PHY-SAP with the

PHY-TXSTART.request(TXVECTOR), as described in 18.3.5. The SIGNAL, SERVICE, and LENGTH

fields of the PLCP header are calculated as described in 18.2.3.

The PLCP shall issue PMD_ANTSEL, PMD_RATE, and PMD_TXPWRLVL primitives to configure the

PHY. The PLCP shall then issue a PMD_TXSTART.request, and the PHY entity shall immediately initiate

data scrambling and transmission of the PLCP preamble based on the parameters passed in the

PHY-TXSTART.request primitive. The time required for transmit power-on ramp, described in 18.4.7.6,

shall be included in the PLCP SYNC field. If the MIB variables dot11MgmtOptionLocationTODImplemented and dot11MgmtOptionLocationTODEnabled exist and are set to TRUE and the TXVECTOR parameter TIME_OF_DEPARTURE_REQUESTED is TRUE then the PLCP shall issue a PHY_TXSTART.confirm(TXSTATUS) primitive to the MAC, forwarding the TIME_OF_DEPARTURE, TIME_OF_DEPARTURE _TOLERANCE and TIME_OF_DEPARTURE_UNITS parameters for the PMD_TXSTART.request within the TXSTATUS vector.

Once the PLCP preamble transmission is complete, data shall be

exchanged between the MAC and the PHY by a series of PHY-DATA.request(DATA) primitives issued by

the MAC and PHY-DATA.confirm primitives issued by the PHY. The modulation and rate change, if any,

shall be initiated with the first data symbol of the PSDU, as described in 18.2.3.7 and 18.2.3.14. The PHY

proceeds with PSDU transmission through a series of data octet transfers from the MAC. At the PMD layer,

the data octets are sent in LSB-to-MSB order and presented to the PHY through PMD_DATA.request

primitives. Transmission can be prematurely terminated by the MAC through the primitive

PHY-TXEND.request. PHY-TXSTART shall be disabled by the issuance of the PHY-TXEND.request.

Normal termination occurs after the transmission of the final bit of the last PSDU octet, calculated from the

number supplied in the PHY preamble LENGTH and SERVICE fields using the equations specified in

18.2.3.5. The PPDU transmission shall be completed and the PHY entity shall enter the receive state (i.e., PHY-TXSTART shall be disabled). It is recommended that modulation continue during power-down to

prevent radiating a continuous wave carrier. Each PHY-TXEND.request is acknowledged with a

PHY-TXEND.confirm primitive from the PHY.

Figure 18-7—Transmit PLCP

A typical state machine implementation of the transmit PLCP is provided in Figure 18-8.

Figure 18-8—PLCP transmit state machine

18.3.5 Vector descriptions

TGv Editor: Insert the following new rows at the end of Table 18-6—Parameter vectors

| TIME_OF_DEPARTURE_REQUESTED | TXVECTOR | FALSE, TRUE. When TRUE, the MAC entity requests that the PHY PLCP entity measure and report time of departure parameters; when FALSE, the MAC entity requests that the PHY PLCP entity neither measure nor report time of departure parameters |
| --- | --- | --- |
| TIME_OF_DEPARTURE | TXSTATUS | 0 to 232-1. The locally-measured time of the PMD_TXSTART.request, in units of TIME_OF_DEPARTURE_UNITS. |
| TIME_OF_DEPARTURE_TOLERANCE | TXSTATUS | 1 to 216-1. The 95% tolerance of TIME_OF_DEPARTURE, in units of TIME_OF_DEPARTURE_UNITS. |
| TIME_OF_DEPARTURE_UNITS | TXSTATUS | Enumerated type: TODU22: 1 / 1408 MHz TODU20: 1 / 1280 MHz TODU40: 1 / 2560 MHz |

18.4.7 PMD transmit specifications

TGv Editor: Insert a new section after 18.4.7.8

18.4.7.9 Time of Departure accuracy test

The Time of Departure accuracy test shall be performed by instrumentation capable of converting signals transmitted on one or more channels into a stream of complex samples at fs sample/s or more, with sufficient accuracy in terms of I/Q arm amplitude and phase balance, dc offsets, phase noise, etc. The minimum sampling rate fs is 22×1061+fH−fL22MHz sample/s where fH is the nominal center frequency in Hz of the highest channel, fL is the nominal center frequency in Hz of the lowest channel, and x equals the smallest integer equal to or larger than x. A possible embodiment of such a setup is converting the signal to a low IF frequency with a microwave synthesizer, sampling the signal with a digital oscilloscope and decomposing it digitally into quadrature components. The sampled signal shall be processed in a manner similar to an actual time of arrival processor, according to the following steps, or an equivalent procedure:

a) Start of frame shall be detected.

b) Channel number, coarse and fine frequency offsets shall be estimated.

c) The packet shall be derotated according to estimated frequency offsets.

d) Transition from the SYNC field to the SFD field shall be detected, and fine timing

(with one sample resolution) shall be established.

e) The type (short or long) of preamble shall be determined

f) The SYNC and SFD portions of the derotated signal shall be up-sampled to above 1 GHz.

g) The up-sampled signal shall be cross-correlated with a reference waveform of the appropriate short or long SYNC and SFD fields. The reference waveform shall use as its chip pulse a rectangular pulse of duration 1/ 11 MHz convolved with a brick-wall low pass filter of bandwidth 11 MHz

h) The measured time of departure shall equal the time of the peak of the magnitude of the cross-correlation.

i) The reported and measured times of departure shall be recorded, and the reported time of departure tolerance.

j) Steps a) to i) shall be repeated for at least 4 packets within 1 second. Time of departure errors shall be calculated as the RMS deviation of the reported values from estimated values. The estimated values shall be determined using the weighted least squares line of best fit, where the X, Y and weight parameters shall be the measured times of departure, the reported times of departure and half the reported tolerances respectively.

k) Step j) shall be repeated 500 times.

l) The Time of Departure accuracy test is passed if 95% or more of the time of departure errors are less than their reported time of departure tolerances, and 95% of the reported time of departure tolerances are less than 2 ns.

Note: One implementation of a time of departure measurement system is a free-running oscillator clocking (a) the digital-to-analog converter(s) used to transmit the packet, (b) a 32-bit continuously-counting counter and (c) a hardware finite state machine such that PMD_TXSTART.request causes a transition within the FSM which in turn causes frame transmission at the DACs a fixed number of cycles later; where the time of departure is recorded as the value of the counter at that transition. In this implementation, the principal source of time of departure error is only short term oscillator imperfection (e.g. phase noise).

19.2 PHY-specific service parameter list

TGV editor: Insert one new row at the end of Table 17-1—TXVECTOR parameters

| TIME_OF_DEPARTURE_REQUESTED | FALSE, TRUE. When TRUE, the MAC entity requests that the PHY PLCP entity measure and report time of departure parameters; when FALSE, the MAC entity requests that the PHY PLCP entity neither measure nor report time of departure parameters |
| --- | --- |

19.2 PHY-specific service parameter list

TGv editor: change

The architecture of the IEEE 802.11 MAC is intended to be PHY independent. Some PHY implementations

require PHY-dependent MAC state machines running in the MAC sublayer in order to meet certain PMD

requirements. The PHY-dependent MAC state machine resides in a sublayer defined as the MLME. In

certain PMD implementations, the MLME may need to interact with the PLME as part of the normal PHY

SAP primitives. These interactions are defined by the PLME parameter list currently defined in the PHY

service primitives as TXVECTOR, TXSTATUS and RXVECTOR. The list of these parameters and the values they may

represent are defined in the specific PHY specifications for each PMD. This subclause addresses the

TXVECTOR, TXSTATUS and RXVECTOR for the ERP. The service parameters for RTXVECTOR, TXSTATUS and TRXVECTOR

shall follow 17.2.2, 17.2.4 and 17.2.3, respectively.

Several service primitives include a parameter vector. DATARATE and LENGTH are described in 12.3.4.4.

The remaining parameters are considered to be management parameters and are specific to this PHY.

The parameters in Table 19-1 are defined as part of the TXVECTOR parameter list in the PHYTXSTART.

request service and PLME_TXTIME.request primitives.

The parameters in Table 19-1a are defined as part of the TXSTATUS parameter list in the PHYTXSTART.

confirm service primitive.

TGv Editor: Insert the following new table after table 19-1

Table 19-1a: TXSTATUS parameters

| Parameter | Value |
| --- | --- |
| TIME_OF_DEPARTURE | 0 to 232-1. The locally-measured time of the PMD_TXSTART.request, in units of TIME_OF_DEPARTURE_UNITS. |
| TIME_OF_DEPARTURE_TOLERANCE | 1 to 216-1. The 95% tolerance of TIME_OF_DEPARTURE, in units of TIME_OF_DEPARTURE_UNITS. |
| TIME_OF_DEPARTURE_UNITS | Enumerated type: TODU22: 1 / 1408 MHz TODU20: 1 / 1280 MHz TODU40: 1 / 2560 MHz |

19.4.7 PMD transmit specifications

TGV editor: Change:

The PMD transmit specifications shall follow 17.3.9 with the exception of the transmit power level

(17.3.9.1), the transmit center frequency tolerance (17.3.9.4), and the symbol clock frequency tolerance

(17.3.9.5), and the time of departure accuracy test (17.3.9.8). Regulatory requirements may have an effect on the combination of maximum transmit power and spectral mask if the resulting signals violate restricted band emission limits.

TGv Editor: Insert a new section after 19.4.7.3

19.4.7.4 Time of Departure accuracy test

The time of departure specifications shall follow 17.3.9.8 for PPDUs transmitted using ERP-OFDM format and 18.4.7.9 for PPDUs transmitted using ERP-DSSS/CCK, ERP-PBCC and DSSS-OFDM formats.

20.2.1 Introduction

TGv Editor: Change

The PHY interfaces to the MAC through the TX_VECTOR, TXSTATUS, RX_VECTOR, and the

PHYCONFIG_VECTOR. The TX_VECTOR supplies the PHY with per packet transmit parameters. Status of the transmission is reported from PHY to MAC by parameters within TXSTATUS. Using

the RX_VECTOR, the PHY informs the MAC of the received packet parameters. Using the

PHYCONFIG_VECTOR, the MAC configures the PHY for operation, independent of frame transmission or

reception.

20.2.2 TXVECTOR parameters

TGV editor: Insert one new row at the end of Table 20-1—TXVECTOR and RXVECTOR parameters

| TIME_OF_DEPARTURE_REQUESTED |  | Enumerated type: TRUE indicates that the MAC entity requests that the PHY PLCP entity measure and report time of departure parameters. FALSE indicates that the MAC entity requests that the PHY PLCP entity neither measure nor report time of departure parameters | O | N |
| --- | --- | --- | --- | --- |

20.2.4 Support for NON_HT formats

TGV editor: Insert new row immediately before the last row (i.e. Note row) of Table 20-3—Mapping of the HT PHY parameters for NON_HT operation

| TIME_OF_DEPARTURE_REQUESTED | TIME_OF_DEPARTURE_REQUESTED | TIME_OF_DEPARTURE_REQUESTED | TIME_OF_DEPARTURE_REQUESTED | TIME_OF_DEPARTURE_REQUESTED |
| --- | --- | --- | --- | --- |

TGV editor: Insert new section after 20.2.4

20.2.5 TXSTATUS parameters

The parameters listed in Table 20-3a are defined as part of the TXSTATUS parameter list in the

PHY-TXSTART.confirm(TXSTATUS) service primitive.

Table 20-3a—TXSTATUS parameters

| Parameter | Value |
| --- | --- |
| TIME_OF_DEPARTURE | 0 to 232-1. The locally-measured time of the PMD_TXSTART.request, in units of TIME_OF_DEPARTURE_UNITS. |
| TIME_OF_DEPARTURE_TOLERANCE | 1 to 216-1. The 95% tolerance of TIME_OF_DEPARTURE, in units of TIME_OF_DEPARTURE_UNITS. |
| TIME_OF_DEPARTURE_UNITS | Enumerated type: TODU22: 1 / 1408 MHz TODU20: 1 / 1280 MHz TODU40: 1 / 2560 MHz |

20.3.21 PMD transmit specification

TGv Editor: Insert a new section after 20.3.21.7

20.3.21.8 Time of Departure accuracy test

The Time of Departure accuracy test shall be performed by instrumentation capable of converting signals transmitted on one or more channels into a stream of complex samples at fs sample/s or more, with sufficient accuracy in terms of I/Q arm amplitude and phase balance, dc offsets, phase noise, etc. For a CH_BANDWIDTH parameter equal to HT_CBW20 or HT_CBW40, the minimum sampling rate fs is 20×1061+fH−fL20MHz or 40×1061+fH−fL20MHz sample/s respectively, where fH is the nominal center frequency in Hz of the highest channel, fL is the nominal center frequency in Hz of the lowest channel, and x equals the smallest integer equal to or larger than x. A possible embodiment of such a setup is converting the signal to a low IF frequency with a microwave synthesizer, sampling the signal with a digital oscilloscope and decomposing it digitally into quadrature components. The sampled signal shall be processed in a manner similar to an actual time of arrival processor, according to the following steps, or an equivalent procedure:

a) Start of frame shall be detected.

b) Channel number, coarse and fine frequency offsets shall be estimated.

c) The packet shall be derotated according to estimated frequency offsets.

d) For HT-mixed format, the transition from the L-STF to the L-LTF shall be detected; for HT-greenfield format, the transition from the HT-GF-STF to the HT-GF-LTF1 shall be detected; and in either case fine timing (with one sample resolution) shall be established.

e) The L-LTF or HT-LTF1 of the derotated signal shall be up-sampled to above 1 GHz.

f) The up-sampled signal shall be cross-correlated with a reference waveform of the L-LTF (for HT-mixed format) or HT-LTF1 (for HT-greenfield format). The reference waveform shall use as its windowing the windowing described in 17.3.2.4 with TTR = 100 ns.

g) The measured time of departure shall equal the time of the peak of the magnitude of the cross-correlation.

h) The reported and measured times of departure shall be recorded, and the reported time of departure tolerance.

i) Steps a) to h) shall be repeated for at least 4 packets with the same CH_BANDWIDTH parameter within 1 second. Time of departure errors shall be calculated as the RMS deviation of the reported values from estimated values. The estimated values shall be determined using the weighted least squares line of best fit, where the X, Y and weight parameters shall be the measured times of departure, the reported times of departure and half the reported tolerances respectively.

j) Step i) shall be repeated 500 times with the same CH_BANDWIDTH parameter.

k) The Time of Departure accuracy test for the tested CH_BANDWIDTH parameter is passed if 95% or more of the time of departure errors are less than their reported time of departure tolerances, and 95% of the reported time of departure tolerances are less than i) 2 ns if the CH_BANDWIDTH parameter equals HT_CBW20, or ii) 1 ns if the CH_BANDWIDTH parameter equals HT_CBW40.

Note: One implementation of a time of departure measurement system is a free-running oscillator clocking (a) the digital-to-analog converter(s) used to transmit the packet, (b) a 32-bit continuously-counting counter and (c) a hardware finite state machine such that PMD_TXSTART.request causes a transition within the FSM which in turn causes frame transmission at the DACs a fixed number of cycles later; where the time of departure is recorded as the value of the counter at that transition. In this implementation, the principal source of time of departure error is only short term oscillator imperfection (e.g. phase noise).

20.3.23 Transmit PLCP

TGV editor: Change, noting changes in diagrams

There are three options for transmit PLCP procedure. The first two options, for which typical transmit procedures

are shown in Figure 20-20 and Figure 20-21, are selected if the FORMAT field of PHY-TXSTART.request(

TXVECTOR) is set to HT_MF or HT_GF, respectively. These transmit procedures do not describe the

operation of optional features, such as LDPC or STBC. The third option is to follow the transmit procedure

as in Clause 17 or Clause 19 if the FORMAT field is set to NON_HT. Additionally, if the FORMAT field is

set to NON_HT, CH_BANDWIDTH indicates NON_HT_CBW20, and NON_HT_MODULATION indicates

OFDM, follow the transmit procedure as in Clause 17. If the FORMAT field is set to NON_HT,

CH_BANDWIDTH indicates NON_HT_CBW20, and NON_HT_MODULATION indicates other than

OFDM, follow the transmit procedure as in Clause 19. And furthermore, if the FORMAT field is set to

NON_HT and CH_BANDWIDTH indicates NON_HT_CBW40, follow the transmit procedure as in Clause

17, except that the signal in Clause 17 is generated simultaneously on each of the upper and lower 20 MHz

channels that comprise the 40 MHz channel as defined in 20.3.8 and 20.3.11.11. In all these options, in order

to transmit data, PHY-TXSTART.request shall be enabled so that the PHY entity shall be in the transmit state.

Further, the PHY shall be set to operate at the appropriate frequency through station management via the

PLME, as specified in 20.4. Other transmit parameters, such as MCS Coding types and transmit power, are

set via the PHY-SAP with the PHY-TXSTART.request(TXVECTOR), as described in 20.2.2.

A clear channel shall be indicated by PHY-CCA.indication(IDLE). Note - under some circumstances, the

MAC uses the latest value of PHY-CCA.indication before issuing the PHY-TXSTART.request. Transmission

of the PPDU shall be initiated after receiving the PHYTXSTART.request(TXVECTOR) primitive. The

TXVECTOR elements for the PHY-TXSTART.request are specified in Table 20-1.

The PLCP shall issue the parameters in the following PMD primitives to configure the PHY:

— PMD_TXPWRLVL

— PMD_TX_PARAMETERS

The PLCP shall then issue a PMD_TXSTART.request, and transmission of the PLCP preamble may start if TIME_OF_DEPARTURE_REQUESTED is FALSE, and shall start immediately if TIME_OF_DEPARTURE_REQUESTED is FALSE,

based on the parameters passed in the PHY-TXSTART.request primitive. If the MIB variables dot11MgmtOptionLocationTODImplemented and dot11MgmtOptionLocationTODEnabled exist and are set to TRUE and the TXVECTOR parameter TIME_OF_DEPARTURE_REQUESTED is TRUE then the PLCP shall issue a PHY_TXSTART.confirm(TXSTATUS) primitive to the MAC, forwarding the TIME_OF_DEPARTURE, TIME_OF_DEPARTURE _TOLERANCE and TIME_OF_DEPARTURE_UNITS parameters for the PMD_TXSTART.request within the TXSTATUS vector.

The data shall then be exchanged

between the MAC and the PHY through a series of PHY-DATA.request(DATA) primitives issued by the

MAC, and PHY-DATA.confirm primitives issued by the PHY. Once PLCP preamble transmission is started,

the PHY entity shall immediately initiate data scrambling and data encoding. The encoding method shall be

based on the FEC_CODING, CH_BANDWIDTH, and MCS parameter of the TXVECTOR. A modulation

rate change, if any, shall be initiated starting with the SERVICE field data, as described in 20.3.2.

The PHY proceeds with PSDU transmission through a series of data octet transfers from the MAC. The SERVICE

field and PSDU are encoded by the encoder selected by the FEC_CODING, CH_BANDWIDTH, and

MCS parameters of the TXVECTOR as described in 20.3.3. At the PMD layer, the data octets are sent in bit

0–7 order and presented to the PHY through PMD_DATA.request primitives. Transmission can be prematurely

terminated by the MAC through the primitive PHY-TXEND.request. PHY-TXSTART shall be disabled

by receiving a PHY-TXEND.request. Normal termination occurs after the transmission of the final bit

of the last PSDU octet, according to the number supplied in the LENGTH field.

The packet transmission shall be completed and the PHY entity shall enter the receive state (i.e., PHYTXSTART

shall be disabled). Each PHY-TXEND.request is acknowledged with a PHY-TXEND.confirm primitive

from the PHY. If the length of the coded PSDU (C-PSDU) is not an integer multiple of the OFDM

symbol length, bits shall be stuffed to make the C-PSDU length an integer multiple of the OFDM symbol

length.

In the PMD, the GI or short GI shall be inserted in every OFDM symbol as a countermeasure against delay

spread.

A typical state machine implementation of the transmit PLCP is provided in Figure 20-22. Requests (.req)

and confirmations(.confirm) are issued once per state as shown. This state machine does not describe the operation

of optional features, such as LDPC or STBC.

Figure 20-20—PLCP transmit procedure (HT-mixed format PPDU)

Figure 20-21—PLCP transmit procedure (HT-greenfield format PPDU)

Figure 20-22—PLCP transmit state machine

- Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| WNM1 | Extended Capabilities element | 7.3.2.62 | CFv:M | Yes, No, N/A |
| WNM2 | Management Diagnostic Alerts | 11.20.2 | CFv:M | Yes, No, N/A |
| WNM2.1 | Measurement Request frame | 7.4.5.1 | CFv:M | Yes, No, N/A |
| WNM2.2 | Measurement Report frame | 7.4.5.2 | CFv:M | Yes, No, N/A |
| WNM2.3 | Protocol for Triggered Measurements | 11.10.7 | CFv:M | Yes, No, N/A |
| WNM2.4 | Triggered STA Statistics Reporting | 7.3.2.21.8, 7.3.2.22.8, 11.10.8.5 | CFv:M | Yes, No, N/A |
| WNM2.5 | Triggered QoS Metrics Reporting | 7.3.2.21.10, 7.3.2.22.10, 11.11.9.8 | (CFv AND CF12):O | Yes, No, N/A |
| WNM2.6 | Multicast Diagnostics Reporting | 7.3.2.21.10a, 7.3.2.22.10a, 11.20.2 | CFv:M | Yes, No, N/A |
| WNM3 | Event | 11.20.3 | CFv:M | Yes, No, N/A |
| WNM3.1 | Event Request | 7.3.2.62, 7.4.11.1 | (CFv AND CF1):M | Yes, No, N/A |
| WNM3.2 | Event Report | 7.3.2.63, 7.4.11.2 | (CFv AND CF12):M | Yes, No, N/A |
| WNM4 | Diagnostic | 11.20.4 | CFv:M | Yes, No, N/A |
| WNM4.1 | Diagnostic Request | 7.3.2.64, 7.4.11.3 | (CFv AND CF1):M | Yes, No, N/A |
| WNM4.2 | Diagnostic Report | 7.3.2.65, 7.4.11.4 | (CFv AND CF12):M | Yes, No, N/A |
| WNM4.3 | Manufacturer Information STA Report Diagnostic Type | 7.3.2.65.2, 11.20.4.3 | CFv:M | Yes, No, N/A |
| WNM4.4 | Configuration Profile Diagnostic Type | 7.3.2.65.3, 11.20.4.2 | CFv:M | Yes, No, N/A |
| WNM4.5 | Association Diagnostic Type | 7.3.2.64.2, 7.3.2.65.4, 11.20.4.4 | CFv:M | Yes, No, N/A |
| WNM4.6 | 802.1X Authentication Diagnostic Type | 7.3.2.64.3, 7.3.2.65.4, 11.20.4.4 | (CFv AND PC34):O | Yes, No, N/A |
| WNM5 | Location | 11.20.5, 7.3.2.66 | CFv:M | Yes, No, N/A |
| WNM5.1 | Location Civic Request |  | CFv:M |  |
| WNM5.2 | Location Civic Report |  | CFv:M |  |
| WNM5.3 | Location Identifier Request |  | CFv:M |  |
| WNM5.4 | Location Identifier Report |  | CFv:M |  |
| WNM5.5 WNM5.5.1 | Location Track Notification Time Of Departure Notifications Motion Detection Notifications |  | CFv:M CFv:O CFv:O |  |
| WNM5.6 | Location Configuration Request | 7.4.11.7 | CFv:M | Yes, No, N/A |
| WNM5.6.1 | Normal Indication |  | CFv:M |  |
| WNM5.6.2 | Motion Indication |  | CFv:O |  |
| WNM5.7 | Location Configuration Response | 7.4.11.8 | CFv:M | Yes, No, N/A |
| WNM5.8 | Time Measurement |  | CFv:O |  |
| WNM6 | Multiple BSSID Support | 11.20.6, 11.20.7 | CFv:O | Yes, No, N/A |
| WNM7 | BSS Transition Management | 11.20.8 | CFv:M | Yes, No, N/A |
| WNM7.1 | Neighbor Report Element | 7.3.2.37 | (CFv AND CF1):M | Yes, No, N/A |
| WNM7.2 | BSS Transition Management Query | 7.4.11.9 | (CFv AND CF1):M | Yes, No, N/A |
| WNM7.3 | BSS Transition Management Request | 7.4.11.10 | (CFv AND CF12):M | Yes, No, N/A |
| WNM7.4 | BSS Transition Management Response | 7.4.11.11 | (CFv AND CF12):M | Yes, No, N/A |
| *WNM8 | FBMS | 9.2.7;11.20.9 | CFv:O | Yes, No, N/A |
| WNM8.1 | FBMS Request | 7.4.11.12 | (CF2&WNM8):M | Yes, No, N/A |
| WNM8.2 | FBMS Response | 7.4.11.13 | (CF1&WNM8):M | Yes, No, N/A |
| WNM9 | Proxy ARP | 11.2.1.4b | CFv:O | Yes, No, N/A |
| WNM10 | Co-located Interference Reporting | 7.4.11.14, 7.4.11.15, 11.20.10 | CFv:O | Yes, No, N/A |
| WNM11 | BSS Max Idle Period | 7.3.2.74, 11.2.1.4c | CFv:O | Yes, No, N/A |
| *WNM12 | TFS | 11.20.13 | CFv:O | Yes, No, N/A |
| WNM12.1 | TFS Request frame | 7.3.2.75, 7.4.11.16 | WNM12:M | Yes, No, N/A |
| WNM12.2 | TFS Response frame | 7.3.2.76, 7.4.11.17 | WNM12:M | Yes, No, N/A |
| WNM12.3 | TFS Notify frame | 7.4.11.18 | (CF1&WNM12):M, (CF2&WNM12):O | Yes, No, N/A |
| *WNM13 | Sleep Mode | 11.20.14 | CFv:O | Yes, No, N/A |
| WNM13.1 | Sleep Mode Request frame | 7.3.2.77, 7.4.11.19 | WNM13:M | Yes, No, N/A |
| WNM13.2 | Sleep Mode Response frame | 7.3.2.77, 7.4.11.20 | WNM13:M | Yes, No, N/A |
| WNM14 | TIM broadcast | 7.3.2.78, 7.3.2.79, 7.4.11.21, 7.4.11.22, 7.4.11.23, 11.2.1.11a | CFv:O | Yes, No. N/A |
| WNM15 | Traffic Generation | 7.4.11.24, 11.20.11 | (CFv AND CF12):O | Yes, No, N/A |
| WNM16 | AC Station Count | 11.20.12 | (CFv AND CF12):O | Yes, No, N/A |

- 11k uses 14. 11r uses 15, 11n uses 16.

Change the “Dot11StationConfigEntry” of the “dotStationConfig TABLE” as follows:

-- *********************************************************************

-- * dotStationConfig TABLE

-- *********************************************************************

Dot11StationConfigEntry ::=
SEQUENCE {
	dot11StationID MacAddress,
	dot11MediumOccupancyLimit INTEGER,
	dot11CFPollable TruthValue,
	dot11CFPeriod INTEGER,
	dot11CFPMaxDuration INTEGER,
	dot11AuthenticationResponseTimeOut Unsigned32,
	dot11PrivacyOptionImplemented TruthValue,
	dot11PowerManagementMode INTEGER,
	dot11DesiredSSID OCTET STRING,
	dot11DesiredBSSType INTEGER,
	dot11OperationalRateSet OCTET STRING,
	dot11BeaconPeriod INTEGER,
	dot11DTIMPeriod INTEGER,
	dot11AssociationResponseTimeOut Unsigned32,
	dot11DisassociateReason INTEGER,
	dot11DisassociateStation MacAddress,
	dot11DeauthenticateReason INTEGER,
	dot11DeauthenticateStation MacAddress,
	dot11AuthenticateFailStatus INTEGER,
	dot11AuthenticateFailStation MacAddress,
	dot11MultiDomainCapabilityImplemented TruthValue,
	dot11MultiDomainCapabilityEnabled TruthValue,
	dot11CountryString OCTET STRING,
	dot11SpectrumManagementImplemented TruthValue,
	dot11SpectrumManagementRequired TruthValue,
	dot11RSNAOptionImplemented TruthValue,
	dot11RSNAPreauthenticationImplemented TruthValue,
	dot11RegulatoryClassesImplemented TruthValue,
	dot11RegulatoryClassesRequired TruthValue,
	dot11QosOptionImplemented TruthValue,
	dot11ImmediateBlockAckOptionImplemented TruthValue,
	dot11DelayedBlockAckOptionImplemented TruthValue,
	dot11DirectOptionImplemented TruthValue,
	dot11APSDOptionImplemented TruthValue,
	dot11QAckOptionImplemented TruthValue,
	dot11QBSSLoadOptionImplemented TruthValue,
	dot11QueueRequestOptionImplemented TruthValue,
	dot11TXOPRequestOptionImplemented TruthValue,
	dot11MoreDataAckOptionImplemented TruthValue,
	dot11AssociatedinNQBSS TruthValue,
	dot11DLSAllowdInQBSS TruthValue,
	dot11DLSAllowed TruthValue,
	dot11AssociateStation MacAddress,
	dot11AssociateID INTEGER,
	dot11AssociateFailStation MacAddress,
	dot11AssociateFailStatus INTEGER,
	dot11ReassociateStation MacAddress,
	dot11ReassociateID INTEGER,
	dot11ReassociateFailStation MacAddress,
	dot11ReassociateFailStatus INTEGER,
	dot11RadioMeasurementCapable TruthValue,
	dot11RadioMeasurementEnabled TruthValue,
	dot11RadioMeasurementProbeDelay INTEGER,
	dot11MeasurementPilotReceptionEnabled TruthValue,
	dot11MeasurementPilotTransmissionEnabled TruthValue,
	dot11MeasurementPilotTransmissionVirtualApSetEnabled TruthValue,
	dot11MeasurementPilotPeriod INTEGER,
	dot11LinkMeasurementEnabled TruthValue,
	dot11NeighborReportEnabled TruthValue,
	dot11ParallelMeasurementsEnabled TruthValue,
	dot11TriggeredMeasurementsEnabled TruthValue,
	dot11RepeatedMeasurementsEnabled TruthValue,
	dot11MeasurementPauseEnabled TruthValue,
	dot11QuietIntervalEnabled TruthValue,
	dot11PassiveBeaconMeasurementEnabled TruthValue,
	dot11ActiveBeaconMeasurementEnabled TruthValue,
	dot11TableBeaconMeasurementEnabled TruthValue,
	dot11ReportingConditionsEnabled TruthValue,
	dot11FrameMeasurementEnabled TruthValue,
	dot11ChannelLoadEnabled TruthValue,
	dot11NoiseHistogramEnabled TruthValue,
	dot11StatisticsReportEnabled TruthValue,
	dot11LCIReportEnabled TruthValue,
	dot11TransmitStreamMeasurementEnabled TruthValue,
	dot11APChannelReportEnabled TruthValue,
	dot11AnnexQMIBSupportEnabled TruthValue,
	dot11NonOperatingChannelMeasurementsEnabled TruthValue,
	dot11MaximumMeasurementDuration Unsigned32,
	dot11MeasurementPilotSupport Unsigned32,
	dot11FastBSSTransitionImplemented TruthValue,
	dot11RRMCivicMeasurementEnabled TruthValue,

dot11RRMIdentifierMeasurementEnabled TruthValue,

dot11WirelessManagementImplemented TruthValue, 
	dot11MaxIdlePeriod 			INTEGER,
	dot11TIMBroadcastInterval     INTEGER,
	dot11TIMBroadcastOffset       INTEGER}

Insert the following elements at the end of the dot11StationConfigTable element definitions:

- last entry in 802.11-2007 has order 42. 11k adds 42-56. 11r adds 57. 11y adds none. 11n adds 1 entry (58). 11s adds none.

dot11RRMCivicMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that dot11RadioMeasurementEnabled is

TRUE and that the station capability for Location Civic Measurement is enabled. FALSE

indicates the station has no Location Civic Measurement capability or that the capability

is present but is disabled. The default value of this attribute is

FALSE."

::= { dot11StationConfigEntry XX }

dot11RRMIdentifierMeasurementEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that dot11RadioMeasurementEnabled is

TRUE and that the station capability for Location Identifier Measurement is enabled. FALSE

indicates the station has no Location Identifier Measurement capability or that the capability

is present but is disabled. The default value of this attribute is

FALSE."

::= { dot11StationConfigEntry XX }

Insert the following element at the end of the list of child objects for dot11smt object identifier:

--***********************************************************

--*Wireless Network Management

--************************************************************

-- dot11WirelessMgmtOptionsTable	::= { dot11smt 17}

Insert the following text at the end of the Station management MIB:

-- **********************************************************************

-- * dot11WirelessMgmtOptions TABLE

-- **********************************************************************

dot11WirelessMgmtOptionsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WirelessMgmtmOptionsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

“Wireless Management attributes. In tabular form to

allow for multiple instances on an agent. This table

only applies to the interface if dot11WirelessManagementImplemented

is set to TRUE in the dot11StationConfigTable. Otherwise

this table should be ignored.”

::= { dot11smt 17 }

dot11WirelessMgmtOptionsEntry OBJECT-TYPE

SYNTAX Dot11WirelessMgmtOptionsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WirelessMgmtOptionsTable. It is

possible for there to be multiple IEEE 802.11 interfaces

on one agent, each with its unique MAC address. The

relationship between an IEEE 802.11 interface and an

interface in the context of the Internet-standard MIB is

one-to-one. As such, the value of an ifIndex object

instance can be directly used to identify corresponding

instances of the objects defined herein.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11WirelessMgmtOptionsTable 1 }

Dot11WirelessMgmtOptionsEntry ::=

SEQUENCE {

dot11MgmtOptionLocationEnabled				TruthValue,

dot11MgmtOptionFBMSImplemented				TruthValue,

dot11MgmtOptionFBMSEnabled				TruthValue,

dot11MgmtOptionEventsEnabled				TruthValue,

dot11MgmtOptionDiagnosticsEnabled				TruthValue,

dot11MgmtOptionMultiBSSIDImplemented				TruthValue,

dot11MgmtOptionMultiBSSIDEnabled				TruthValue,

dot11MgmtOptionTFSImplemented 				TruthValue,

dot11MgmtOptionTFSEnabled				TruthValue,

dot11MgmtOptionSleepModeImplemented				TruthValue, 																			 			dot11MgmtOptionSleepModeEnabled				TruthValue,

dot11MgmtOptionTIMBroadcastImplemented				TruthValue,

dot11MgmtOptionTIMBroadcastEnabled				TruthValue,

dot11MgmtOptionProxyARPEnabled				TruthValue,

dot11MgmtOptionBSSTransitionEnabled				TruthValue,

dot11MgmtOptionTrafficGenerationImplemented				TruthValue,

dot11MgmtOptionTrafficGenerationEnabled				TruthValue,

dot11MgmtOptionACStationCountImplemented				TruthValue,

dot11MgmtOptionACStationCountEnabled				TruthValue,

dot11MgmtOptionMulticastDiagnosticsEnabled				TruthValue,
		dot11MgmtOptionCLIReportingEnabled				TruthValue,

dot11MgmtOptionMotionDetectionImplemented		TruthValue,

dot11MgmtOptionMotionDetectionEnabled		TruthValue,

dot11MgmtOptionTODImplemented 			TruthValue,

dot11MgmtOptionTODEnabled			TruthValue,

dot11MgmtOptionTimingMsmtImplemented 		TruthValue,

dot11MgmtOptionTimingMsmtEnabled			TruthValue}

dot11MgmtOptionLocationEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the

capability of the station to provide location is

enabled. The capability is disabled, otherwise.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 1 }

dot11MgmtOptionFBMSImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station

implementation is capable of supporting FBMS when the dot11WirelessManagementImplemented is TRUE.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 2 }

dot11MgmtOptionFBMSEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the

capability of the station to provide FBMS is

enabled. The capability is disabled, otherwise.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 3 }

dot11MgmtOptionEventsEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the

capability of the station to provide Event Reporting is

enabled. The capability is disabled, otherwise.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 4 }

dot11MgmtOptionDiagnosticsEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the

capability of the station to provide Diagnostic Reporting is

enabled. The capability is disabled, otherwise.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 5 }

dot11MgmtOptionMultiBSSIDImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station

implementation is capable of supporting Multiple BSSID when the dot11WirelessManagementImplemented is TRUE.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 6 }

dot11MgmtOptionMultiBSSIDEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the

capability of the station to provide Multi BSSID is

enabled. The capability is disabled, otherwise.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 7 }

dot11MgmtOptionTFSImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting TFS when the dot11WirelessManagementImplemented is TRUE.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 8 }

dot11MgmtOptionTFSEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that TFS is enabled. TFS is disabled otherwise.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 9 }

dot11MgmtOptionSleepModeImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting Sleep Mode when the dot11WirelessManagementImplemented is TRUE.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 10 }

dot11MgmtOptionSleepModeEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that Sleep Mode is enabled. Sleep Mode is disabled otherwise.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 11 }

dot11MgmtOptionTIMBroadcastImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting TIM Broadcast when the dot11WirelessManagementImplemented is TRUE.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 12}

dot11MgmtOptionTIMBroadcastEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that TIM broadcast is enabled. TIM broadcast is disabled otherwise.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 13}

dot11MgmtOptionProxyARPEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability of the AP to provide the Proxy ARP service is enabled. The capability is disabled, otherwise. The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 14 }

dot11MgmtOptionBSSTransitionEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability of the station to provide BSS Transition is enabled. The capability is disabled, otherwise. The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 15 }

dot11MgmtOptionTrafficGenerationImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting Traffic Generation when the dot11WirelessManagementImplemented is TRUE. The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 16 }

dot11MgmtOptionTrafficGenerationEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability of the station to provide Traffic Generation is enabled. Traffic Generation is disabled otherwise. The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 17 }

dot11MgmtOptionACStationCountImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station implementation is capable of supporting AC Station Count when the dot11WirelessManagementImplemented is TRUE. The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 18 }

dot11MgmtOptionACStationCountEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability of the station to provide AC Station Count is enabled. AC Station Count is disabled otherwise. The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 19 }

dot11MgmtOptionMotionDetectionImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station

implementation is capable of supporting motion detection when the dot11WirelessManagementImplemented is TRUE.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 20 }

dot11MgmtOptionMotionDetectionEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability to support motion detection is enabled. The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 21 }

dot11MgmtOptionTODImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station

implementation is capable of supporting Time Of Departure for transmitted Clause 15, 17, 18, 19 and 20 frames when the dot11WirelessManagementImplemented is TRUE.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 22 }

dot11MgmtOptionTODEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the capability to support Time Of Departure frames for transmitted Clause 15, 17, 18, 19 and 20 frames is enabled.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 23 }

dot11MgmtOptionTimeMsmtImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station

implementation is capable of supporting Timing Measurement capability when the dot11WirelessManagementImplemented is TRUE.

The default value of this attribute is FALSE."

::= { dot11WirelessMgmtOptionsEntry 24 }

dot11MgmtOptionTimeMsmtEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the station capability for Timing Measurement is enabled. FALSE indicates the station has no Timing Measurement capability or that the capability is present but is disabled. The default value of this attribute is FALSE"

::= { dot11WirelessMgmtOptionsEntry 25 }

-- ********************************************************************

-- * dot11LocationServices TABLE

-- ********************************************************************

dot11LocationServices OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11LocationServices

MAX-ACCESS READ-WRITE

STATUS current

DESCRIPTION

"Group contains conceptual table of attributes for

Wireless Network Management LocationServices."

::= { dot11smt xx }

dot11LocationServicesEntry OBJECT-TYPE

SYNTAX Dot11LOCATIONSERVICESEntry

MAX-ACCESS READ-WRITE

STATUS current

DESCRIPTION

"An entry in the dot11LOCATIONSERVICESTable

Indexed by dot11LOCATIONSERVICESIndex."

INDEX { dot11LOCATIONSERVICESIndex }

::= { dot11LOCATIONSERVICESTable 1 }

Dot11LOCATIONSERVICESEntry ::=

SEQUENCE {

dot11LOCATIONSERVICESIndex Unsigned32,

dot11LOCATIONSERVICESMACAddress MacAddress,

dot11LOCATIONSERVICESLocationIndicationParametersReportIntervalUnits INTEGER,

dot11LOCATIONSERVICESLocationIndicationParametersNormalReportInterval INTEGER,

dot11LOCATIONSERVICESLocationIndicationParametersNormalFramesperChannel INTEGER,

dot11LOCATIONSERVICESLocationIndicationParametersInMotionReportInterval INTEGER,

dot11LOCATIONSERVICESLocationIndicationParametersInMotionFramesperChannel INTEGER,

dot11LOCATIONSERVICESLocationIndicationParametersBurstInterframeInterval INTEGER,

dot11LOCATIONSERVICESLocationIndicationChannelsChannel OCTET STRING,

dot11LOCATIONSERVICESLocationStatus INTEGER,

dot11LOCATIONSERVICESRadioInformationTransmitPower INTEGER,

dot11LOCATIONSERVICESRadioInformationAntennaID INTEGER,

dot11LOCATIONSERVICESRadioInformationAntennaGain INTEGER,

dot11LOCATIONSERVICESRadioInformationReceivedRSNI INTEGER,

dot11LOCATIONSERVICESRadioInformationRCPI INTEGER,

dot11LOCATIONSERVICESTimingMeasurementsTimestampDifference OCTET STRING,

dot11LOCATIONSERVICESTimingMeasurementsTimestampDifferenceUnits INTEGER,

dot11LOCATIONSERVICESTimingMeasurementsTimestampDifferenceAccuracy INTEGER,

dot11LOCATIONSERVICESTimingMeasurementsIngressTimestamp OCTET STRING,

dot11LOCATIONSERVICESMotionIndicator INTEGER,

dot11LOCATIONSERVICESMotionSpeed INTEGER

dot11LOCATIONSERVICESLocationDataAccuracyEstimate INTEGER,

dot11LOCATIONSERVICESLocationDataLocationValue DisplayString,

dot11LOCATIONSERVICESIndex OBJECT-TYPE

SYNTAX Unsigned32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute is the Index for LOCATIONSERVICES Report elements in dot11LOCATIONSERVICESTable,

greater than 0."

::= { dot11LOCATIONSERVICESEntry 1 }

dot11LOCATIONSERVICESMACAddress OBJECT-TYPE

SYNTAX MacAddress

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute is the MAC address of the STA reporting location information."

::= { dot11LOCATIONSERVICESEntry 2 }

dot11LOCATIONSERVICESLocationIndicationParametersReportIntervalUnits OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Location Indication Parameters Report Interval Units value."

::= { dot11LOCATIONSERVICESEntry 3 }

dot11LOCATIONSERVICESLocationIndicationParametersNormalReportInterval OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Location Indication Parameters Normal Report Interval value."

::= { dot11LOCATIONSERVICESEntry 4 }

dot11LOCATIONSERVICESLocationIndicationParametersNormalFramesperChannel OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Location Indication Parameters Normal Frames per Channel value."

::= { dot11LOCATIONSERVICESEntry 5 }

dot11LOCATIONSERVICESLocationIndicationParametersInMotionReportInterval OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Location Indication Parameters in-motion Report Interval value."

::= { dot11LOCATIONSERVICESEntry 6 }

dot11LOCATIONSERVICESLocationIndicationParametersInMotionFramesperChannel OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Location Indication Parameters in-motion Frames per Channel value."

::= { dot11LOCATIONSERVICESEntry 7 }

dot11LOCATIONSERVICESLocationIndicationParametersBurstInterframeInterval OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Location Indication Parameters inter-frame Interval value."

::= { dot11LOCATIONSERVICESEntry 8 }

dot11LOCATIONSERVICESLocationIndicationChannels OBJECT-TYPE

SYNTAX OCTET STRING (SIZE (2..254))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Location Indication Channels Channel and Regulatory Class octet fields."

::= { dot11LOCATIONSERVICESEntry 9 }

dot11LOCATIONSERVICESLocationStatus OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Location Status value."

::= { dot11LOCATIONSERVICESEntry 11 }

dot11LOCATIONSERVICESRadioInformationTransmitPower OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Radio Information Transmit Power value."

::= { dot11LOCATIONSERVICESEntry 14 }

dot11LOCATIONSERVICESRadioInformationAntennaID OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Radio Information Antenna ID value."

::= { dot11LOCATIONSERVICESEntry 15 }

dot11LOCATIONSERVICESRadioInformationAntennaGain OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Radio Information Antenna Gain value."

::= { dot11LOCATIONSERVICESEntry 16 }

dot11 LOCATIONSERVICESRadioInformationReceivedRSNI OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Radio Information Received RSNI value.

::= { dot11LOCATIONSERVICESEntry 17 }

dot11LOCATIONSERVICESRadioInformationRCPI  OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute conains the Radio Information RCPI value."

::= { dot11LOCATIONSERVICESEntry 18 }

dot11LOCATIONSERVICESTimingMeasurementsTimestampDifference  OBJECT-TYPE

SYNTAX OCTET STRING (SIZE (4))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Timing Measurements Timestamp Difference value."

::= { dot11LOCATIONSERVICESEntry 19 }

dot11LOCATIONSERVICESTimingMeasurementsTimestampDifferenceUnits  OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Timing Measurements Timestamp Difference Units value."

::= { dot11LOCATIONSERVICESEntry 20 }

dot11LOCATIONSERVICESTimingMeasurementsTimestampDifferenceAccuracy  OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Timing Measurements Timestamp Difference Accuracy value."

::= { dot11LOCATIONSERVICESEntry 21 }

dot11LOCATIONSERVICESTimingMeasurementsIngressTimestamp  OBJECT-TYPE

SYNTAX OCTET STRING (SIZE (12))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Timing Measurements Ingress Timestamp value."

::= { dot11LOCATIONSERVICESEntry 22 }

dot11LOCATIONSERVICESMotionIndicator OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Motion Indicator value."

::= { dot11LOCATIONSERVICESEntry 23 }

dot11LOCATIONSERVICESMotionSpeed OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Motion Speed value."

::= { dot11LOCATIONSERVICESEntry 24 }

dot11LOCATIONSERVICESLocationDataAccuracyEstimate  OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Location Data Accuracy Estimate value."

::= { dot11LOCATIONSERVICESEntry 28 }

dot11LOCATIONSERVICESLocationDataLocationValue  OBJECT-TYPE

SYNTAX OCTET STRING (SIZE (3..254))
	MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Location data value."

::= { dot11LOCATIONSERVICESEntry 29 }

dot11LOCATIONSERVICESLocationSourceIdentifierTimeZoneOffset  OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Location Source Identifier Time Zone Offset value."

::= { dot11LOCATIONSERVICESEntry 30 }

dot11LOCATIONSERVICESLocationSourceIdentifierLocationTimestamp  OBJECT-TYPE

SYNTAX OCTET STRING (SIZE (6))

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Location Source Identifier Location Timestamp value."

::= { dot11LOCATIONSERVICESEntry 31 }

dot11LOCATIONSERVICESLocationSourceIdentifierPublicIdentifierLink OBJECT-TYPE

SYNTAX DisplayString

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute contains the Public Identifier Link value."

::= { dot11LOCATIONSERVICESEntry 32 }