IEEE P802.11
Wireless LANs

| LB124: TxBF Comment Resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-04-22 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Bjorn A. Bjerke | Qualcomm, Inc. | 9 Damonmill Sq. Suite 2A Concord, MA 01742, USA | +1 781-276-0912 | bbjerke@qualcomm.com |

Abstract

This submission proposes resolutions to CIDs 6100, 6105, 6192, and 6288. The proposed modifications are based on D4.0.

| 6100 | 74.10 | 10 | 7.3.2.57.6 | This bothered me for a while, so I'll throw in a comment: this field has two bits that could be confused: "Implicit Transmit Beamforming Receiving Capable" and "Implicit Transmit Beamforming Capable" | Change "Implicit Transmit Beamforming Capable" to "Implicit Transmit Beamforming Transmitting Capable" for symmetry. | Reject. The definitions clearly differentiate between the two fields. Symmetry is therefore not needed. Refer to 08/468r2. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Reject.

The definitions clearly differentiate between the two fields. Symmetry is therefore not needed.

| 6105 | 159.21 | 21 | 9.17 | "NOTE—The maximum number of space time streams for which channel coefficients can be simultaneously estimated during the data portion of the HT long training fields is limited by 1) the Rx MCS Bitmask subfield of the Supported MCS Set field and 2) the Rx STBC subfield, both of the HT Capabilities element, and" and what? | Finish the sentence. | Counter. Refer to CID 6285. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter.

This is a typo left hanging from a previous draft. Resolved by the same resolution as EDITOR comment CID 6285.

| 6192 | 162.53 | 53 | 9.19.2.1 | In table 9-8, required support for beamformee, the second paragraphs says either transmit staggered sounding or transmit NDP must be supported by beamformee. The third paragraph says supporting squared soundings (SOUNDING=1 and NUM_EXTEN_SS=0, and non NDP) is required, regardless of transmitting staggerred orNDP capabilities. Since squared sounding is neither staggerred nor NDP, these two paragraphs are contradictory. It makes no sense that the beamformee has to support one of staggerred and NDP soundings. | remove the second paragraph. | Counter. Refer to 08/468r2. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Counter.

Support for square sounding is required, whereas support for staggered or NDP sounding is not. Delete the paragraph in question, and, for consistency, also a similar paragraph in the Beamfomer section of the same table.

TGn Editor: on page 162, Table 9-8, delete 1) the paragraph that begins on line 41 and ends on line 43, and 2) the paragraph that begins on line 57 and ends on line 59.

| 6288 | 171.52 | 52 | 9.19.3 | "— If NDP sounding is used, the transmission of the feedback response frame may follow the NDP, but the control response frame is transmitted a SIFS after reception of the PPDU that elicited the control response." This is redundant to the first list item. | Delete cited text. | Accept. Refer to 08/468r2. |
| --- | --- | --- | --- | --- | --- | --- |

Suggested resolution: Accept.

Remove redundancy.

TGn Editor: on page 171, delete the paragraph that begins on line 52 and ends on line 55.