IEEE P802.11
Wireless LANs

| Normative Text for Extended Airtime Link Metric |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-05-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Yongho Seok | LG Electronics | 16 Woomyeon-Dong, Seocho-Gu, Seoul 137-724, Korea | +8225264225 | yhseok@lge.com |
| Guenael Strutt | Motorola | 1064 Greenwood Blvd – Lake Mary, FL 32771 -- USA | +14075624050 | Guenael.Strutt@motorola.com |
| Alexander Safonov | IITP RAS | 19 Bolshoy Karetny per, Moscow, 127994, Russia | +74956503617 | safa@iitp.ru |

Abstract

This document is a normative text proposal to provide comment resolution for TGs LB 126 comment 1 and 1150. Changes are shown with respect to TGs draft D2.0.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Change 11B.8 as shown

- Airtime link metric computation procedures

In order to compute the forwarding table for individually addressed frames, the MP shall first calculate the link metric for each pairwise link to its neighbor peer MPs in the Mesh.  This subclause defines a default link metric that may be used by a path selection protocol to identify an efficient radio-aware path. The extensibility framework allows this metric to be overridden by any path selection metric as specified in the active profile.

The default link metric is the airtime metric. Airtime reflects the amount of channel resources consumed by transmitting the frame Nt frames over a particular link. This measure is approximate and designed for ease of implementation and interoperability.

The airtime for each link is calculated as, according to the supporting MAC feature:

Where Oc, Oa , Nt and Bt are constants listed in Error: Reference source not found, and the input parameters r and Ef are the data rate in Mb/s and the frame error rate for the single MSDU, or for the A-MSDU, or for an MPDU of the A-MPDU, respectively. The rate r represents the data rate at which the MP would transmit a frame of standard size Bt based on current conditions and its estimation is dependent on local implementation of rate adaptation. Ef is calculated as indicated in Table s57, and ef is the probability that when a frame of standard size Bt is transmitted at the current transmission bit rate r, the frame is corrupted due to transmission error; its estimation is a local implementation choice. Frame drops due to exceeding TTL should not be included in this estimate as they are not correlated with link performance.

The airtime link metric shall be measured in increments of 10.24 microseconds, or one hundredth of a TU.

| - Airtime cost parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| Parameter | No Aggregation | MSDU Aggregation | MPDU Aggregation | Description |
| Oc | Nt Channel access overhead | Channel access overhead | Channel access overhead | Channel access overhead, which varies depending on PHY and includes frame headers, training sequences, access protocol frames |
| Oa | 0 | A-MSDU subframe header | MPDU delimiter | Aggregation overhead |
| Nt | 4 | 4 | 4 | Number of test frames for calculating the airtime cost |
| Bt | 8192 | 8192 | 8192 | Number of bits in test frame |
| Ef | ef |  | e f | Frame error rate for single MSDU, or A-SDU, or Aggregated MPDU, respectively |

Parameters of the Airtime Link Metric for Extensible Path Selection Framework gives the parameters of the airtime link metric for the Extensible Path Selection Framework.

| - Parameters of the Airtime Link Metric for Extensible Path Selection Framework |  |
| --- | --- |
| Path Selection Metric ID | See Table s6 in 7.3.2.81.2. |
| Data type | Unsigned integer, |
| Length of metric field | 4 octets |
| Operator for metric aggregation | addition ( + ) |
| Comparison operator | less than, equal to, greater than as used with integers - metric a is better than metric b iff a < b - metric a is equal to metric b iff a = b - metric a is worse than metric b iff a > b |
| Initial value of path metric | 0 |