IEEE P802.11
Wireless LANs

| Higher Throughput MACs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Bill Marshall | AT&T Labs Research | 180 Park Ave, Florham Park, NJ 07932 | 973-360-8718 | wtm@research.att.com |

Abstract

This submission contains all the draft D6.03 changes needed to resolve LB134 CIDs 9066, 9068, 9069, 9070, 9301, 9303, and various other unaccepted comments from prior letter ballots.

# Introduction

This submission contains all the draft D6.03 changes needed to resolve LB134 CIDs 9066, 9068, 9069, 9070, 9301, 9303, and various other unaccepted comments from prior letter ballots.

MSWord “Track Changes” is used in this submission to show the changes to TGn draft P802.11n_D6.03.

# Draft P802.11n-D6.03 changes

7.1.3.1.9 Order field (#978)

Change 7.1.3.1.9 as follows (#979, 5547):

The Order field is 1 bit in length and is set to 1 in any non-QoS data frame that contains an MSDU, or fragment thereof, which is being transferred using the StrictlyOrdered service class. This field is set to 0 in all other frames. All QoS STAs set this subfield to 0.

The Order field is 1 bit in length.

It is used for two purposes:

— When set to 1 in a non-QoS data frame transmitted by a non-QoS STA, it indicates that the frame contains an MSDU, or fragment thereof, which is being transferred using the StrictlyOrdered service class.

— When set to 1 in a QoS data or management frame

, it indicates that the frame contains an HT Control field.

Otherwise the Order field is set to 0.

7.3.2.27 Extended Capabilities information element

Table 7-35a – Capabilities field

| Bit | Information | Notes |
| --- | --- | --- |
| <ANA> | PSMP Capability | This bit in the Extended Capabilities information element is set to 1 if the STA supports PSMP operation described in 9.16. |
| <ANA> | A-MSDU Capability | This bit in the Extended Capabilities information element is set to 1 if the STA supports A-MSDU. |
| <ANA> | Extended Block Ack Capability | This bit in the Extended Capabilities information element is set to 1 if the STA supports Compressed bitmap and Multi-TID BlockAck variants. |

9.7c A-MSDU operation

A STA (#7228)shall not transmit an A-MSDU to a STA from which it has not received an indication of support of A-MSDUs.

9.10.6 Selection of BlockAck and BlockAckReq variants (#5565)

The Compressed Bitmap subfield of the BA Control field or BAR Control field (#5565) shall be set to 1 in all BlockAck and BlockAckReq frames (#2292) sent from one HT STA to another HT STA

.

The Multi-TID subfield of the BA Control field shall be set to 1 in all BlockAck frames related to an HTimmediate agreement transmitted inside a PSMP sequence, and shall be set to 0 otherwise. The Multi-TID subfield of the BAR Control field shall be set to 1 in all BlockAckReq frames related to an HT-immediate agreement transmitted inside a PSMP sequence.

# LB134 Comment Resolutions

Resolve LB134 CIDs 9066, 9068, 9069, 9070, 9301, 9303 as “Counter. Text changes given in 08/1084r0”