IEEE P802.11
Wireless LANs

| TGn LB134 Submission for CIDs 9149 and 9185 in COEX ad-hoc |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-09-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ali Raissinia | Qualcomm Incorporated |  |  | alirezar@qualcomm.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB134 comment:

- 9149 and 9185

The changes marked in this document are based on TGn Draft version 6.03.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

TGn Editor: Please delete all those indicating the related CID as "(# XXXX)" after appropriate time.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# 9149 (R), 9185 (R)

# CID 9149

| CID | Page | Clause | Comment | Proposed change |
| --- | --- | --- | --- | --- |
| 9149 | 230.00 | 111.14.9 | The draft states "the STA may transmit a pending 40 MHz mask PPDU only if the secondary channel has also been idle during the times the primary channel CCA is performed (defined in 9.2.10) during an interval of a PIFS immediately preceding the expiration of the backoff counter." The PIFS duration is chosen so that the time is longer than SIFS period (which is used for frame exchange sequence) plus the uncertainty between primary and secondary MAC clock as well as possible detection error on "absence of energy". If there's no mention of PIFS "using long timeslot" then the PIFS would be 19us (case where secondary channel consisted of 11.b only devices). The 19us is short to be used as ACK time-out value. | Add the phrase "(using long timeslot)" right after the word " PIFS". |

# CID 9185

| CID | Page | Clause | Comment | Proposed change |
| --- | --- | --- | --- | --- |
| 9185 | 230.57 | 11.14.10 | By deleting "(using short timeslot for 5 GHz band and long timeslot for 2.4 GHz band)" after PIFS, the PIFS could now be 10 us SIFs plus a short slot time of 9 us is a total of 19 us. In my opinion this is too short to make sure to not stap on a data - ack sequence on the secondary channel. | Include "(using short timeslot for 5 GHz band and long timeslot for 2.4 GHz band)" after PIFS again, or change PIFS to DIFS, or introduce a fixed time of, say, 25 us |

Note: The place that the commenter is referring to is 11.14.9 not 11.14.10.

## Proposed resolutions to CIDs 9149 and 9185:

Counter.  Modify sensing interval as DIFS for 2.4 GHz Band and PIFS for 5 GHz band

Instruction to TGn Editer:

Edit following text in section 11.14.9 as following:

At the specific slot boundaries (defined in 9.2.10) determined by the STA based on the 20

MHz primary channel CCA, when the transmission begins a TXOP using EDCA (as described in

9.9.1.3), the STA may transmit a pending 40 MHz mask PPDU only if the secondary channel has also been idle during the times the primary channel CCA is performed (defined in 9.2.10) during an interval of a PIFS for 5 GHz band and DIFS for 2.4 GHz band immediately preceding the expiration of the backoff counter. If a STA was unable to transmit a 40 MHz mask PPDU because the secondary channel was occupied during this PIFS interval, it may:

## References:

[1] Draft P802.11n_D6.03.pdf

[2] IEEE Std 802.11TM-2007