IEEE P802.11
Wireless LANs

| Timing Information Element |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-01-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| R. Roy | Affiliation: Connexis |  | 650-861-3351 | dickroy@alum.mit.edu |

Abstract

This document contains a preliminary description of the timing information element which is optionally included in Timing Advertisement management frames and proposes changes to the text of 11p D5.0.  (DRAFT)

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Change table 7-18b to the Timing IE as shown below:

| Order | Information | Notes |
| --- | --- | --- |
| 1 | Timestamp |  |
| 2 | Capability |  |
| 3 | Timing IE | ID = <ANA> Optional |
| 4 | Supported Rates IE | ID=1 Optional |
| 5 | Country IE | ID=7 Included if dot11MultiDomainCapabilityEnabled or dot11SpectrumManagementRequired is true. Optional |
| 6 | EDCA IE | ID=22 Optional |
| 7 | Higher-layer information element(s) (HLIE) (s) | ID=69 Optional (Zero or more HLIEs may be included in one frame. No HLIEs may be included if the frame is intended to transmit timing information only.) |
| 8 | Extended Capabilities IE | ID=127 Optional |
| Last | Vendor Specific | One or more vendor-specific information elements may appear in this frame. This information element follows all other information elements. |

Insert the following new subclause  7.3.2.b at the end of subclause 7.3.2:

7.3.2.b Timing Information element (TIE)

The TIE is optionally present in Timing and Information management frames.  The TIE format is shown in Figure 7-95a2.  The element ID is set to <ANA>.  The value of the length field varies with and determines the number of parameters included.

| Element ID | Length | Content |
| --- | --- | --- |

Octets		1		        1                  Variable (1-255)

Figure 7-95a2—Timing Information element format

The format of the content field of the TIE is shown in Figure 7-95a3. It contains information describing the source of time in the Timing capabilities field as well as an estimate of the parameters, and their related estimate error covariance, of a polynomial model that relates TSF timer values to those of the external clock.  The numbers in parentheses are the size of the field in octets.

| Timing capabilities (1) | TSF timer offset estimate (TTOE) in nsec (10) | TTOE error std dev in nsec (5) |  |  |
| --- | --- | --- | --- | --- |
| TSF timer t0 reference (8) | TSF timer frequency offset estimate (TTFOE) in nsec/sec (4) | TTFOE error std dev (2) | LTTFOE-TTOE (2) |  |
|  | TSF timer frequency drift estimate (TTFDE) in nsec/sec/sec (4) | TTFDE error std dev (2) | LTTFDE-TTOE (2) | LTTFDE-TTFOE (2) |

Figure 7-95a3—Content field format

This information can be used by recipients of Timing and Information frames to estimate corrections to their internal TSF timer to estimate the current time of the external clock on the STA transmitting the TIE in a Timing and Information frame. The content field contains a timing capabilities field (1 octet), and a variable number of subsequent fields.

| Time Source | Availability of Time Source | Reserved |
| --- | --- | --- |

bits		0-2		        3                            4-7

Figure 7-95a4—Timing capabilities field format

The timing capabilities field is a single-byte (octet) field used to inform recipients of the TIE of the sender’s source of external time information as well as the current condition of the timing estimator at the sender. Bits 0-2 of this field shall be set to 0 if there is no source of external time associated with the Timing information.  Bits 0-2 are set to 0b001 if UTC is the external source (e.g., if the STA has a GPS unit onboard capable of generating UTC time).  Other values for bits 0-2 are currently reserved for future use. Bit 3 of this field shall be set to 1 if this time source is currently available and being used to estimate time, and 0 if not. Bits 4–7 are reserved for future enhancements.

The TSF timer offset field is an 10-byte field and shall contain a TSF timer offset estimate (TTOE) as a 2’s complement integer in nanoseconds, which when added to the STA’s TSF timer value yields an estimate of the STA’s best estimate of external time (e.g., UTC time) at the instant the first bit of the Timestamp in the Timing and Information frame is transmitted from that STA’s antenna.

The TSF timer standard deviation field is a 5-byte field containing an estimate of the standard deviation of the error in the STA’s estimate of external time (including biases that have not been calibrated out) in unsigned integer format in units of nanoseconds. The TSF timer standard deviation field shall be set to its maximum value (2^(40)-1) to indicate that the value of the TSF timer offset field is not meaningful and should not be used to estimate UTC time.  If only the TTOE is to be included in the TIE, the length field is set to 16.

Optionally, a TSF timer frequency offset estimate (TTFOE) may be included in the TIE.  If the TTFOE is included, the next field in the TIE shall contain the value of the transmitting STA's TSF timer to which the TTFOE is being referenced (t0).  The t0 field is an 8-octet field.  The next field then contains the TTFOE and is a 4-octet field containing a 2's complement signed integer

Upon startup, the TSF timer offset shall be set to zero and the TSF timer standard deviation shall be set to its maximum value (2^(40)-1), indicating that the TSF timer offset estimate is not currently valid. Note that once any lower variance source of time becomes available, that source and its error variance can be adopted as initial conditions for the estimator. The value of the TSF timer offset standard deviation, i.e., the error in the estimate of external time (or equivalently, the TSF timer offset for zeroth order estimators) is generally a function of various environmental and implementation factors and may vary with time.

For improved estimates of external time, the Timing Reference Information Element can be extended to include higher order information as follows.  The Timing Reference Information Element can be extended to include a first order correction (i.e., a correction for the difference between the clock frequencies of the external source and the local TSF Timer).  The value in the length field is increased by a multiple of 7 octets.  For first order corrections, a 4-byte TSF Timer frequency offset field is appended and contains an estimate of the difference in the external clock’s rate and the rate of the local TSF Timer.  It is a signed integer in units of microseconds/second.  Following that, a 3-byte unsigned integer, in units of microseconds/second, containing an estimate of the uncertainty in the estimated clock rate difference is appended.  These estimates can be used to yield improved estimates of the external clock.  The implementation details are beyond the scope of this standard.

Insert the following new subclause  11.18.3 after subclause 11.18.2:

11.18.3 Time base synchronization using TIEs

A STA choosing to synchronize to the time base of another STA may do so using the TSF Timer primitives provided along with the information contained in the TIE.  For example, STAs may synchronize to Universal Coordinates Time (UTC) time relative to UTC instant 2004-01-01T00:00:00.000000.  UTC is defined in ITU-R TF.460-4 (1986) [ed: add this reference to the list of references and insert the associated pointer here] and can be obtained externally from many sources including GPS. GPS receivers typically provide a precise 1 pulse per second (PPS) UTC signal (with an error less than 100 ns), and these precise 1 PPS signals can be used for timing and synchronization.  A STA that chooses to synchronize to UTC time may estimate UTC time as the value of the STA’s TSF timer plus any corrections estimated as described in 11.1.1.3.1.  This estimate of UTC time can be used for many purposes including synchronization of channel changes.

STAs that have a source of UTC time (e.g., from a GPS unit) can easily implement an estimator in a variety of ways, all of which are beyond the scope of this standard.  STAs that do not have an internal source of time (UTC or any other) may nonetheless estimate time from Timing and Information frames containing a Timestamp and a Timing Information Element received from STAs that do have an estimate of time (UTC or any other).   While the details of how this can be accomplished are beyond the scope of this standard, it is easy to see that sufficient information is present in the received Timing and Information frames.  The Timestamp field gives an estimate of the value of the TSF timer of the transmitting STA at the moment the first bits of the Timestamps were transmitted out the transducer (antenna).  The TIE provides sufficient information to estimate the necessary corrections to the Timestamp value so that it equals the value of its external time source (which must be the same for all Timing and Information frames obviously, and may be, e.g., UTC time) at the transmission time.  The local timestamp attached to the received Timing and Information frame gives an estimate of the local TSF timer value at the instant of reception of each Timing and Information frame.  This information is clearly sufficient for the receiving STA to produce an estimate (biased if the propagation delays from the transmitters to the receiver is not accounted for properly) of the external clock on the transmitting STAs.  The details of how this is implemented are beyond the scope of this standard, but statistically unbiased (this is a bit difficult to achieve in these applications) and efficient estimators are preferred.  Such estimators also produce estimates of the uncertainties of the parameters they estimate, and these uncertainties are included in the TIE as well.

In the subclauses of clause 10 describing the MLME_TIMING_INFO.request and MLME_TIMING_INFO.indication primitives, add "Timing Information" as a parameter after "Capability Information" and in the tables that follow, add the following row:

| Timing Information | As defined in frame format | As defined in frame format | Timing Information announced by the STA |
| --- | --- | --- | --- |

Motion: Move to accept these changes to P802.11p D4.02 and instruct the editor to update the latest 11p draft to include these changes.

Moved:

Seconded:

YES:

NO:

ABSTAIN: