IEEE P802.11
Wireless LANs

| Normative Text for Amendment for EASN |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ding Zhiming | Huawei | Bldg 7,Vision Business Park, Nan Shan Science Park, Shenzhen P.R. China | +86 755 36835663 | dingzhiming@huawei.com |
| Stephen McCann | Roke Manor Research Ltd | Roke Manor, Old Salisbury Lane, Romsey, SO51 0ZN, UK | +44 1794 833341 | stephen.mccann@roke.co.uk |
| Shu Guiming | Huawei | Bldg 7,Huawei Technologies Vision Business Park, Nan Shan Science Park, Shenzhen P.R. China | +86 755 36835543 | shugm@huawei.com |
| Celine Liu | Huawei | Bldg 7,Vision Business Park, Nan Shan Science Park, Shenzhen P.R. China | +86 755 36835837 | celineliu@huawei.com |
| Amy Zhang | Huawei | Huawei Industry Park, Bantian, Shenzhen, P.R. China |  | Amyzhang@huawei.com |

Abstract

This document is a normative text proposal for EASN. The text is aligned with IEEE P802.11u/D4.01, October, 2008.

- Insert the following definitions in “4. Abbreviations and acronyms” (Page 4)

4. Abbreviations and acronyms

- Insert the following new abbreviations and acronyms into clause 4 in alphabetical order:

802.x LAN	IEEE 802 based local area networks such as 802.3 and 802.11

AAA		authentication, authorization, and accounting

ASRA		additional step required for access

DN		destination network

EAS		emergency alert system

EASN		emergency alert system notification

EBR		expedited bandwidth request

EDT		eastern daylight time

EICI		Emergency Information Change Indication

ES		emergency services

ESO		emergency services only

EST		eastern standard time

GAS		 generic advertisement Service

HESSID 	homogenous ESS identifier

MICS		media independent command service

MIES 		media independent event service

MIH 		media independent handover

MIIS		media independent information service

MLPP		multi-level precedence and preemption

NAI 		network access identifier

PHB		per-hop behavior

PoS		point of service

Add a row into Table 7-42s in subclause 7.3.2.27 (page 18)

7.3.2.27 Extended capabilities information element

Insert the following additional rows at the end of Table 42s.

Table 7-42s—Capabilities field

| Bit(s) | Information | Notes |
| --- | --- | --- |
| <ANA> | Interworking | When dot11InterworkingServiceEnabled is set to TRUE, the Interworking field is set to 1 to indicate the STA supports Interworking Service as described in 11.18. When dot11InterworkingServiceEnabled is set to FALSE, the Interworking field is set to 0 to indicate the STA does not support this capability. |
| <ANA> | QoS Map | When dot11QosMapEnabled is set to TRUE, the QoS Map field is set to 1 to indicate the STA supports QoS Map service as described in 11.18.6. When dot11QosMapEnabled is set to FALSE, the QoS Map field is set to 0 to indicate the STA does not support this capability. |
| <ANA> | EBR | When dot11EBREnabled is set to TRUE, the EBR field is set to 1 to indicate the STA supports EBR as described in 7.3.2.62. When dot11EBREnabled is set to FALSE, the EBR field is set to 0 to indicate the STA does not support this capability. |
| <ANA> | SSPN Interface | When dot11SSPNInterfaceEnabled is set to TRUE, the SSPN Interface field is set to 1 to indicate the AP supports SSPN Interface service as described in 11.18.4. When dot11SSPNInterfaceEnabled is set to FALSE, the SSPN Interface is set to 0 to indicate the AP does not support this capability. Non-AP STAs shall set this field to 0. |
| <ANA> | EASN | When dot11EASNEnabled is set to TRUE, the EASN field is set to 1 to indicate the STA supports EASN mechanism as described in 11.18.5. When dot11EASNEnabled is set to FALSE, the EASN field is set to 0 to indicate the STA does not support this capability. |

- Modify Figure 7-95ao in subclause 7.3.2.59  (Page 19)as follows:

7.3.2.59 Interworking information element

The Interworking information element contains information about the interworking service capabilities of a STA as shown in Figure 7-95ao.

| - Interworking element format |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B0 - B3 | B4 | B5 | B6 | B7 |  |  |  |  |
|  | Element ID | Length | Network Type | Internet | ASRA | EASN Available | Emergency Network | Venue Group | 802.11 Venue Type | HESSID (optional) | EICI (optional) |
| Bits |  |  | 4 | 1 | 1 | 1 | 1 |  |  |  |  |
| Octets: | 1 | 3 or 9 |  |  |  |  |  | 1 | 1 | 0 or 6 | 1 |

- Figure 7-95ao—Interworking element format

The value of the Length field is the length of the Interworking capabilities field. The value of the length field is 3 if no HESSID is present or 9 if a HESSID is present.

The Network Types are shown in Table 7-43r. A non-AP STA uses this field to indicate the desired Network Type in an active scan. See U.4 for informative text on usage of fields contained within the Interworking element.

- Modify the text in Clause 7.3.2.59 (Page 20-23) as follows:

- Table 7-43r—Network Type codes

Bit 4 is the Internet field. If this field is set to 1, the network provides connectivity to the Internet; otherwise it is set to 0 indicating that it is unspecified whether the network provides connectivity to the Internet. If the 802.11 Network Type is set to 2, 3, or 14, then this field is set to 1 when the network provides connectivity to the Internet, and it is set to 0 when the network provides unspecified connections to external networks. When the 802.11 Network Type is set to a value other than 2, 3, or 14, this field is always set to 0.

Bit 5 is the Additional Step Required for Access (ASRA) field. It is set to 1 to indicate that the network requires a further step for access. For more information, refer to Native Query Protocol Native Info Network Authentication Type Information in 7.3.3.5.

Bit 6 is the Emergency Alert System Notification (EASN) Available field. It is set to 1 to indicate that a higher layer EAS is available at the AP for download. It is a reserved field for the non-AP STA. The EASN Available field set to 0 indicates that no higher layer EAS is available at the AP for download. When the field is changed to 1 from 0, a non-AP STA requests further information from the higher layers to receive the full EAS information. This information is transported over GAS using a suitable advertisement protocol as described in 11.18.5. The EASN Available field is set to 1 while EAS messages are active.

EAS protocols define when messages are active.

Bit 7 is the Emergency Network field. When set to 0, this field indicates that no emergency services are directly configured to be reachable through te SSID. When set to 1, this field indicates that higher layer emergency services are reachable through the SSID, in addition to any other services offered. Emergency services may be supported either by an ESO (emergency services only) network or a network that supports emergency services accessible with public credentials. An AP advertises this capability if at least one of the two methods is supported.

For a network to be identified as an Emergency Services (ES) network, location capability is enabled on an AP if the AP is located in a regulatory domain that requires location capabilities. In addition, this field is only advertised if all of the following are true (see U.4.2 for further information):

—If an ESO network, the SSID is configured for open authentication and used exclusively for providing emergency services

—If an Emergency Services network with public credentials, the SSID is configured for RSN authentication

—dot11QosOptionImplemented is true

—Expedited Bandwidth Request Service capability field is set to 1

—Network supports end-to-end QoS from non-AP STA to all APs in ESS to PSAP. It is outside the scope of the Interworking Service how this information is known to the AP

The Venue Group and 802.11 Venue Type fields are both one octet values selected from Table 7-43s and Table 7-43t respectively. The entries in Table 7-43s and Table 7-43t are drawn from the International Building Code’s Use and Occupancy Classifications [B33]. An AP sets the default value of Venue Group to Residential and the default value of 802.11 Venue Type to unspecified. A non-AP STA sets the default value of Venue Group to unspecified and the default value of Venue Type to unspecified.

- Table 7-43s—Venue Group codes and descriptions

- Table 7-43t—802.11 Venue Type assignments

The HESSID field specifies the value of HESSID, see 11.18.1. A non-AP STA uses this field to indicate the desired HESSID in an active scan. The HESSID field is optionally present depending on the setting of the HESSID Present field in the Interworking capabilities field.

The EICI (Emergency Information Change Indication) field is an optional field used to indicate the change of the emergencies. While no EAS message is active, the EASN Available field is set to 0, and the EICI field does not exist. When a first EAS message becomes available, the EASN Available field is set to 1, the EICI field appears and its value is set to non-zero; then the value of EICI is increment by 1 upon a new EAS message is active or an existing EAS message is inactive. The EICI field disappears if all the existing EAS messages become inactive. If a new EAS message becomes available thereafter, the EICI field appears again and its value is incremented upon its previous value. The EICI value restores to 1 after it reaches 255 upon one more change. When the EICI field changes value while the EASN field keeps value 1, a non-AP STA requests further information from the higher layers to receive the full EAS information. This information is transported over GAS using a suitable advertisement protocol as described in 11.18.5.

- Modify the text in 11.18.5 ( Page 73) as follows:

on this access category and deny all subsequent ADDTS request frames with TID=1 or 2, or if the ACM field for AC_BK is set to 0 then the non-AP STA shall be disassociated.

—dot11NonAPStationHCCAOctetCount is compared to dot11NonAPStationAuthMaxHCCAOctets.When the value of the authorized maximum octet count is exceeded, then the HC shall delete all admitted TSs with access policy = HCCA or HEMM and deny all subsequent ADDTS request frames with access policy = HCCA or HEMM.

—The sum of dot11NonAPStationVoiceOctetCount, dot11NonAPStationVideoOctetCount, dot11NonAPStationBestEffortOctetCount, dot11NonAPStationAuthMaxBackgroundOctets, and dot11NonAPStationHCCAOctetCount is compared to dot11NonAPStationAuthMaxTotalOctets. When the value of the authorized maximum octet count is exceeded, the non-AP STA shall be disassociated.

When a non-AP STA has exceeded the resource limits listed above, the AP shall terminate the connection. The AP SME invokes the disassociation procedure defined in 11.3.2.7 by issuing an MLME-DISASSOCIATE.request with the reason code “Authorized Access Limit Reached”.

- 11.18.5 Interworking Procedures: Emergency Alert System (EAS) Support

The Emergency Alert System (EAS) would be supported if the AP with dot11EASNEnabled set to TRUE,  and the EASN field in the Extended capabilities element is set to 1; otherwise the EASN field in the Extended capabilities element is set to 0, the EASN Available field in Interworking element is useless and shall be set to 0.

The Emergency Alert System (EAS) provides alerts, typically issued by authorities, to be transmitted, upon request, to non-AP STAs using the GAS protocol, from an AP within the IEEE 802.11 infrastructure. The mechanism provided by the interworking service uses a single bit EASN Available field and an optional octet EICI field in the Interworking element Figure 7-95ao, transmitted from an AP, to indicate whether or not any alert is available within the IEEE 802.11 infrastructure, and the change of the EAS messages information. The EASN field is set to 0 by a non-AP STA.

If an external network emergency alert is received by the IEEE 802.11 infrastructure, or existing emergency alerts are updated, each AP then advertises the option for the EAS mechanism, setting the Interworking element EASN  Available field to 1, and modifies the EICI field. Once the non-AP STA receives this advertisement, it utilizes GAS to retrieve the alert message from the AP, typically formatted using the Common Alert Protocol (CAP).

By utilizing GAS, the emergency functionality of the interworking service is extended to provide EAS support for client devices.

The CAP allows the collection and distribution of “all-hazard” safety notifications and emergency warnings across information networks and public alert systems used by first responders.

Authorization and integrity protection of the message is the responsibility of the authority and is out of scope of this standard.

- Modify the mib in Annex D  as follows:

Annex D

Insert new entries in Dot11StationConfigEntry

Dot11StationConfigEntry::=

SEQUENCE {	dot11StationID 							MACAddress,

dot11MediumOccupancyLimit 					INTEGER,

dot11CFPollable 							TruthValue,

dot11CFPPeriod 							INTEGER,

dot11CFPMaxDuration 						INTEGER,

dot11AuthenticationResponseTimeOut 				Unsigned32,

dot11PrivacyOptionImplemented 					TruthValue,

dot11PowerManagementMode 					INTEGER,

dot11DesiredSSID 						OCTET STRING,

dot11DesiredBSSType						 INTEGER,

dot11OperationalRateSet 					OCTET STRING,

dot11BeaconPeriod 						INTEGER,

dot11DTIMPeriod 						INTEGER,

dot11AssociationResponseTimeOut 			Unsigned32,

dot11DisassociateReason 					INTEGER,

dot11DisassociateStation 					MACAddress,

dot11DeauthenticateReason 				INTEGER,

dot11DeauthenticateStation 				MACAddress,

dot11AuthenticateFailStatus 				INTEGER,

dot11AuthenticateFailStation 				MACAddress,

dot11MultiDomainCapabilityImplemented 		TruthValue,

dot11MultiDomainCapabilityEnabled 			TruthValue,

dot11CountryString 					OCTET STRING,

dot11SpectrumManagementImplemented 			TruthValue,

dot11SpectrumManagementRequired 			TruthValue,

dot11RSNAOptionImplemented 				TruthValue,

dot11RSNAPreauthenticationImplemented 		TruthValue,

dot11RegulatoryClassesImplemented 			TruthValue,

dot11RegulatoryClassesRequired 				TruthValue,

dot11QosOptionImplemented 				TruthValue,

dot11ImmediateBlockAckOptionImplemented 		TruthValue,

dot11DelayedBlockAckOptionImplemented 		TruthValue,

dot11DirectOptionImplemented 				TruthValue,

dot11APSDOptionImplemented 				TruthValue,

dot11QAckOptionImplemented 				TruthValue,

dot11QBSSLoadOptionImplemented 				TruthValue,

dot11QueueRequestOptionImplemented 			TruthValue,

dot11TXOPRequestOptionImplemented 			TruthValue,

dot11MoreDataAckOptionImplemented 			TruthValue,

dot11AssociateinNQBSS 					TruthValue,

dot11DLSAllowedInQBSS 					TruthValue,

dot11DLSAllowed 						TruthValue,

dot11InterworkingServiceImplemented 			TruthValue,

dot11InterworkingServiceEnabled 			TruthValue,

dot11QosmapImplemented 					TruthValue,

dot11QosMapEnabled 					TruthValue,

dot11EBRImplemented 					TruthValue,

dot11EBREnabled 						TruthValue,

dot11ESNetwork 						TruthValue,

dot11SSPNInterfaceImplemented 				TruthValue,

dot11SSPNInterfaceEnabled 				TruthValue,

dot11EASNImplemented					TruthValue,

dot11EASNEnabled						TruthValue,

dot11GASResponseBufferingTime				INTEGER

}

Insert the following elements to the dot11StationConfigTable definitions in Annex D.

dot11InterworkingServiceImplemented dot11EASNImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute when TRUE, indicates the STA is capable of emergency alert system notification with external networks. A STA setting this to TRUE implements emergency alert system notification. When this is set to FALSE, the STA does not implement emergency alert system notification.”

DEFVAL (false)

::= {dot11StationConfigEntry 1}

dot11InterworkingServiceEnabled dot11EASNEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute when TRUE, indicates the capability of the STA to support emergency alert system when interwork with external networks is enabled. The capability is disabled otherwise.”

DEFVAL (false)

::= {dot11StationConfigEntry 2}