IEEE P802.11
Wireless LANs

| Normative Text for Amendment for EASN |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ding Zhiming | Huawei Technologies | Bldg 7,Vision Business Park, Nan Shan Science Park, Shenzhen P.R. China | +86 755 36835663 | dingzhiming@huawei.com |
| Stephen McCann | Roke Manor Research Ltd | Roke Manor, Old Salisbury Lane, Romsey, SO51 0ZN, UK | +44 1794 833341 | stephen.mccann@roke.co.uk |
| Shu Guiming | Huawei Technologies | Bldg 7,Huawei Technologies Vision Business Park, Nan Shan Science Park, Shenzhen P.R. China | +86 755 36835543 | shugm@huawei.com |
| Celine Liu | Huawei Technologies | Bldg 7,Vision Business Park, Nan Shan Science Park, Shenzhen P.R. China | +86 755 36835837 | celineliu@huawei.com |
| Amy Zhang | Huawei Technologies | Huawei Industry Park, Bantian, Shenzhen, P.R. China |  | Amyzhang@huawei.com |
| Necati Canpolat | Intel Corporation | 2111 NE. 25th Ave, Hillsboro, OR 97124 | +1 503 264 8014 | necati.canpolat@intel.com |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |
| Gabor Bajko | Nokia | 313 Fairchild dr. Mountain View, CA, 94043 | +1 858 717 6650 | gabor.bajko@nokia.com |

Abstract

This document is a normative text proposal for EASN. The text is aligned with IEEE P802.11u/D4.01, October, 2008.

- Insert the following definitions in “4. Abbreviations and acronyms” (Page 4)

4. Abbreviations and acronyms

- Insert the following new abbreviations and acronyms into clause 4 in alphabetical order:

802.x LAN	IEEE 802 based local area networks such as 802.3 and 802.11

AAA		authentication, authorization, and accounting

ASRA		additional step required for access

DN		destination network

EAS		emergency alert system

EASN		emergency alert system notification

EBR		expedited bandwidth request

EDT		eastern daylight time

EICI		emergency information change indication

ES		emergency services

ESO		emergency services only

EST		eastern standard time

GAS		 generic advertisement Service

HESSID 	homogenous ESS identifier

MICS		media independent command service

MIES 		media independent event service

MIH 		media independent handover

MIIS		media independent information service

MLPP		multi-level precedence and preemption

NAI 		network access identifier

PHB		per-hop behavior

PoS		point of service

- Add a row into Table 7-42s in 7.3.2.27

7.3.2.27 Extended capabilities information element

Insert the following additional row at the end of Table 42s.

Table 7-42s—Capabilities field

| Bit(s) | Information | Notes |
| --- | --- | --- |
| <ANA> | Interworking | When dot11InterworkingServiceEnabled is set to TRUE, the Interworking field is set to 1 to indicate the STA supports Interworking Service as described in 11.18. When dot11InterworkingServiceEnabled is set to FALSE, the Interworking field is set to 0 to indicate the STA does not support this capability. |
| <ANA> | QoS Map | When dot11QosMapEnabled is set to TRUE, the QoS Map field is set to 1 to indicate the STA supports QoS Map service as described in 11.18.6. When dot11QosMapEnabled is set to FALSE, the QoS Map field is set to 0 to indicate the STA does not support this capability. |
| <ANA> | EBR | When dot11EBREnabled is set to TRUE, the EBR field is set to 1 to indicate the STA supports EBR as described in 7.3.2.62. When dot11EBREnabled is set to FALSE, the EBR field is set to 0 to indicate the STA does not support this capability. |
| <ANA> | SSPN Interface | When dot11SSPNInterfaceEnabled is set to TRUE, the SSPN Interface field is set to 1 to indicate the AP supports SSPN Interface service as described in 11.18.4. When dot11SSPNInterfaceEnabled is set to FALSE, the SSPN Interface is set to 0 to indicate the AP does not support this capability. Non-AP STAs shall set this field to 0. |
| <ANA> | EASN | When dot11EASNEnabled is set to TRUE, the EASN field is set to 1 to indicate the STA supports EASN mechanism as described in 11.18.5. When dot11EASNEnabled is set to FALSE, the EASN field is set to 0 to indicate the STA does not support this capability. |

7.3.2.59 Interworking information element

- Change Figure 7-95ao in  7.3.2.59  as follows:

| - Interworking element format |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B0 - B3 | B4 | B5 | B6 | B7 |  |  |  |  |
|  | Element ID | Length | Network Type | Internet | ASRA | EASN Available | Emergency Network | Venue Group | 802.11 Venue Type | HESSID (optional) | EICI (optional) |
| Bits |  |  | 4 | 1 | 1 | 1 | 1 |  |  |  |  |
| Octets: | 1 | 1 |  |  |  |  |  | 1 | 1 | 0 or 6 | 0 or 1 |

- Figure 7-95ao—Interworking element format

The value of the Length field is the length of the Interworking capabilities field. The value of the length field dependent on the presence of the optional fields: It is 3 when no optional field is present, it is 4 when only EICI present, it is 9 if only  HESSID is present and it is 10 when all optional fields are present.

The Network Types are shown in Table 7-43r. A non-AP STA uses this field to indicate the desired Network Type in an active scan. See U.4 for informative text on usage of fields contained within the Interworking element.

- Change the text as follows:

Bit 6 is the Emergency Alert System Notification (EASN) Available field. It is set to 1 by the AP to indicate that a higher layer EASN is available at the AP for download. The EASN Available field set to 0 indicates that no higher layer EASN message is available at the AP for download. When this field is set to 1 by an AP, the EICI optional field is present. For more information, refer to 11.8.5. EAS protocols define when message are active.

- Insert  the following text below at the end of clause 7.3.2.59

The Emergency Information Change Indication (EICI) is an optional field used to indicate the presence of a new EASN message. When there is no active EASN message, the EASN Available field is set to 0 and the EICI optional field is not present. When an EASN message is available, the EASN Available field is set to 1, the EICI field is present. For more information on EICI field see 11.18.5.

- 11.18.5 Interworking Procedures: Emergency Alert System (EAS) Support

- Change  11.18.5  as follows:

The Emergency Alert System Notification (EASN) provides alerts, typically issued by authorities, to be transmitted, upon request, to non-AP STAs using the GAS protocol, from an AP within the IEEE 802.11 infrastructure. The mechanism provided by the interworking service uses the EASN Available and EICI fields in the Interworking element in Figure 7-95ao. When dot11EASNEnabled in APs is set to TRUE, EASN is supported and APs shall set the EASN field in the Extended capabilities element to 1; otherwise the dot11EASNEnabled is set to FALSE and APs shall set the EASN field in the Extended capabilities to 0.

When a new EASN message is received by the IEEE 802.11 infrastructure, or existing emergency alerts are updated, each AP then advertises the availability of the EASN message by setting the Interworking element EASN Available field to 1, and incrementing the EICI field by 1.

When there is no EASN message, the EICI field is not present and EASN available field shall be set to 0. If a new EAS message becomes available thereafter, the EICI field reappears and its value is incremented upon its previous value.

Once the non-AP STA receives this advertisement, and find that the EASN Available field is set to 1 from 0 or the EICI field changes value, it should utilize non-native GAS procedures to retrieve the alert message from the AP, typically formatted using the Common Alert Protocol (CAP). By utilizing GAS, the emergency functionality of the interworking service is extended to provide EAS support for client devices.

The CAP allows the collection and distribution of “all-hazard” safety notifications and emergency warnings across information networks and public alert systems used by first responders.

- Authorization and integrity protection of the message is the responsibility of the authority and is out of scope of this standard.

- Change the mib in Annex D  as follows:

Annex D

Insert new entries in Dot11StationConfigEntry

Dot11StationConfigEntry::=

SEQUENCE {	dot11StationID 							MACAddress,

dot11MediumOccupancyLimit 					INTEGER,

dot11CFPollable 							TruthValue,

dot11CFPPeriod 							INTEGER,

dot11CFPMaxDuration 						INTEGER,

dot11AuthenticationResponseTimeOut 				Unsigned32,

dot11PrivacyOptionImplemented 					TruthValue,

dot11PowerManagementMode 					INTEGER,

dot11DesiredSSID 						OCTET STRING,

dot11DesiredBSSType						 INTEGER,

dot11OperationalRateSet 					OCTET STRING,

dot11BeaconPeriod 						INTEGER,

dot11DTIMPeriod 						INTEGER,

dot11AssociationResponseTimeOut 			Unsigned32,

dot11DisassociateReason 					INTEGER,

dot11DisassociateStation 					MACAddress,

dot11DeauthenticateReason 				INTEGER,

dot11DeauthenticateStation 				MACAddress,

dot11AuthenticateFailStatus 				INTEGER,

dot11AuthenticateFailStation 				MACAddress,

dot11MultiDomainCapabilityImplemented 		TruthValue,

dot11MultiDomainCapabilityEnabled 			TruthValue,

dot11CountryString 					OCTET STRING,

dot11SpectrumManagementImplemented 			TruthValue,

dot11SpectrumManagementRequired 			TruthValue,

dot11RSNAOptionImplemented 				TruthValue,

dot11RSNAPreauthenticationImplemented 		TruthValue,

dot11RegulatoryClassesImplemented 			TruthValue,

dot11RegulatoryClassesRequired 				TruthValue,

dot11QosOptionImplemented 				TruthValue,

dot11ImmediateBlockAckOptionImplemented 		TruthValue,

dot11DelayedBlockAckOptionImplemented 		TruthValue,

dot11DirectOptionImplemented 				TruthValue,

dot11APSDOptionImplemented 				TruthValue,

dot11QAckOptionImplemented 				TruthValue,

dot11QBSSLoadOptionImplemented 				TruthValue,

dot11QueueRequestOptionImplemented 			TruthValue,

dot11TXOPRequestOptionImplemented 			TruthValue,

dot11MoreDataAckOptionImplemented 			TruthValue,

dot11AssociateinNQBSS 					TruthValue,

dot11DLSAllowedInQBSS 					TruthValue,

dot11DLSAllowed 						TruthValue,

dot11InterworkingServiceImplemented 			TruthValue,

dot11InterworkingServiceEnabled 			TruthValue,

dot11QosmapImplemented 					TruthValue,

dot11QosMapEnabled 					TruthValue,

dot11EBRImplemented 					TruthValue,

dot11EBREnabled 						TruthValue,

dot11ESNetwork 						TruthValue,

dot11SSPNInterfaceImplemented 				TruthValue,

dot11SSPNInterfaceEnabled 				TruthValue,

dot11EASNImplemented					TruthValue,

dot11EASNEnabled						TruthValue,

dot11GASResponseBufferingTime				INTEGER

}

Insert the following elements to the dot11StationConfigTable definitions in Annex D.

dot11EASNImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute when TRUE, indicates the STA is capable of emergency alert system notification with external networks. A STA setting this to TRUE implements emergency alert system notification. When this is set to FALSE, the STA does not implement emergency alert system notification.”

DEFVAL (false)

::= {dot11StationConfigEntry ANA }

dot11EASNEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute when TRUE, indicates the capability of the STA to support emergency alert system when interwork with external networks is enabled. The capability is disabled otherwise.”

DEFVAL (false)

::= {dot11StationConfigEntry ANA+1}