IEEE P802.11
Wireless LANs

| Normative Text for Amendment for EASN |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Ding Zhiming | Huawei | Bldg 7,Vision Business Park, Nan Shan Science Park, Shenzhen P.R. China | +86 755 36835663 | dingzhiming@huawei.com |
| Stephen McCann | Roke Manor Research Ltd | Roke Manor, Old Salisbury Lane, Romsey, SO51 0ZN, UK | +44 1794 833341 | stephen.mccann@roke.co.uk |
| Shu Guiming | Huawei Technologies | Bldg 7,Huawei Technologies Vision Business Park, Nan Shan Science Park, Shenzhen P.R. China | +86 755 36835543 | shugm@huawei.com |
| Celine Liu | Huawei Technologies | Bldg 7,Vision Business Park, Nan Shan Science Park, Shenzhen P.R. China | +86 755 36835837 | celineliu@huawei.com |
| Amy Zhang | Huawei Technologies | Huawei Industry Park, Bantian, Shenzhen, P.R. China |  | Amyzhang@huawei.com |
| Necati Canpolat | Intel Corporation | 2111 NE. 25th Ave, Hillsboro, OR 97124 | +1 503 264 8014 | necati.canpolat@intel.com |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |
| Gabor Bajko | Nokia | 313 Fairchild dr. Mountain View, CA, 94043 | +1 858 717 6650 | gabor.bajko@nokia.com |

Abstract

This document is a normative text proposal for EASN. The text is aligned with IEEE P802.11u/D4.01, November 2008.

- Insert the following definitions in “4. Abbreviations and acronyms” (Page 4)

4. Abbreviations and acronyms

- Insert the following new acronyms into clause 4 in alphabetical order:

EICI		emergency information change indication

- Change  Table 7-42s by inserting a new row as follows

7.3.2.27 Extended capabilities information element

Insert the following additional row at the end of Table 42s.

#### Table 7-42s—Capabilities field

| Bit(s) | Information | Notes |
| --- | --- | --- |
| <ANA> | Interworking | When dot11InterworkingServiceEnabled is set to TRUE, the Interworking field is set to 1 to indicate the STA supports Interworking Service as described in 11.18. When dot11InterworkingServiceEnabled is set to FALSE, the Interworking field is set to 0 to indicate the STA does not support this capability. |
| <ANA> | QoS Map | When dot11QosMapEnabled is set to TRUE, the QoS Map field is set to 1 to indicate the STA supports QoS Map service as described in 11.18.6. When dot11QosMapEnabled is set to FALSE, the QoS Map field is set to 0 to indicate the STA does not support this capability. |
| <ANA> | EBR | When dot11EBREnabled is set to TRUE, the EBR field is set to 1 to indicate the STA supports EBR as described in 7.3.2.62. When dot11EBREnabled is set to FALSE, the EBR field is set to 0 to indicate the STA does not support this capability. |
| <ANA> | SSPN Interface | When dot11SSPNInterfaceEnabled is set to TRUE, the SSPN Interface field is set to 1 to indicate the AP supports SSPN Interface service as described in 11.18.4. When dot11SSPNInterfaceEnabled is set to FALSE, the SSPN Interface is set to 0 to indicate the AP does not support this capability. Non-AP STAs shall set this field to 0. |
| <ANA> | EASN | When dot11EASNEnabled is set to TRUE, the EASN field is set to 1 to indicate the STA supports EASN mechanism as described in 11.18.5. When dot11EASNEnabled is set to FALSE, the EASN field is set to 0 to indicate the STA does not support this capability. |

7.3.2.59 Interworking information element

Change Figure 7-95ao as follows:

|  |  |  | B0 - B3 | B4 | B5 | B6 | B7 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Element ID | Length | Network Type | Internet | ASRA | EASN Available | Emergency Network | Venue Group | 802.11 Venue Type | HESSID (optional) | EICI (optional) |
| Bits |  |  | 4 | 1 | 1 | 1 | 1 |  |  |  |  |
| Octets: | 1 | 1 |  |  |  |  |  | 1 | 1 | 0 or 6 | 0 or 1 |

- Figure 7-95ao—Interworking element format

- Change the text as follows:

The Length field is the length of the Interworking element fields. The value of the Length field is 3 when no optional field is present, 4 when only EICI present, 9 when only HESSID is present and 10 when all optional fields are present.

Bit 6 is the Emergency Alert System Notification (EASN) Available field. It is set to 1 by the AP to indicate that a higher layer EASN is available at the AP for download. When EASN Available field is set to 0, the EICI field is not present. When the EASN field is set to 1, EICI field is present and set to a non-zero value, and it indicates that a higher layer EASN message is available at the AP for download. For more information, refer to 11.8.5.

- Insert the following text below at the end of clause 7.3.2.59

The Emergency Information Change Indication (EICI) field is used to indicate the availability of a new EASN message. When the EASN Available field is set to 0, the EICI field is not present. When the EASN Available field is set to 1, the EICI field is present and is set to a non-zero value. The value of the EICI field is incremented by 1 whenever a new EAS message arrives to the AP.

- 11.18.5 Interworking Procedures: Emergency Alert System (EAS) Support

Change 11.18.5 as follows:

The Emergency Alert System Notification (EASN) provides alerts, typically issued by authorities, to be transmitted, from APS to non-AP STAs upon request from non-AP STAs. The mechanism uses the EASN Available and EICI fields in the Interworking element in Figure 7-95ao. When the Emergency Alert System Notification (EASN) is supported, dot11EASNEnabled shall be set to TRUE. When EASN is not supported, dot11EASNEnabled shall be set to FALSE.

Each time a new EASN alert is received by the IEEE 802.11 infrastructure, each AP shall advertise the availability of the new message by setting the EASN Available field in the Interworking element to 1, and incrementing the last value of EICI field by 1 regardless of whether EICI was present in the Interworking element in the most recently transmitted beacon or Probe Response frame. When the current EICI value is 255, the EICI value is set to 1. When all EASN messages in the AP expire, the EASN Available field in the Interworking element shall be set to 0.

Once the non-AP STA receives EASN alert advertisement through EASN Available set to 1 and EICI set to value between 1 and 255, it will retrieve the EASN message from AP using the non-Native GAS procedures. Later on, when the non-AP STA sees the EASN Available set to 1, and receives an EICI value different from the last EICI value it has received, the non-AP STA will retrieve the EASN message from the AP; otherwise if the EICI values are the same, it will ignore the EASN Available alert since the EASN alert is for the same message which the non-AP STA has already retrieved.

The EASN messages are typically formatted using the Common Alert Protocol (CAP). The CAP allows the collection and distribution of “all-hazard” safety notifications and emergency warnings across information networks and public alert systems used by first responders. Authorization and integrity protection of the message is the responsibility of the authority and is out of scope of this standard.

When there is no EASN message, tis not present and EASN available field shall be set to 0. the EICI field re

and find that the EASN Available field is set to 1 from 0 or the EICI field changes value,

- Change

- Annex D

Change  Dot11StationConfigEntry by inserting two new entries as follows:

Dot11StationConfigEntry::=

SEQUENCE {	dot11StationID 							MACAddress,

dot11MediumOccupancyLimit 					INTEGER,

dot11CFPollable 							TruthValue,

dot11CFPPeriod 							INTEGER,

dot11CFPMaxDuration 						INTEGER,

dot11AuthenticationResponseTimeOut 				Unsigned32,

dot11PrivacyOptionImplemented 					TruthValue,

dot11PowerManagementMode 					INTEGER,

dot11DesiredSSID 						OCTET STRING,

dot11DesiredBSSType						 INTEGER,

dot11OperationalRateSet 					OCTET STRING,

dot11BeaconPeriod 						INTEGER,

dot11DTIMPeriod 						INTEGER,

dot11AssociationResponseTimeOut 			Unsigned32,

dot11DisassociateReason 					INTEGER,

dot11DisassociateStation 					MACAddress,

dot11DeauthenticateReason 				INTEGER,

dot11DeauthenticateStation 				MACAddress,

dot11AuthenticateFailStatus 				INTEGER,

dot11AuthenticateFailStation 				MACAddress,

dot11MultiDomainCapabilityImplemented 		TruthValue,

dot11MultiDomainCapabilityEnabled 			TruthValue,

dot11CountryString 					OCTET STRING,

dot11SpectrumManagementImplemented 			TruthValue,

dot11SpectrumManagementRequired 			TruthValue,

dot11RSNAOptionImplemented 				TruthValue,

dot11RSNAPreauthenticationImplemented 		TruthValue,

dot11RegulatoryClassesImplemented 			TruthValue,

dot11RegulatoryClassesRequired 				TruthValue,

dot11QosOptionImplemented 				TruthValue,

dot11ImmediateBlockAckOptionImplemented 		TruthValue,

dot11DelayedBlockAckOptionImplemented 		TruthValue,

dot11DirectOptionImplemented 				TruthValue,

dot11APSDOptionImplemented 				TruthValue,

dot11QAckOptionImplemented 				TruthValue,

dot11QBSSLoadOptionImplemented 				TruthValue,

dot11QueueRequestOptionImplemented 			TruthValue,

dot11TXOPRequestOptionImplemented 			TruthValue,

dot11MoreDataAckOptionImplemented 			TruthValue,

dot11AssociateinNQBSS 					TruthValue,

dot11DLSAllowedInQBSS 					TruthValue,

dot11DLSAllowed 						TruthValue,

dot11InterworkingServiceImplemented 			TruthValue,

dot11InterworkingServiceEnabled 			TruthValue,

dot11QosmapImplemented 					TruthValue,

dot11QosMapEnabled 					TruthValue,

dot11EBRImplemented 					TruthValue,

dot11EBREnabled 						TruthValue,

dot11ESNetwork 						TruthValue,

dot11SSPNInterfaceImplemented 				TruthValue,

dot11SSPNInterfaceEnabled 				TruthValue,

dot11EASNImplemented					TruthValue,

dot11EASNEnabled						TruthValue,

dot11GASResponseBufferingTime				INTEGER

}

Insert the following elements to the dot11StationConfigTable definitions in Annex D.

dot11EASNImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute when TRUE, indicates the STA is capable of emergency alert system notification with external networks. A STA setting this to TRUE implements emergency alert system notification. When this is set to FALSE, the STA does not implement emergency alert system notification.”

DEFVAL (false)

::= {dot11StationConfigEntry ANA }

dot11EASNEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute when TRUE, indicates the capability of the STA to support emergency alert system when interwork with external networks is enabled. The capability is disabled otherwise.”

DEFVAL (false)

::= {dot11StationConfigEntry ANA+1}