IEEE P802.11
Wireless LANs

| UTC Sync Information Element |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2008-11-4 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Allan Thomson | Cisco Systems | 170 W Tasman Drive, San Jose, CA95134, USA | +1-408-853-5570 | allant@cisco.com |
| Dave Stephenson | Cisco Systems | 170 W Tasman Drive, San Jose, CA95134, USA | +1-408-527-7991 | daves@cisco.com |

Abstract

This document contains proposed edits to TGv draft 3.02 for a new information element that provides UTC time synchronization.

7.2.3.1 Beacon frame format

Change Table 7-8 as follows:.

| Table 7-8—Beacon frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| 31 | Multiple BSSID | One or more The Multiple BSSID elements is are present if dot11RRMMeasurementPilotCapability is set to a value between 2 and 7 and the AP is a member of a Multiple BSSID Set (see 11.10.11) with two or more members, or if dot11MgmtOptionMultiBSSIDEnabled is set to true. |
| 42 | FMS Descriptor | The FMS Descriptor element is present if dot11MgmtOptionFMSEnabled is set to true. |
| 43 | QoS Traffic Capability | The QoS Traffic Capability element is optionally present if dot11MgmtOptionACStationCountEnabled is set to true. |
| 44 | UTC Time Sync | The UTC Time Sync element is present if dot11UTCTimeSyncEnabled is set to true |

7.2.3.9 Probe Response frame format

Change Table 7-15 as follows:

| Table 7-15—Probe Response frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| 24 | Multiple BSSID | One or more Multiple BSSID elements are present Included if dot11RRMMeasurementPilotCapability is set to a value between 2 and 7 and the AP is a member of a Multiple BSSID Set (see 11.10.11) with two or more members, or if dot11MgmtOptionMultiBSSIDEnabled is set to true. |
| 40 | QoS Traffic Capability | The QoS Traffic Capability element is optionally present if dot11MgmtOptionACStationCountEnabled is set to true. |
| 41 | UTC Time Sync | The UTC Time Sync element is present if dot11UTCTimeSyncEnabled is set to true |

7.3.2 Information Elements

Insert Element IDs <ANA> and change the Reserved row in Table 7-26 as follows (note that the entire table is not shown here):

| Table 7-26—Element IDs |  |  |  |
| --- | --- | --- | --- |
| Information Element | Element ID | Length (in octets) | Extensible |
| Event Request (see 7.3.2.62) | <ANA> | 5 to 257 | Subelements |
| Event Report (see 7.3.2.63) | <ANA> | 5 to 257 |  |
| Diagnostic Request (see 7.3.2.64) | <ANA> | 6 to 257 | Subelements |
| Diagnostic Report (see 7.3.2.65) | <ANA> | 5 to 257 | Subelements |
| Location Parameters (see 7.3.2.66) | <ANA> | 2 to 257 | Subelements |
| Non-transmitted BSSID Capability (see 7.3.2.67) | <ANA> | 4 |  |
| SSID List (see 7.3.2.68) | <ANA> | 2 to 257 |  |
| Multiple BSSID-Index (see 7.3.2.69) | <ANA> | 3 to 5 |  |
| FMS Descriptor (see 7.3.2.70) | <ANA> | 2 to 257 |  |
| FMS Request (see 7.3.2.71) | <ANA> | 3 to 257 | Subelements |
| FMS Response (see 7.3.2.72) | <ANA> | 18 to 257 | Subelements |
| QoS Traffic Capability (see 7.3.2.73) | <ANA> | 3-5 | Yes |
| BSS Max Idle Period (see 7.3.2.74) | <ANA> | 5 | Yes |
| TFS Request (see 7.3.2.75) | <ANA> | 6 to 257 | Subelements |
| TFS Response (see 7.3.2.76) | <ANA> | 6 to 256 | Subelements |
| WNM-Sleep Mode (see 7.3.2.77) | <ANA> | 4 or 5 | Yes |
| TIM Broadcast Request (see 7.3.2.78) | <ANA> | 3 | Yes |
| TIM Broadcast Response (see 7.3.2.79) | <ANA> | 3 or 10 | Yes |
| Co-Located Interference Report (see 7.3.2.80) | <ANA> | 21 | Yes |
| Channel Usage (see 7.3.2.81) | <ANA> | 4 to 256 | Subelements |
| UTC Time Sync (see 7.3.2.82) | <ANA> | 11 | Yes |
| Reserved | (<ANA>+1) 74 — 220 |  |  |

7.3.2.27 Extended Capabilities information element

Insert Bit(s) <ANA> and change the Reserved row in Table 7-35a as follows (note that the entire table is not shown here):

| Table 7-35a—Capabilities field |  |  |
| --- | --- | --- |
| Bit(s) | Information | Notes |
| <ANA> | Event | The STA sets the Event field to 1 when the MIB attribute dot11MgmtOptionEventsEnabled is set to true, and sets it to 0 otherwise. See 11.20.2. |
| <ANA> | Diagnostics | The STA sets the Diagnostics field to 1 when the MIB attribute dot11MgmtOptionDiagnosticsEnabled is set to true, and sets it to 0 otherwise. See 11.20.3. |
| <ANA> | Multicast Diagnostics | The STA sets the Multicast Diagnostics field to 1 when the MIB attribute dot11MgmtOptionMulticastDiagnosticsEnabled is set to true, and sets it to 0 otherwise. See 11.20.2. |
| <ANA> | Location Tracking | The STA sets the Location Tracking field to 1 when dot11MgmtOptionLocationEnabled is set to true and supports Location Tracking as described in 11.20.4. Otherwise, the STA sets the Location Tracking field to 0. |
| <ANA> | FMS | The STA sets the FMS field to 1 when the MIB attribute dot11MgmtOptionFMSEnabled is set to true, and sets it to 0 otherwise. See 11.2.1.4a and 11.20.7. |
| <ANA> | Proxy ARP Service | The AP sets the Proxy ARP Service field to 1 when the MIB attribute dot11MgmtOptionProxyARPEnabled is set to true, and sets it to 0 otherwise. See 11.20.13. A non-AP STA sets the Proxy ARP Service field to 0. |
| <ANA> | Co-located Interference Reporting | The STA sets the Co-located Interference Reporting field to 1 when the MIB attribute dot11MgmtOptionCOLOINTReportingEnabled is set to true, and sets it to 0 otherwise. See 11.20.9. |
| <ANA> | ES CIVIC Location | The STA sets the ES CIVIC Location field to 1 when the MIB attribute dot11RRMCivicMeasurementEnabled is set to true, and sets it to 0 otherwise. See 11.10.8.9. |
| <ANA> | ES Geo Location | The STA sets the ES Geo Location field to 1 when the MIB attribute dot11RRMLCIMeasurementEnabled is set to true, and sets it to 0 otherwise. See 11.10.8.6. |
| <ANA> | TFS | The STA sets the TFS field to 1 when the MIB attribute dot11MgmtOptionTFSEnabled is set to true, and sets it to 0 otherwise. See 11.20.11. |
| <ANA> | WNM-Sleep Mode | The STA sets the WNM-Sleep Mode field to 1 when the MIB attribute dot11MgmtOptionWNMSleepModeEnabled is set to true, and sets it to 0 otherwise. See 11.2.2. |
| <ANA> | TIM Broadcast | The STA sets the TIM Broadcast field to 1 when the MIB attribute dot11MgmtOptionTIMBroadcastEnabled is set to true, and sets it to 0 otherwise. See 11.2.1.13. |
| <ANA> | BSS Transition | The STA sets the BSS Transition field to 1 when the MIB attribute dot11MgmtOptionBSSTransitionEnabled is set to true, and sets it to 0 otherwise. See 11.20.6. |
| <ANA> | QoS Traffic Capability | The STA sets the QoS Traffic Capability field to 1 when the MIB attribute dot11MgmtOptionTrafficGenerationEnabled is set to true, and sets it to 0 otherwise. See 11.20.9. |
| <ANA> | AC Station Count | The STA sets the AC Station Count field to 1 when the MIB attribute dot11MgmtOptionACStationCountEnabled is set to true, and sets it to 0 otherwise. See 11.20.10. |
| <ANA> | Multiple BSSID | The STA sets the Multiple BSSID field to 1 when the MIB attribute dot11MgmtOptionMultiBSSEDEnabled is set to true, and sets it to 0 otherwise. See 11.10.11 and 11.1.2.3a. |
| <ANA> | Timing Measurement | The STA sets the Timing Measurement field to 1 when the MIB attribute dot11MgmtOptionTimingMsmtEnabled is set to true, and sets it to 0 otherwise. See 11.20.5. |
| <ANA> | Channel Usage | The STA sets the Channel Usage field to 1 when dot11MgmtOptionChannelUsageEnabled is set to true and sets it to 0 otherwise. See 11.20.14. |
| <ANA> | SSID List | The STA sets the SSID List field to 1 when the MIB attributedot11MgmtOptionSSIDListEnabled is set to true, and sets it to 0 otherwise. See 11.1.3. |
| <ANA> | UTC Time Sync | The STA sets the UTC Time Sync field to 1 when dot11UTCTimeSyncEnabled is set to true and sets it to 0 otherwise. See 11.20.15 |
| (<ANA>+1) — n*8 | Reserved | All other bits are reserved, and are set to 0 on transmission and ignored on reception. |

7.3.2.82 UTC Time Sync element

The UTC Time Sync element contains the UTC time associated with the TSF timer value from the AP. The format of the UTC Time Sync element is shown in Figure vXX.

|  | Element ID | Length | UTC Reference |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 9 |

The Element ID field is set to the UTC Time Sync value in Table 7-26.

The value of the Length field is 9

The UTC Reference field is set to the UTC time at which the TSF timer is equal to the value in the TSF field in beacons or probe responses. The UTC Reference time is in UTC format, including all subfields shown in Table v6.

11.20.15 UTC Time Sync procedures

When dot11UTCTimeSyncEnabled is true, the UTC Time Sync element shall be included in the Beacon frame every 1024ms and all Probe Response frames. When the dot11UTCTimeSyncEnabled is false, the UTC Time Sync element shall not be included  Beacon and Probe Response frames.

An AP that has dot11UTCTimeSyncEnabled set to true shall maintain a UTC timer that is in synchronization with the AP’s TSF timer to a resolution of 256us. The method the AP uses to synchronize with a UTC reference clock is out of scope of the standard.

A non-AP STA shall not set dot11UTCTimeSyncEnabled to true.

A.4.21 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| WNM1 | Extended Capabilities element | 7.3.2.27 | CFv:M | Yes, No, N/A |
| WNM2 | Management Diagnostic Alerts | 11.20.2 | CFv:M | Yes, No, N/A |
| WNM2.1 | Radio Measurement Request frame | 7.4.6.1 | CFv:M | Yes, No, N/A |
| WNM2.2 | Radio Measurement Report frame | 7.4.6.2 | CFv:M | Yes, No, N/A |
| WNM2.3 | Protocol for Triggered Measurements | 11.10.7 | CFv:M | Yes, No, N/A |
| WNM2.4 | Triggered STA Statistics Reporting | 7.3.2.21.8, 7.3.2.22.8, 11.10.8.5 | CFv:O | Yes, No, N/A |
| WNM2.5 | Triggered QoS Metrics Reporting | 7.3.2.21.10, 7.3.2.22.10, 11.11.9.8 | (CFv AND CF2):O | Yes, No, N/A |
| WNM2.6 | Multicast Diagnostics Reporting | 7.3.2.21.12, 7.3.2.22.11, 11.20.2 | CFv:M | Yes, No, N/A |
| WNM3 | Event | 11.20.2 | CFv:M | Yes, No, N/A |
| WNM3.1 | Event Request frame | 7.3.2.62, 7.4.11.2 | (CFv AND CF1):M | Yes, No, N/A |
| WNM3.2 | Event Report frame | 7.3.2.63, 7.4.11.3 | (CFv AND CF2):M | Yes, No, N/A |
| WNM4 | Diagnostic | 11.20.3 | CFv:M | Yes, No, N/A |
| WNM4.1 | Diagnostic Request | 7.3.2.64, 7.4.11.4 | (CFv AND CF1):M | Yes, No, N/A |
| WNM4.2 | Diagnostic Report | 7.3.2.65, 7.4.11.5 | (CFv AND CF2):M | Yes, No, N/A |
| WNM4.3 | Manufacturer Information STA Report Diagnostic Type | 7.3.2.65.2, 11.20.3.3 | CFv:M | Yes, No, N/A |
| WNM4.4 | Configuration Profile Diagnostic Type | 7.3.2.65.3, 11.20.3.2 | CFv:M | Yes, No, N/A |
| WNM4.5 | Association Diagnostic Type | 7.3.2.64.2, 7.3.2.65.4, 11.20.3.4 | CFv:M | Yes, No, N/A |
| WNM4.6 | 802.1X Authentication Diagnostic Type | 7.3.2.64.3, 7.3.2.65.4, 11.20.3.4 | (CFv AND PC34):M | Yes, No, N/A |
| WNM5 | Location | 11.20.4, 7.3.2.66 | CFv:M | Yes, No, N/A |
| WNM5.1 | Location Civic Request/Report | 11.10.8.9 | CFv:M | Yes, No, N/A |
| WNM5.2 | Location Identifier Request/Report | 11.10.8.10 | CFv:M | Yes, No, N/A |
| WNM5.3 WNM5.3.1 WNM5.3.2 | Location Track Notification Time Of Departure Notifications Motion Detection Notifications | 11.20.4;7.4.7.10 11.20.4;7.3.2.66 11.20.4;7.3.2.66 | CFv:O CFv:O CFv:O | Yes, No, N/A |
| WNM5.4 | Location Configuration Request | 7.4.11.7;7.3.2.66 | CFv:M | Yes, No, N/A |
| WNM5.5.1 | Normal Indication | 7.4.11.7;7.3.2.66 | CFv:O | Yes, No, N/A |
| WNM5.5.2 | Motion Indication | 7.4.11.7;7.3.2.66 | CFv:O | Yes, No, N/A |
| WNM5.6 | Location Configuration Response | 7.4.11.8;7.3.2.66 | CFv:M | Yes, No, N/A |
| WNM6 | Multiple BSSID Support | 11.1.2.3a, 11.1.3 | CFv:O | Yes, No, N/A |
| WNM7 | BSS Transition Management | 11.20.6 | CFv:M | Yes, No, N/A |
| WNM7.1 | Neighbor Report Element | 7.3.2.37 | (CFv AND CF1):M | Yes, No, N/A |
| WNM7.2 | BSS Transition Management Query | 7.4.11.8 | (CFv AND CF1):M | Yes, No, N/A |
| WNM7.3 | BSS Transition Management Request | 7.4.11.9 | (CFv AND CF2):M | Yes, No, N/A |
| WNM7.4 | BSS Transition Management Response | 7.4.11.10 | (CFv AND CF2):M | Yes, No, N/A |
| *WNM8 | FMS | 9.2.7;11.20.7 | CFv:O | Yes, No, N/A |
| WNM8.1 | FMS Request frame | 7.4.11.11 | (CF2&WNM8):M | Yes, No, N/A |
| WNM8.2 | FMS Response frame | 7.4.11.12 | (CF1&WNM8):M | Yes, No, N/A |
| WNM9 | Proxy ARP | 11.20.13 | CFv:O | Yes, No, N/A |
| WNM10 | Co-located Interference Reporting | 7.4.11.13, 7.4.11.14, 11.20.9 | CFv:O | Yes, No, N/A |
| WNM11 | BSS Max Idle Period | 7.3.2.74, 11.20.12 | CFv:M | Yes, No, N/A |
| *WNM12 | TFS | 11.20.11 | CFv:O | Yes, No, N/A |
| WNM12.1 | TFS Request frame | 7.3.2.75, 7.4.11.15 | WNM12:M | Yes, No, N/A |
| WNM12.2 | TFS Response frame | 7.3.2.76, 7.4.11.16 | WNM12:M | Yes, No, N/A |
| WNM12.3 | TFS Notify frame | 7.4.11.17 | (CF1&WNM12):M, (CF2&WNM12):O | Yes, No, N/A |
| *WNM13 | WNM-Sleep Mode | 11.2.2 | CFv:O | Yes, No, N/A |
| WNM13.1 | WNM-Sleep Mode Request frame | 7.3.2.77, 7.4.11.18 | WNM13:M | Yes, No, N/A |
| WNM13.2 | WNM-Sleep Mode Response frame | 7.3.2.77, 7.4.11.19 | WNM13:M | Yes, No, N/A |
| WNM14 | TIM broadcast | 7.3.2.78, 7.3.2.79, 7.4.11.20, 7.4.11.21, 7.4.11.22, 11.2.1.13 | CFv:O | Yes, No. N/A |
| WNM15 | QoS Traffic Capability | 7.4.11.23, 11.20.9 | (CFv AND CF2):O | Yes, No, N/A |
| WNM16 | AC Station Count | 11.20.10 | (CFv AND CF2):O | Yes, No, N/A |
| WNM17 | Timing Measurement | 11.20.5 | CFv:O | Yes, No, N/A |
| WNM18 | Channel Usage | 11.20.14; 7.3.2.81; 7.4.7.12;7.4.7.13 | CFv:O | Yes, No, N/A |
| WNM19 | UTC Time Sync | 11.20.15, 7.3.2.82 | CFv:O | Yes, No, N/A |

Annex D

-- **********************************************************************

-- * dot11WirelessMgmtOptions TABLE

-- **********************************************************************

dot11WirelessMgmtOptionsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11WirelessMgmtmOptionsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

“Wireless Management attributes. In tabular form to

allow for multiple instances on an agent. This table

only applies to the interface if dot11WirelessManagementImplemented

is set to TRUE in the dot11StationConfigTable. Otherwise

this table should be ignored.”

::= { dot11smt 17 }

dot11WirelessMgmtOptionsEntry OBJECT-TYPE

SYNTAX Dot11WirelessMgmtOptionsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11WirelessMgmtOptionsTable. It is

possible for there to be multiple IEEE 802.11 interfaces

on one agent, each with its unique MAC address. The

relationship between an IEEE 802.11 interface and an

interface in the context of the Internet-standard MIB is

one-to-one. As such, the value of an ifIndex object

instance can be directly used to identify corresponding

instances of the objects defined herein.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11WirelessMgmtOptionsTable 1 }

Dot11WirelessMgmtOptionsEntry ::=

SEQUENCE {

dot11MgmtOptionLocationEnabled				TruthValue,

dot11MgmtOptionFMSImplemented				TruthValue,

dot11MgmtOptionFMSEnabled				TruthValue,

dot11MgmtOptionEventsEnabled				TruthValue,

dot11MgmtOptionDiagnosticsEnabled				TruthValue,

dot11MgmtOptionMultiBSSIDImplemented				TruthValue,

dot11MgmtOptionMultiBSSIDEnabled				TruthValue,

dot11MgmtOptionTFSImplemented 				TruthValue,

dot11MgmtOptionTFSEnabled				TruthValue,

dot11MgmtOptionWNMSleepModeImplemented				TruthValue, 																			 			dot11MgmtOptionWNMSleepModeEnabled				TruthValue,

dot11MgmtOptionTIMBroadcastImplemented				TruthValue,

dot11MgmtOptionTIMBroadcastEnabled				TruthValue,

dot11MgmtOptionProxyARPEnabled				TruthValue,
		dot11MgmtOptionProxyARPImplemented 				Truthvalue

dot11MgmtOptionBSSTransitionEnabled				TruthValue,

dot11MgmtOptionTrafficGenerationImplemented				TruthValue,

dot11MgmtOptionTrafficGenerationEnabled				TruthValue,

dot11MgmtOptionACStationCountImplemented				TruthValue,

dot11MgmtOptionACStationCountEnabled				TruthValue,

dot11MgmtOptionMulticastDiagnosticsEnabled				TruthValue,
		dot11MgmtOptionCOLOINTReportingEnabled				TruthValue,

dot11MgmtOptionCOLOINTReportingImplemented 				TruthValue,

dot11MgmtOptionMotionDetectionImplemented				TruthValue, 
		dot11MgmtOptionMotionDetectionEnabled				TruthValue,
		dot11MgmtOptionTODImplemented 				TruthValue,

dot11MgmtOptionTODEnabled				TruthValue,

dot11MgmtOptionTimingMsmtImplemented 				TruthValue,

dot11MgmtOptionTimingMsmtEnabled				TruthValue
		dot11MgmtOptionChannelUsageImplemented				TruthValue,
		dot11MgmtOptionChannelUsageEnabled				TruthValue,

dot11MgmtOptionTriggerSTAStatisticsEnabled					TruthValue,

dot11MgmtOptionSSIDListImplemented 				TruthValue,

dot11MgmtOptionSSIDListEnabled 				TruthValue,

dot11MgmtOptionUTCTimeSyncImplemented				TruthValue,

dot11MgmtOptionUTCTimeSyncEnabled				TruthValue}

dot11MgmtOptionUTCTimeSyncImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the

Station implementation is capable of supporting UTC Time Synchronization when the dot11WirelessManagementImplemented is set to TRUE."

DEFVAL { FALSE }

::= { dot11WirelessMgmtOptionsEntry 34 }

dot11MgmtOptionUTCTimeSyncEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the

capability of the station to provide UTC Time synchronization is

enabled. The capability is disabled, otherwise."

DEFVAL { FALSE }

::= { dot11WirelessMgmtOptionsEntry 35 }