IEEE P802.11
Wireless LANs

Normative Text for Intra-AC Differentiated Service in Non-802.11n

Date: 2009-01-20

| Name | Company | Address | Phone | email |
| --- | --- | --- | --- | --- |
| Xiaojun Ma | Thomson Inc. | 8/F Bld A, No. 8 Xueqing Road, Haidian Dist, Beijing 100086, PRC | +86 10 5883 7105 | Xiao-jun.ma@thomson.net |
| Jun LI | Thomson Inc. | 8/F Bld A, No. 8 Xueqing Road, Haidian Dist, Beijing 100086, PRC | +86 10 5883 7107 | lijun.bj@thomson.net |
| Hang Liu | Thomson Inc. | 2 Independence Way | +1 609 987 7335 | Hang.liu@thomson.net |

Abstract

This submission contains the normative text for Intra-AC differentiated service which allows link layer Intra-AC prioritization of transport streams and packet discarding without any requirement for deep packet inspection. The proposed text is aligned with 802.11-2007.

##### 3 Definitions

Insert the following new definitions:

3.x1 Intra-AC differentiated service (IACDS): A service which allows link layer Intra-AC prioritization of transport streams and packet discarding without any requirement for deep packet inspection. It enables modifying the EDCA timing and parameter selection for frames belonging to the same AC for intra-AC prioritization and also enables frame discarding for graceful degradation of audio video streams when there is insufficient channel capacity.

- Abbreviations and acronyms

Insert the following new acronyms:

IACDS	Intra-AC Differentiated Service

IACDSF	IACDS Flag

      - QoS control field

TGaa editor: Change Table 4-Qos Control Field as follows:

EDITORIAL NOTE—The changes comprise using bit 7 to indicate IACDS present in only QoS Data

Subtypes for non-802.11n data

| Applicable frame (sub) types | Bits 0–3 | Bit 4 | Bits 5-6 | Bit 7 | Bits 8–15 |
| --- | --- | --- | --- | --- | --- |
| QoS (+)CF-Poll frames sent by HC | TID | EOSP | Ack Policy | Reserved | TXOP Limit |
| QoS Data, QoS Null, and QoS Data+CF-Ack frames sent by HC | TID | EOSP | Ack Policy | Reserved | QAP PS Buffer State |
|  | TID | EOSP | Ack Policy | IACDSF=0 | QAP PS Buffer State |
|  | TID | EOSP | Ack Policy | IACDSF=1 | IACDS information |
| QoS data frames sent by non-AP QSTAs | TID | 0 | Ack Policy | Reserved | TXOP Duration Requested |
|  | TID | 0 | Ack Policy | IADSF=0 | TXOP Duration Requested |
|  | TID | 0 | Ack Policy | IADSF=1 | IACDS information |
|  | TID | 1 | Ack Policy | Reserved | Queue Size |
|  | TID | 1 | Ack Policy | IACDSF=0 | Queue Size |
|  | TID | 1 | Ack Policy | IACDSF=1 | IACDS information |

TGaa editor: Insert the following two new subclauses after 7.1.3.5.7 and its subclauses, if any:

7.1.3.5.8 IACDSF subfield

The IACDSF subfield is 1 bit in length and is set to 1 to indicate IACDS information present in Bit 8-15, and is set to 0 otherwise.

7.1.3.5.9 IACDS information subfield

The IACDS information subfield is an 8-bit field that contains the information used for Intra-AC Differentiated Service. The format of IACDS information subfield is defined in Figure 7-4a.

|  | B8 B10 | B11 B12 | B13 B15 |
| --- | --- | --- | --- |
|  | Reserved | Drop Precedence | Priority |
| Bits: | 2 | 2 | 3 |

Figure 7-4a—IACDS information subfield

Drop Precedence subfield is 2 bits in length and identifies frame dropping policy. The interpretation of these 2 bits is given in Table 7-6a. Frames with “drop randomly” flag set may be dropped randomly based on predetermined criteria or on-line determination when necessary. Frames with “drop all” flag set may be dropped in burst in a time period during which there is insufficient channel capacity.

Table 7-6a—Drop Precedence subfield

| B11 B12 | Description |
| --- | --- |
| 00 | No special action |
| 01 | Drop randomly if necessary |
| 10 | Drop all if necessary |
| 11 | Reserved |

Priority subfield indicates the priority of the frame in the access category.