IEEE P802.11
Wireless LANs

Normative Text for Intra-AC Differentiated Service in 802.11n

Date: 2009-01-20

| Name | Company | Address | Phone | email |
| --- | --- | --- | --- | --- |
| Xiaojun Ma | Thomson Inc. | 8/F Bld A, No. 8 Xueqing Road, Haidian Dist, Beijing 100086, PRC | +86 10 5883 7105 | Xiao-jun.ma@thomson.net |
| Jun LI | Thomson Inc. | 8/F Bld A, No. 8 Xueqing Road, Haidian Dist, Beijing 100086, PRC | +86 10 5883 7107 | lijun.bj@thomson.net |
| Hang Liu | Thomson Inc. | 2 Independence Way, Princeton NJ 08540 | 609-987-7335 | Hang.liu@thomson.net |

Abstract

This submission contains the normative text for Intra-AC differentiated service which allows link layer Intra-AC prioritization of transport streams and packet discarding without any requirement for deep packet inspection. The proposed text is aligned with P802.11n-D7.0.

##### 3 Definitions

Insert the following new definitions:

3.x1 Intra-AC differentiated service (IADS): A service which allows link layer Intra-AC prioritization of transport streams and packet discarding without any requirement for deep packet inspection. It enables modifying the EDCA timing and parameter selection for frames belonging to the same AC for intra-AC prioritization and also enables frame discarding for graceful degradation of audio video streams when there is insufficient channel capacity.

- Abbreviations and acronyms

Insert the following new acronyms:

IACDS	Intra-AC Differentiated Service

IACDSF	IACDS Flag

      - HT Control field

TGaa editor: Change Figure 7-4a—HT Control Field as follows:

EDITORIAL NOTE—The changes comprise using bit 20 by setting it to 1 to indicate IACDS information subfield present in only QoS Data Subtypes

|  | B0 B15 | B16 B17 | B18 B19 | B20 | B21 | B22 B23 | B24 | B25 B29 | B30 | B31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Link Adaptation Control | Calibration Position | Calibration Sequence | IACDSF | Reserved | CSI / Steering | NDP Announcement | IACDS Information | AC Constraint | RDG / More PPDU |
| Bits: | 16 | 2 | 2 | 1 | 1 | 2 | 1 | 5 | 1 | 1 |

Figure 7-4a—HT Control field

TGaa editor: Insert the following sentence before line 52 on page 22 of 7.1.3.5a

The IACDSF subfield is 1 bit in length and indicates an IACDS information subfield is present in the HT control filed if it is set to 1, otherwise it is set to 0.

IACDS information subfield is a 5-bit field that contains the information used for Intra-AC Differentiated Service. The format of IACDS information subfield is defined in Figure 7-4e.

|  | B25 B26 | B27 B29 |
| --- | --- | --- |
|  | Drop Precedence | Priority |
| Bits: | 2 | 3 |

Figure 7-4e—IACDS information subfield

Drop Precedence subfield is 2 bits in length and identifies frame dropping policy. The interpretation of these 2 bits is given in Table 7-6da. Frames with “drop randomly” flag set may be dropped randomly based on predetermined criteria or on-line determination when necessary. Frames with “drop all” flag set may be dropped in burst in a time period during which there is insufficient channel capacity.

Table 7-6da—Drop Precedence subfield

| B25 B26 | Description |
| --- | --- |
| 00 | No special action |
| 01 | Drop randomly if necessary |
| 10 | Drop all if necessary |
| 11 | Reserved |

Priority subfield indicates the priority of the frame in the access category.