IEEE P802.11
Wireless LANs

| ERP Protection in Mesh Network |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-01-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Ashish Shukla Sandesh Goel | Marvell India Pvt Ltd | 1st Floor, Muttha Towers, Off Airport Road, Pune, India - 411006 | +91-20-40130016 +91-99605-96612 | ashishs@marvell.com sgoel@marvell.com |
| Raja Bannerjea Harish Rammurthy | Marvell Semiconductor | 5488 Marvell Lane, Santa Clara, CA 94087 | +1-408-222-3713 +1-408-222-9683 | rajab@marvell.com harishr@marvell.com |
| Liwen Chu | ST Microelectronics | 1060 East Brokaw Rd. San Jose, CA 95131 | +1 408-467-8436 | Liwen.chu@st.com |
| Jarkko Kneckt | Nokia |  |  | Jarkko.Kneckt@nokia.com |
| Guenael Strutt | Motorola | 1064 Greenwood Blvd Lake Mary, FL 32771 | +1 407-562-4050 | Guenael.Strutt@motorola.com |

Abstract

This document describes set of recommended changes to the IEEE 802.11s draft 2.06 to incorporate ERP protection in mesh network. The IEEE 802.11s draft amendment does not include any specific PHY requirement and therefore Mesh STA can be implemented on NonERP PHYs and or a Mesh STA may overlap with BSS that includes NonERP STAs.

Change 7.3.2.13 first paragraph as follows:

7.3.2.13 ERP Information Element

The ERP Information element contains information on the presence of Clause 15 or Clause 18 STAs in the

BSS that are not capable of Clause 19 (ERP-OFDM) data rates. It also contains the requirement of the ERP

Information element sender (AP in a BSS, or STA in an IBSS or Mesh STA in an MBSS) as to the use of protection mechanisms to optimize BSS performance and as to the use of long or short Barker preambles. See Figure 7-50 for a

definition of the frame element.

Insert the following text after first paragraph of 7.3.2.13:

7.3.2.13 ERP Information element

The NonERP_Present and Use_Protection bits shall be set to 1 when a NonERP Mesh STA establishes a peering with this Mesh STA transmitting the ERP information element.

The Use_Protection bit should be additionally set to 1 when

- A Mesh STA detects the overlapped presence of either a NonERP BSS, a NonERP IBSS, or a NonERP MBSS, or

- A beacon frame is received from a Mesh STA where the supported rate set contains only clause 15 or clause 18 rates, or

- A management frame (excluding probe request) is received where the supported rate set includes only clause 15 or clause 18 rates.

ERP mesh STAs shall invoke the use of a protection mechanism after the transmission of the Use_Protection bit with a value of 1 in an MMPDU. ERP mesh STAs may additionally invoke protection mechanism at other times.

NOTE:  Mesh STAs don't have to protect ERP transmissions after reception of a mesh beacon with the Use_Protection bit set to 1.

The Barker_Preamble_mode bit shall be set to 1 if there is a NonERP Mesh  neighbor STA that that does not support the short preamble and has established a peering with the Mesh STA that is transmitting the ERP information element.

ERP Mesh STAs shall use long preambles when transmitting Clause 15, Clause 18, and Clause 19 frames after transmission or reception of an ERP Information Element with a Barker_Preamble_Mode value of 1 in an MMPDU to or from the MBSS that the ERP Mesh STA belongs to.

Change table s14 as follows:

| - Peer Link Open frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action Value |  |
|  | Capability |  |
|  | Supported rates |  |
|  | ERP Information | The ERP Information element shall be present if ERP Mesh STA detects NonERP STAs in its vicinity , and is optional otherwise. |
|  | Extended Supported Rates | The Extended Supported Rates element is present whenever there are more than eight supported rates, and it is optional otherwise. |
|  | Power Capability | The Power Capability element shall be present if dot11SpectrumManagementRequired is true. |
|  | Supported Channels | The Supported Channels element shall be present if dot11SpectrumManagementRequired is true and dot11ExtendedChannelSwitchEnabled is false. |
|  | RSN | The RSN information element is only present within Peer Link Open frames generated by mesh STAs that have dot11RSNAEnabled set to TRUE. |
|  | Mesh ID | The Mesh ID information element is present when dot11MeshEnabled is true. |
|  | Mesh Configuration | The Mesh Configuration information element is present when dot11MeshEnabled is true. |
|  | Peer Link Management | The Peer Link Management information element is present only when dot11MeshEnabled is true. The subtype of the Peer Link Management Element is set to 0. |
|  | MSCIE | The MSCIE element is present when dot11MeshEnabled is true. |
|  | MSAIE | The MSAIE element is present when dot11MeshEnabled is true. |
|  | MIC | This field is present when dot11MeshEnabled is true and the abbreviated handshake is enabled |
|  | HT Capabilities | The HT Capabilities element is present when dot11HighThroughputOptionImplemented attribute is true. |
|  | HT Information | The HT Information element is included when dot11HighThroughputOptionImplemented attribute is true. |
|  | Extended Capabilities element | The Extended Capabilities element shall be present if the dot112040BSSCoexistenceManagementSupport attribute is true and may be present otherwise. |
|  | 20/40 BSS Coexistence element | The 20/40 BSS Coexistence element may appear in this frame. |
|  | Supported MBSS Regulatory Classes and Channels | The Supported MBSS Regulatory Classes and Channels information element is present if dot11ExtendedChannelSwitchEnabled is true. |

Change table s15  as follows:

| - Peer Link Confirm frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Category |  |
|  | Action Value |  |
|  | Capability |  |
|  | Status code |  |
|  | AID |  |
|  | Supported rates |  |
|  | ERP Information | The ERP Information element shall be present if ERP Mesh STA detects NonERP STAs in its vicinity, and is optional otherwise. |
|  | Extended Supported Rates | The Extended Supported Rates element is present whenever there are more than eight supported rates, and it is optional otherwise. |
|  | RSN | The RSN information element is only present when dot11RSNAEnabled is set to TRUE. |
|  | EDCA Parameter Set |  |
|  | Mesh ID | The Mesh ID information element is present when dot11MeshEnabled is true. |
|  | Mesh Configuration | The Mesh Configuration information element is present when dot11MeshEnabled is true. |
|  | Peer Link Management | The Peer Link Management information element is present only when dot11MeshEnabled is true. The subtype of the Peer Link Management Element is set to 1. |
|  | MSCIE | The MSCIE element is present when dot11MeshEnabled is true. |
|  | MSAIE | The MSAIE element is present when dot11MeshEnabled is true. |
|  | MIC | This field is present when dot11MeshEnabled is true and the abbreviated handshake is enabled |
|  | HT Capabilities | The HT Capabilities element is present when dot11HighThroughputOptionImplemented attribute is true. |
|  | HT Information | The HT Information element is included when dot11HighThroughputOptionImplemented attribute is true. |
|  | Extended Capabilities element | The Extended Capabilities element shall be present if the dot112040BSSCoexistenceManagementSupport attribute is true and may be present otherwise. |
|  | 20/40 BSS Coexistence element | The 20/40 BSS Coexistence element may appear in this frame. |

References:  IEEE 802.11-08/1468r2 “ERP Protection in IEEE 802.11s Mesh Network”