IEEE P802.11
Wireless LANs

| Normative Texts for 11s MIB Structure |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-01-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Liwen Chu | STMicroelectronics, Inc. | 1060 East Brokaw Rd. San Jose, CA 95131 | +1-408-467-8436 | Liwen.chu@st.com |
| George Vlantis | STMicroelectronics, Inc. | 1060 East Brokaw Rd. San Jose, CA 95131 | +1 408 451-8109 | George.vlantis@st.com |

Abstract

This submission provides updated MIB structure to resolve the following comments: 984, 985, 1947, 1950

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Background

This contribution provides MIB structure to aline with 802.11 Baseline Standard and also resolve the following comments: 984, 985, 1947, 1950..

The following is normative text proposed as an amendment to P802.11s/D2.06.

Add  the following objects to the Major sections:

dot11MeshStationConfigTable ::= { dot11smt 18 }

dot11MeshHWMPConfigTable ::= {dot11smt19}

Change the “Dot11StationConfigEntry” of the “dotStationConfigTable” as follows:

Dot11StationConfigEntry ::=

SEQUENCE {

dot11StationID 					MacAddress,

dot11MediumOccupancyLimit 			INTEGER,

dot11CFPollable 					TruthValue,

dot11CFPeriod					 INTEGER,

dot11CFPMaxDuration 				INTEGER,

dot11AuthenticationResponseTimeOut 		Unsigned32,		dot11PrivacyOptionImplemented 		TruthValue,

dot11PowerManagementMode 			INTEGER,

dot11DesiredSSID OCTET 				STRING,

dot11DesiredBSSType 				INTEGER,

dot11OperationalRateSet 				OCTET STRING,

dot11BeaconPeriod 				INTEGER,

dot11DTIMPeriod 				INTEGER,

dot11AssociationResponseTimeOut 			Unsigned32,

dot11DisassociateReason 				INTEGER,

dot11DisassociateStation 				MacAddress,

dot11DeauthenticateReason 			INTEGER,

dot11DeauthenticateStation 				MacAddress,

dot11AuthenticateFailStatus 			INTEGER,

dot11AuthenticateFailStation 			MacAddress,

dot11MultiDomainCapabilityImplemented 		TruthValue,

dot11MultiDomainCapabilityEnabled 		TruthValue,

dot11CountryString 				OCTET STRING,

dot11SpectrumManagementImplemented 		TruthValue,

dot11SpectrumManagementRequired 		TruthValue,

dot11RSNAOptionImplemented 			TruthValue,

dot11RSNAPreauthenticationImplemented 		TruthValue,

dot11RegulatoryClassesImplemented	 		TruthValue,

dot11RegulatoryClassesRequired 			TruthValue,

dot11QosOptionImplemented 			TruthValue,

dot11ImmediateBlockAckOptionImplemented 		TruthValue,

dot11DelayedBlockAckOptionImplemented 		TruthValue,

dot11DirectOptionImplemented 			TruthValue,

dot11APSDOptionImplemented 			TruthValue,

dot11QAckOptionImplemented 			TruthValue,

dot11QBSSLoadOptionImplemented 			TruthValue,

dot11QueueRequestOptionImplemented 		TruthValue,

dot11TXOPRequestOptionImplemented 		TruthValue,

dot11MoreDataAckOptionImplemented 				TruthValue,

dot11AssociatedinNQBSS 						TruthValue,

dot11DLSAllowdInQBSS 						TruthValue,

dot11DLSAllowed 						TruthValue,

dot11AssociateStation 						MacAddress,

dot11AssociateID 							INTEGER,

dot11AssociateFailStation 						MacAddress,

dot11AssociateFailStatus 						INTEGER,

dot11ReassociateStation 						MacAddress,

dot11ReassociateID 						INTEGER,

dot11ReassociateFailStation 					MacAddress,

dot11ReassociateFailStatus 					INTEGER,

dot11RadioMeasurementCapable 					TruthValue,

dot11RadioMeasurementEnabled 					TruthValue,

dot11RRMMeasurementProbeDelay 				INTEGER,

dot11RRMMeasurementPilotPeriod 				INTEGER,

dot11RRMLinkMeasurementEnabled 				TruthValue,

dot11RRMNeighborReportEnabled 					TruthValue,

dot11RRMParallelMeasurementsEnabled TruthValue,

dot11RRMRepeatedMeasurementsEnabled TruthValue,

dot11RRMBeaconPassiveMeasurementEnabled TruthValue,

dot11RRMBeaconActiveMeasurementEnabled TruthValue,

dot11RRMBeaconTableMeasurementEnabled TruthValue,

dot11RRMBeaconMeasurementReportingConditionsEnabled TruthValue,

dot11RRMFrameMeasurementEnabled TruthValue,

dot11RRMChannelLoadMeasurementEnabled TruthValue,

dot11RRMNoiseHistogramMeasurementEnabled TruthValue

dot11RRMStatisticsMeasaurementEnabled TruthValue,

dot11RRMLCIMeasurementEnabled TruthValue,

dot11RRMLCIAzimuthEnabled TruthValue,

dot11RRMTransmitStreamCategoryMeasurementEnabled TruthValue,

dot11RRMTriggeredTransmitStreamCategoryMeasurementEnabled

TruthValue,

dot11RRMAPChannelReportEnabled TruthValue,

dot11RRMMIBEnabled TruthValue,

dot11RRMMaxMeasurementDuration Unsigned32,

dot11RRMNonOperatingChannelMaxMeasurementDuration Unsigned32,

dot11RRMMeasurementPilotTransmissionInformationEnabled TruthValue,

dot11RRMMeasurementPilotCapability Unsigned32,

dot11RRMNeighborReportTSFOffsetEnabled TruthValue,

dot11RRMRCPIMeasurementEnabled TruthValue,

dot11RRMRSNIMeasurementEnabled TruthValue,

dot11RRMBSSAverageAccessDelayEnabled TruthValue,

dot11RRMBSSAvailableAdmissionCapacityEnabled TruthValue,

dot11RRMAntennaInformationEnabled TruthValue,

dot11FastBSSTransitionImplemented TruthValue

dot11LCIDSEImplemented TruthValue,

dot11LCIDSERequired TruthValue,

dot11DSERequired TruthValue,

dot11ExtendedChannelSwitchEnabled TruthValue,

dot11RSNAProtectedManagementFramesEnabled TruthValue,

dot11RSNAUnprotectedManagementFramesAllowed TruthValue,

dot11AssociationPingResponseTimeout Unsigned32,

dot11AssociationMaximumPingAttempts INTEGER,

dot11HighThroughputOptionImplemented TruthValue,

dot11RSNAPBACRequired TruthValue,

dot11MeshEnabled							TruthValue

}

Add “dot11MeshEnabled” to the end of  “dotStationConfigTable”:

dot11MeshEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This attribute shall specify whether or not mesh services are supported by a station.”

::= { dot11StationConfigEntry 1}

Insert the following dot11MeshSTATable to the end of dot11HTStationConfigTable:

**********************************************************************

* dot11MeshSTAConfig TABLE

**********************************************************************

dot11MeshStationConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11MeshStationConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"MeshStation Configuration attributes. In tablular form to allow for

multiple instances on an agent."

dot11MeshStationConfigEntry  OBJECT-TYPE

SYNTAX Dot11 MeshStationConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11MeshStationConfigTable. It is

possible for there to be multiple IEEE 802.11 interfaces

on one agent, each with its unique MAC address. The

relationship between an IEEE 802.11 interface and an

interface in the context of the Internet-standard MIB is

one-to-one. As such, the value of an ifIndex object

instance can be directly used to identify corresponding

instances of the objects defined herein.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11MeshStationConfigTable 1 }

Dot11MeshStationConfigEntry ::=

SEQUENCE {

dot11MeshID				OCTET STRING,

dot11MeshHWMPEnabled			TruthValue,

dot11MeshForwarding			TruthValue,

dot11MeshMaxRetries			INTEGER,

dot11MeshRetryTimeout			INTEGER,

dot11MeshTTL				INTEGER,

dot11MeshPortalAnnouncementInterval	INTEGER,

dotMeshConfirmTimeout			INTEGER,

dotMeshHoldingTimeout			INTEGER,

dot11MeshFirstLevelKeyLifetime		Unsigned32

dot11MeshKeyDistributorDomainID	OCTET STRING

dot11MeshMKDNASID			OCTET STRING

dot11MeshKHHandshakeAttempts		INTEGER

dot11MeshKHHandshakeTimeout		INTEGER

dot11MeshKeyTransportTimeout		INTEGER

dot11SAERetransPeriod			INTEGER

dot11SAEThresh			INTEGER

dot11MeshBeaconPeriod				INTEGER,

dot11MeshDTIMPeriod				INTEGER,

dot11MeshDelayedBeaconTxInterval			INTEGER,

dot11MeshDelayedBeaconDelayAmount		INTEGER,

dot11MeshExpectedBeaconFrameDuration		INTEGER,

dot11MeshSTAMissingAckRetryLimit		INTEGER,

dot11MDAImplemented 				TruthValue,

dot11MDAEnabled 				TruthValue,

dot11MAFlimit 					INTEGER,

dot11MDAScanDuration				INTEGER,

dot11MDAdvertPeriodMax				INTEGER,

dot11MDAOPtimeout				INTEGER,

dot11MeshNbrOffEnabled				TruthValue

}

dot11MeshID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(0..32))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute reflects the Mesh ID configured in this entity."

::= { dot11MeshSTAConfigEntry 1}

dot11MeshHWMPEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object shall specify whether or not the station is HWMP enabled."

::= { dot11MeshSTAConfigEntry 2}

dot11MeshForwarding OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the ability of a mesh STA to forward frames. The default value for this attribute is 1."

::= { dot11MeshSTAConfigEntry 3}.

dot11MeshMaxRetries OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

“This object specifies the maximum number of Peer Link Open retries that can be sent to establish a new peer link instance in a mesh. The default value for this attribute is 2.”

::= { dot11MeshSTAConfigEntry 4}

dot11MeshRetryTimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the initial retry timeout, in millisecond units, used by the Peer Link Open message. The default value for this attribute is 40."

::= { dot11MeshSTAConfigEntry 5}

dot11MeshTTL OBJECT-TYPE

SYNTAX INTEGER (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the value of TTL field set at a source mesh STA. The default value for this attribute is 31."

::= { dot11MeshSTAConfigEntry 6}

dot11MeshPortalAnnouncementInterval OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The Portal Announcement Interval is the number of seconds between the transmission of two Portal Announcements. The default value for this attribute is 10."

::= { dot11MeshSTAConfigEntry 7}

dot11MeshConfirmTimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the initial retry timeout, In millisecond units, used by the Peer Link Open message. The default value for this attribute is 40."

::= { dot11MeshSTAConfigEntry 8}

dot11MeshHoldingTimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the confirm timeout, in millisecond units, used by the peer link management to close a peer link. The default value for this attribute is 40."

::= { dot11MeshSTAConfigEntry 9}

dot11MeshFirstLevelKeyLifetime OBJECT-TYPE

SYNTAX Unsigned32 (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the default lifetime of the PMK-MKD, in minutes when a Session-Timeout attribute is not provided during the EAP authentication."

::= { dot11MeshSTAConfigEntry 10}

dot11MeshKeyDistributorDomainID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(6))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the MKD domain identifier of this entity."

::= { dot11MeshSTAConfigEntry 11}

dot11MeshMKDNASID OBJECT-TYPE

SYNTAX OCTET STRING (SIZE(1..48))

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute shall specify the MKD Key Holder identifier of the Authenticator of this entity.

NOTE: Backend protocol may allow longer NAS Client identifiers (e.g., RADIUS allows up to 253 octet NAS-Identifier), but when used with Initial MSA Authentication, the maximum length is limited to 48 octets. The same value must be used for the NAS Client identifier and dot11MeshMKDNASID to allow EAP channel binding.

::= { dot11MeshSTAConfigEntry 12}

dot11MeshKHHandshakeAttempts OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The number of times transmission of mesh key holder security handshake messages 1 and 3 will be attempted before indicating failure of the mesh key holder security handshake protocol."

::= { dot11MeshSTAConfigEntry 13}

dot11MeshKHHandshakeTimeout OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The time in milliseconds between transmission attempts of mesh key holder security handshake messages 1 and 3, and between the final transmission attempt and indicating failure of the mesh key holder security handshake protocol."

::= { dot11MeshSTAConfigEntry 14}

dot11MeshKeyTransportTimeout OBJECT-TYPE

SYNTAX INTEGER (1..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"The timeout value in milliseconds that a mesh entity waits for a response message in a key transport protocol before indicating failure of the key transport protocol."

::= { dot11MeshSTAConfigEntry 15}

dot11SAERetransPeriod OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the initial retry timeout, in millisecond units, used by the SAE authentication and key establishment protocol. The default value of this attribute is 40."

::= { dot11MeshSTAConfigEntry 16}

dot11SAEThresh OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object specifies the maximum number of SAE protocol instances allowed to simultaneously be in either Commit or Confirmed state. The default value of this attribute is 5."

::= { dot11MeshSTAConfigEntry 17}

dot11MeshBeaconPeriod OBJECT-TYPE

SYNTAX INTEGER (0..65535)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object specifies the number of TUs that a mesh STA shall use for scheduling beacon transmissions. This value is transmitted in the Beacon Interval field of Beacon and Probe Response frames. The default value of this attribute is 200."

::= { dot11MeshSTAConfigEntry 18}

dot11MeshDTIMPeriod OBJECT-TYPE

SYNTAX INTEGER  (1..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This object specifies the number of beacon intervals that shall elapse between transmission of beacon frames containing a Mesh TIM element whose Mesh DTIM Count field is 0. This value is transmitted in the Mesh DTIM Period field of Beacon and Probe Response frames. The default value of this attribute is 5."

::= { dot11MeshSTAConfigEntry 19}

dot11MeshDelayedBeaconTxInterval OBJECT-TYPE

SYNTAX INTEGER  (0..255)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute shall specify the interval for the delayed beacon transmission from TBTT, for MBCA purpose. The value is expressed in unit of beacon interval, whereas the value 0 indicates that the delayed beacon transmission is disabled exceptionally. The default value of this attribute is 0."

::= { dot11MeshSTAConfigEntry 20}

dot11MeshDelayedBeaconDelayAmount OBJECT-TYPE

SYNTAX INTEGER  (0..4023)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute shall specify the delay amount for the delayed beacon transmission from TBTT, for MBCA purpose. The value is expressed in units of microseconds. The default value of this attribute is 0."

::= { dot11MeshSTAConfigEntry 21}

dot11MeshExpectedBeaconFrameDuration OBJECT-TYPE

SYNTAX INTEGER  (0..4023)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

“This attribute shall specify the expected duration of beacon frames received by the neighboring mesh STAs. The value is expressed in units of microseconds."

::= { dot11MeshSTAConfigEntry 22}

dot11MeshSTAMissingAckRetryLimit OBJECT-TYPE

SYNTAX INTEGER (0..100)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute indicates the number of times the mesh STA may retry a frame for which it does not receive an ACK for a STA in power-save mode after the mesh STA does not receive an ACK to a directed MPDU sent with the EOSP set to 1."

::= { dot11MeshSTAConfigEntry 23}

dot11MDAImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object shall specify whether or not the MDA is implemented in this station."

::= { dot11MeshSTAConfigEntry 24}

dot11MDAEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify whether or not the station is MDA enabled."

::= { dot11MeshSTAConfigEntry 25}

dot11MAFlimit OBJECT-TYPE

SYNTAX INTEGER (0..16)

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the maximum MDA access fraction allowed at the mesh STA. This number is always a multiple of (1/16)."

::= { dot11MeshSTAConfigEntry 26}

dot11MDAScanDuration OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute specifies the duration in TUs that a mesh STA scans on the current operating channel before it may accept or setup MDAOPs."

::= { dot11MeshSTAConfigEntry 27}

dot11MDAdvertPeriodMax OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the minimum interval with which an MDA active mesh STA advertises its MDAOP Advertisements information element. It is expressed in number of mesh DTIM intervals."

::= { dot11MeshSTAConfigEntry 28}

dot11MDAOPtimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object specifies the timeout value for an MDAOP teardown. it is expressed in TU units."

::= { dot11MeshSTAConfigEntry 29}

dot11MeshNbrOffEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify whether or not the station is neighbor offset protocol enabled."

::= { dot11MeshSTAConfigEntry 30}

Insert the following dot11MeshHWMPTable to the end of dot11MeshSTAConfigTable:

**********************************************************************

* dot11MeshHWMPConfig TABLE

**********************************************************************

dot11MeshHWMPConfigTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11MeshHWMPConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"MeshStation HWMP Configuration attributes. In tablular form to allow for

multiple instances on an agent."

dot11MeshHWMPConfigEntry  OBJECT-TYPE

SYNTAX Dot11 MeshHWMPConfigEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11MeshHWMPConfigTable. It is

possible for there to be multiple IEEE 802.11 interfaces

on one agent, each with its unique MAC address. The

relationship between an IEEE 802.11 interface and an

interface in the context of the Internet-standard MIB is

one-to-one. As such, the value of an ifIndex object

instance can be directly used to identify corresponding

instances of the objects defined herein.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11MeshStationConfigTable 1 }

Dot11MeshHWMPConfigEntry ::=

SEQUENCE {

dot11MeshHWMPmaxPREQretries			INTEGER,

dot11MeshHWMPnetDiameterTraversalTime		INGETER,

dot11MeshHWMPpreqMinInterval			INTEGER,

dot11MeshHWMPperrMinInterva			INTEGER,

dot11MeshHWMPactiveRootTimeout		INTEGER,

dot11MeshHWMPactivePathTimeout		INTEGER,

dot11MeshHWMPpathToRootInterval		INTEGER,

dot11MeshHWMPrannInterval			INTEGER

}

dot11MeshHWMPmaxPREQretries OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the number of action frames containing a PREQ that a mesh STA can send to a particular path target. The default value of this attribute is 3"

::= { dot11MeshHWMPConfigEntry 1}

dot11MeshHWMPnetDiameterTraversalTime OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the estimate of the interval of time (in TUs) that it takes for an HWMP information element to propagate across the Mesh. The default value of this attribute is 500"

::= { dot11MeshHWMPConfigEntry 2}

dot11MeshHWMPpreqMinInterval OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the minimum interval of time (in TUs) during which a mesh STA can send only one action frame containing a PREQ information element. The default value of this attribute is 100"

::= { dot11MeshHWMPConfigEntry 3}

dot11MeshHWMPperrMinInterva OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the minimum interval of time (in TUs) during which a mesh STA can send only one action frame containing a PERR information element. The default value of this attribute is 100"

::= { dot11MeshHWMPConfigEntry 4}

dot11MeshHWMPactiveRootTimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the time (in TUs) for which mesh STAs receiving a PREQ shall consider the forwarding information from the root to be valid. The default value of this attribute is 5000"

::= { dot11MeshHWMPConfigEntry 5}

dot11MeshHWMPactivePathTimeout OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the time (in TUs) for which mesh STAs receiving a PREQ shall consider the forwarding information to be valid. The default value of this attribute is 5000"

::= { dot11MeshHWMPConfigEntry 6}

dot11MeshHWMPpathToRootInterval OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the time (in TUs) for which mesh STAs receiving a PREQ shall consider the forwarding information to be valid; it must be greater than dot11MeshHWMPrannInterval. The default value of this attribute is 5000"

::= { dot11MeshHWMPConfigEntry 7}

dot11MeshHWMPrannInterval OBJECT-TYPE

SYNTAX INTEGER

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object shall specify the minimum interval of time (in TUs) during which a mesh STA can send only one action frame containing a RANN information element. The default value of this attribute is 1000"

::= { dot11MeshHWMPConfigEntry 8}

Delete V.2 from the draft