IEEE P802.11
Wireless LANs

| Liaison Response from 3GPP CT1 to IEEE 802.11 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-06-26 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| 3GPP CT1 | 3GPP |  |  |  |

Abstract

This document contains a liaison response from 3GPP CT1 to IEEE 802.11 regarding WLAN related information in ANDSF MO and ANID for WLAN access.

.

- 3GPP TSG-CT WG1 Meeting #59							C1-093159

- Los Angeles (USA), 22-26 June 2009.

- Title:		WLAN related information in ANDSF MO and ANID for WLAN access

- Response to:	C1-092292/ IEEE 802.11-09/0645r0 on Liaison response to 3GPP CT1 C1-085530

- Release:	Release 8

- Work Item:

- Source:	3GPP TSG CT WG1

- To:	IEEE P 802.11, 3GPP TSG SA WG2

- Cc:	OMA Device Management Working Group

- Contact Person:

- Name:		Chen-Ho CHIN

- Tel. Number:		+32-474-520.341

- E-mail Address:	chchin@rim.com

- Attachments:	None

- Overall Description:

- 3GPP CT1 thanks IEEE 802.11 for their LS (11-09-0645r1 / C1-092292) responding to an earlier CT1 LS on the ANDSF MO (Access Network Discovery and Selection Function Management Object).

- To recap, the LS from IEEE 802.11 requested CT1 to consider:

- o	Introduction of the HESSID parameter

- o	change the term "WiFi" to "WLAN"

- o	specify frequency and/or frequency bands for WLAN

- within CT1's ANDSF MO, i.e. TS 24.312

- 2. 3GPP CT1 Response:

- 3GPP CT1 has considered the requested actions and would like inform IEEE 802.11 that CT1 can attempt to complete some of this work, but that there will be related work to be done in other committees.

- a)	Changes to ANDSF MO (Access Network Discovery and Selection Function Management Object): HESSID and Frequency Bands

- The ANDSF MO is written in such a manner that SDOs responsible for specifying their non-3GPP access technology that access to the 3GPP EPC can define their own MOs which are then referenced within the ANDSF MO. Thus it can be seen that the ANDSF MO provides the vehicle for non-3GPP access technology MOs.

- Within the existing ANDSF MO, there is a reference to a WLAN MO where WLAN parameters reside. However, as this reference is currently OMA-DDS-DM_ConnMO_WLAN-V1_0-20081024-A: “Standardized Connectivity Management Objects WLAN Parameters”, it is this WLAN MO that most of the necessary changes that IEEE 802.11 requested have to be made.

- Since this MO is owed by OMA, CT1 cannot do this work and so CT1 kindly request IEEE 802.11 to consider contacting OMA directly.

- Specifically these changes cover the introduction of HESSID and for inclusion of frequency and/or frequency bands.

- b)  Changing the term "WiFi" to "WLAN"

- At the level of specification work done within CT1 for WLAN as a non-3GPP access, this request does not pose a problem. However, CT1 wishes to note that the term "WLAN" is used in the 3GPP architecture to refer to other wireless local access technologies other than IEEE 802.11, such as that defined by ETSI BRAN.  Hence CT1 is unsure that the use of the term "WLAN" if introduced to ANDSF MO TS 24.312 can be interpreted that other WLAN technologies (other than IEEE 802.11) is covered in our ANDSF MO.

- As 3GPP SA2 is the 3GPP WG responsible for 3GPP system architecture, CT1 would kindly ask SA2 to consider this point and decide if CT1 can proceed to replace "WiFi" with "WLAN" in TS 24.312.

- 3. New Item: ANID parameter

- CT1 kindly asks IEEE 802.11, whether it has considered the introduction of an ANID (Access Network Identity) parameter, into IEEE 802.11, as specified within TS 24.302 (6.4.2.4.5).  This document was referenced in our original liaison (C1-085530), from November 2008.

- To recap, this parameter would be exchanged during an EAP authentication exchange between the 3GPP EPC and a mobile terminal accessing through the non-3GPP access technology (i.e. IEEE 802.11), to identify the specific type of non-3GPP access technology.

- TS 24.302 currently recommends that the ANID parameter, to be defined within IEEE 802.11, is assigned the value “WLAN”. However, this term might be reviewed when SA2 answers the question CT1 poses in section 2b) above.

- 4. Actions:

- To IEEE 802.11

- ACTION:

- i)	Can IEEE 802.11 consider sending a liaison to OMA regarding adding the extra IEEE 802.11 parameters to the OMA WLAN MO?

- Ii)	Can IEEE 802.11 consider the introduction of an ANID parameter in the IEEE 802.11 specification to assist 3GPP interworking?

- To SA2

- ACTION:

- Can SA2 consider if the replacing of the term "WiFi" with "WLAN" can be performed without giving the interpretation that other WLAN technologies considered in 23.234 are included?

- 5. Date of Next TSG-CT WG1 Meetings:

- TSG-CT WG1 Meeting #60	24-28 August 2009	Sevilla, Spain.

- TSG-CT WG1 Meeting #61	12-16 October 2009	TBD, USA.