IEEE P802.11
Wireless LANs

| Implementation of 60 GHz WLAN Channel Model |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 18-02-2010 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Roman Maslennikov | University of Nizhny Novgorod | Gagarina ave., 23, Nizhny Novgorod, 603022, Russia | +79047855055 | roman.maslennikov@ wcc.unn.ru |
| Artyom Lomayev | Intel | Turgeneva str., 30, Nizhny Novgorod, 603024, Russia | +78312969444 | artyom.lomayev@ intel.com |

Abstract

Description of the source code implementing 60 GHz WLAN channel models proposed in [1].

Revision History

- July 2009 – Initial version.

- February 2010 – description of Matlab/C source code for Living Room channel model added, appendix describing beamforming algorithm deleted.

Channel Model Source Code

The source code for Conference Room and Living Room channel models is in the zip file embedded below.

Table of Contents

1	Channel Models Code Description	3

1.1	General Code Structure	3

1.2	Description of Data Structures Used in Channel Models	4

1.3	Directory Tree for Channel Models Source Code	7

1.4	Channel Models Test Script	9

2	References	11

# Channel Models Code Description

## General Code Structure

This document describes a structure of the software code developed for channel models described in [1]. The channel model source code is a combination of Matlab software functions and C-code routines that are integrated to Matlab through MEX (Matlab Executable) interface.

The functional block diagram describing general code organization and relations between different high-level functions is shown in Figure 1 below.

Figure 1. General code structure implementing Conference Room and Living Room channel models

As it can be seen from Figure 1 the top functions of the Conference Room and Living Room channel models are cr_ch_model.m and lr_ch_model.m accordingly. These functions have the same structure shown in Figure 1. Each top level function subsequently calls the input parameters initialization function cr_ch_cfg.m (or lr_ch_cfg.m), channel impulse response generation function gen_cr_ch.m (or gen_lr_ch.m), function implementing beamforming algorithm beamforming.m, and a simple subroutine ct2dt.m implementing analog to discrete time conversion of channel impulse response in accordance with the target sample rate. Initialization function and function generating impulse response are different for the Conference Room and Living Room channel models. Beamforming function and analog to discrete conversion subroutine are the same for all channel models.

Both the channel realization generation and beamforming algorithm functions have very many input parameters. In the Matlab implementation of the channel model these parameters were organized into data structures so that to simplify the interfaces of these functions.

The diagram in Figure 1 illustrates the order in which software functions are executed during top function call. The first function in this pipeline is the configuration function where all input channel model parameters are set. It returns structure cfg containing all channel model settings. Configuration structure includes common parameters used by the top function and two substructures. The first substructure cfg.cr (or cfg.lr) is unique for the considering Conference Room (or Living Room) channel model. The second substructure cfg.bf is common for all channel models and contains beamforming settings. These two substructures provide input parameters for gen_cr_ch.m (or gen_lr_ch.m) and beamforming.m functions.

Functions gen_cr_ch.m and gen_lr_ch.m implement generation of space-temporal channel impulse response realization for the Conference Room and Living Room environments including angular, temporal, amplitude and phase characteristics. These functions have the same interface:

[ch] = gen_cr_ch(cfg.cr,cfg.bf.ps,cfg.bf.pol),

[ch] = gen_lr_ch(cfg.lr,cfg.bf.ps,cfg.bf.pol),

where ch is an output channel structure keeping all space-time characteristics listed above.

Function beamforming.m implements a max power ray and a max power exhaustive search beamforming algorithms described in [1]. The output of this function is a channel impulse response between the transmitter and receiver with antenna patterns optimally steered by the selected beamforming algorithm. The provided channel impulse response is in the continued time. The interface of the beamforming function is:

[imp_res,toa] = beamforming(cfg.bf,ch),

where output arrays imp_res and toa provide complex amplitudes of rays after beamforming and their relative time values. Max power exhaustive search beamforming was realized in both Matlab and C languages. The user has an option to choose between Matlab or C subroutine by setting a special flag in the cfg.bf structure. exhaustive_search_mex.c gateway routine is provided to integrate C-code into Matlab environment through MEX interface. Note that C subroutines fully duplicate Matlab subfunctions that are executed during beamforming.m call. They have the same names and the same functionality except for negligible details relating to C-code realization aspects. The main reason to provide C-code version is to accelerate computations related to exhaustive search (especially for the search with a great number of space positions) and therefore to reduce the amount of time for the channel realizations generation.

Note, that a user can implement its own beamforming algorithm and use it instead of standard function beamforming.m, because as remarked in EVM [2], TGad proposals may also include system evaluation results based on their proposed antenna and beamforming.

## Description of Data Structures Used in Channel Models

This section provides description of the configuration structure cfg and channel impulse response structure ch used in the developed channel models.

Table 1 shows all fields of cfg structure, provides short description for every field and gives default values of the fields.

Table 1. Description of cfg structure fields

| # | Structure Field | Field Description | Default Value |
| --- | --- | --- | --- |
| 1 | cfg.Pnorm | Normalization parameter: 0 - w/o normalization, 1 - apply normalization for output channel impulse response. | 1 |
| 2 | cfg.sample_rate | Sample rate in [GHz] applied for continuous time to discrete time channel impulse response conversion. | 2.56 |
| 3 | cfg.cr | Substructure contains settings for the Conference Room channel model. | n/a |
|  | cfg.lr | Substructure contains settings for the Living Room channel model. | n/a |
| 4 | cfg.bf | Substructure contains beamforming algorithm settings. | n/a |

Table 2 shows all fields of cfg.cr substructure, provides short description for every field and gives default values of the fields.

Table 2. Description of cfg.cr substructure fields

| # | Structure Field | Field Description | Default Value |
| --- | --- | --- | --- |
| 1 | cfg.cr.ap_sp | Parameter selects subscenario, permitted values: 0 – STA-STA subscenario, 1 - STA-AP subscenario. | 0 |
| 2 | cfg.cr.D | Distance in meters between transmitter and receiver sides. Note that for the STA-AP subscenario D is set in projection to horizontal plane. | 2 |
| 3 | cfg.cr.Plos | LOS (Line-of-Sight) parameter, permitted values: 0 – corresponds to NLOS scenario, 1 – corresponds to LOS scenario. | 0 |
| 4 | cfg.cr.Psta_1st_c | Probability that the cluster is present (i.e. not blocked) for the 1st order reflections from ceiling for STA-STA subscenario. | 1 |
| 5 | cfg.cr.Psta_1st_w | Probability that the cluster is present (i.e. not blocked) for the 1st order reflections from walls for STA-STA subscenario. | 0.76 |
| 6 | cfg.cr.Psta_2nd_wc | Probability that the cluster is present (i.e. not blocked) for the 2nd order wall-ceiling (ceiling-wall) reflections for STA-STA subscenario. | 0.963 |
| 7 | cfg.cr.Psta_2nd_w | Probability that the cluster is present (i.e. not blocked) for the 2nd order reflections from walls for STA-STA subscenario. | 0.825 |
| 8 | cfg.cr.Pap_1st | Probability that the cluster is present (i.e. not blocked) for the 1st order reflections from walls for the STA-AP subscenario. | 0.874 |
| 9 | cfg.cr.Pap_2nd | Probability that the cluster is present (i.e. not blocked) for the 2nd order reflections from walls for the STA-AP subscenario. | 0.93 |

Table 3 shows all fields of cfg.lr substructure, provides short description for every field and gives default values of the fields.

Table 3. Description of cfg.lr substructure fields

| # | Structure Field | Field Description | Default Value |
| --- | --- | --- | --- |
| 1 | cfg.lr.D | Distance in meters between transmitter and receiver sides. | 4 |
| 2 | cfg.lr.Plos | LOS (Line-of-Sight) parameter, permitted values: 0 – corresponds to NLOS scenario, 1 – corresponds to LOS scenario. | 0 |
| 3 | cfg.lr.P_1st_c | Probability that the cluster is present (i.e. not blocked) for the 1st order reflections from ceiling. | 1 |
| 4 | cfg.lr.P_1st_f | Probability that the cluster is present (i.e. not blocked) for the 1st order reflections from floor. | 0.3 |
| 5 | cfg.lr.P_1st_w | Probability that the cluster is present (i.e. not blocked) for the 1st order reflections from walls. | 0.6 |
| 6 | cfg.lr.P_2nd_w | Probability that the cluster is present (i.e. not blocked) for the 2nd order reflections from two walls. | 0.2 |
| 7 | cfg.lr.P_2nd_cf | Probability that the cluster is present (i.e. not blocked) for the 2nd order reflections from ceiling and floor. | 0.2 |
| 8 | cfg.lr.P_2nd_wc | Probability that the cluster is present (i.e. not blocked) for the 2nd order wall-ceiling (ceiling-wall) reflections. | 0.7 |
| 9 | cfg.lr.P_2nd_wf | Probability that the cluster is present (i.e. not blocked) for the 2nd order wall-floor (floor-wall) reflections. | 0.3 |

Table 4 shows all fields of cfg.bf substructure, provides short description for every field and gives default values of the fields.

Table 4. Description of cfg.bf substructure fields

| # | Structure Field | Field Description | Default Value |
| --- | --- | --- | --- |
| 1 | cfg.bf.bf_alg | Beamforming algorithm: 0 - max power ray algorithm, 1 - max power exhaustive search algorithm. | 0 |
| 2 | cfg.bf.matlab_c | This flag selects Matlab or C function to perform exhaustive search in beamforming procedure: 0 - use Matlab function, 1 - use C function. | 0 |
| 3 | cfg.bf.tx_ant_type | TX antenna type, permitted values: 0 - isotropic radiator, 1 - basic steerable directional antenna. | 1 |
| 4 | cfg.bf.rx_ant_type | RX antenna type, permitted values: 0 - isotropic radiator, 1 - basic steerable directional antenna. | 1 |
| 5 | cfg.bf.tx_sec_bound | TX sector bound in exhaustive search beamforming algorithm. | 900/1200 |
| 6 | cfg.bf.rx_sec_bound | RX sector bound in exhaustive search beamforming algorithm. | 900/1200 |
| 7 | cfg.bf.tx_hpbw | TX antenna HPBW. | 300 |
| 8 | cfg.bf.rx_hpbw | RX antenna HPBW. | 300 |
| 9 | cfg.bf.ps | Polarization support parameter: 0 - TX/RX polarization vectors are not applied, 1 - polarization is applied. | 0 |
| 10 | cfg.bf.pol | Antenna polarization type vector: 0 - linear in θ direction, 1 - linear in φ direction, 2 - LHCP, 3 – RHCP. First dimension corresponds to antenna polarization on TX side; second dimension corresponds to antenna polarization type on RX side. | [0,0] |
| 11 | cfg.bf.tx_az | Array of azimuth angles specifying angles grid for exhaustive beamsearch procedure on TX side – generated during configuration based on other parameters. | n/a |
| 12 | cfg.bf.tx_el | Array of elevation angles specifying angles grid for exhaustive beamsearch procedure on TX side – generated during configuration based on other parameters. | n/a |
| 13 | cfg.bf.rx_az | Array of azimuth angles specifying angles grid for exhaustive beamsearch procedure on RX side – generated during configuration based on other parameters. | n/a |
| 14 | cfg.bf.rx_el | Array of elevation angles specifying angles grid for exhaustive beamsearch procedure on RX side – generated during configuration based on other parameters. | n/a |

Table 5 shows all fields of ch structure and provides a short description for every field.

Table 5. Description of ch structure fields

| # | Structure Field | Field Description |
| --- | --- | --- |
| 1 | ch.am | Complex amplitudes corresponding to LOS/NLOS rays existing between transmitter and receiver parts. |
| 2 | ch.toa | Times of arrival (in [ns]) of rays calculated relatively to time of arrival of LOS component. |
| 3 | ch.tx_az | Transmit azimuth angles for different rays. |
| 4 | ch.tx_el | Transmit elevation angles for different rays. |
| 5 | ch.rx_az | Receive azimuth angles for different rays. |
| 6 | ch.rx_el | Receive elevation angles for different rays. |

## Directory Tree for Channel Models Source Code

The directory tree and all files for the channel models source code are shown in Figure 2.

Figure 2. Directory tree for the channel models source code

To start up channel simulations it is necessary to run startup.m script. This script adds the specified in Figure 2 directories to the top of the current Matlab search path and changes current directory to /work folder. After that the cr_ch_model.m (or lr_ch_model.m) function may be used to generate channel impulse response realizations.

## Channel Models Test Script

The /work folder contains also cr_test.m and lr_test.m files developed for a simple check that the channel model functions are working normally. Figure 3 and Figure 4 show examples of the figures obtained by running cr_test.m and lr_test.m scripts with default settings of the configuration files for initial seed value equal to zero in randn/rand generators (Matlab 7.6.0 R2008a).

Figure 3. Example of the figure obtained by running cr_test.m script with default parameters

Figure 4. Example of the figure obtained by running lr_test.m script with default parameters

# References

- A. Maltsev et al. “Channel Models for 60 GHz WLAN Systems,” IEEE doc. 802.11-09/0334r6, January, 2010.

- E. Perahia, “TGad evaluation methodology,” IEEE doc. 802.11-10/0296r16, January, 2010.

900 – for Conference Room channel model, 1200 – for Living Room channel model.