IEEE P802.11
Wireless LANs

Normative Text for BSS Transition with Bearing

Date: 2009-11-16

| Name | Company | Address | Phone | email |
| --- | --- | --- | --- | --- |
| Allan Thomson | Cisco Systems | 170 W. Tasman Drive, San Jose, CA95134 | +1-408-853-5570 | allant@cisco.com |

Abstract

This submission contains the normative text for the SB comment #7355400023.

This proposal text is aligned with P802.11v-D7.02.

7.3.2.37 Neighbor Report element

Change Table 7-43b in IEEE 802.11k-2008 as follows:

| Table 7-43b—Optional Subelement IDs for Neighbor Report |  |  |  |
| --- | --- | --- | --- |
| Subelement ID | Name | Length field (octets) | Extensible |
| 0 | Reserved |  |  |
| 1 | TSF Information | 4 | Yes |
| 2 | Condensed Country String | 2 | Yes |
| 3 | BSS Transition Candidate Preference | 1 |  |
| 4 | BSS Termination Duration | 12 |  |
| 5 | Bearing | 4 |  |
| 36-65 | Reserved |  |  |
| 66 | Measurement Pilot Transmission Information | 1 to 238 | Subelements |
| 67-69 | Reserved |  |  |
| 70 | RRM Enabled Capabilities | 4 | Yes |
| 71 | Multiple BSSID | 1 to 238 | Subelements |
| 72-220 | Reserved |  |  |
| 221 | Vendor Specific | 1 to 238 |  |
| 222-255 | Reserved |  |  |

Insert the following text at the end of 7.3.2.37 in IEEE 802.11k-2008 as indicated below:

The format of the BSS Transition Candidate Preference subelement field is shown in Figure 7-95e1.

|  | Subelement ID | Length | Preference |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |
| Figure 7-95e1— BSS Transition Candidate Preference subelement field format |  |  |  |

The value of the BSS Transition Candidate Information subelement length field is 1.

The Preference field indicates the network preference for BSS transition to the BSS listed in this BSS Transition Candidate List Entries field in the BSS Transition Management Request frame. The Preference field value is a number ranging from 0 to 255, as defined in Table 7-43b1, indicating an ordering of preferences for the BSS transition candidates for this STA. Additional details describing Preference are provided in 11.22.6.3.

| Table 7-43b1—Preference field values |  |
| --- | --- |
| Preference field value | Description |
| 0 | Excluded BSS |
| 1-255 | Relative values used to indicate the preferred ordering of BSSs, with 255 indicating the most preferred candidate and 1 indicating the least preferred candidate. |

The BSS Termination TSF field contained in the BSS Termination Duration subelement is set to the TSF time of the BSS transmitting the Neighbor Report that corresponds to the time when termination of the neighbor BSS will occur. How the BSS determines the neighbor BSS termination time is out of scope of the standard. The format of the BSS Termination Duration subelement field is shown in Figure 7-95e2.

|  | Subelement ID | Length | BSS Termination TSF | Duration |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 8 | 2 |
| Figure 7-95e2—BSS Termination Duration subelement field format |  |  |  |  |

The value of the BSS Termination Duration Information subelement length field is 10.

The BSS Termination TSF field indicates the value of the TSF counter when the BSS termination will occur in the future. A BSS Termination TSF field value of 0 indicates that termination of the BSS will occur imminently. Prior to termination of the BSS, all associated STAs are disassociated by the AP.

The Duration field is an unsigned 2 octet integer that indicates the number of minutes for which the BSS is not present. The Duration field value is set to 65535 when the BSS is terminated for a period longer than or equal to 65535 minutes.

| Figure 7-XX— Bearing subelement field format |  |  |  |
| --- | --- | --- | --- |

The format of the Bearing subelement field is shown in Figure 7-XX.

|  | Subelement ID | Length | Bearing | Relative Height |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | 2 |
| Figure 7-XX— Bearing subelement field format |  |  |  |  |

The value of the Bearing Information subelement length field is 4.

The Bearing field, defined by a 2 octet unsigned integer, specifies the direction that the neighbor is positioned, relative to the reporting BSS and defined in relation to true north, increasing clockwise, measured in degrees from 0 degree to 359 degrees. If the Bearing value is unknown, the sub-element is not included. Additional details describing Preference are provided in 11.22.6.3.

The Relative Height field, defined by a 2 octet signed integer, specifies the relative height in meters that the neighbor is positioned, relative to the reporting BSS. If the Relative height is unknown or at the same height as the reporting BSS the field is set to 0.

11.22.6.2 BSS transition management query

A non-AP STA supporting BSS transition management may request a BSS Transition Candidate list by sending a BSS Transition Management Query frame to its associated AP if the associated AP indicates that it supports the BSS Transition Capability in the Extended Capabilities information element.

11.22.6.3 BSS transition management request

An AP that supports BSS transition management shall respond to a BSS Transition Management Query frame with a BSS Transition Management Request frame. The AP may send an unsolicited BSS Transition Management Request frame to a non-AP STA at any time if the non-AP STA indicates that it supports such capability in the Extended Capabilities information element. The AP may transmit a group addressed BSS Transition Management Request frame to associated non-AP STAs if all associated non-AP STAs support the BSS Transition Management capability. When the BSS Transition Management Request frame is transmitted as a group addressed frame, a receiving non-AP STA shall not respond with a BSS Transition Management Response frame.

The AP shall include the BSS Transition Candidate List Entries field in the BSS Transition Management Request frame if the AP has information in response to the BSS Transition Management Query frame. The BSS Transition Candidate List Entries field contains one or more Neighbor Report elements describing the preferences for target BSS candidates. Preference field value of 0 indicates that the BSS to be listed is an excluded BSS, and the STA refrains from associating to the listed AP unless the STA is unable to associate with any non-excluded APs that are compatible with the STA’s choice of SSID and, if the STA is associated with the AP that sent the request, the STA has transmitted a BSS Transition Candidate Response frame to the associated AP with the reason of reject. The Preference field values are used only to establish the relative order of entries within the given list at the given time, and for the given AP.

The values between 1 and 255 provide the indication of order, with 255 indicating the most preferred BSS within the given candidate list, decreasing numbers representing decreasing preference relative only to entries with lower values of the Preference field, and equal numbers representing equal preference. The preference value is only valid before the Validity Interval has expired. The AP may include zero or more subelements in the BSS Transition Candidate List Entries field. Load Subelements contain additional information describing the BSS load and QoS BSS Admission Control Capacity. The subelement is used by a non-AP QoS STAs to select a QoS AP that is likely to accept future admission control requests, but it does not represent a guarantee that the HC shall admit these requests.

The Bearing sub-element may be included in the BSS Transition Candidate List Entries field to indicate the relative direction and height of each neighboring BSS to the AP.

The non-AP STA that receives the abridged bit with a value of 1 shall treat any BSSID that does not appear in the BSS Transition Candidate List as if it were present in the BSS Transition Candidate List with a Preference value of 0.

The AP may also include one BSS Termination Duration subelement for each BSS in the BSS Transition Candidate List entry, including the BSS Termination Duration value and a BSS Termination TSF value. The BSS Termination duration may be different for each BSS.

An AP's SME may have accounting session control information, such as a notice of session expiry. The means by which the AP's SME obtains accounting session control information is out of scope of this specification. Accounting session control information can include a time duration after which the non-AP STA will be disassociated from the ESS and an optional session information URL at which information may be obtained to extend the accounting session. When an AP's SME has accounting session control information, it shall issue an MLME-BTM.request to the AP's MLME and shall encode the time to session expiry in the Disassociation Timer parameter and shall encode the URL, if available, in the SessionInformationURL parameter. A non-AP STA's SME receiving an MLME-BTM.indication forwards the MLME-BTM.indication parameters to the appropriate entity within the device (e.g., web-browser) to inform the end-user; the means and protocol by which the SME accomplishes this is outside the scope of this specification.

A STA's SME receiving an MLME-BTM.indication primitive containing the BSS Transition Candidate List Entries field may use this list of candidates, with individual transition preference values, to make BSS transition decisions. Upon receiving an MLME-BTM.indication primitive, the STA's SME shall disregard any previous MLME-BTM.indication primitive received from the same AP. The STA shall not use the corresponding BSS Transition Candidate List Entries field information after the indicated Validity Interval. The STA may send a BSS Transition Management Query frame at any time to obtain an updated BSS Transition Candidate List Entries field.

A STA's SME receiving an MLME-BTM.indication primitive containing a non-zero value of the Disassociation Timer field shall attempt to re-associate with some other AP before the indicated disassociation time. If the receiving STA cannot find a suitable AP with which to associate, the STA's SME shall issue an MLME-BTM.response primitive containing a Status Code indicating reject before the indicated disassociation time.

11.22.6.4 BSS transition management response

When the STA's SME receives an MLME-BTM.indication primitive, it may issue an MLME-BTM.response primitive.

The STA's SME may include the result of its BSS transition decision in the Target BSSID field and Status Code field in the MLME-BTM.response primitive. A Status Code set to 0 (i.e., Accept) indicates the STA will transition from the current BSS. The STA's SME receiving an MLME-BTM.indication primitive may issue an MLME-BTM.response primitive with a valid nonzero status code indicating rejection if it is unable to comply with this BSS transition management request.

When a non-AP STA's SME receives an MLME-BTM.indication with the BSS Termination Included field in the Request Mode field set to 1 it may send an MLME-BTM.response primitive with the Status code set to one of the following values:

—	0 - Accept. Accept the BSS Termination request.

—	4 - Deny, BSS Termination Undesired. Request for deferral of BSS Termination, interval not specified.

—	5 - Deny, BSS Termination Delay. Request for deferral of BSS Termination interval specified in the BSS Termination Delay field in the BSS Transition Management Response frame.

The AP's SME may choose to accept a response to terminate or delay BSS Termination based on policies that are out of the scope of this standard. The MLME-RESET.request primitive is invoked to terminate the BSS. The AP shall issue a Disassociation frame to each associated STA immediately prior to termination of the BSS.