IEEE P802.11
Wireless LANs

Normative Text for Sponsor Ballot Time Advertisement Comments

Date: 2009-11-16

| Name | Company | Address | Phone | email |
| --- | --- | --- | --- | --- |
| Allan Thomson | Cisco Systems | 170 W. Tasman Drive, San Jose, CA95134 | +1-408-853-5570 | allant@cisco.com |

Abstract

This submission contains the normative text for the SB comments .

This proposal text is aligned with P802.11v-D7.02.

7.3.2.65 Time Advertisement information element

The Time Advertisement information element, shown in Figure 7-95a1, specifies fields describing the source of time corresponding to a time standard, which may correspond with an external clock (external time source), an estimate of the offset between that time standard and the TSF timer, and the standard deviation of the error in the offset estimate. This information may be used by a receiving STA to align its own estimate of the time standard based on that of another STA.

|  | Element ID | Length | Timing capabilities | Time Value (optional) | Time Error (optional) | Time Update Counter (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 10 | 5 | 1 |
| Figure 7-95a1—Time Advertisement element format |  |  |  |  |  |  |

The Timing capabilities field specifies the STA's source and encoding of the Time Value field. The encoding of the Timing capabilities field is specified in Table 7-37b.

| Table 7-37b—Encoding of the Timing Capabilities field |  |
| --- | --- |
| Value | Usage |
| 0 | No standardized external time source |
| 1 | Timestamp offset based on UT0 (See clause 2 ITU-R TF: 460-6) |
| 2 | UTC UT0 time at which the TSF timer is 0 |
| 23-255 | Reserved |

When the value of the Timing Capabilities field is set to 0, no optional fields are included in the Time Advertisement information element;

When the value of the Timing Capabilities is set to 1, the following fields are included in the Time Advertisement information element:

- the Time Value field is set to a two’s complement integer in nanoseconds which, when added to the Timestamp present in the same transmitted frame, gives the receiving STA an estimate of the time standard at the time the frame was transmitted. .The Timestamp is derived from the TSP Timer as defined in 11.21

- the Time Error field is set to an unsigned integer in nanoseconds that defines the standard deviation of the error in the Time Value estimate

When the Timing Capabilities field is set to 2 the following fields are included in the Time Advertisement information element:

- the Time Value field is set to the UTC UT0 time at which the TSF Timer is 0, assuming that the TSF Timer units are exactly 1 microsecond. The format, including all subfields is shown in Table 7-37c. For any subfield not known in the Time Value field, the subfield value is set to 0.

- the Time Error field is set to

- an unsigned integer in milliseconds that defines the standard deviation of the error in the Time Value estimate.

- The Time Update Counter field is set to a modulo 256 counter that increments each time the AP updates the Time Value UTC UT0 at which the TSF Timer is 0.

| Table 7-37c—Time Value field format when Timing Capabilities is 2 |  |
| --- | --- |
| Octet | Description |
| 0 - 1 | Year (0-65534) |
| 2 | Month (0-12) |
| 3 | Day of month (0-31) |
| 4 | Hours (0-23) |
| 5 | Minutes (0-59) |
| 6 | Seconds (0-59) |
| 7-8 | Milliseconds (0-999) |
| 9 | Reserved |

The Time Error field specifies an unsigned integer in nanoseconds that defines the standard deviation of the error in the Time Value estimate. When the Timing Capabilities field is set to 1 or 2, the Time Error field is the error in the estimate of the external time standard (e.g. UT0) compared with the true value of time corresponding to that external time standard, as well as any additional errors introduced when computing the Time Value.