IEEE P802.11
Wireless LANs

| Timing Measurement Trigger Frame |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Gabor Bajko | Nokia | 313 Fairchild dr. Mountain View, CA, 94043 | +1 858 717 xxxx | gabor.bajko@nokia.com |

Introduction

This proposal addresses comments 100, 101 and 327

This proposal is based on Draft P802.11v_D7.04

The purpose of the submission is to define an action frame which could be used by a STA to trigger a Timing Measurement procedure from its peer.

- Make the following changes to  Table 7-57ac:

7.4.12.1 WNM Action fields

Several Action frame formats are defined for Wireless Network Management (WNM) purposes. An Action

field, in the octet field immediately after the Category field, differentiates the formats. The Action field values

associated with each frame format are defined in Table 7-57ac.

Table 7-57ac—WNM Action field values

| Action field value | Description |  |
| --- | --- | --- |
| 0 | Event Request |  |
| 1 | Event Report |  |
| 2 | Diagnostic Request |  |
| 3 | Diagnostic Report |  |
| 4 | Location Configuration Request |  |
| 5 | Location Configuration Response |  |
| 6 | BSS Transition Management Query |  |
| 7 | BSS Transition Management Request |  |
| 8 | BSS Transition Management Response |  |
| 9 | FMS Request |  |
| 10 | FMS Response |  |
| 11 |  | Collocated Interference Request |
| 12 |  | Collocated Interference Response |
| 13 |  | TFS Request |
| 14 |  | TFS Response |
| 15 |  | TFS Notify |
| 16 |  | WNM-Sleep Mode Request |
| 17 |  | WNM-Sleep Mode Response |
| 18 |  | TIM Broadcast Request |
| 19 |  | TIM Broadcast Response |
| 20 |  | QoS Traffic Capability Update |
| 21 |  | Channel Usage Request |
| 22 |  | Channel Usage Response |
| 23 |  | DMS Request |
| 24 |  | DMS Response |
| 25 |  | Timing Measurement Request |
| 26-255 |  | Reserved |

- Add the following subsection into the draft:

- 7.4.12.27 Timing Measurement Request Frame format

| - Category | - Action | - Trigger |
| --- | --- | --- |

octets:			1		1	1

Figure 7-101xyx

- The Category field is set to the value indicating the WNM category, as specified in Table 7-24 in 7.3.1.11.

- The Action field is set to the value indicating Timing Measurement Request frame, as specified in Table 7-57ac in 7.4.12.1.

- The trigger field set to the value 1 indicates that the sending STA would like to trigger a Timing Measurement procedure from its peer. The trigger field set to zero indicates that the sending STA would like its peer to stop sending Timing Measurement frames.

- Modify section 11.22.5 as indicated below:

11.22.5 Timing measurement procedure

Implementation of Timing Measurement is optional for a WNM STA. A STA that implements Timing Measurement

has the MIB attribute dot11MgmtOptionTimingMsmtImplemented set to true. When

dot11MgmtOptionTimingMsmtImplemented is true, dot11WirelessManagementImplemented shall be true.

If dot11MgmtOptionTimingMsmtEnabled is set to true, the Timing Measurement field in the Extended

Capabilities information element shall be set to 1 and the STA supports the timing measurement procedure.

If dot11MgmtOptionTimingMsmtEnabled is false the STA shall set the Timing Measurement field in the

Extended Capabilities information element to 0 and STA does not support the timing measurement procedure.

A STA that does not support the timing measurement procedure shall ignore a received Timing Measurement

frame.

A STA that supports the timing measurement procedure may transmit Timing Measurement frames

addressed to a peer STA that also supports the timing measurement procedure. The means by which a STA

determines that it should transmit Timing Measurement frames to a peer STA is beyond the scope of this

standard.

A STA may send a Timing Measurement Request frame to its peer to request it to initiate or stop the Timing Measurement procedure, depending on the value of the Trigger field in the request frame.

One higher-layer protocol for synchronizing time between STAs using this feature is specified in IEEE 802.1AS.

- Replace figure Figure 11-17d—Timing Measurement Procedure with the figure below:

- Note: *Request represents the optional Timing Measurement Request Action Frame