IEEE P802.11
Wireless LANs

| Timing Measurement Trigger Frame |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-03-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Gabor Bajko | Nokia | 313 Fairchild dr. Mountain View, CA, 94043 | +1 858 717 xxxx | gabor.bajko@nokia.com |

Introduction

This proposal addresses comments 100, 101 and 327

This proposal is based on Draft P802.11v_D7.04

The purpose of the submission is to define an action frame which could be used by a STA to trigger a Timing Measurement procedure from its peer.

- Make the following changes to  Table 7-57ac:

7.4.12.1 WNM Action fields

Several Action frame formats are defined for Wireless Network Management (WNM) purposes. An Action

field, in the octet field immediately after the Category field, differentiates the formats. The Action field values

associated with each frame format are defined in Table 7-57ac.

Table 7-57ac—WNM Action field values

| Action field value | Description |  |
| --- | --- | --- |
| 0 | Event Request |  |
| 1 | Event Report |  |
| 2 | Diagnostic Request |  |
| 3 | Diagnostic Report |  |
| 4 | Location Configuration Request |  |
| 5 | Location Configuration Response |  |
| 6 | BSS Transition Management Query |  |
| 7 | BSS Transition Management Request |  |
| 8 | BSS Transition Management Response |  |
| 9 | FMS Request |  |
| 10 | FMS Response |  |
| 11 |  | Collocated Interference Request |
| 12 |  | Collocated Interference Response |
| 13 |  | TFS Request |
| 14 |  | TFS Response |
| 15 |  | TFS Notify |
| 16 |  | WNM-Sleep Mode Request |
| 17 |  | WNM-Sleep Mode Response |
| 18 |  | TIM Broadcast Request |
| 19 |  | TIM Broadcast Response |
| 20 |  | QoS Traffic Capability Update |
| 21 |  | Channel Usage Request |
| 22 |  | Channel Usage Response |
| 23 |  | DMS Request |
| 24 |  | DMS Response |
| 25 |  | Timing Measurement Request |
| 26-255 |  | Reserved |

- Add the following subsection into the draft:

- 7.4.12.27 Timing Measurement Request Frame format

| - Category | - Action | - Trigger |
| --- | --- | --- |

octets:			1		1	1

Figure 7-101xyx

- The Category field is set to the value indicating the WNM category, as specified in Table 7-24 in 7.3.1.11.

- The Action field is set to the value indicating Timing Measurement Request frame, as specified in Table 7-57ac in 7.4.12.1.

- The trigger field set to the value 1 indicates that the sending STA would like to trigger a Timing Measurement procedure from its peer. The trigger field set to zero indicates that the sending STA would like its peer to stop sending Timing Measurement frames.

- Modify section 11.22.5 as indicated below:

11.22.5 Timing measurement procedure

Implementation of Timing Measurement is optional for a WNM STA. A STA that implements Timing Measurement shall also implement the Timing Measurement Request. A STA that implements Timing Measurement and Timing Measurement Request,,

has the MIB attribute dot11MgmtOptionTimingMsmtImplemented set to true. When

dot11MgmtOptionTimingMsmtImplemented is true, dot11WirelessManagementImplemented shall be true.

If dot11MgmtOptionTimingMsmtEnabled is set to true, the Timing Measurement field in the Extended

Capabilities information element shall be set to 1 and the STA supports the timing measurement procedure.

If dot11MgmtOptionTimingMsmtEnabled is false the STA shall set the Timing Measurement field in the

Extended Capabilities information element to 0 and STA does not support the timing measurement procedure.

A STA that does not support the timing measurement procedure shall ignore a received Timing Measurement

frame.

A STA that supports the timing measurement procedure may transmit Timing Measurement frames

addressed to a peer STA that also supports the timing measurement procedure. The means by which a STA

determines that it should transmit Timing Measurement frames to a peer STA is beyond the scope of this

standard.

A STA may send a Timing Measurement Request frame to its peer to request it to initiate or stop the Timing Measurement procedure, depending on the value of the Trigger field in the request frame.

One higher-layer protocol for synchronizing time between STAs using this feature is specified in IEEE 802.1AS.

- Replace figure Figure 11-17d—Timing Measurement Procedure with the figure below:

- Note: *Request represents the optionally generated Timing Measurement Request Action Frame

- Add the following subsection into the draft:

10.3.70 Timing Measurement Request

The following set of primitives supports triggering a Timing Measurement procedure or stopping an ongoing Timing Measurement procedure.

10.3.70.1 MLME-TIMINGMSMTRQ.request

10.3.70.1.1 Function

This primitive requests the transmission of a Timing Measurement Request frame to a peer entity.

10.3.70.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-TIMINGMSMTRQ.request(

Peer MAC Address,

Trigger)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual addressed MAC Address | The address of the peer MAC entity to which the Timing Measurement Request frame shall be sent. |
| Trigger | Integer | 0-1 | The trigger to identify the action (0=stop, 1=trigger) |

10.3.70.1.3 When generated

This primitive is generated by the SME to request that a Timing Measurement Request frame be sent to a peer entity.

10.3.70.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs a Timing Measurement Request frame with the specified parameters. This frame is then scheduled for transmission.

10.3.70.3 MLME-TIMINGMSMT.indication

10.3.70.3.1 Function

This primitive indicates that a Timing Measurement Request frame has been received and the corresponding ACK has been transmitted.

10.3.70.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-TIMINGMSMT.indication(

Peer MAC Address,

Trigger)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual addressed MAC Address | The address of the peer MAC entity to which the Timing Measurement Request frame shall be sent. |
| Trigger | Integer | 0-1 | The trigger to identify the action (0=stop, 1=trigger) |

10.3.70.3.3 When generated

This primitive is generated by the MLME when a valid Timing Measurement Request frame is received.

10.3.70.3.4 Effect of receipt

On receipt of this primitive, the SME uses the information contained within the notification.