IEEE P802.11
Wireless LANs

| WGLB 160 TGmb D2- Clause Gen Adhoc 11.3 Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-03-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Jon Rosdahl | CSR | Highland, UT, USA | 801-492-4023 | jrosdahl@ieee.org |

Abstract

This submission addresses the 25 CIDs from LB160 relevant to Clause 11.3.  The detail edits are made based on text from the D2.02 Clause 11.3.

R1: March 12 TGmb Telecon Changes added and updates applied

R2: March Plenary Tues PM 2 Changes added.

R3: Changes listed in 11-10/390r0 and the final 7 Gen AdHoc comments were added in this revision.  Motions to accept the comment resolutions were made based on Revision 2 of this document. This third and final revision was to put all the changes into one place.

# Introduction

| CID | Page | Clause | Resn Status | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 2160 | 634.00 | 11.3.0a |  | Figure 11-11 has various missing transitions, e.g.: - successful auth in State 2 leaves you in State 2 - unsuccessful auth in State 2 takes you to State 1 - unsuccessful (re)assoc in State 3 takes you to State 2 - unsuccessful reassoc (not just assoc) in State 4 takes you to State 2 - successful (re)assoc in State 3 leaves you in State 3 - successful (re)assoc in State 4 when RSNA is required takes you to state 3 - successful (re)assoc in State 4 when RSNA is not required leaves you in state 4 | Add missing state transitions as cited. |
| 2155 | 633.45 | 11.3.0a | A | "These two variables create four local states for each remote STA". This sentence should indicate that it is the relationship between the STA and the remote STA not the STAs themselves that are in the states described. | Change sentence to be "These two variables create four local states for the relationship between the STA and the remote STA." |

| 2128 | 633.34 | 11.3 | D | Figure 11-10 should be replaced by Figure 11-11. | Remove Figure 11-10 |
| --- | --- | --- | --- | --- | --- |
| 2110 | 634.33 | 11.3.0a |  | Why would unsuccessful association move the state from State 4 to State 2? This seems to break any possible protection we introduce against DoS attacks (e.g., the protections added in FT) since the association would be dropped in case of such an attack (e.g., fake association request sent to the AP). Furthermore, this type of state change is not defined for State 3. Is there a difference on failed (re)association behavior between the States 3 and 4? | Remove the transition from State 4 to State 2 with condition “Unsuccessful Association”. On page 638 line 19 in 11.3.2.0a, replace “Unsuccessful association when not in State 1 sets the STA's state to State 2” with “Unsuccessful association does not change STA's state”. On page 638 line 22, replace “Unsuccessful reassociation when not in State 1 sets the STA's state to State 2” with “Unsuccessful reassociation does not change STA's state”. |
| 2156 | 634.42 | 11.3.0a |  | The diagram shows the state of the STA with respect to a remote STA. When in State 3 or 4, the STA would not execute an "802.11 Authentication" with the remote STA. It may do so with a new STA (AP) that it was wanting to transition to, but that would not be included in this digraph because the state of the STA with the other Remote STA would be a different state diagram. So while the transition of "Unsuccessful 802.11 Authentication" doing nothing (looping back to the same state) is valid (it the new remote STA did not authenticate with the STA, then the relationship of the STA and the current remote STA would not change), it is not correct to be on this diagram. | remove the "Unsuccessful 802.11 Authentication" loops for state 3 and 4. |
| 2229 | 635.56 | 11.3.1.0a |  | The text states "Unsuccessful authentication sets the STA's state to State 1." However, in figure 11-6, if in states 3 or 4, the new state after unsuccessful authentication is not state 1. | Align the text and the figure. |
| 2111 | 635.57 | 11.3.1.0a |  | The text describing unsuccessful authentication is in conflict with the transitions shown in Figure 11-6. The text claims a failure to result in transition to State 1 while the figure claims there is no transition in such a case. I believe the behavior in the figure is correct since it is needed to maintain DoS protection provided for FT. | Replace “Unsuccessful authentication sets the STA's state to State 1” with “Unsuccessful authentication does not change STA's state”. |
| 2230 | 635.58 | 11.3.1.0a |  | The text states "If previously in State 3 or 4, the STA shall become associated again prior to sending Class 3 frames." If the AP has to keep track if a device is previously associated, then that implies a 5th state is needed. | Delete the cited sentence. |
| 2231 | 635.58 | 11.3.1.0a |  | In several places, section 11.3.1.0a discusses what has to happen when a new state is entered after a STA has been in a higher-numbered state. E.g., "The STA shall become authenticated again prior to sending Class 2 frames." These descriptions are generally confusing. Each state, by itself, defines what class of frames may be sent regardless of the previous state. | Re-write the text so that each state defines exactly what frames can be sent without regard to the previous state the STA was in. |
| 2161 | 636.01 | 11.3.1.0a |  | 11.3.1.0a the rules about class errors are normative, not informative. So drop the "For example," and add some "shall"s. | Rewrite the paragraph using normative language. |
| 2163 | 636.12 | 11.3.1.1 |  | In 11.3.1.1 I don't understand why the PTKSA and temporal key deletion is a "may". Everywhere else it's a "shall". | Change "may" to "shall" |
| 2196 | 636.39 | 11.3.1.2 |  | Editor’s Note: Bad “if and only if” | Replace "if and only if" with "if <condition> and shall not <verb> otherwise" |
| 2166 | 637.04 | 11.3.1.2 |  | 11.3.1.2.d needs to say that if the auth was unsuccessful the state is set to State 1. | Add to the end of the sentence at d) "if the authentication was unsuccessful, the state for the originating STA shall be set to State 1." |
| 2157 | 637.36 | 11.3.1.3 |  | Remove Editor's Note. | Remove Editor's Note -- No action required. |
| 2197 | 637.36 | 11.3.1.3 |  | Editor’s Note: The previous statement makes legacy STA potentially non-compliant when a new reason code is added. Ditto 640.05 and 641.61 and 642.38 | Replace "The use of Unspecified reason shall indicate the indicated STA was deauthenticated for a reason unrelated to every any defined reason code defined in Table 7-22 (Reason codes)." with "The use of Unspecified reason indicates the STA was deauthenticated for a reason unrelated to any other reason code defined in Table 7-22 (Reason codes) that is understood by the deauthenitcating STA." |
| 2164 | 637.54 | 11.3.1.4 |  | shouldn't there be blurb in e.g. 11.3.1.4 and 11.3.2.6/8 in the same way there is in 11.3.1.2, about not deleting keys if MFP was in force? | Make the conditional deletion of Keys consistent in 11.3.1.2, 11.3.14, 11.3.2.6 and 11.3.2.8. |
| 2129 | 637.61 | 11.3.1.4 |  | The second sentence of the paragraph does not provide any useful information to the reader. | Replace the paragraph with the following text: "The STA shall indicate that reason code from Table 7-28 (Reason codes) of 7.3.1.7(Reason Code field)." |
| 2162 | 638.34 | 11.3.2.0a |  | 11.3.2.0a the rules about class errors are normative, not informative. So drop the "For example," and add some "shall"s. | Rewrite the paragraph using normative language. |
| 2167 | 638.55 | 11.3.2.1 |  | 11.3.2.1.b should not say "STA shall transmit, if contained RSN; included in frame." but "STA shall transmit. If contained RSN, included in frame." (Change comma to period and make two sentances. | Change "The STA shall transmit an Association Request frame to the AP, if the MLMEASSOCIATE.request primitive contained an RSN..." to "The STA shall transmit an Association Request frame to the AP. If the MLMEASSOCIATE.request primitive contained an RSN..." |
| 2112 | 640.44 | 11.3.2.3 |  | The introduction of the new State 4 did not seem to take into account FT protocol for reassociation cases. When FT is used, 4-way handshake does not follow the reassociation. Instead, the STA should end up in State 4 at the successful completion of reassociation. | Replace “State 3 if RSNA establishment is required” with “State 3 if RSNA establishment is required and FT Protocol is not used”. Similarly, on page 641 line 49 in 11.3.2.4, replace “State 3 if RSNA establishment is required on the new AP” with “State 3 if RSNA establishment is required on the new AP and FT Protocol is not used”. |
| 2198 | 642.05 | 11.3.2.4 |  | Editor’s Note: 802.11 includes no protocol that allows a new AP to talk to an old AP, so this shall statement is not realizable. "h) The SME shall inform the DS (including the old AP, if this is not the same as the new AP) of any changes in the association state." | Remove the cited parenthetical text and editor's note. |
| 2159 | 642.08 | 11.3.2.4 |  | As noted in the Editor Note, the Parenthetical Statement is not realizable, but the main statement is correct. | Remove the parenthetical statement. "(including the old AP……)" |
| 2168 | 642.17 | 11.3.2.5 |  | 11.3.2.5 should say "disassociation initiation" not just "disassociation". We should make sure the wording is consistent in all the categories (e.g. between all the "initiation" procedures and between "association" and "reassociation" and between "deauthentication" and "disassociation") to avoid causing Fear Uncertainty and Doubt. | add "initiation" to title cited and a followup submission to made on consistency efforts. |
| 2165 | 642.49 | 11.3.2.6 |  | The first para of 11.3.2.6 needs to be fixed because it talks about invoking .request but this is about getting a .indication. | change paragraph to use .indication. |
| 2130 | 643.28 | 11.3.2.8 |  | The second sentence of the paragraph does not provide any useful information to the reader. | Replace the paragraph with the following text: "The STA shall indicate that reason code from Table 7-28 (Reason codes) of 7.3.1.7(Reason Code field)." |

# Proposed Resolutions

CID 2110:

AGREE IN PRINCIPLE (GEN: 2010-03-17 21:51:11Z) Apply the changes in doc 11-10/390r0.

CID 2111:

AGREE IN PRINCIPLE (GEN: 2010-03-12 13:27:33Z) Replace the second paragraph in 11.3.1.0a with "Successful authentication sets the STA's state to State 2. Unsuccessful authentication leaves the STA's state unchanged. The STA shall not transmit Class 3 frames unless in State 2, or State 3 or State 4.  Authentication notification when in State 3 or 4 implies disassociation as well."

CID 2112

AGREE (GEN: 2010-03-16 21:40:17Z) [Replace “State 3 if RSNA establishment is required” with “State 3 if RSNA establishment is required and FT Protocol is not used”. Similarly, on page 641 line 49 in 11.3.2.4, replace “State 3 if RSNA establishment is required on the new AP” with “State 3 if RSNA establishment is required on the new AP and FT Protocol is not used”.]

CID 2128:

DISAGREE (GEN: 2010-01-20 06:48:31Z) - No change required. The figure is correct in the non-redline version of the draft.

CID 2129:

AGREE IN PRINCIPLE (GEN: 2010-03-12 05:50:43Z) A consistency to the cited sentence was made.  CID 2197 has the specific changes listed in this clause and several others.

CID 2130:

AGREE IN PRINCIPLE (GEN: 2010-03-12 15:13:22Z) The specific text changes are noted in CID 2197.  Also for consistency, other places where this text was reiterated was fixed as well.

CID 2155:

AGREE (GEN: 2010-01-21 01:21:48Z) in 11.3.0a Change sentence to be "These two variables create four local states for the relationship between the STA and the remote STA."

CID 2156:

AGREE IN PRINCIPLE (GEN: 2010-03-17 21:52:43Z) remove the four unsuccessful self loops. [there is one loop for each state that currently loops back to self.  All 4 are to be removed.]

CID 2157:

AGREE (GEN: 2010-03-12 03:52:43Z). The editor note is to be removed; CID 2197 addresses the Editor's concern.

CID 2159:

AGREE IN PRINCIPLE (GEN: 2010-01-20 07:28:49Z) - Remove Editor's Note and parenthetical in 11.3.2.4 h).

CID 2160:

AGREE IN PRINCIPLE (GEN: 2010-03-17 21:54:03Z) Change the introduction to the figure to read "Figure 11-6 shows the state transition diagram for these STA states.  Note that only events causing states changes are shown."

CID 2161:

AGREE IN PRINCIPLE (GEN: 2010-03-17 22:22:40Z) Delete "For example, " and change "discards" to "shall discard" in 11.3.1.0a. And add a line to page 801(D2.04)  a) 3) ii) "Data frames between peers using DLS"

CID 2162:

AGREE (GEN: 2010-03-12 05:54:16Z) in 11.3.2.0a  the 6th paragraph 1st sentence: Delete "For example, " and change "discards" to "shall discard".

CID 2163:

AGREE GEN: 2010-01-20 07:07:06Z - Accept. Change "may" to "shall" in 11.3.1.1

CID 2164:

AGREE (GEN: 2010-03-12 03:58:24Z) Make the following changes

in 11.3.1.2:

Delete "if and only if Management Frame Protection had not been negotiated when the PTKSA(s) were created"

Add "or if Management Frame Protection was not negotiated when the PTKSA(s) were created," after the first "if conditional phrase" at the start of the paragraph and correct capitalization.

in 11.3.1.4:

add "if Management Frame Protection was not negotiated when the PTKSA(s) were created," to the start of the paragraph(line 4) and correct capitalization.

in 11.3.2.6:

add "If Management Frame Protection was not negotiated when the PTKSA(s) were created," to the start of the paragraph and correct capitalization.

and in 11.3.2.8:

add "If Management Frame Protection was not negotiated when the PTKSA(s) were created," to the start of the paragraph and correct capitalization.

CID 2165:

AGREE IN PRINCIPLE (GEN: 2010-03-15 23:27:37Z) The specific change to the first para is not necessary, but we did find that along the line of understanding why this is the case, that the 2nd paragraph should change from "originating STA's” to "originating STA's MLME" (or AP as the case may be.)  This clarification would help point to the part of the STA that is logically acting on the information.

Changes made to 11.3.1.1, 11.3.1.2, 11.3.1.3, 11.3.1.4, 11.3.2.1, 11.3.2.2, 11.3.2.3, 11.3.2.4, 11.3.2.5, 11.2.3.6, 11.2.3.7 and 11.3.2.8:  2nd par add “s MLME” to the STA, Non-AP STA or AP as appropriate.

CID 2166

AGREE IN PRINCIPLE (GEN: 2010-03-17 21:57:49Z) make the change as noted in 11-10/390r0, which insert  the recommended text in a different location -- at the end of b.

CID 2167:

AGREE (GEN: 2010-03-12 06:03:41Z) Change "The STA shall transmit an Association Request frame to the AP, if the MLMEASSOCIATE.request primitive contained an RSN..."

to

"The STA shall transmit an Association Request frame to the AP.  If the MLMEASSOCIATE.request primitive contained an RSN..."

CID 2168:

AGREE IN PRINCIPLE (GEN: 2010-03-12 14:56:52Z) Change the following titles by adding the quoted word:

11.3.2.1 Non-AP STA association "initiation" procedures

11.3.2.2 AP association "receipt" procedures

11.3.2.3 Non-AP STA reassociation"initiation" procedures

11.3.2.4 AP reassociation "receipt" procedures

11.3.2.5 Non-AP STA disassociation "initiation" procedures

CID 2196:

AGREE IN PRINCIPLE (GEN: 2010-03-12 12:59:18Z) The Cited "if and only If" was deleted by CID 2164 and the sentence was relocated and rewritten.

CID 2197:

AGREE IN PRINCIPLE (GEN: 2010-03-12 03:03:28Z)

In 11.3.1.3 b), Replace paragraph with

"The STA shall indicate the appropriate reason code for the STA deathentication as defined in Table 7-28 (Reason codes) of 7.3.1.7(Reason Code field)."

Similar change to be made in 11.3.2.2 d) Replace paragraph with

"When the status code of the association is not Successful, the AP shall indicate a specific reason for the failure to associate in the status code of the Association Response frame as defined in Table 7-23 (Status Codes) in 7.3.1.9 (Status Code Field).  The state for the STA shall be set to State 2.(#1342)"

Similar change to be made in 11.3.2.4 e) Replace paragraph with

"When the status code of the reassociation is not Successful, the AP shall indicate a specific reason for the failure to reassociate in the status code of the Reassociation Response frame as defined in Table 7-23 (Status Codes) in 7.3.1.9 (Status Code Field).  The state for the STA shall be set to State 2 on the AP the Reassociation Request frame was sent to."

Similar change to be made in 11.3.2.5 b) Replace paragraph with

"The STA shall indicate the appropriate reason code for the AP dissacociation as defined in Table 7-28 (Reason codes) of 7.3.1.7(Reason Code field)."

Similar change to be made in 11.3.2.7 b) Replace paragraph with

"The STA shall indicate the appropriate reason code for the STA disassociation as defined in Table 7-28 (Reason codes) of 7.3.1.7(Reason Code field)."

CID 2198

AGREE (GEN: 2010-03-16 21:20:33Z) - Remove Editor's Note and parenthetical in 11.3.2.4 h). See CID 2159

CID 2229:

AGREE IN PRINCIPLE (GEN: 2010-03-12 14:15:23Z) See CID 2111 for specific text changes: (from CID 2111:AGREE IN PRINCIPLE (GEN: 2010-03-12 13:27:33Z)Accept in principle. Replace the second paragraph in 11.3.1.0a with "Successful authentication sets the STA's state to State 2. Unsuccessful authentication leaves the STA's state unchanged. The STA shall not transmit Class 3 frames unless it becomes associated.  Authentication notification when in State 3 or 4 implies disassociation as well.")

CID 2230:

AGREE IN PRINCIPLE (GEN: 2010-03-12 13:47:30Z) - The cited text is to clarify the original statement that on disassociation "This STA shall become associated again if it wishes to utilize the DS."  The "if previously in State 3 or 4" part qualifies the "associated again", not the "prior to sending Class 3 frames". There is no need for a 5th state.  However, CID 2111 changes the paragraph and removes the cited text.

CID 2231:

AGREE (GEN: 2010-03-12 12:42:56Z) Change  "The STA shall become authenticated again prior to sending Class 2 frames."

To

"The STA shall not transmit Class 2 frames unless in State 2, or State 3 or State 4."

## Detailed Changes:

# 11.3 STA authentication and association

## 11.3.0a General

A STA keeps two state variables for each STA with which direct communication via the WM is needed:

- Authentication state: The values are unauthenticated and authenticated.

- Association state: The values are unassociated and associated.

These two variables create four local states for the relationship between the STA and the remote STA.(#2155):

- State 1: Initial start state, unauthenticated, unassociated.

- State 2: Authenticated, not associated.

- State 3: Authenticated and associated (Pending RSN Authentication).(#1342)

- State 4: Authenticated and associated.(#1342)

Figure 11-6 shows the state transition diagram for these STA states.  Note that only events causing states changes are shown." (#2160) (Error: Reference source not found).

The current state existing between the source and destination STAs determines the IEEE 802.11 frame types that may be exchanged between that pair of STAs (see Clause\ Error: Reference source not found (Error: Reference source not found)). A unique state exists for each pair of source and destination STAs.(#1342) The state of the sending STA given by Figure\ Error: Reference source not found (Error: Reference source not found) is with respect to the intended receiving STA. The allowed frame types are grouped into classes and the classes correspond to the STA state. In State 1, only Class 1 frames are allowed. In State 2, either Class 1 or Class 2 frames are allowed. In State 3 and State 4(#1342), all frames are allowed (Classes 1, 2, and 3). The frame classes are defined as follows:

a)	Class 1 frames(#1342)

1)	Control frames

i)	RTS(#1342)

ii)	CTS(#1342)

iii)	ACK(#1342)

iv)	CF-End+ACK(#1342)

v)	CF-End

vi)	Within an IBSS, Block Ack (BlockAck)(#1342)

vii)	Within an IBSS, Block Ack Request (BlockAckReq)(#1342)

2)	Management frames

i)	Probe Request/Response(#1342)

ii)	Beacon

iii)	Authentication(#1342)

iv)	Deauthentication(#1342)

v)	ATIM(#1342)

vi)	Public Action(11k)(#2170)

vii)	Within an IBSS, all Action frames(11k) and all Action No Ack frames(11n)

3)	Data frames

i)	Data frames between STAs in an IBSS(#1342)

ii) Data Frames between peers using DLS(#2161)

b)	Class 2 frames(#1342)

1)	Management frames

i)	Association Request/Response(#1342)

ii)	Reassociation Request/Response(#1342)

iii)	Disassociation(#1342)

c)	Class 3 frames(#1342)

1)	Data frames

i)	Data frames between STAs in an(#2210) infrastructure BSS(#1342)

2)	Management frames

i)	(#1468)Within an infrastructure BSS, all Action and Action No Ack(11n) frames except those that are declared to be Class 1 or Class 2 frames (above)(#1342)(#2171)

3)	Control frames

i)	PS-Poll(#1342)

ii)	Within an infrastructure BSS,(#1342) Block Ack (BlockAck)(11k)

iii)	Within an infrastructure BSS,(#1342) Block Ack Request (BlockAckReq)

Class 2 and Class 3 frames are not allowed in an IBSS.  If a STA in an IBSS receives a Class 2 or Class 3 frame, it shall ignore the frame.(#1342)

The use of the word “receive” in  (11.3 STA authentication and association)(#1342) refers to a frame that meets all of the filtering criteria specified in Clause\ Error: Reference source not found (Error: Reference source not found) and Clause\ Error: Reference source not found (Error: Reference source not found).

11.3.1 Authentication and deauthentication

11.3.1.0a General(#28)

This subclause describes the procedures used for IEEE 802.11 authentication and deauthentication. The states used in this description are defined in  (11.3.0a Genera).(#1342)

Successful authentication sets the STA's state to State 2. Unsuccessful authentication leaves the STA's state unchanged. The STA shall not transmit Class 3 frames unless in State 3 or State 4.(#2111) .(#2110).(#1342)

Deauthentication notification sets the STA's state to State 1. The STA shall not transmit Class 2 frames unless in State 2 or State 3 or State 4.(#2231)Deauthentication notification when in State 3 or 4 implies disassociation as well.(#1342)

If(#2161) STA A in an infrastructure BSS receives a Class 2 or Class 3 frame from STA B that is not authenticated with STA A (i.e., the state for STA B is State 1), STA A shall discard(#2161) the frame.  If the frame has a unicast address in the Address 1 field, STA A sends a Deauthentication frame to STA B.(#1342)

Authentication is optional in an IBSS.  In an infrastructure BSS, authentication is required. APs do not initiate authentication.(#1342)

11.3.1.1 Authentication—originating STA

If the requested authentication mechanism is other than FT authentication, the(11r) STA’s SME shall (#2163) delete any PTKSA and temporal keys held for communication with the indicated STA by using MLME-DELETEKEYS.request primitive (see 8.4.10 (RSNA security association termination)) before invoking MLME-AUTHENTICATE.request primitive. (#1342)

Upon receipt of an MLME-AUTHENTICATE.request primitive, the originating STA’s MLME(#2165) shall authenticate with the indicated STA using the following procedure:(11r)

a)	(#1342)The STA shall execute one of the following:(11r)

1)	For the Open System or Shared Key authentication algorithm, the authentication mechanism described in 8.2.2.2 (Open System authentication) or 8.2.2.3 (Shared Key authentication), respectively.(11r)

2)	For the FT authentication algorithm in an ESS, the authentication mechanism described in Error: Reference source not found (FT Protocol).(11r)

b)	If the authentication was successful within the AuthenticateFailureTimeout(#1342), the state(#1342) for the indicated STA shall be set to State 2.

c)	The MLME(#1342) shall issue an MLME-AUTHENTICATE.confirm primitive to inform the SME of the result of the authentication.

11.3.1.2 Authentication—destination STA

(#2211)If the requested authentication mechanism is other than FT authentication, or if Management Frame Protection was not negotiated when the PTKSA(s) were created,(#2164) the(11r) STA’s SME shall delete any PTKSA and temporal keys held for communication with the originating(#1342) STA by using the MLME-DELETEKEYS.request primitive (see Error: Reference source not found (RSNA security association termination) upon receiving a MLME-AUTHENTICATE.indication primitive (#2164)(11w). (#1342)

Upon receipt of an Authentication frame with authentication transaction sequence number equal to 1, the destination STA’s MLME shall authenticate with the originating(#1342) STA using the following procedure:

a)	The MLME shall issue an MLME-AUTHENTICATE.indication primitive to inform the SME of the authentication request.

b)	Upon receipt of an MLME-AUTHENTICATE.response primitive, if the ResultCode is not SUCCESS, the STA shall transmit an Authentication frame with authentication transaction sequence number set to 2 with a status code, as defined in Error: Reference source not found (Status Code Field), other than Successful, and the state for the originating STA shall be left unchanged(#2166).

c)	(#1468)(#1342)If the ResultCode in the MLME-AUTHENTICATE.response primitive was SUCCESS, the STA shall execute one of the following:(11r)

1)	For the Open System or Shared Key authentication algorithm, the authentication mechanism described in 8.2.2.2 (Open System authentication) or 8.2.2.3 (Shared Key authentication), respectively.(11r)

2)	For the FT authentication algorithm in an ESS(#1342), the authentication mechanism described in 11A.5 (FT Protocol).(11r)

d)	If the authentication was successful, the state for the originating(#1342) STA shall be set to State 2. if the authentication was unsuccessful, the state for the originating STA shall be set to State 1.

e)	If the destination STA is an AP, its SME shall inform the DS of the disassociation, if the state for the originating STA was State 3 or 4.(#1342)

If the STA is in an IBSS, if the SME decides to initiate an RSNA, and if the SME does not know the security policy of the peer, it may issue an individually addressed(#1359) Probe Request frame to the peer by invoking an MLME-SCAN.request primitive(#1342) to discover the peer’s security policy.

11.3.1.3 Deauthentication—originating STA

The STA’s SME shall delete any PTKSA and temporal keys held for communication with the indicated STA by using the MLME-DELETEKEYS.request primitive (see 8.4.10 (RSNA security association termination)) and by invoking MLME-SETPROTECTION.request(None) before invoking the MLME-DEAUTHENTICATE.request primitive.

Upon receipt of an MLME-DEAUTHENTICATE.request primitive, the originating STA’s MLME(#2165) shall deauthenticate with the indicated STA using the following procedure:

a)	If the state for the indicated STA is State 2, State 3, or State 4, the STA shall transmit a Deauthentication frame to the indicated STA.(#1342)

b)	The STA shall indicate the appropriate reason code for the STA deathentication as defined in Table 7-28 (Reason codes) of 7.3.1.7(Reason Code field).(#2197)

c)	(#2211)The state for the indicated STA shall be set to State 1.(#1342)

d)	The MLME(#1342) shall issue an MLME-DEAUTHENTICATE.confirm primitive to inform the SME of the completion of the deauthentication.

e)	If the STA is an AP, its SME shall inform the DS of the disassociation, if the state for the indicated STA was State 3 or State 4.

11.3.1.4 Deauthentication—destination STA

If Management Frame Protection was not negotiated when the PTKSA(s) were created, the (#2164) SME shall delete any PTKSA and temporal keys held for communication with the originating STA by using the MLME-DELETEKEYS.request primitive (see Error: Reference source not found (RSNA security association termination)) and by invoking MLME-SETPROTECTION.request(None) upon receiving an MLME-DEAUTHENTICATE.indication primitive.(#1342)

Upon receipt of a Deauthentication frame from a STA for which the state is State 2, State 3, or State 4, the destination STA’s MLME(#2165) shall deauthenticate with the originating STA using the following procedure:(#1342)

a)	The state for the originating(#1342) STA shall be set to State 1.

b)	The MLME(#1342) shall issue an MLME-DEAUTHENTICATE.indication primitive to inform the SME of the deauthentication.

c)	If the STA is an AP, its SME shall inform the DS of the disassociation, if the state for the originating STA was State 3 or State 4.(#1342)(#2158)

11.3.2 Association, reassociation, and disassociation

11.3.2.0a General(#28)

Subclause  (11.3.2 Association, reassociation, and disassociation) describes the procedures used for IEEE 802.11 association, reassociation and disassociation.(#1342)

The states used in this description are defined in  (11.3.0a Genera)(#1342).

Successful association enables a STA to exchange Class 3 frames.(#2110). Successful association sets the STA's state to State 3 or State 4.(#1342)

Successful reassociation enables a STA to exchange Class 3 frames. Unsuccessful reassociation when not in State 1 leaves the STA's state unchanged (#2110)(with respect to the AP that was sent the Reassociation Request (which may be the current STA)). Successful reassociation sets the STA's state to State 3 or State 4 (with respect to the AP that was sent the Reassociation Request). Successful reassociation when not in State 1 sets the STA's state to State 2 (with respect to the current AP, if this is not the AP that was sent the Reassociation Request).  Reassociation shall only be performed if the originating STA is already associated in the same ESS.(#1342)

Disassociation notification when not in State 1 sets the STA's state to State 2.  The STA shall become associated again prior to sending Class 3 frames.(#1342)

If(#2162)STA A in an infrastructure BSS receives a Class 3 frame from STA B that is authenticated but not associated with STA A (i.e., the state for STA B is State 2), STA A shall discard (#2162) the frame.  If the frame has a unicast address in the Address 1 field, STA A sends a Disassociation frame to STA B.(#1342)

Association is not applicable in an IBSS. In an infrastructure BSS, association is required. APs do not initiate association.(#1342)

11.3.2.1 Non-AP STA association initiation (#2168) procedures(#1133)

The SME shall delete any PTKSA and temporal keys held for communication with the AP by using MLME-DELETEKEYS.request primitive (see 8.4.10 (RSNA security association termination)) before invoking MLME-ASSOCIATE.request primitive.(#1342)

Upon receipt of an MLME-ASSOCIATE.request primitive, a non-AP(#1133) STA’s MLME(#2165) shall associate with an AP using(#1342) the following procedure:

a)	If the state for the AP is State 1, the STA shall inform the SME of the failure of the association by issuing an MLME-ASSOCIATE.confirm primitive.

b)	The STA shall transmit an Association Request frame to the AP., If (#2167) the MLME-ASSOCIATE.request primitive contained an RSN (#1684)element with only one pairwise cipher suite and only one authenticated key suite; this RSN (#1684)element shall be included in the Association Request frame.(#1342)

c)	If an Association Response frame is received with a status code of Successful, the state for the AP shall be set to State 4 or State 3 if RSNA Establishment is required. The state for any other AP which is State 3 or State 4 prior to the association request shall be set to State 2, and the MLME shall issue an MLME-ASSOCIATE.confirm primitive to inform the SME of the successful completion of the association.(#1342)

d)	If an Association Response frame is received with a status code other than Successful or the AssociateFailureTimeout expires the state for the AP shall be set to State 2, and the MLME shall issue an MLME-ASSOCIATE.confirm primitive to inform the SME of the failure of the association.(#1342) The status code returned in the Association Response frame indicates the cause of the failed association attempt. Any misconfiguration or parameter mismatch, e.g., data rates required as basic rates that the STA did not indicate as supported in the STA’s Supported Rates (#1684)element, shall be corrected before the SME issues an MLME-ASSOCIATE.request primitive(#1342) for the same AP. If the status code indicates the association failed because of a reason that is not related to configuration, e.g., the AP is unable to support additional associations, the SME shall not issue an MLME-ASSOCIATE.request primitive(#1342) for the same AP until a period of at least 2 s has elapsed.

e)	If an Association Response frame is received with a status code of Successful, and RSNA is required, and the STA is in State 3, then the SME shall establish an RSNA. After a successful 4-way handshake, the STA shall transition to State 4.(#1342)

f)	When the STA enters State 4, if policy within the SME requires secure communication the SME shall enable protection by invoking (#2173)MLME-SETPROTECTION.request(Rx_Tx)(#1509).

11.3.2.2 AP association receipt (#2168)procedures

The SME shall delete any PTKSA and temporal keys held for communication with the STA by using the(Ed) MLME-DELETEKEYS.request primitive (see 8.4.10 (RSNA security association termination)) upon receiving a MLME-ASSOCIATE.indication primitive that results in a successful MLME-ASSOCIATE.response primitive.(11w) (#1342)

Upon receipt of an Association Request frame from a non-AP STA for which the state is State 1, the AP shall transmit an Association Response frame with an appropriate status code.(#2111)

Upon receipt of(#1342) an Association Request frame from a non-AP(#1133) STA for which the state is State 2, State 3, or State 4, the AP’s MLME shall associate with the STA using the following procedure:(#1342)

a)	The MLME shall issue an MLME-ASSOCIATE.indication primitive to inform the SME of the association request.(#1342)

b)	In an RSNA, the AP shall check the values received in the RSN (#1684)element to see whether the values received match the AP’s security policy. If not, the association shall not be accepted.

b1)	If the STA is associated, has a valid security association, and has negotiated Management Frame Protection, the AP shall reject the Association Request with status code “Association request rejected temporarily; try again later”. The AP shall not modify any association state for the non-AP STA, and shall include in the Association Response a Timeout Interval element(Ed) with Timeout interval type set to 3 (Association Comeback time), specifying a comeback time when the AP would be ready to accept an association with this STA. Following this, if the AP is not already engaging in an SA Query with the STA, the AP shall issue one MLME-SAQuery.request primitive to the STA every dot11AssociationSAQueryRetryTimeout TUs until a matching MLME-SAQuery.confirm primitive(#2172)(#2174) is received or dot11AssociationSAQueryMaximumTimeout TUs from the beginning of the SA Query procedure have passed. The STA shall insert the TransactionIdentifier field value in the SA Query Request frame, and increment the value by 1 for each subsequent SA Query Request frame, rolling over the value to 0 after the maximum allowed value is reached. An AP may interpret reception of a valid protected frame as an indication of a successfully completed SA Query. If an MLME-SAQuery.confirm primitive(#2172)(#2174) with an outstanding transaction identifier is not received within dot11AssociationSAQueryMaximumTimeout period, the AP shall allow the association process to be started without starting an additional SA Query procedure.(11w)

b2)	An AP shall refuse an association request from a STA that does not support all the rates in the BSSBasicRateSet parameter.(11n)

b3)	An AP shall refuse an association request from an HT STA that does not support all the MCSs in the BSSBasicMCSSet parameter.(11n)

c)	Upon receipt of an MLME-ASSOCIATE.response primitive(#1342), the AP shall transmit an Association Response frame(#1342) with a status code as defined in 7.3.1.9 to the STA. If the status value is Successful, the association identifier assigned to the STA shall be included in the response.

d)	When the status code of the association is not Successful, the AP shall indicate a specific reason for the failure to associate in the status code of the Association Response frame as defined in Table 7-23 (Status Codes) in 7.3.1.9 (Status Code Field)(#2197). The state for the STA shall be left unchanged.(#2110).(#1342)

e)	(#2211)When the Association Response frame with a status code of Successful is acknowledged by the STA, the state for the STA shall be set to State 4 or State 3 if RSNA establishment is required.(#1342)

f)	If the state of the STA is set to State 3, upon a successful completion of a 4-way handshake, the state for the STA shall be set to State 4. (#1342)

g)	When the STA enters State 4, if policy within the SME requires secure communication the SME shall enable protection by invoking (#2173)MLME-SETPROTECTION.request(Rx_Tx)(#1509).

h)	The SME shall inform the DS of any changes in the association state.(#1342)

In case of a failed SA Query procedure, if the AP receives an Association frame from the STA with which it has an existing SA, then the AP shall send a protected Disassociation frame to this STA prior to terminating the old SA, with Reason Code “Previous Authentication no longer valid”.(11w)

11.3.2.3 Non-AP STA reassociation initiation (#2168)procedures(#1133)

Except when the association is part of a fast BSS transition, the(11r) SME shall delete any PTKSA and temporal keys held for communication with the AP by using the(Ed) MLME-DELETEKEYS.request primitive (see 8.4.10 (RSNA security association termination)) before invoking MLME-REASSOCIATE.request primitive. (#1342)

Upon receipt of an MLME-REASSOCIATE.request primitive, a non-AP(#1133) STA’s MLME(#2165) shall reassociate with an AP using(#1342) the following procedure:

a)	If the STA is not associated in the same ESS or the state for the new AP is State 1, the STA shall inform the SME of the failure of the reassociation by issuing an MLME-REASSOCIATE.confirm primitive.

b)	The STA shall transmit a Reassociation Request frame to the new AP. If the MLME-REASSOCIATE.request primitive contained an RSN (#1684)element with only one pairwise cipher suite and only one authenticated key suite, this RSN (#1684)element shall be included in the Reassociation Request frame.

c)	If a Reassociation Response frame is received with a status code of Successful, the state variable shall be set to State 4, or to State 3 if RSNA establishment is required and FT Protocol is not used (#2112) with respect to the new AP and, unless the old AP and new AP are the same, to State 2 with respect to the old AP, and the MLME shall issue an MLME-REASSOCIATE.confirm primitive to inform the SME of the successful completion of the reassociation.(#1342)

d)	If a Reassociation Response frame is received with a status code other than Successful or the ReassociateFailureTimeout expires, the state for the AP shall be set to State 2 with respect to the new AP, and the MLME shall issue an MLME-REASSOCIATE.confirm primitive to inform the SME of the failure of the reassociation. The status code returned in the Reassociation Response frame indicates the cause of the failed reassociation attempt. Any misconfiguration or parameter mismatch, e.g., data rates required as basic rates that the STA did not indicate as supported in the STA’s Supported Rates (#1684)element, shall be corrected before the SME issues an MLME-REASSOCIATE.request primitive for the same AP. If the status code indicates the reassociation failed because of a reason that is not related to configuration, e.g., the AP is unable to support additional associations, the SME shall not issue an MLME-REASSOCIATE.request primitive for the same AP until a period of at least 2 s has elapsed.(#1342)

e)	If a Reassociation Response frame is received with a status code of Successful, and RSNA is required, and the STA is in State 3, then the SME shall establish an RSNA. After a successful 4-way handshake, the STA shall transition to State 4.

f)	When the STA enters State 4, if policy within the SME requires secure communication the SME shall enable protection by invoking (#2173)MLME-SETPROTECTION.request(Rx_Tx)(#1509).

11.3.2.4 AP reassociation receipt (#2168) procedures

Except when the association is part of a fast BSS transition, the(11r) SME shall delete any PTKSA and temporal keys held for communication with the STA by using MLME-DELETEKEYS.request primitive (see 8.4.10 (RSNA security association termination)) upon receiving a MLME-REASSOCIATE.indication primitive that results in a successful MLME-REASSOCIATE.response primitive.(11w)(#1342)

Upon receipt of an Reassociation Request frame from a non-AP STA for which the state is State 1, the AP shall transmit an Reassociation Response frame with an appropriate status code.

Upon receipt of a Reassociation Request frame from a STA for which the state is State 2, State 3, or State 4, the AP’s MLME(#2165) shall reassociate with the STA using the following procedure:(#1342)

a)	The MLME shall issue an MLME-REASSOCIATE.indication primitive to inform the SME of the reassociation request.(#1342)

b)	In an RSNA, the AP shall check the values received in the RSN (#1684)element to see whether the values received match the AP’s security policy. If not, the association shall not be accepted.

b1)	If the STA is associated and has a valid security association, has negotiated management frame protection, and the reassociation is not a part of a Fast BSS Transition, the AP shall reject the Reassociation Request with status code “Association request rejected temporarily; Try again later”. The AP shall not modify any association state for the non-AP STA, and shall include in the Reassociation Response a Timeout Interval element(Ed) with type set to 3 (Association Comeback time), specifying a comeback time when the AP would be ready to accept an association with this STA. Following this, if the AP is not in an ongoing SA Query with the STA, the AP shall issue one MLME-SAQuery.request primitive to the STA every dot11AssociationSAQueryRetryTimeout TUs until a matching MLME-SAQuery.confirm primitive(#2172)(#2174) is received or dot11AssociationSAQueryMaximumTimeout TUs from the beginning of the SA Query procedure have passed. The STA shall insert the TransactionIdentifier in SA Query Request, and increment this by 1 for each subsequent SA Query Request, and rolling over to 0 after the maximum allowed value in this field. An AP may interpret reception of a valid protected frame as an indication of a successfully completed SA Query. If an MLME-SAQuery.confirm primitive(#2172)(#2174) with an outstanding transaction identifier is not received within dot11AssociationSAQueryMaximumTimeout period, the AP shall allow the association process to be started without starting additional SA Query procedure. (11w)

b2)	An AP shall refuse a reassociation request from a STA that does not support all the rates in the BSSBasicRateSet parameter.(11n)

b3)	An AP shall refuse a reassociation request from an HT STA that does not support all the MCSs in the BSSBasicMCSSet parameter.(11n)

c)	Upon receipt of an MLME-REASSOCIATE.response primitive, the AP shall transmit a Reassociation Response frame with a status code as defined in Error: Reference source not found (Status Code Field). If the status code is Successful, the association identifier assigned to the STA shall be included in the response. (#1342)

d)	When the Reassociation Response frame with a status value of Successful is acknowledged by the STA, the state variable for the STA shall be set to State 4, or to State 3 if RSNA establishment is required on the new AP and FT Protocol is not used (#2112)

e)	When the status code of the reassociation is not Successful, the AP shall indicate a specific reason for the failure to reassociate in the status code of the Reassociation Response frame as defined in Table 7-23 (Status Codes) in 7.3.1.9 (Status Code Field)(#2197). ).  The state for the STA shall be set left unchanged.(#2110) on the AP the Reassociation Request frame was sent to.

f)	(#2211)If  the state of the STA is set to State 3, upon a successful completion of a 4-way handshake, the state for the STA shall be set to State 4.

g)	When the STA enters State 4, if policy within the SME requires secure communication the SME shall enable protection by invoking (#2173)MLME-SETPROTECTION.request(Rx_Tx)(#1509).

h)	(#2211)The SME shall inform the DS (#2159) (of any changes in the association state.

In case of a failed SA Query procedure, if the AP receives a Reassociation frame from the STA with which it has an existing SA, then the AP shall send a protected Disassociate frame to this STA prior to terminating the old SA, with Reason Code “Previous Authentication no longer valid”.(11w)

11.3.2.5 Non-AP STA disassociation initiation(2168) procedures(#1133)

The SME shall delete any PTKSA and temporal keys held for communication with the AP by using the MLME-DELETEKEYS.request primitive (see 8.4.10 (RSNA security association termination)) and by invoking MLME-SETPROTECTION.request(None) before invoking the MLME-DISASSOCIATE.request primitive. (#1342)

Upon receipt of an MLME-DISASSOCIATE.request primitive, a non-AP(#1133) STA’s MLME(#2165) shall disassociate from an AP using the following procedure:(#1342)

a)	If the state for the AP is State 3 or State 4, the STA shall transmit a Disassociation frame to the AP.(#1342)

b)	The STA shall indicate the appropriate reason code for the AP dissacociation as defined in Table 7-28 (Reason codes) of 7.3.1.7(Reason Code field).(#2197)

c)	(#2211)The state for the AP shall be set to State 2 if it was not State 1.(#1342)

d)	The MLME shall issue an MLME-DISASSOCIATE.confirm primitive to inform the SME of the successful completion of the disassociation.(#1342)

11.3.2.6 Non-AP STA disassociation receipt procedure

If Management Frame Protection was not negotiated when the PTKSA(s) were created, the(#2164) SME shall delete any PTKSA and temporal keys held for communication with the AP by using the MLME-DELETEKEYS.request primitive (see 8.4.10 (RSNA security association termination)) and by invoking MLME-SETPROTECTION.request(None) before invoking the MLME-DISASSOCIATE.request primitive.(#1342)

Upon receipt of a Disassociation frame from an AP for which the state is State 3 or State 4, a non-AP(#1133) STA’s MLME(#2165) shall disassociate from the AP using the following procedure:(#1342)

a)	The state for the AP shall be set to State 2.(#1342)

b)	The MLME shall issue an MLME-DISASSOCIATE.indication primitive to inform the SME of the disassociation.(#1342)

c)	If the reason code indicates a configuration or parameter mismatch as the cause of the disassociation, the STA shall not attempt to associate or reassociate with the AP until the configuration or parameter mismatch has been corrected.(#1342)

d)	If the reason code indicates the STA was disassociated for a reason other than configuration or parameter mismatch, the STA shall not attempt to associate or reassociate with the AP until a period of 2 s has elapsed.(#1342)

11.3.2.7 AP disassociation initiation procedure

The SME shall delete any PTKSA and temporal keys held for communication with the STA by using the MLME-DELETEKEYS.request primitive (see 8.4.10 (RSNA security association termination)) and by invoking MLME-SETPROTECTION.request(None) upon receiving a MLME-DISASSOCIATE.indication primitive.(#1342)

Upon receipt of an MLME-DISASSOCIATE.request primitive, an AP’s MLME shall disassociate a STA using the following procedure:

a)	If the state for the STA is State 3 or State 4, the AP shall send a Disassociation frame to the STA.

b)	The STA shall indicate the appropriate reason code for the STA disassociation as defined in Table 7-28 (Reason codes) of 7.3.1.7(Reason Code field).(#2197)).(#1342)

c)	The state for the STA shall be set to State 2, if it was not State 1.(#1342)

d)	The SME shall inform the DS of the disassociation.

11.3.2.8 AP disassociation receipt procedure

If Management Frame Protection was not negotiated when the PTKSA(s) were created, the(#2164) SME shall delete any PTKSA and temporal keys held for communication with the STA by using the MLME-DELETEKEYS.request primitive (see 8.4.10 (RSNA security association termination)) and by invoking MLME-SETPROTECTION.request(None) upon receiving a MLME-DISASSOCIATE.indication primitive.(#1342)

Upon receipt of a Disassociation frame from a STA for which the state is State 3 or State 4, the AP’s MLME(#2165 shall disassociate the STA using the following procedure:(#1342)

a)	The state for the STA shall be set to State 2.(#1342)

b)	The MLME shall issue an MLME-DISASSOCIATE.indication primitive to inform the SME of the disassociation.

c)	The SME shall inform the DS of the disassociation.(#1342)

References:

TGmb Draft 2.02 for baseline text.

Gen AdHoc Comment File:

https://mentor.ieee.org/802.11/dcn/10/11-10-0038-05-000m-wglb-160-tgmb-d2-gen-adhoc-comments.xls