IEEE P802.11
Wireless LANs

| 11v Annex L update |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-03-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Qi Wang | Broadcom Coporation | 150 Mathilda Place Sunnyvale CA 94086 USA | +1 408 922 8798 | Qi.wang@broadcom.com |

Abstract

This submission contains the revised sample C code to replace the sample code included in Annex L.3 of IEEE 802.11v/D9.0.

rev: de2004-11-18

Introduction

This submission contains the improved sample C code to replace the sample code in Annex L.3 of IEEE 802.11v/D9.0 [1], and to resolve CID-21 from the 11v recirculation sponsor ballot.

The proposed modifications are in reference to the text in IEEE P802.11v/D9.0 and are indicated by the change marks as follows:

Replace the sample code in L.3 with the following:

#include <stdio.h>

#include <limits.h>

#define ADD_TIM_BIT     0

#define REMOVE_TIM_BIT  1

#define TIM_ELEMENT_ID  5

#define TIM_BASE_SIZE   3	/* size of TIM fixed fields */

#define AID_SIZE        2008	/* valid AIDs are numBssids thru 2007 */

#define VBM_SIZE        251	/* size of VBM array = 2008/8 = 251 */

#define MAX_BSSIDS  128		/* maximum possible number of BSSIDs per AP, is a power of 2 */

typedef unsigned char UINT8;

typedef unsigned short int UINT16;

struct _tim

{

UINT8 Element_id;

UINT8 IELength;

UINT8 DtimCount;

UINT8 DtimPeriod;

UINT8 BitMapControl;

UINT8 PartialVirtualBitMap[VBM_SIZE];

};

UINT8 virtualBitMap[VBM_SIZE];

UINT8 mcast_pending[MAX_BSSIDS] = { 0 };

UINT8 dtimCount[MAX_BSSIDS] = { 0 };

UINT8 dtimPeriod[MAX_BSSIDS] = { 5 };

UINT16 numBssids = 1;

void

Build_TIM (struct _tim *Tim, char TIM_method, UINT16 numBssids)

{

UINT8 octetIndex = 0;

UINT8 octetIndex0 = 0;

UINT8 octetIndex1 = 0;

UINT8 offset = 0;

UINT8 lengthOfPartialVirtualBitMap = 0;

UINT8 bcast_octet = 0;

UINT8 bcast_bit = 0;

UINT8 max_bcast_octetIndex = 0;

UINT16 bssidIndex = 0;

UINT16 N1 = 0;

UINT16 N2 = 0;

printf ("numBssids = %d.\n", numBssids);

/* Compute the largest octet_index for bcast_indication */

max_bcast_octetIndex = (numBssids - 1) / 8;

/* Initialize PartialVirtualBitMap */

for (octetIndex = 0; octetIndex < VBM_SIZE; octetIndex++)

Tim->PartialVirtualBitMap[octetIndex] = 0;

octetIndex = 0;

if (numBssids == 1)

{

/* Find first nonzero octet in the virtual bit map */

for (octetIndex = 0;

((virtualBitMap[octetIndex] == 0) && (octetIndex < VBM_SIZE));

octetIndex++)

/* empty */ ;

if (octetIndex < VBM_SIZE)

offset = octetIndex & 0xFE;

/* Find last nonzero octet in the virtual bit map */

for (octetIndex = (VBM_SIZE - 1);

((virtualBitMap[octetIndex] == 0) && (octetIndex > 0));

octetIndex--)

/* empty */ ;

lengthOfPartialVirtualBitMap = octetIndex - offset + 1;

Tim->IELength = lengthOfPartialVirtualBitMap + TIM_BASE_SIZE;

Tim->BitMapControl = offset;

/* Copy the virtual bit map octets that are nonzero */

/* Note: A NULL virtualBitMap will still add a single octet of zero */

for (octetIndex = 0; octetIndex < lengthOfPartialVirtualBitMap;

octetIndex++)

Tim->PartialVirtualBitMap[octetIndex] =

virtualBitMap[offset + octetIndex];

}

if (numBssids > 1)

{

/* Update the broadcast/multicast bits, when numBssids > 1 */

for (bssidIndex = 1; bssidIndex < numBssids; bssidIndex++)

{

bcast_octet = (UINT8) (bssidIndex >> 3);

bcast_bit = (UINT8) (0x01 << (bssidIndex & 0x07));

if (mcast_pending[bssidIndex])

{

virtualBitMap[bcast_octet] |= bcast_bit;

}

else

{

virtualBitMap[bcast_octet] &= ~bcast_bit;

}

}

for (octetIndex0 = 0;

(octetIndex0 < VBM_SIZE) && (virtualBitMap[octetIndex0] == 0);

octetIndex0++)

/* empty */ ;

/* PVB contains neither bcast nor buffered ucast traffic, PVB is a single all-0 byte */

if (octetIndex0 == VBM_SIZE)

{

lengthOfPartialVirtualBitMap = 1;

offset = 0;

Tim->IELength = lengthOfPartialVirtualBitMap + TIM_BASE_SIZE;

Tim->BitMapControl = offset;

Tim->PartialVirtualBitMap[0] = virtualBitMap[0];

}

for (octetIndex1 = (max_bcast_octetIndex + 1);

((octetIndex1 < VBM_SIZE) && (virtualBitMap[octetIndex1] == 0));

octetIndex1++)

/* empty */ ;

/* PVB only contains bcast inidation, no buffered ucast traffic */

if ((octetIndex1 == VBM_SIZE) && (octetIndex0 < VBM_SIZE))

{

lengthOfPartialVirtualBitMap = max_bcast_octetIndex + 1;

offset = 0;

Tim->IELength = lengthOfPartialVirtualBitMap + TIM_BASE_SIZE;

Tim->BitMapControl = offset;

for (octetIndex = 0; octetIndex < (max_bcast_octetIndex + 1);

octetIndex++)

printf ("octetIndexCase2 = %d.\n", octetIndex);

Tim->PartialVirtualBitMap[octetIndex] = virtualBitMap[octetIndex];

}

/* PVB contains ucast indication with or without buffered bcast traffic */

for (octetIndex = 0; octetIndex < (max_bcast_octetIndex + 1);

octetIndex++)

Tim->PartialVirtualBitMap[octetIndex] = virtualBitMap[octetIndex];

if ((octetIndex1 < VBM_SIZE) && (octetIndex0 < VBM_SIZE))

{

if (TIM_method == 'A')

{

offset = 0;

for (octetIndex = (VBM_SIZE - 1);

(virtualBitMap[octetIndex] == 0)

&& (octetIndex > (max_bcast_octetIndex + 1)); octetIndex--)

/* empty */ ;

N2 = octetIndex;

lengthOfPartialVirtualBitMap = N2 - offset + 1;

for (octetIndex = (max_bcast_octetIndex + 1);

(octetIndex <= N2); octetIndex++)

Tim->PartialVirtualBitMap[octetIndex] =

virtualBitMap[octetIndex];

Tim->IELength = lengthOfPartialVirtualBitMap + TIM_BASE_SIZE;

Tim->BitMapControl = offset;

}

if (TIM_method == 'B')

{

offset = octetIndex1 - (max_bcast_octetIndex + 1);

offset = offset & 0xFE;

N1 = (max_bcast_octetIndex + 1) + offset;

for (octetIndex = (VBM_SIZE - 1);

((virtualBitMap[octetIndex] == 0)

&& (octetIndex > (max_bcast_octetIndex + 1)));

octetIndex--)

/* empty */ ;

N2 = octetIndex;

lengthOfPartialVirtualBitMap = N2 - offset + 1;

for (octetIndex = (max_bcast_octetIndex + 1);

octetIndex <= (lengthOfPartialVirtualBitMap - 1);

octetIndex++)

Tim->PartialVirtualBitMap[octetIndex] =

virtualBitMap[offset + octetIndex];

Tim->IELength = lengthOfPartialVirtualBitMap + TIM_BASE_SIZE;

Tim->BitMapControl = offset;

}

}

}

Tim->Element_id = TIM_ELEMENT_ID;

Tim->DtimCount = dtimCount[0];

Tim->DtimPeriod = dtimPeriod[0];

/* Update broadcast/ multicast indication bit for transmitted BSSID if necessary */

if ((Tim->DtimCount == 0) && mcast_pending[0])

Tim->BitMapControl |= 0x01;

}

void

Update_VirtualBitMap (UINT16 station_id, UINT8 Action)

{

UINT16 aid = station_id;

UINT8 aid_octet;

UINT8 aid_bit;

if ((aid > 0) && (aid < AID_SIZE))

{

/* Get aid position in Virtual Bit Map. */

aid_octet = (UINT8) (aid >> 3);

aid_bit = (UINT8) (0x01 << (aid & 0x07));

if (Action == REMOVE_TIM_BIT)

virtualBitMap[aid_octet] &= ~aid_bit;

else

virtualBitMap[aid_octet] |= aid_bit;

}

}

int

main (void)

{

struct _tim Tim;

UINT8 ExampleCase;

UINT16 count = 0;

char TIM_method;

/* The value of ExampleCase depends on the test case, allowed values are 1 through 22 */

ExampleCase = 1;

/* The value of TIM_method depends on the method to use, allowed values are 'A' and 'B' */

TIM_method = 'A';

switch (ExampleCase)

{

/* Nine examples with numBssids = 1, no difference between Method A and B */

case 1:

mcast_pending[0] = 0;

Update_VirtualBitMap (2, ADD_TIM_BIT);

Update_VirtualBitMap (7, ADD_TIM_BIT);

break;

case 2:

mcast_pending[0] = 1;

Update_VirtualBitMap (2, ADD_TIM_BIT);

Update_VirtualBitMap (7, ADD_TIM_BIT);

Update_VirtualBitMap (22, ADD_TIM_BIT);

Update_VirtualBitMap (24, ADD_TIM_BIT);

break;

case 3:

mcast_pending[0] = 1;

Update_VirtualBitMap (24, ADD_TIM_BIT);

break;

case 4:

mcast_pending[0] = 0;

Update_VirtualBitMap (3, ADD_TIM_BIT);

Update_VirtualBitMap (37, ADD_TIM_BIT);

Update_VirtualBitMap (43, ADD_TIM_BIT);

break;

case 5:

mcast_pending[0] = 0;

Update_VirtualBitMap (35, ADD_TIM_BIT);

break;

case 6:

mcast_pending[0] = 0;

Update_VirtualBitMap (43, ADD_TIM_BIT);

break;

case 7:

mcast_pending[0] = 0;

Update_VirtualBitMap (35, ADD_TIM_BIT);

Update_VirtualBitMap (35, REMOVE_TIM_BIT);

break;

case 8:

mcast_pending[0] = 1;

Update_VirtualBitMap (13, ADD_TIM_BIT);

Update_VirtualBitMap (43, ADD_TIM_BIT);

Update_VirtualBitMap (63, ADD_TIM_BIT);

Update_VirtualBitMap (73, ADD_TIM_BIT);

break;

case 9:

mcast_pending[0] = 1;

Update_VirtualBitMap (2007, ADD_TIM_BIT);

break;

/* Thirteen examples with numBssids > 1, TIM_method = 'A' or 'B' */

case 10:

numBssids = 8;

Update_VirtualBitMap (9, ADD_TIM_BIT);

Update_VirtualBitMap (11, ADD_TIM_BIT);

break;

case 11:

numBssids = 8;

mcast_pending[0] = 1;

mcast_pending[3] = 1;

Update_VirtualBitMap (12, ADD_TIM_BIT);

Update_VirtualBitMap (17, ADD_TIM_BIT);

Update_VirtualBitMap (22, ADD_TIM_BIT);

Update_VirtualBitMap (24, ADD_TIM_BIT);

break;

case 12:

numBssids = 16;

mcast_pending[3] = 1;

Update_VirtualBitMap (39, ADD_TIM_BIT);

break;

case 13:

numBssids = 8;

mcast_pending[5] = 1;

mcast_pending[7] = 1;

Update_VirtualBitMap (23, ADD_TIM_BIT);

break;

case 14:

numBssids = 8;

mcast_pending[2] = 1;

mcast_pending[7] = 1;

Update_VirtualBitMap (10, ADD_TIM_BIT);

break;

case 15:

numBssids = 15;

mcast_pending[5] = 1;

mcast_pending[7] = 1;

Update_VirtualBitMap (2007, ADD_TIM_BIT);

break;

case 16:

numBssids = 15;

mcast_pending[5] = 1;

mcast_pending[7] = 1;

Update_VirtualBitMap (1997, ADD_TIM_BIT);

Update_VirtualBitMap (1999, ADD_TIM_BIT);

break;

case 17:

numBssids = 32;

for (count = 0; count < numBssids; count += 2)

mcast_pending[count] = 1;

Update_VirtualBitMap (32, ADD_TIM_BIT);

Update_VirtualBitMap (33, ADD_TIM_BIT);

Update_VirtualBitMap (39, ADD_TIM_BIT);

break;

default:

break;

case 18:

numBssids = 14;

mcast_pending[5] = 1;

mcast_pending[7] = 1;

break;

case 19:

numBssids = 14;

mcast_pending[0] = 1;

mcast_pending[12] = 1;

break;

case 20:

numBssids = 15;

Update_VirtualBitMap (38, ADD_TIM_BIT);

break;

case 21:

numBssids = 15;

mcast_pending[0] = 1;

Update_VirtualBitMap (44, ADD_TIM_BIT);

break;

case 22:

numBssids = 16;

break;

}

Build_TIM (&Tim, TIM_method, numBssids);

printf ("Element_id = %d.\n", Tim.Element_id);

printf ("IELength = %d.\n", Tim.IELength);

printf ("DtimCount = %d.\n", Tim.DtimCount);

printf ("DtimPeriod = %d.\n", Tim.DtimPeriod);

printf ("BitMapControl = 0x%02X\n", Tim.BitMapControl);

if (Tim.IELength - TIM_BASE_SIZE > 0)

{

int octetIndex;

for (octetIndex = 0; octetIndex < Tim.IELength - TIM_BASE_SIZE;

octetIndex++)

printf ("PartialVirtualBitMap [%d] = 0x%02X\n", octetIndex,

Tim.PartialVirtualBitMap[octetIndex]);

}

return 0;

}

/* The End. */

References:

1. IEEE P802.11v/D9.0, Draft standard for information technology – telecommunications and information exchange between systems – local and metropolitan area networks – specific requirements – Part 11: Wireless LAN medium access control (MAC) and physical layer (PHY) specifications; Amendment 8: Wireless Network Management