IEEE P802.11
Wireless LANs

| Proposed changes regarding to DSE in IEEE 802.11af D0.03 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-06-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Chen Sun | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 | +81 45 847 5062 | sun@nict.go.jp |
| Yohannes D. Alemseged | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | yohannes@nict.go.jp |
| Zhou Lan | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | lan@nict.go.jp |
| Ha Nguyen Tran | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | haguen@nict.go.jp |
| Hiroshi Harada | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | harada@nict.go.jp |

Abstract

This document presents the update of the J2.4 in IEEE 802.11af/D0.02, to be incorporated in IEEE 802..1af/D0.03.

TGaf editor: Make the following changes at subclause of J2.4:

J.2.4 TVWS band in the United States (54 MHz to 688 MHz)

As shown in Figure J2.1, the state of a dependent STA changes from Unenabled to Becoming enabled once an enabling message is received. Before a dependent STA can become enabled on a TVWS channel to operate on, it shall perform successfully channel availability check for a minimum interval of 30 seconds in the TVWS channel. The channel availability check can be performed either/both in Unenabled state or/and in Becoming enabled state of the dependent STA to sense primary service signals. The primary service signals include TV signals (digital and analog), wireless microphone signals, and signals of private land mobile radio, commercial mobile radio, broadcast auxiliary, offshore radio and cable system head-ends.

# NOTE— The enablement process involves message exchange between the enabling station and the dependent STA (cf. subclause 11.11.2). If this message exchange is carried out in a TVWS channel, the dependent STA shall complete successfully the channel availability check on that channel before transmitting messages to the enabling STA. It is recommended that the channel availability check is performed after a list of available channels is received or is performed on the channel on which the dependent STA has received the enabling signals. This expedites the spectrum sensing task with reduced power by scanning a limited number of channels. The channel availability check can also be done in parallel to listening for enabling signals as an implementer’s option if the list of available channels is a known a-priori. For example, the STA can scan the channels used in the previous operation.

Under any of the following conditions, the state of the dependent STA changes from Becoming enabled back to Unenabled. 1) The STA fails enablement within dot11DSEEnablementTimeLimit (in seconds) and shall not transmit for dot11DSEEnablementFailHoldTime (in seconds) before it can again attempt to attain enablement. 2) The STA detects primary service signals in all the frequency bands that are indicated as available TVWS channels by the enabling STA, i.e., none of the available channel passes the channel availability check.

If the dependent STA detects primary service signals in one of the available TVWS channels, the dependent STA can stays in Becoming enabled state and can perform TV channel availability check on other available TVWS channels within the dot11DSEEnablementTimeLimit. Further, dependent STA shall exchange enablement request and enablement response with the enabling STA for the available TWVS channel that passes the TV channel availability check. The availability check result can be used by the enabling STA to update the list of available channels.

If the enablement is successful and no primary service signals are detected during channel availability check, the state of the dependent STA changes from Becoming enabled to Enabled. A dependent STA once becomes enabled and starts operation shall perform in-service monitoring successfully during operation to keep its Enabled state. The term “in-service monitoring” in 802.11 refers to sensing primary service signals once in every dot11DSERenewalTime (in seconds) and listening for enabling signals from the enabling STA that the dependent STA is enabled with.

Under any of the following conditions, the STA changes its state from Enabled back to Unenabled. 1) A deenablement message is received from the enabling STA. 2) The dependent STA fails to receive enabling signal from the enabling STA with which it is enabled within dot11DSERenewalTime (in seconds). The reason for this might be that the enabling STA finds that the operating channel is not available anymore or it detects a signal during the in-service monitoring. 3) The dependent STA detects primary service signals on the operating channel during in-service monitoring performed at the dependent STA and no available TVWS channels. All transmissions in that channel shall cease within 2 seconds.

If the dependent STA detects primary service signals in operating channel during in-service monitoring and there are still available TVWS channels, the STA changes its state from Enabled to Becoming enabled. Subsequently, within dot11DSEEnablementTimeLimit the STA can perform TV channel availability on other available TVWS channels and seek enablement with the enabling station for the TVWS channel that passes the channel availability check. The in-service monitoring result can be used by the enabling STA to update the list of available channels.

Figure J2.1 Enabling procedure for communication in TV band.

STAs shall set the value of dot11DSETransmitDivisor to 512 and the other dot11DSE timer values as shown in Table J.5 (TVWS DSE timer limits).

Table J.5—TVWS DSE timer limits

| Parameter | Seconds |
| --- | --- |
| dot11DSEEnablementTimeLimit | 32 |
| dot11DSEEnablementFailHoldTime | 512 |
| dot11DSERenewalTime | 60 |