IEEE P802.11
Wireless LANs

| Proposed changes regarding Measurement in IEEE 802.11af D0.02 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-06-22 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Yohannes Alemseged | NICT | 3-4 Hikarionooka, 239-0847 Yokosuka, Japan |  | yohannes@nict.go.jp |
| Zhou Lan | NICT | 3-4 Hikarionooka, 239-0847 Yokosuka, Japan |  |  |
| Chen Sun | NICT | 3-4 Hikarionooka, 239-0847 Yokosuka, Japan |  |  |
| Ha Nguyen Tran | NICT | 3-4 Hikarionooka, 239-0847 Yokosuka, Japan |  |  |
| Sum Chin Sean | NICT | 3-4 Hikarionooka, 239-0847 Yokosuka, Japan |  |  |
| Tuncer Baykas | NICT | 3-4 Hikarionooka, 239-0847 Yokosuka, Japan |  |  |
| Hiroshi Harada | NICT | 3-4 Hikarionooka, 239-0847 Yokosuka, Japan |  |  |

Abstract

This document presents an extension to the proposal 448r01. The rational to add additional items is to address the comments provided in the comment spreadsheet DCN 595 v07, comments 45, 100 and 148.  The proposed text is highlighted in yellow. It is intended that the new proposal shall be included as an amendment to the 802.11af D 0.03 document.

3. Definitions

EDITORIAL NOTE—The subclause numbering of definitions is of the form “3.af<x>” where <x> is an increasing number. The 802.11 technical editor will assign numbers when merging this list into the baseline document.

Insert new definition 3.af1 retaining the alphabetic ordering:

3.af1 PSSI: a measurement type that can be used to provide detail measurement of signal features and related parameters to facilitate primary service signal detection

4. Abbreviations and acronyms

Insert the following new acronym into Clause 4, while maintaining alphabetic ordering:

PSSI			Primary Service Signal Identification

7. Frame formats

7.3 MAC frame formats

7.3.2 Information elements

7.3.2.21 Measurement Request element

Replace Table 7-29 by the following table

Table 802.11af1 – Measurement Type definitions for measurement requests

| Name | Measurement Type |
| --- | --- |
| Basic request | 0 |
| Clear channel assessement (CCA) request | 1 |
| Received power indication (RPI) histogram request | 2 |
| Primary service signal Identification (PSSI) request | 3 |
| Reserved | 4-255 |

7.3.2.21.1 Basic request

Modify Figure 7-60- of Measurement Request field format for a basic request as follows

|  | Regulatory Class | Channel Number | Measurement Start Time | Measurement Duration | Measurement Interval | Measurement Reptition |
| --- | --- | --- | --- | --- | --- | --- |
| Octets | 1 | 1 | 8 | 2 | 2 | 1 |

Figure 802.11af1—Measurement request field format for a Basic request

Regulatory Class indicates the channel set for which the measurement request applies. Country, regulatory Class, and Channel Number together indicate the channel center frequency and bandwidth for which the measurement request applies. Valid values of Regulatory Class containing the channel set are shown in Annex J.

The Channel Number field shall be set to the channel number of the given regulatory class to which the Basic request applies.

The Measurement Start Time field shall be set to the TSF at the time (± 32 μs) at which the requested Basic measurement shall start. A value of 0 shall indicate it shall start immediately.

The Measurement Duration field shall be set to the duration of the requested measurement, expressed

in TUs.

The Measurement Duration field shall be set to the duration over which the Basic measurement should be performed. The value in this field is determined based on either regulatory requirement or target quality of detection.  Measurement duration is expressed in TUs.

Measurement interval indicates the interval that should be kept to perform successive measurements. Measurement interval is specified in ms (1000*TUs).  A value of 0 shall indicate successive measurements should be performed continuously.

Measurement repetition indicates the number of independent measurements to be performed. A value of 1 indicates measurement is to be performed only once.

7.3.2.21.4 PSSI request

A Measurement Type in the Measurement Request element may indicate a PSSI request. A response to a PSSI request is a PSSI report. It is optional for a STA to generate a PSSI report in response to a PSSI request. The Measurement Request field corresponding to a PSSI request is shown in Figure 802.11af2.

|  | Regulatory Class | Channel Number | Measurement Start Time | Measurement Duration | Measurement Interval | Measurement Reptition |
| --- | --- | --- | --- | --- | --- | --- |
| Octets | 1 | 1 | 8 | 2 | 2 | 1 |

Figure 802.11af2—Measurement request field format for a PSSI request

Regulatory Class indicates the channel set for which the measurement request applies. Country, regulatory Class, and Channel Number together specify the channel center frequency and bandwidth for which the measurement request applies. Valid values of Regulatory Class containing the channel set are shown in Annex J.

The Channel Number field shall be set to the channel number of the given regulatory class to which the PSSI request applies.

The Measurement Start Time field shall be set to the TSF at the time (± 32 μs) at which the requested PSSI measurement shall start. A value of 0 shall indicate it shall start immediately.

The Measurement Duration field shall be set to the duration of the requested measurement, expressed

in TUs.

The Measurement Duration field shall be set to the duration over which the PSSI measurement should be performed. The value in this field is determined based on either regulatory requirement or target quality of detection.  Measurement duration is expressed in TUs.

Measurement interval indicates the interval that should be kept to perform successive measurements. Measurement interval is specified in ms (1000*TUs).

Measurement repetition indicates the number of independent measurements to be performed. A value of 1 indicates measurement is to be performed only once.

7.3.2.22 Measurement Report element

Replace Table 7-30 by the following table

Table 802.11af1 – Measurement Type definitions for measurement reports

| Name | Measurement Type |
| --- | --- |
| Basic Report | 0 |
| CCA report | 1 |
| RPI histogram report | 2 |
| PSSI report | 3 |
| Reserved | 4-255 |

7.3.2.22.1 Basic Report

Replace Figure 7-66 by the following figure

Figure 802.11af3 Map field format

Replace the description of the Radar bit in  7.3.2.22.1 by the following paragraph

- Primary service signal/radar bit, which shall be set to 1 when the measured signal is identified to be a primary service signal/radar that is entitled to protection, was detected operating in the channel during the measurement period. Otherwise, the primary service signal/ radar bit shall be set to 0.

Note: Primary service signal is a signal that requires protection by regulation such as TV broadcast signal, PLMR/CMR signal, offshore radiotelephone signal, low power auxiliary service signal including microphone signal, and other signals that require protection from harmful interference. The algorithm to detect the primary service signal/radar shall satisfy regulatory requirements and is outside the scope of this standard.

7.3.2.22.4 PSSI report

A Measurement Type in the Measurement Report element may indicate a PSSI report. It is optional for a STA to support the generation of this report. STA generates PSSI report only when the PSSI measurement type is indicated in the measurement request frame. The format of the Measurement Report field corresponds to a PSSI report is shown in Fig. 7af4

|  | Channel Number | Measurement Start Time | Measurement Duration | Signal Energy | Noise Threshold | Optional sub-elements |
| --- | --- | --- | --- | --- | --- | --- |
| Octets | 1 | 8 | 2 | 2 | 2 | Variable |

Figure 7af4—Measurement Report field format for a PSSI report

The Channel Number field shall be set to the channel number to which the PSSI report applies. The  channel number in the Measurement report field refers to a channel number of particular regulatory class specified in the PSSI measurement request.

The Measurement Start Time field shall be set to the TSF at the time (± 32 s) at which the PSSI measurement started.

The Measurement Duration field shall be set to the duration over which the PSSI report was actually measured expressed in TUs.

The signal energy contains a quantized level of the primary service signlal energy measured just before a decision device.  The quantized signal energy is obtained after the primary service signal is separated from a background noise and interference. The specific methods involved to extract the primary signal are not the scope of this standard.

Noise Threshold contains the quantized noise threshold level which is proportional to the background noise and interference and is used as a detection threshold to obtain the result of primary service signal/radar bit. The specific methods   involved estimating the noise and interference level to derive the threshold level is out of the scope of the standard.

Other relevant information can also be included in the optional subelements.  PSSI measurement can be used as a measure of the quality of signal detection.  PSSI reports collected by a master STA from different dependent STAs is provided to a higher layer protocol for different application purposes. For example, advanced signal detection can be achieved by aggregating the PSSI report from different dependent STAs.