IEEE P802.11
Wireless LANs

| D0.1 comment resolution part 2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-08-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Carlos Cordeiro | Intel |  |  | carlos.cordeiro@intel.com |
| Solomon Trainin | Intel |  |  | solomon.trainin@intel.com |

Abstract

This document proposes a resolution to CID348 on D0.1.

| 348 | A.4 |  |  | TR | The PICS proforma may not be accurate for implementation | need to revisit it |
| --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution: Accept

## A.4 PICS proforma

### A.4.3 IUT configuration

.11 Editor note: Modify the Table as follows:

| Item | IUT configuration | References | Status | Support |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| *CF2.1 | Operation in an infrastructure BSS | 5.2 (Components of the IEEE 802.11 architecture) | CF2:M CF2&CF17:O | Yes, No, N/A |
| *CF2.2 | Operation in an IBSS | 5.2 (Components of the IEEE 802.11 architecture) | CF2:O CF2&CF17:O | Yes, No, N/A |
| *CF2.3 | Operation in a PBSS | 5.2.1a (The Personal BSS (PBSS) as an ad hoc network) |  |  |
| *CF2.3.1 | Operation as a PCP | 5.2.1a (The Personal BSS (PBSS) as an ad hoc network) | CF2&CF17:M | Yes, No |
| *CF2.3.2 | Operation Not as a PCP | 5.2.1a (The Personal BSS (PBSS) as an ad hoc network) | CF2&CF17:M | Yes, No |
| … | … | … | … | … |
| * CF10 | Is spectrum management operation supported? | 7.3.1.4, 11.6 | (CF6 OR CF16 OR CF17):O | Yes, No |
| * CF11 | Is regulatory classes capability implemented? | 7.3.2.12, 17.3.8.3.2, 17.3.8.6, 17.4.2, Annex I, Annex J | (CF6 OR CF16 OR CF17)& CF8&CF10:O | Yes, No, N/A |
| * CF12 | Quality of service (QoS) supported (in the OBand) | 9.9, 9.10, 5.2.9 | O CF16:M | Yes, No, N/A |
| * CF17 | mmWave features | 7.3.2.91 | O | Yes, No |
| * CF18 | At least two radios in different bands present | 7.4.14, 11.34 | CF4 OR CF6 OR CF7 OR CF9 OR CF16 OR CF17:O | Yes, No, N/A |

#### A.4.4 MAC protocol

#### A.4.4.1 MAC protocol capabilities

.11 Editor note: Modify the Table as follows:

| Item | Protocol capability | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | Are the following MAC protocol capabilities supported? |  |  |  |
| PC1 | Authentication service (in the OBand) | 5.4.3.1 (Authentication), 5.4.3.2 (Deauthentication), 8.1 (Framework), Annex C | M | Yes, No |
| … | … | … | … | … |
| PC3 | Distributed coordination function (DCF) (in the OBand) | 9.1 (MAC architecture), 9.2 (DCF), Annex C | M | Yes, No |
| … | … | … | … | … |
| *PC4 | Point coordinator (PC) (in the OBand) | 9.1 (MAC architecture), 9.3 (PCF), Annex C | CF1:O | Yes, No, N/A |
| … | … | … | … | … |
| *PC5 | Contention-free (CF)-Pollable (in the OBand) | 9.1 (MAC architecture), 9.3 (PCF), Annex C | CF2.1:O | Yes, No, N/A |
| … | … | … | … | … |
| PC11.7 | Passive scanning | 11.1.3 (Acquiring synchronization, scanning) | (CF2.1 or CF2.2 or CF2.3.2):M | Yes, No, N/A |
| PC11.8 | Active scanning | 11.1.3 (Acquiring synchronization, scanning) | (CF2.1 or CF2.2 or CF2.3.2):M | Yes, No, N/A |
| … | … | … | … | … |
| PC12 | Infrastructure power management (in the OBand) | 11.2.1 (Power management in an infrastructure network), Annex C | M | Yes, No |
| … | … | … | … | … |
| PC14 | Association and reassociation (in the OBand) | 5.4 (Overview of the services), 11.3 (STA authentication and association), 11.3.2 (Association, reassociation, and disassociation), Annex C | M | Yes, No |
| … | … | … | … | … |
| PC34.1.2.3 | Galois/Counter mode protocol (GCMP) data confidentiality protocol | 8.3.5 | (CF17 and PC34):M | Yes, No, N/A |
| … | … | … | … | … |
| *PC34.1.10 | Management Frame Protection (in the OBand) | 7.2.2.1 (Data frame format), 7.3.1.4 (Capability Information field), 5.4.3.3 (Data confidentiality), 8.7.2 (RSNA frame pseudo-code), 8.7.2.1 (Per-MSDU/Per-AMSDU Tx pseudo-code), 8.7.2.2 (Per-MPDU Tx pseudo-code), 8.7.2.3 (Per-MPDU Rx pseudo-code), 8.7.2.4 (Per-MSDU/Per-AMSDU Rx pseudo-code), 11.3.1 (Authentication and deauthentication), 11.3.2 (Association, reassociation, and disassociation), 8.3.3 (CTR with CBC-MAC Protocol (CCMP)) | PC34:O | Yes, No, N/A |
| … | … | … | … | … |
| PC35.12 | QoS procedures for fast basic service set (BSS) transition | 11A.11 | (CF12&PC35 OR CF17&PC35):M | Yes, No, N/A |
| … | … | … | … | … |
| *PC37 | Power save multi-poll (PSMP) (in the OBand) | 7.4.10.4 (PSMP frame format), 9.16 (PSMP Operation) | O | Yes, No |
| … | … | … | … | … |
| *PC38 | Multi-band Operation | 11.34 | CF18:O | Yes, No, N/A |
| *PC38.1 | FST Setup | 11.34.1, 7.3.2.101 Multi-band element, 7.4.14.2 FST Setup Request, 7.4.14.3 FST Setup Response, 7.4.14.5 FST Ack Request, 7.4.14.6 FST Ack Response, 7.3.2.106, 7.3.2.107 | PC38:M | Yes, No, N/A |
| PC38.2 | FST TS switching | 11.34.1.1, 7.4.2.1.2 Extended mmWave ADDTS Request frame variant, 7.4.2.2.2 Extended mmWave ADDTS Response frame variant, 7.4.2.3 DELTS frame format, 7.4.4.1 ADDBA Request frame format, 7.4.4.2 ADDBA Response frame format, 7.4.4.3 DELBA frame format, 7.3.2.30 TSPEC element, 7.3.2.97Extended mmWave TSPEC element | PC38.1:M | Yes, No, N/A |
| PC38.3 | FST Teardown |  |  |  |
| PC38.3.1 | Transmission of FST Teardown | 11.34.2, 7.4.14.4 FST Tear down | PC38.1:O | Yes, No, N/A |
| PC38.3.2 | Reception of FST Teardown | 11.34.2, 7.4.14.4 FST Tear down | PC38.1:M | Yes, No, N/A |

#### A.4.4.2 MAC frames

.11 Editor note: Modify the table as follows:

| Item | MAC frame | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | Is the transmission of the following MAC frames supported? | Clause 7 (Frame formats), Annex C |  |  |
| FT1 | Association Request | Clause 7 (Frame formats) | CF2.1:M CF2.3.2:O | Yes, No, N/A |
| FT2 | Association Response | Clause 7 (Frame formats) | (CF1 OR CF2.3.1):M | Yes, No, N/A |
| FT3 | Reassociation Request | Clause 7 (Frame formats) | CF2.1:M CF2.3.2:O | Yes, No, N/A |
| FT4 | Reassociation Response | Clause 7 (Frame formats) | (CF1 OR CF2.3.1):M | Yes, No, N/A |
| FT5 | Probe Request | Clause 7 (Frame formats) | (CF2.1 OR CF2.2 OR CF2.3):M | Yes, No, N/A |
| FT6 | Probe Response | Clause 7 (Frame formats) | M | Yes, No |
| FT7 | Beacon (in the OBand) | Clause 7 (Frame formats) | M | Yes, No |
| FT8 | ATIM | Clause 7 (Frame formats) | (CF2.1 OR CF2.2 OR CF2.3):M | Yes, No, N/A |
| FT9 | Disassociation | Clause 7 (Frame formats) | M | Yes, No |
| FT10 | Authentication (in the OBand) | Clause 7 (Frame formats) | M | Yes, No |
| FT11 | Deauthentication (in the OBand) | Clause 7 (Frame formats) | M | Yes, No |
| FT12 | Power save (PS)-Poll (in the OBand) | Clause 7 (Frame formats) | CF2.1:M | Yes, No, N/A |
| FT13 | RTS | Clause 7 (Frame formats) | M | Yes, No |
| FT14 | CTS (in the OBand) | Clause 7 (Frame formats) | M | Yes, No |
| FT15 | Acknowledgement (ACK) | Clause 7 (Frame formats) | M | Yes, No |
| FT16 | CF-End (in the OBand) | Clause 7 (Frame formats) | PC4:M | Yes, No, N/A |
| FT17 | CF-End+CF-Ack (in the OBand) | Clause 7 (Frame formats) | PC4:M | Yes, No, N/A |
| FT18 | Data (in the OBand) | Clause 7 (Frame formats) | M | Yes, No |
| FT19 | Data+CF-Ack (in the OBand) | Clause 7 (Frame formats) | (PC4 OR PC5):M | Yes, No, N/A |
| FT20 | Data+CF-Poll (in the OBand) | Clause 7 (Frame formats) | PC4.3:M | Yes, No, N/A |
| FT21 | Data+CF-Ack+CF-Poll (in the OBand) | Clause 7 (Frame formats) | PC4.3:M | Yes, No, N/A |
| FT22 | Null (in the OBand) | Clause 7 (Frame formats) | M | Yes, No |
| FT23 | CF-Ack (no data) (in the OBand) | Clause 7 (Frame formats) | (PC4 OR PC5):M | Yes, No, N/A |
| FT24 | CF-Poll (no data) (in the OBand) | Clause 7 (Frame formats) | PC4.3:M | Yes, No, N/A |
| FT25 | CF-Ack+CF-Poll (no data) (in the OBand) | Clause 7 (Frame formats) | PC4.3:M | Yes, No, N/A |
| FT26 | QoS Data (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FT27 | QoS Null (no data) (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FT28 | BlockAckReq (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FT29 | BlockAck (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FT30 | Poll (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FT31 | SPR (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FT32 | Grant (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FT33 | mmWaveCTS (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FT34 | mmWaveDTS (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FT35 | mmWaveCF-End (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FT36 | ScS (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FT37 | SS-Feedback (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FT38 | SS-ACK (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FT39 | mmWave Beacon (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
|  | Is the reception of the following MAC frames supported? | Clause 7 (Frame formats), Annex C |  |  |
| FR1 | Association Request | Clause 7 (Frame formats) | (CF1 OR CF2.3.1):M | Yes, No, N/A |
| FR2 | Association Response | Clause 7 (Frame formats) | (CF2.1 OR CF2.3.2):M | Yes, No, N/A |
| FR3 | Reassociation Request | Clause 7 (Frame formats) | (CF1 OR CF2.3.1):M | Yes, No, N/A |
| FR4 | Reassociation Response | Clause 7 (Frame formats) | (CF2.1 OR CF2.3.2):M | Yes, No, N/A |
| FR5 | Probe Request | Clause 7 (Frame formats) | M | Yes, No |
| FR6 | Probe Response | Clause 7 (Frame formats) | M | Yes, No |
| FR7 | Beacon (in the OBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR8 | ATIM | Clause 7 (Frame formats) | (CF2.2 OR CF2.3):M | Yes, No, N/A |
| FR9 | Disassociation | Clause 7 (Frame formats) | M | Yes, No |
| FR10 | Authentication (in the OBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR11 | Deauthentication (in the OBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR12 | Power save (PS)-Poll (in the OBand) | Clause 7 (Frame formats) | CF1:M | Yes, No, N/A |
| FR13 | RTS | Clause 7 (Frame formats) | M | Yes, No |
| FR14 | CTS (in the OBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR15 | Acknowledgement (ACK) | Clause 7 (Frame formats) | M | Yes, No |
| FR16 | CF-End (in the OBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR17 | CF-End+CF-Ack (in the OBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR18 | Data (in the OBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR19 | Data+CF-Ack (in the OBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR20 | Data+CF-Poll (in the OBand) | Clause 7 (Frame formats) | PC5:M | Yes, No, N/A |
| FR21 | Data+CF-Ack+CF-Poll (in the OBand) | Clause 7 (Frame formats) | PC5:M | Yes, No, N/A |
| FR22 | Null (in the OBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR23 | CF-Ack (no data) (in the OBand) | Clause 7 (Frame formats) | (PC4 OR PC5):M | Yes, No, N/A |
| FR24 | CF-Poll (no data) (in the OBand) | Clause 7 (Frame formats) | PC5:M | Yes, No, N/A |
| FR25 | CF-Ack+CF-Poll (no data) (in the OBand) | Clause 7 (Frame formats) | PC5:M | Yes, No, N/A |
| FR26 | QoS Data (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR27 | QoS Null (no data) (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR28 | BlockAckReq (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR29 | BlockAck (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR30 | Poll (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR31 | SPR (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR32 | Grant (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR33 | mmWaveCTS (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR34 | mmWaveDTS (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR35 | mmWaveCF-End (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR36 | ScS (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR37 | SS-Feedback (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR38 | SS-ACK (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |
| FR39 | mmWave Beacon (in the DBand) | Clause 7 (Frame formats) | M | Yes, No |

#### A.4.14 QoS base functionality

.11 Editor note: modify the table as follows

| Item | Protocol capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| QB1 | QoS frame format | 7.2.1.1 (RTS frame format) – 7.2.1.3 (ACK frame format), 7.2.2.1 (Data frame format), 7.2.3.1 (Beacon frame format), 7.2.3.4 (Association Request frame format) – 7.2.3.7 (Reassociation Response frame format), 7.2.3.9 (Probe Response frame format), 7.2.3.12 (Action frame format) | CF12:M | Yes, No, N/A |
| QB1.1 | QoS frame format (in the OBand) | 7.2.1.1 (RTS frame format) – 7.2.1.3 (ACK frame format), 7.2.2.1 (Data frame format), 7.2.3.1 (Beacon frame format), 7.2.3.4 (Association Request frame format) – 7.2.3.7 (Reassociation Response frame format), 7.2.3.9 (Probe Response frame format), 7.2.3.12 (Action frame format) | CF12:M | Yes, No, N/A |
| QB1.2 | QoS frame format (in the DBand) | 7.2.1.1 (RTS frame format), 7.2.1.3 (ACK frame format), 7.2.1.7 (Block Ack Request frame format), 7.2.1.8 (Block Ack frame format), 7.2.1.10 (Poll) – 7.2.1.18 (SS-ACK), 7.2.2.1 (Data frame format), 7.2.4.1 (mmWave Beacon frame format), 7.2.3.4 (Association Request frame format) – 7.2.3.7 (Reassociation Response frame format), 7.2.3.9 (Probe Response frame format), 7.2.3.12 (Action frame format) | CF17:M | Yes, No, N/A |
| QB2 | Per traffic identifier (TID) duplicate detection | 7.1.3.4 (Sequence Control field), 7.1.3.5 (QoS Control field), 9.2.0b.11 (Duplicate detection and recovery) | CF12 OR CF17:M | Yes, No, N/A |
| QB3 | Decode of no-acknowledgment policy in QoS data frames | 7.1.3.5.3 (Ack Policy subfield), 9.9.1.4 (Multiple frame transmission in an EDCA TXOP), 9.9.1.5 (EDCA backoff procedure), 9.9.3.1 (Contention-based admission control procedures), 9.9.3.2 (Controlled- access admission control) | CF12 OR CF17:M | Yes, No, N/A |
| QB4 | Block Acknowledgments (Block Acks) |  |  |  |
| QB4.1 | Immediate Block Ack | 7.2.1.7.1 (Overview), 7.2.1.7.2 (Basic BlockAckReq variant), 7.2.1.8.1 (Overview), 7.2.1.8.2 (Basic Block Ack variant), 7.4.4 (Block Ack Action frame details), 9.10 (Block Acknowledgment (Block Ack)) (except 9.10.7 (HT-immediate Block Ack extensions) and 9.10.8 (HT-delayed Block Ack extensions)), 11.5 (Block Ack operation) | CF12:O CF16 OR CF17:M | Yes, No, N/A |
| QB4.2 | Delayed Block Ack | 7.2.1.7.1 (Overview), 7.2.1.7.2 (Basic BlockAckReq variant), 7.2.1.8.1 (Overview), 7.2.1.8.2 (Basic Block Ack variant), 7.4.4 (Block Ack Action frame details), 9.10 (Block Acknowledgment (Block Ack)) (except 9.10.7 (HT-immediate Block Ack extensions) and 9.10.8 (HT-delayed Block Ack extensions)), 11.5 (Block Ack operation) | CF12:O | Yes, No, N/A |
| QB4.3 | Compressed Block Ack | 7.2.1.7.3 (Compressed BlockAckReq variant) | CF12:O CF16:M | Yes, No, N/A |
| QB4.3.1 | Compressed Block Ack | 7.2.1.7.3 (Compressed BlockAckReq variant) | CF12:O CF16 OR CF17:M | Yes, No, N/A |
| QB4.3.2 | Extended Compressed Block Ack | 7.2.1.8.5 (Extended Compressed BlockAckReq variant) | CF17:O | Yes, No, N/A |
| QB4.4 | MultiTID Block Ack | 7.2.1.7.4 (Multi-TID BlockAckReq variant) | CF12:O CF16: M | Yes, No, N/A |
| QB5 | Automatic power-save delivery (APSD) | 7.4.2 (QoS Action frame details), 11.2.1 (Power management in an infrastructure network) | CF12:O | Yes, No, N/A |
| QB6 | Direct-link setup (DLS) | 7.3.2.20 (Channel Switch Announcement element), 7.4.3 (DLS Action frame details), 10.3.12 (Measurement request), 11.7 (DLS operation) | (CF1 AND CF12):M (CF2.1 AND CF12):O | Yes, No, N/A |

#### A.4.15 QoS enhanced distributed channel access (EDCA)

.11 Editor note: modify the table as follows

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| QD1 | Support for four transmit queues with a separate channel access entity associated with each (in the OBand) | 9.1.3.1 (HCF contention-based channel access (EDCA)), 9.9.1.1 (Reference implementation) | CF12:M | Yes, No, N/A |
| QD2 | Per-channel access function differentiated channel access (in the OBand) | 9.9.1.2 (EDCA TXOPs), 9.9.1.3 (Obtaining an EDCA TXOP), 9.9.1.5 (EDCA backoff procedure) | CF12:M | Yes, No, N/A |
| QD3 | Multiple frame transmission support | 9.9.1.4 (Multiple frame transmission in an EDCA TXOP) | CF12 OR CF17:O | Yes, No, N/A |
| QD4 | Maintenance of within-queue ordering, exhaustive retransmission when sending non-QoS data frames | 9.9.1.6 (Retransmit procedures) | CF12 OR CF17:M | Yes, No, N/A |
| QD5 | Interpretation of admission control mandatory (ACM) bit in EDCA Parameter Set element | 7.3.2.14 (Extended Supported Rates element), 9.9.3.1 (Contention-based admission control procedures) | (CF2.1 & (CF12 OR CF17)):M (CF2.3.2 & CF17):M | Yes, No, N/A |
| QD6 | Contention-based admission Control | 9.9.3.1 (Contention-based admission control procedures), 7.3.2.15 (Power Constraint element), 7.3.2.16 (Power Capability element), 7.4.2.1 (ADDTS Request frame format)–7.4.2.3 (DELTS frame format), 11.4 (TS operation) | (CF1 & (CF12 OR CF17)):O (CF2.1 & (CF12 OR CF17)):O (CF2.3.1 & CF17):O (CF2.3.2 & CF17):O | Yes, No, N/A |
| QD7 | Power management | 11.2 (Power management) | CF12:O | Yes, No, N/A |

#### A.4.17 Radio management extensions

.11 Editor note: Modify the Table as follows:

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| RM2.4 | Link Measurement Report frame | 7.4.6.4 (Link Measurement Report Frame Format) | CF13:M | Yes, No, N/A |
| RM2.4.1 | Link Measurement Report frame (in the OBand) | 7.4.6.4 (Link Measurement Report Frame Format) | CF13:M | Yes, No, N/A |
| RM2.4.2 | Link Measurement Report frame (in the DBand) | 7.4.6.4 (Link Measurement Report Frame Format), 7.3.2.105 (mmWave Link Margin element), 9.27 (mmWave Link Adaptation) | (CF13 AND CF17):M | Yes, No, N/A |
| … | … | … | … | … |
| *RM10 | Transmit Stream/Category Measurement Type | 11.10 (Radio measurement procedures), 11.10.8.8 (Transmit Stream/Category Measurement Report) | (CF13 AND (CF12 OR CF17)):O | Yes, No, N/A |
| … | … | … | … | … |
| RM14.1 | Respond with RCPI element when requested | 11.1.3.2.2 (Active scanning procedure) | (CF13 AND (CF12 OR CF17) AND CF1):M | Yes, No, N/A |
| RM14.2 | Measurement of RCPI on Probe Request frames | 11.1.3.2.2 (Active scanning procedure) | (CF13 AND (CF12 OR CF17) AND CF1):O | Yes, No, N/A |
| … | … | … | … | … |
| RM24 | BSS Available Admission Capacity | 7.3.2.43 (BSS Available Admission Capacity element) | (CF1 AND (CF12 OR CF17) AND CF13):M | Yes, No, N/A |
| RM25 | BSS AC Access Delay (in the OBand) | 7.2.3.1 (Beacon frame format), 7.2.3.9 (Probe Response frame format), 7.3.2.44 (BSS AC Access Delay element) | (CF1 AND CF12 AND CF13):M | Yes, No, N/A |
| RM26 | BSS AC Access Delay (in the DBand) | 7.2.4.1 (mmWave Beacon frame format), 7.2.3.9 (Probe Response frame format), 7.3.2.44 (BSS AC Access Delay element) | (CF1 AND CF17 AND CF13):M | Yes, No, N/A |
| RM27 | Directional Channel Quality Measurement | 11.10, 11.33 |  |  |
| RM27.1 | Directional Channel Quality Request | 7.3.2.21.12 (Directional Channel Quality request), 11.33 (Spatial frequency sharing and Interference mitigation) | (CF13 AND mmWaveM19):O | Yes, No, N/A |
| RM27.2 | Directional Channel Quality Response | 7.3.2.22.11 (Directional Channel Quality report), 11.33 (Spatial frequency sharing and Interference mitigation) | (CF13 AND mmWaveM19):M | Yes, No, N/A |
| RM28 | Directional Measurement Type | 11.10 |  |  |
| RM28.1 | Directional Measurement Request | 7.3.2.21.13 Directional Measurement request | (CF13 AND CF17):O | Yes, No, N/A |
| RM28.2 | Directional Measurement Response | 7.3.2.22.12 Directional Measurement report | (CF13 AND CF17):O | Yes, No, N/A |
| RM29 | Directional Statistics Measurement Type | 11.10 |  |  |
| RM29.1 | Directional Statistics Measurement Type | 7.3.2.21.14 Directional Statistics request | (CF13 AND CF17):O | Yes, No, N/A |
| RM29.2 | Directional Statistics Measurement Type | 7.3.2.22.13 Directional Statistics report | (CF13 AND CF17):O | Yes, No, N/A |

.11 Editor note: Insert the following subclause

### A.4.21 mmWave features

#### A.4.21.1 mmWave MAC features

| Item | Protocol capability | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | Are the following MAC protocol features supported? |  |  |  |
| mmWaveM1 | mmWave capabilities signaling |  |  |  |
| mmWaveM1.1 | mmWave Capabilities element | 7.3.2.91 | CF17:M | Yes, No, N/A |
| mmWaveM1.2 | Signaling of STA capabilities in Probe Request, (Re)Association Request frames | 7.3.2.91, 7.2.3.8, 7.2.3.4, 7.2.3.6 | (CF17 and (CF2.1 OR CF2.2 OR CF2.3.2)):M | Yes, No, N/A |
| mmWaveM1.3 | Signaling of STA and BSS capabilities in mmWave Beacon, Probe Response, (Re)Association Response frames | 7.3.2.91, 7.2.4.1, 7.2.3.9, 7.2.3.5, 7.2.3.7 | (CF17 and (CF1 OR CF2.3.1)):M | Yes, No, N/A |
| mmWaveM2 | Signaling of mmWave operation | 7.3.2.92 | (CF17 and (CF1 OR CF2.3.1)):M | Yes, No, N/A |
| mmWaveM3 | MSDU aggregation |  |  |  |
| mmWaveM3.1 | Reception of Standard A-MSDUs | 7.1.3.5, 7.2.2.2 (except 7.2.2.2.1) | CF17:M | Yes, No, N/A |
| mmWaveM3.2 | Standard A-MSDU format | 7.2.2.2 (except 7.2.2.2.1) | CF17:M | Yes, No, N/A |
| mmWaveM3.3 | Standard A-MSDU content | 7.2.2.2 (except 7.2.2.2.1) | CF17:M | Yes, No, N/A |
| mmWaveM3.4 | Transmission of Standard A-MSDUs | 7.2.2.2 (except 7.2.2.2.1), 7.1.3.5 | CF17:O | Yes, No, N/A |
| *mmWaveM3.5 | Reception of Short A-MSDU | 7.1.3.5, 7.2.2.2.1 | CF17:O | Yes, No, N/A |
| mmWaveM3.6 | Short A-MSDU format | 7.2.2.2.1 | (mmWaveM3.5 OR mmWaveM3.8):M | Yes, No, N/A |
| mmWaveM3.7 | Short A-MSDU content | 7.2.2.2.1 | (mmWaveM3.5 OR mmWaveM3.8):M | Yes, No, N/A |
| *mmWaveM3.8 | Transmission of Short A-MSDU | 7.2.2.2.1, 7.1.3.5 | CF17:O | Yes, No, N/A |
| mmWaveM3.9 | Negotiation of Short A-MSDU usage | 7.3.2.30 | CF17:M | Yes, No, N/A |
| mmWaveM4 | MPDU aggregation |  |  |  |
| mmWaveM4.1 | Reception of A-MPDU | 7.3.2.91.1 (A-MPDU parameters), 8.3, 9.7d.2, 7.3.2.102 | CF17:M | Yes, No, N/A |
| mmWaveM4.2 | A-MPDU format | 7.4a.1a | CF17:M | Yes, No, N/A |
| mmWaveM4.3 | A-MPDU content | 7.4a.3 | CF17:M | Yes, No, N/A |
| mmWaveM4.4 | Transmission of A-MPDU | 7.3.2.91.1 (A-MPDU parameters), 8.3, 7.3.2.102 | CF17:O | Yes, No, N/A |
| mmWaveM5 | A-PPDU aggregation | 9.7j |  |  |
| *mmWaveM5.1 | Reception of A-PPDU | 7.3.2.91.1 (A-PPDU Supported) | CF17:O | Yes, No, N/A |
| mmWaveM5.2 | A-PPDU format | 21.5.2, 21.6.2 | (mmWaveM5.1 OR mmWaveM5.3):M | Yes, No, N/A |
| *mmWaveM5.3 | Transmission of A-PPDU | 7.3.2.91.1 (A-PPDU Supported) | CF17:O | Yes, No, N/A |
| *mmWaveM6 | Reverse direction aggregation exchanges | 9.15, 7.3.2.91.1 (Reverse Direction) | CF17:O | Yes, No, N/A |
| mmWaveM6.1 | Constraints regarding responses | 9.15.4 | mmWaveM6:M | Yes, No, N/A |
| mmWaveM7 | mmWave channel access |  |  |  |
| mmWaveM7.1 | AT transmission |  |  |  |
| mmWaveM7.1.1 | Transmission of Request | 9.23.3 | CF17 AND (CF1 OR CF2.3.1):O | Yes, No, N/A |
| mmWaveM7.1.2 | Reception of Request | 9.23.3 | CF17 AND (CF2.1 OR CF2.3.2):M | Yes, No, N/A |
| mmWaveM7.1.3 | Transmission of Response | 9.23.3 | CF17 AND (CF2.1 OR CF2.3.2):M | Yes, No, N/A |
| mmWaveM7.1.4 | Reception of Response | 9.23.3 | CF17 AND (CF1 OR CF2.3.1):M | Yes, No, N/A |
| mmWaveM7.2 | DTT transmission | 9.23.4 | CF17:M | Yes, No, N/A |
| mmWaveM7.3 | Time allocation |  |  |  |
| *mmWaveM7.3.1 | Service period (SP) allocation | 9.23.6.1, 7.3.2.95, 7.3.2.97 (Extended mmWave TSPEC element) | CF17 AND (CF1 OR CF2.3.1):O | Yes, No, N/A |
| *mmWaveM7.3.2 | CBP allocation | 9.23.6.2, 7.3.2.95 | CF17 AND (CF1 OR CF2.3.1):O | Yes, No, N/A |
| mmWaveM7.3.3 | Interpretation of allocation | 9.23.6.1, 9.23.6.2 7.3.2.95 | CF17 AND (CF1 OR CF2.1 OR CF2.2 OR CF2.3.1 OR CF2.3.2):M | Yes, No, N/A |
| *mmWaveM7.4 | Contention-based period | 9.23.5, 9.23.6.2 | CF17:M | Yes, No, N/A |
| mmWaveM7.4.1 | Distributed coordination function (DCF) |  |  |  |
| mmWaveM7.4.1.1 | Network allocation vector (NAV) function | 9.2.0b.2 (CS mechanism), 9.2.5 (DCF access procedure), 9.23.10 | mmWaveM7.4:M | Yes, No, N/A |
| mmWaveM7.4.1.2 | Interframe space usage and timing | 9.2.0b.4 (IFS), 9.2.5 (DCF access procedure), 9.2.10 (DCF timing relations) | mmWaveM7.4:M | Yes, No, N/A |
| mmWaveM7.4.1.3 | Random Backoff function | 9.2.4 (Random backoff time) | mmWaveM7.4:M | Yes, No, N/A |
| mmWaveM7.4.1.4 | DCF Access procedure | 9.2 (DCF), 9.2.5.1 (Basic access), 9.2.5.5 (Control of the channel) | mmWaveM7.4:M | Yes, No, N/A |
| mmWaveM7.4.1.5 | Random Backoff procedure | 9.2 (DCF), 9.2.5.2 (Backoff procedure for DCF) | mmWaveM7.4:M | Yes, No, N/A |
| mmWaveM7.4.1.6 | Recovery procedures and retransmit limits | 9.2.5.3 (Recovery procedures and retransmit limits) | mmWaveM7.4:M | Yes, No, N/A |
| mmWaveM7.4.1.7 | Request to send (RTS)/mmWave clear to send (mmWaveCTS) procedure | 9.2.5.4 (Setting and resetting the NAV), 9.23.10 (NAV Update), 9.2.0b.6 (RTS/CTS with fragmentation), 9.2.0b.7 (CTS procedure) | mmWaveM7.4:M | Yes, No, N/A |
| mmWaveM7.4.1.8 | Directed MAC protocol data unit (MPDU) transfer | 9.2.6 (Individually addressed MPDU transfer procedure) | mmWaveM7.4:M | Yes, No, N/A |
| mmWaveM7.4.1.9 | Group addressed MPDU transfer | 9.2.7 (Group addressed MPDU transfer procedure) | mmWaveM7.4:M | Yes, No, N/A |
| mmWaveM7.4.1.10 | MAC-level acknowledgment | 9.2.0b.3 (MAC Level Acknowledgements), 9.2.0b.9 (ACK procedure) | mmWaveM7.4:M | Yes, No, N/A |
| mmWaveM7.4.1.11 | Duplicate detection and recovery | 9.2.0b.11 (Duplicate detection and recovery) | mmWaveM7.4:M | Yes, No, N/A |
| mmWaveM7.4.2 | Enhanced DCF (EDCA) |  |  |  |
| *mmWaveM7.4.2.1 | Support for one transmit queue with AC_BE access category | 9.1.3.1 (HCF contention-based channel access (EDCA)), 9.9.1.1 (Reference implementation) | mmWaveM7.4:M | Yes, No, N/A |
| mmWaveM7.4.2.2 | Support for four transmit queues with a separate channel access entity associated with each | 9.1.3.1 (HCF contention-based channel access (EDCA)), 9.9.1.1 (Reference implementation) | mmWaveM7.4.2.1:O | Yes, No, N/A |
| mmWaveM7.4.2.3 | AC_BE access category differentiated channel access | 9.9.1.2 (EDCA TXOPs), 9.9.1.3 (Obtaining an EDCA TXOP), 9.9.1.5 (EDCA backoff procedure) | mmWaveM7.4.2.1:M | Yes, No, N/A |
| mmWaveM7.5 | Pseudo-static allocation |  |  |  |
| mmWaveM7.5.1 | Scheduling of pseudo-static allocation | 9.23.6.1, 7.3.2.95, 7.3.2.97 (Extended mmWave TSPEC element), 9.23.6.4 (Guard time), 11.4.11.1 (Isochronous TS support) | mmWaveM7.3.1:O | Yes, No, N/A |
| mmWaveM7.5.2 | Operation within pseudo-static allocation | 9.23.6.1, 9.23.6.3 (Pseudo-static allocations) 7.3.2.95, 7.3.2.97 (Extended mmWave TSPEC element) | CF17 AND (CF1 OR CF2.1 OR CF2.3.1 OR CF2.3.2):M | Yes, No, N/A |
| mmWaveM7.6 | Guard time | 9.23.6.4 | mmWaveM7.3.1:M | Yes, No, N/A |
| mmWaveM7.7 | mmWave protected period |  |  |  |
| *mmWaveM7.7.1 | Establishment of mmWave protected period with RTS at Source STA | 9.23.6.5 | mmWaveM7.3.2:O | Yes, No, N/A |
| *mmWaveM7.7.2 | Acceptance to establish mmWave protected period with mmWaveCTS at Destination STA | 9.23.6.5 | mmWaveM7.3.2:M | Yes, No, N/A |
| mmWaveM7.7.3 | Transmission of mmWaveDTS at Destination STA | 9.23.6.5 | mmWaveM7.7.2:O | Yes, No, N/A |
| mmWaveM7.7.4 | Reception of mmWaveDTS at Source STA | 9.23.6.5 | mmWaveM7.7.1:M | Yes, No, N/A |
| mmWaveM7.8 | Service period recovery | 9.23.6.6 | mmWaveM7.3.1&(CF1 OR CF2.3.1):O | Yes, No, N/A |
| mmWaveM7.9 | Dynamic allocation of service period | 9.23.7 |  |  |
| mmWaveM7.9.1 | Polling period (PP) | 9.23.7.1, 7.3.2.96 |  |  |
| mmWaveM7.9.1.1 | Transmission of Poll | 7.2.1.10 | CF17 AND (CF1 OR CF2.3.1):O | Yes, No, N/A |
| mmWaveM7.9.1.2 | Reception of Poll | 7.2.1.10 | CF17 AND (CF2.1 OR CF2.3.2):M | Yes, No, N/A |
| mmWaveM7.9.1.3 | Transmission of SPR | 7.2.1.11 | CF17 AND (CF2.1 OR CF2.3.2):M | Yes, No, N/A |
| mmWaveM7.9.1.4 | Reception of SPR | 7.2.1.11 | CF17 AND (CF1 OR CF2.3.1):M | Yes, No, N/A |
| mmWaveM7.9.2 | Grant period (GP) | 9.23.7.2 |  |  |
| mmWaveM7.9.2.1 | Transmission of Grant | 7.2.1.12 | CF17 AND (CF1 OR CF2.3.1):O | Yes, No, N/A |
| mmWaveM7.9.2.2 | Reception of Grant | 7.2.1.12 | CF17 AND (CF2.1 OR CF2.3.2):M | Yes, No, N/A |
| mmWaveM7.10 | Dynamic truncation of service period | 9.23.8 |  |  |
| mmWaveM7.10.1 | Transmission of mmWaveCF-End | 7.2.1.15 | CF17:O | Yes, No, N/A |
| mmWaveM7.10.2 | Reception of mmWaveCF-End | 7.2.1.15 | CF17:M | Yes, No, N/A |
| mmWaveM7.11 | Dynamic extension of service period | 9.23.9 |  |  |
| mmWaveM7.11.1 | Transmission of SPR | 7.2.1.11 | CF17 AND (CF2.1 OR CF2.3.2):O | Yes, No, N/A |
| mmWaveM7.11.2 | Reception of SPR | 7.2.1.11 | CF17 AND (CF1 OR CF2.3.1):M | Yes, No, N/A |
| mmWaveM7.11.3 | Transmission of Grant | 7.2.1.12 | CF17 AND (CF1 OR CF2.3.1):O | Yes, No, N/A |
| mmWaveM7.11.4 | Reception of Grant | 7.2.1.12 | CF17 AND (CF2.1 OR CF2.3.2):M | Yes, No, N/A |
| mmWaveM7.12 | Isochronous and Asynchronous TS support |  |  |  |
| mmWaveM7.12.1 | Isochronous operation | 11.4.11.1, 7.3.2.95, 7.3.2.97 (Extended mmWave TSPEC element) | mmWaveM7.3.1 AND (CF1 OR CF2.1 OR CF2.3.1 OR CF2.3.2):M | Yes, No, N/A |
| mmWaveM7.12.2 | Asynchronous operation | 11.4.11.2, 7.3.2.95, 7.3.2.97 (Extended mmWave TSPEC element), 7.2.1.11(Service period request) | mmWaveM7.3.1 AND (CF1 OR CF2.1 OR CF2.3.1 OR CF2.3.2):M | Yes, No, N/A |
| mmWaveM8 | PCP/AP clustering | 9.24 | CF17:O | Yes, No, N/A |
| mmWaveM9 | mmWave Beamforming |  |  |  |
| mmWaveM9.1 | Sector level sweep | 9.25.1, 9.25.5.1 | CF17:M | Yes, No, N/A |
| mmWaveM9.2 | Beamforming in BT | 9.25.3 | CF17:M | Yes, No, N/A |
| mmWaveM9.3 | Beamforming in A-BFT | 9.25.4 | CF17:M | Yes, No, N/A |
| mmWaveM9.4 | BRP setup | 9.25.2.1 | CF17:M | Yes, No, N/A |
| mmWaveM9.5 | MID | 9.25.5.2.2 | CF17:O | Yes, No, N/A |
| mmWaveM9.6 | BC | 9.25.5.2.3 | CF17:O | Yes, No, N/A |
| mmWaveM9.7 | BRP |  |  |  |
| *mmWaveM9.7.1 | BRP with BS-FBCK | 9.25.2, 9.25.5.3 | CF17:M | Yes, No, N/A |
| mmWaveM9.7.2 | BRP with channel measurement | 9.25.2, 9.25.5.3 | mmWaveM9.7.1:O | Yes, No, N/A |
| mmWaveM9.8 | Beam tracking | 9.25.6 | CF17:M | Yes, No, N/A |
| mmWaveM10 | mmWave Block ACK with flow control | 9.26, 7.3.2.91.1 (BA with flow control) | CF17:O | Yes, No, N/A |
| mmWaveM11 | mmWave link adaptation | 9.27 | CF17:O | Yes, No, N/A |
| mmWaveM12 | mmWave dynamic tone pairing (DTP) | 9.28, 7.3.2.91.1 (DTP Supported) | CF17:O | Yes, No, N/A |
| mmWaveM13 | Timing synchronization function (TSF) in a PBSS |  |  |  |
| mmWaveM13.1 | Timing in a PBSS network | 11.1.1.1 (TSF for infrastructure Networks and PBSS Networks), 11.1.4 (Adjusting STA timers) | CF2.3.1&CF17:M | Yes, No, N/A |
| mmWaveM13.2 | PBSS initialization | 11.1.3 (Acquiring synchronization, scanning) | CF2.3.1&CF17:M | Yes, No, N/A |
| mmWaveM14 | Power management |  |  |  |
| mmWaveM14.1 | STA power management without wakeup schedule | 11.2.3.1.1, 11.2.3.1.3 | CF17:M | Yes, No, N/A |
| mmWaveM14.2 | STA power management with wakeup schedule | 11.2.3.1.2, 11.2.3.1.3, 7.3.2.94 | CF17:O | Yes, No, N/A |
| mmWaveM14.3 | PCP power management | 11.2.3.2, 7.3.2.94 | CF17:O | Yes, No, N/A |
| mmWaveM15 | Authentication and association |  |  |  |
| mmWaveM15.1 | Association state | 11.3.3 | M | Yes, No |
| mmWaveM15.2 | STA association procedure | 11.3.3.1 | CF2.1 & CF17:M CF2.3.2 & CF17:O | Yes, No, N/A |
| mmWaveM15.3 | PCP/AP association procedure | 11.3.3.2 | CF1 & CF17:M CF2.3.1 & CF17:O | Yes, No, N/A |
| mmWaveM15.4 | Communicating PBSS information | 11.3.4 | (CF2.3.1 OR CF2.3.2) & CF17:M | Yes, No, N/A |
| mmWaveM16 | mmWave beamformed link and BSS maintenance |  |  |  |
| mmWaveM16.1 | Beamformed link maintenance | 11.30.1 | CF17:M | Yes, No, N/A |
| mmWaveM16.2 | PCP handover | 11.30.2 | CF17:O | Yes, No, N/A |
| mmWaveM17 | mmWave BSS Peer and Service Discovery | 11.31 | CF17:O | Yes, No, N/A |
| mmWaveM18 | Changing mmWave BSS parameters | 11.32 | CF17:O | Yes, No, N/A |
| *mmWaveM19 | Spatial sharing and interference mitigation | 11.33 | CF17:O | Yes, No, N/A |
| mmWaveM20 | mmWave coexistence with other mmWave technologies | 11.35 | CF17:O | Yes, No, N/A |
| mmWaveM21 | Traffic specification (TSPEC and Extended mmWave TSPEC) and associated frame formats | 7.4.2 (QoS Action frame details) | CF17:M | Yes, No, N/A |
| mmWaveM22 | mmWave frame formats | 7.4.13 (mmWave Action frame details) | CF17:M | Yes, No, N/A |

#### A.4.21.2 mmWave PHY features

| Item | Protocol capability | References | Status | Support |
| --- | --- | --- | --- | --- |
|  | Are the following PHY protocol features supported? |  |  |  |
| mmWaveP1 | PHY operating modes |  |  |  |
| mmWaveP1.1 | Operation according to clause 21 | 21 | CF17:M | Yes, No, N/A |
| mmWaveP2 | PLCP frame format |  |  |  |
| *mmWaveP2.1 | Control PHY PLCP format | 21.4 | CF17:M | Yes, No, N/A |
| *mmWaveP2.2 | SC PHY PLCP format | 21.6 | CF17:M | Yes, No, N/A |
| *mmWaveP2.3 | OFDM PHY PLCP format | 21.5 | CF17:O | Yes, No, N/A |
| *mmWaveP2.4 | Low power SC PHY PLCP format | 21.7 | CF17:O | Yes, No, N/A |
| mmWaveP2.5 | Modulation and coding schemes (MCS) |  |  |  |
| mmWaveP2.5.1 | MCS 0 of Control PHY |  | mmWaveP2.1:M | Yes, No, N/A |
| mmWaveP2.5.2 | MCS 1-12 of SC PHY |  |  |  |
| mmWaveP2.5.2.1 | MCS 1-4 |  | mmWaveP2.2:M | Yes, No, N/A |
| mmWaveP2.5.2.2 | MCS 5-12 |  | mmWaveP2.2:O | Yes, No, N/A |
| mmWaveP2.5.3 | MCS 13-24 of OFDM PHY |  |  |  |
| mmWaveP2.5.3.1 | MCS 13-17 |  | mmWaveP2.3:M | Yes, No, N/A |
| mmWaveP2.5.3.2 | MCS 18-24 |  | mmWaveP2.3:O | Yes, No, N/A |
| mmWaveP2.5.4 | MCS 25-27 of Low Power SC PHY |  | mmWaveP2.4:M | Yes, No, N/A |
| mmWaveP2.6 | Common preamble format | 21.3.6 | CF17:M | Yes, No, N/A |
| mmWaveP2.7 | Use of LDPC codes | 21.3.8 | CF17:M | Yes, No, N/A |