IEEE P802.11
Wireless LANs

| Normative Text for White Space Map Notification |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-09-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Eunsun Kim | LG Electronics | Mobile Comm. Lab, LG R&D Complex 533, Hogye1, Dongan, Anyang, Korea | +82-31-450-1860 | esun.kim@lge.com |
| Stephen McCann | Research in Motion (RIM) UK Ltd | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@rim.com |
| Yongho Seok | LG Electronics | Mobile Comm. Lab, LG R&D Complex 533, Hogye1, Dongan, Anyang, Korea | +82-31-450-1947 | yongho.seok@lge.com |

Abstract

This document is a normative text proposal for a white space map notification mechanism, so that the TVWS Database can push updated information to devices.

The mechanism is similar to that of the Emergency Alerts defined in IEEE 802.11u.

The text is aligned with P802.11af-D0.05.

Introduction

- Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGaf Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGaf Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGaf Editor:  Editing instructions preceded by “TGaf Editor” are instructions to the TGaf editor to modify existing material in the TGaf draft.   As a result of adopting the changes, the TGaf editor will execute the instructions rather than copy them to the TGaf Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

Discussion:

The TG approved the use of Registered Location Server and RLQP (Registered Location Query Protocol) as a mean to communicate with external database. In this document, WSM request/response method using RLQP  is proposed. TVWS database can push update information(notfication message)  to devicesand the device having the notification should query WSM and retrive the available channel list using RLQP protocol.

Proposed Resolution:

Approve editing instructions in document 10/1040r0.

## Editing instructions:

- 8.3.3.2 Beacon frame format

TGaf editor: Change the following row in Table 8-19:

| Order | Information | Notes |
| --- | --- | --- |
|  | WSM Notification | The WSM Notification information element may be present if dot11WSMNotificationEnabled is true. |

- 8.3.3.9 Probe Response frame format

TGaf editor: Change the following row in Table 8-25:

| Order | Information | Notes |
| --- | --- | --- |
|  | WSM Notification | The WSM Notification information element may be present if dot11WSMNotificationEnabled is true. |

- 8.4.2 Information Elements

TGaf editor: Change the following row in table 8-50, and renumber the reserved values accordingly:

Table 8-50—Element IDs

| Information element | Element ID | Length (in octets) | Extensible |
| --- | --- | --- | --- |
| WSM Notification (see 8.4.2.af5) | < ANA > | variable | Yes |

TGaf editor: Insert the following new sub clause 8.4.2.af5, as shown:

- 8.4.2.af5 White Space Map Notification element

The White Space Map (WSM) Notification element provides a hash to indicate that the WSM has been updated within the network. The hash allows the dependent STA to assess whether a WSM Notification element advertised by an enabling STA has been previously received and therefore whether it is necessary to download from the network. The format of the WSM Notification element is provided in Figure 8-32f6.

|  | Element ID | Length | WSM Notification Hash |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 8 |

Figure 8-32f6—WSM Notification element format

The Element ID field is equal to the WSM Notification value in Table 8-50.

The Length is a 1-octet field whose value is equal to 8.

The WSM Notification Hash (WSNH) is an 8-octet field. It is a unique value used to indicate an instance of a White Space Map notification. The value of this field is the hash produced by the HMAC-SHA1-64 hash algorithm operating on the White Space Map notification.

WSNH =HMAC-SHA1-64(“WSN”, WSM_Information)

Where WSNH is then truncated to the first 64 bits of this function.

WSM_ Information is the WSM Information field (Figure 8-35af4).

8.4.5 Registered Location Query Protocol elements

TGaf Editor: Change the Table 8-35af1, as shown:

Table 8-35af1 – Registered Location Query Protocol info ID definitions

| Info Name | Info ID | RLQP Info element (clause) |
| --- | --- | --- |
| Reserved | 0 | N/A |
| DSE Enablement | 1 | 7 8.4.5.1 |
| Channel Power Management | 2 | 8.4.5.2 |
| White Space Map | 3 | 8.4.5.3 |
| Reserved | 3 4 – 220 | N/A |
| Vendor Specific | 221 | 8.4.2.28 |
| Reserved | 222 – 255 | N/A |

TGaf Editor: Insert the following new subclause after 8.3.5.2, as shown:

8.4.5.3 White Space Map

The White Space Map of RLQP is used to request/respond to a WSM Notification using the GAS protocol rather than using dedicated Public Action frames. The element is in the format shown in Figure 8-35af4 (White Space Map).

|  | Info ID | Length | WSM Type | WSM Information |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure 8-35af4 White Space Map format

The Info ID field is set to the value for White Space Map defined in Table 8-35af1.

The Length field indicates the length of the remaining element fields in octets, and the value is variable.  The minimum value of the Length field is 1.

The remaining fields are as described in the White Space Map element (see section 8.4.2.af1).

8.5 Action frame format details

8.5.8 Public Action frame details

8.5.8.1 Public Action frames

Insert the following rows in Table 8-131:

Table 8-131 Public Action field values

| Action field value | Description |
| --- | --- |
| <ANA> | WSM Notification |

TGaf Editor: Insert the following new sub clause after 8.5.8.af3, as shown:

8.5.8.af4 WSM Notification frame format

The WSM Notification frame is a Public Action frame that is transmitted by an enabling STA to notify that the available channel information from the regulatory database has been updated. The format of the WSM Notification frame body is shown in Figure 8-47af4.

|  | Category | Public Action | Length | WSM Notification Hash |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 8 |

Figure 8-47f4—WSM Notification frame body format

The Category field is set to the value for public action defined in Table 8-36 (Category values).

The Action Value field is set to indicate a WSM Notification frame, as defined in

Table 8-131 (Public Action field values).

The remaining fields are as described in the WSM Notification element (see 8.4.2.af5).

TGaf editor: Change the sub clause 10.af2 as follows:

- 10.af2 Regulatory Domain Operation

10.af2.1 General

TGaf editor: Change 2nd paragraph in a sub clause 10.af2.1, as follows:

Enablement procedures are described in 10.12. Upon the reception of the enabling signal, the dependent STAs shall receive the WSM element in order to operate in the specific regulatory domain, which indicates the available channel list in that regulatory domain. After transmission of a DSE Enablement frame (8.5.8.4) granting a dependent STA, the enabling STA shall transmit a WSM. An enabling STA transmits a WSM within the Beacon frame, Probe Response frame and White Space Map Announcement frame. A WSM can be transmitted within a Beacon frame, Probe Response frame, WSM RLQP element using GAS protocol and White Space Map Announcement frame.

TGaf editor: Change the sub clause 10.af2.3 and insert text, as follows:

10.af2.3 Multi-band Operation(472r1) White Space Map Update Procedures

<TBD>

Whenever the available channel information from the regulatory domain database is updated, the enabling STA shall retrieve the information and transmit the updated channel information to its dependent STAs that the enabling STA has enabled. In general, an enabling STA controls several dependent STAs including dependent APs. Available channels are decided from the database depending on the enabling STA and/or dependent AP’s geo-location information.

When an enabling STA is informed of update of channel information from the geo-location database (the mechanism by which is out of scope of this standard), it shall advertise the notification message that the channel information including maximum power limit has changed by including a WSM Notification element (8.4.2.af5) in its Beacon, Probe Response and White Space Identifier Public Action frames.

The WSM Notification element (8.4.2.af5) notifies that the channel information from the database has changed. Whenever an enabling STA become aware of an update of channel information including powers from the database it shall transmit the notification message. An enabling STA with dot11WSMNotificationEnabled set to true issues the WSM Notification within Probe Response frame, Beacon frame and Public Action frame. The WSM Notification element provides a WSM Notification Hash (WSNH), a unique indicator of the notification to the dependent STA. The WSNH allows the dependent STA to determine whether this is a new notification.

The WSNH in the WSM Notification element shall be computed using HMAC-SHA1-64 hash algorithm as shown in 8.4.2.af5.

After receiving a WSNH for an update of WSM which has not already been retrieved from the enabling STA, a dependent STA having dot11WSMNotificationEnabled set to true can retrieve the updated WSM from the enabling STA using the White Space Map RLQP  element using GAS.

Whenever a dependent STA wants to have a White Space Map, it can transmit the White Space Map RLQP element using GAS protocol and gets a valid White Space Map from its enabling STA.

An Enabling STA that receives White Space Map request element using GAS protocol, shall transmit the valid White Space Map RLQP element using GAS protocol.

TGaf editor: Insert the following new row in B.4.af:

### B.4.af TVWS functions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| *WS1 | Fixed STA TVWS Operation | 10.12.3, Annex D, Annex E.2.4 | CFaf:O | Yes, No. N/A |
| *WS2 | Master STA TVWS Operation | 10.12.3, 10.12.4, Annex D, Annex E.2.4 | CFaf:O | Yes, No. N/A |
| *WS3 | Client STA TVWS Operation | 10.12.5, Annex D, Annex E.2.4 | CFaf:O | Yes, No. N/A |
| WS3.1 | Dependent STA TVWS Behavior | 10.12.5, Annex D, Annex E.2.4 | WS3:M | Yes, No. N/A |
| WS4 | White Space Map Announcement | 8.4.2.af1, 8.5.8.af1, 10.af2.2 | CFaf: M | Yes, No. N/A |
| WS5 | Multi-band Operation | 8.4.2.af2, 10.af2.3 | CFaf:O | Yes, No. N/A |
| WS6 | Channel Power Management Announcement | 8.4.2.af1, 8.5.8.af2, 10.af1, Annex E.2.4 | CFaf:M | Yes, No. N/A |
| WS7 | White Space Map Notification | 8.4.2.af5, 8.4.5.3, 8.5.8.af4, 10.af2.2 | CFaf:M | Yes, No. N/A |

Annex E (normative) Country elements and regulatory classes

E.2 Band-specific operating requirements

E.2.4 TVWS band in the United States (54 MHz to 698 MHz)

TGaf editor: Insert one dashed item in the seventh paragraph as follows:

STAs shall have the following elements set to “true”

— dot11LCIDSERequired,

— dot11OperatingClassesRequired,

— dot11SpectrumManagementRequired,

— dot11MultiDomainCapabilityActivated,

— dot11ChannelPowerManagementActivated. ,

— dot11WSMNotificationEnabled.

Annex C

(normative)

ASN.1 encoding of the MAC and PHY MIB

TGaf editor: Insert a new MIB variable dot11WSMNotificationEnabled after dot11WhiteSpaceMapEnabled , as shown :

Dot11StationConfigEntry::=

SEQUENCE {

dot11TVWSMapEnabled TruthValue,(472r1)

dot11TVWSMultiBandOperationEnabled TruthValue,(472r1)

dot11TVWSMapPeriod Unsigned32,(472r1)

dot11TVWSMapValidTime Integer, (472r1)

dot11RLSImplemented TruthValue, (737r3)

dot11RLSActivated TruthValue,(737r3)

dot11WhiteSpaceMapEnabled TruthValue,(790r2)

dot11WSMNotificationEnabled TruthValue,

dot11WhiteSpaceMapPeriod Unsigned32,(790r2)

dot11WhiteSpaceMapValidTime Integer, (790r2)

dot11ChannelPowerManagementActivated TruthValue (767r1)

}

TGaf editor: add definition of  dot11WSMNotificationEnabled after definition of dot11WhiteSpaceMapEnabled , as shown :

dot11WSMNotificationEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity.

Changes take effect for the next MLME-START.request

primitive.

This attribute, when true, indicates that the system capability for White Space Map Notifications is enabled. False indicates that the station has no White Space Map Notification cor that the capability is present but is disabled.”

DEFVAL { FALSE }

::= { dot11StationConfigEntry <ANA> }