IEEE P802.11
Wireless LANs

| Channel list request/response for multiple geo-locations |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-11-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Yohannes D. Alemseged | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | yohannes@nict.go.jp |
| Chen Sun | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | sun@nict.go.jp |
| Zhou Lan | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | lan@nict.go.jp |
| Ha Nguyen Tran | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | haguen@nict.go.jp |
| Chin Sean Sum | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | sum@nict.go.jp |
| Junyi Wang | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | Junyi.wang@nict.go.jp |
| Tuncer Baykas | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | tuncerbaykas@nict.go.jp |
| Hiroshi Harada | NICT | 3-4, Hikarino-oka, Yokosuka, Kanagawa, Japan, 239-0847 |  | harada@nict.go.jp |

Abstract

This submission proposes to include the multiple geo-location channel list request/response capability by Mode II devices as part of comment resolution for comment 08 in document 595r14. According to the FCC 10-174, such capability is intended to support operating in a bounded geographic area while reducing the number of database query for available channel list.  This proposal is intended to be included in the IEEE 802.11af D0.05 as part of a reflection from the new FCC rule.

Comment

| 8 | 8.19 | 11.af1.1 | It is required by the FCC that a TV Band Device requests and receives a list of available TV channels from an authorized database using its geolocation coordinates. The statement must be normative. | Change the text to read: "An enabling STA shall be capable of identifying its geographic location and shall access a TV bands database system." |
| --- | --- | --- | --- | --- |

Resolution: Agree in principle. The suggested resolution is however not enough is as it doesn’t capture the possible capability of Mode II devices to obtain available TV channels from authorized database using not only its geolocation but also multiple geolocations in the vicinity of the current geolocation.  In addition to the suggested resolution we proposed as indicated in the editor instruction of this document.

Discussion:

The following paragraphs are excerpts from the FCC 10-174 document on the matter of multiple geo-location channel lists loading.

A Mode II personal/portable device may load channel availability information for multiple locations around, i.e., in the vicinity of, its current location and use that information in its operation. A Mode II TVBD may use such available channel information to define a geographic area within which it can operate on the same available channels at all locations, for example a Mode II TVBD could calculate a bounded area in which a channel or channels are available at all locations within the area and operate on a mobile basis within that area. A Mode II TVBD using such channel availability information for multiple locations must contact the database again if/when it moves beyond the boundary of the area where the channel availability data is valid, and must access the database daily even if it has not moved beyond that range.  Appendix B Final Rule, Page 75

The current 802.11af D 0.06 should take the advantage of the newly added capability for Mode II personal portable devices for better and efficient operation during mobility. The modification we introduce is just to add a new information element, where the current geo-location and the vicinity- geo-location and the WSM associated with each geo-location.  The vicinity geo-location can be represented by a set of geo-location points around the current geo-location. However sending these geo-location positions as separate location information may create unnecessary overhead. To circumvent this problem we use the LCI altitude and longitude resolution bits to indicate the range in which the channel lists are applicable to create a geographic bound in which the Mode II device can operate without querying the database once it obtains the valid channel list.

3.3 Abbreviations and acronyms

Insert the following new acronyms in alphabetical order

MLCL	Multiple Locations Channel List

6. Layer Management

6.3 MLME SAP Interface

TGaf Editor: Insert following new clause after the last sentence of subclause 6.3.af3:

6.3.af4 Multiple Locations Channel list management

The following MLME primitives support the signaling of multiple locations channel list (MLCL) management.

6.3.af4.1 MLME- MLCLManagement.request

6.3.af4.1.1 Function

This primitive requests that a (Protected) MLCL Management frame be sent by a STA.

6.3.af4.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME- MLCLManagement.request (

Multiple Locations Channel List Management

Protected,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Multiple Locations Channel List management | A set of information subfields | As described in 8.4.2.af4 | As described in 8.4.2.af4 |
| Protected | Boolean | true, false | Specifies whether the request is sent using a Robust Management frame. If true, the request is sent using the Protected Multiple locations Channel List Management frame. If false, the request is sent using the Multiple Locations Channel List Management Announcement frame. |
| VendorSpecificInfo | A set of elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.af4.1.3 When generated

This primitive is generated by the STA management entity (SME) to request that a (Protected) Multiple Locations Channel List Management frame be sent by a STA.

6.3.af4.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs and transmits a (Protected) Multiple Location Channel List Management frame.

6.3.af4.2 MLME- MLCLManagement.confirm

6.3.af4.2.1 Function

This primitive reports the result of a request to Multiple Locations Channel List management.

6.3.af4.2.1 Semantics of the service primitive

The primitive parameters as follows:

MLME- MLCLManagement.confirm (

ResultCode,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_PARAMETERS, UNSPECIFIED_FAILURE | Reports the result of a Multiple Locations Channel List management request |
| VendorSpecificInfo | A set of elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.af4.2.2 When generated

This primitive is generated by the MLME when a Multiple Locations Channel List management request completes.  Possible unspecified failure causes include an inability to provide the Multiple Locations Channel List information.

6.3.af4.2.3 Effect of receipt

The SME is notified of the results of the Multiple Locations Channel List management procedure.

6.3.af4.3 MLME-MLCLManagement.indication

6.3.af4.3.1  Function

This primitive indicates that a (Protected) Multiple Locations Channel list management frame was received.

6.3.af4.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME- MLCLManagement.indication (

PeerMACAddress,

Multiple Locations Channel List Management

Protection,

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| PeerMACAddress | MACAddress | Any valid individual MAC address | The address of the peer MAC entity from which the Multiple Locations Channel List Management frame was received. |
| Multiple Locations Channel List Management | A set of information subfields | As described in 8.4.2.af4 | As described in 8.4.2.af4 |
| Protected | Boolean | true, false | Specifies whether the request was received using a Robust Management frame. If true, the request was received using the Protected Multiple Locations Channel List Management frame. If false, the request was received using the Multiple Locations Channel List Management frame. |
| VendorSpecificInfo | A set of elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.af5.3.3 When generated

This primitive is generated by the MLME when a valid (Protected) Multiple Locations channel list management frame is received.

6.3.af5.3.4 Effect of receipt

On receipt of this primitive, the SME decides whether to provide the Multiple Locations channel list information.

6.3.af5.4 MLME-MLCLManagement.response

6.3.af5.4.1  Function

This primitive is used to provide Multiple Locations channel list information on channel availability.

6.3.af5.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME- MLCLMManagement.response (

Multiple Locations Channel List Management

VendorSpecificInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Multiple Locations Channel List Management | A set of information subfields | As described in 8.4.2.af4 | As described in 8.4.2.af4 |
| VendorSpecificInfo | A set of elements | As defined in 8.4.2.28 (Vendor Specific element) | Zero or more elements. |

6.3.af5.4.3 When generated

This primitive is generated by the SME to provide the Multiple Locations Channel List information.

6.3.af5.4.4 Effect of receipt

On receipt of this primitive, the MLME provides Multiple Locations Channel List information on channel availability.

8.4 Management frame body components

8.4.2 Information elements

8.5.2.1 General

TGaf Editor: Insert following rows in Table 8-50 Element Ids in the correct position to preserve ordering by the “Element ID” column:

Table 8-50 – Element IDs

| Information Element | Element ID | Length (in octets) | Extensible |
| --- | --- | --- | --- |
| Multiple Locations Channel List Management (see 8.4.2.af4) | <ANA> | 3 to 255 |  |

8.5.2.29 Extended capabilities element

TGaf Editor: Add the following rows to Table 8-88 and renumber the reserved values accordingly:

Table 8-88 – Capabilities field

| Bit | Information | Notes |
| --- | --- | --- |
| <ANA> | Multiple Locations Channel List Management | The STA sets the Multiple Locations Channel List Management field to 1 when the MIB attribute dot11MLCLManagementActivated is true, and set it to zero otherwise, see 8.4.2.af4. |

TGaf Editor: Insert following new clause after the last sentence of subclause 8.4.2.af3:

8.4.2.af5 Multiple Locations Channel List Management information element

The Multiple Locations Channel List Management information element shall have the following format.

|  | Element ID | Length | Current Geo-Location | WSM1 | Vicinity Geo-Location | WSM2 |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 16 | variable | 16 | variable |

Figure 8-42af5 – Multiple Locations Channel List Management Information element format

The Length field indicates the length of the remaining fields in octets, and the value is variable depending on the WSM1 field length and WSM2 field length.  The minimum value of the length field is 4 assuming that the minimum length of WSM is 1 as specified in 8.4.2.af1.

Current Geo-Location: refers to the geo-location of the STA represented by LCI with in accuracy of +-50 meter as specified in the resolution bits of the LCI.

WSM1: refers to the available channel list valid for the current geo-location specified. When the Multiple Locations Channel List Management information element is generated by the requesting STA, WSM1 indicates no available channel list information.  When the Multiple Locations Channel List Management information element is generated by the responding STA or television white space database, WSM1 indicates the valid available channel list.

Vicinity Geo-Location: refers to the geo-location of the STA around the current geo-location of the STA. It is represented by LCI with in accuracy of  less  than +-50 meter (for example +-200 meter) as specified in the resolution bits of the LCI. This is to indicate that the channel list requested for this location should be valid for the geographic bound defined by contents of the LCI including the LCI resolution.

WSM2: refers to the available channel list valid for the vicinity geo-location specified. When the Multiple Locations Channel List Management information element is generated by the requesting STA, WSM2 indicates no available channel list information.  When the Multiple Locations Channel List Management information element is generated by the responding STA or television white space database, WSM2 indicates the valid available channel list.

8.4.5 Registered Location Query Protocol elements

TGaf Editor: Modify and add the following rows to Table 8-45af1 and renumber the reserved values accordingly:

Table 8-45af1 – Registered Location Query Protocol info ID definitions

| Info Name | Info ID | RLQP Info element (clause) |
| --- | --- | --- |
| MLCL Management | 7 | 8.4.5.6 |
| Reserved | 68 – 220 | N/A |
| Vendor Specific | 221 | 8.4.2.28 |
| Reserved | 222 – 255 | N/A |

TGaf Editor: Insert the following new subclause after 8.4.5.5, as shown:

8.4.5.6 Multiple Locations Channel List Management information element

Multiple Locations Channel List Management information element is used by.

|  | Info ID | Length | Requestor STA address | Requestor STA address | Current Geo-Location | WSM1 |  | Vicinity Geo-Location | WSM2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 16 | variable |  | 16 | variable |

Figure 8-456 – Multiple Locations Channel List Management Information element format

The Info ID field is set to the value for Multiple Locations Channel List Management defined in Table 8-45af1.

The Length field indicates the length of the remaining fields in octets, and the value is variable depending on the WSM1 field length and WSM2 field length.  The minimum value of the length field is 4 assuming that the minimum length of WSM is 1 as specified in 8.4.2.af1.

The Requester STA Address field is the MAC address of the requesting STA that initiates the Multiple Locations Channel List Management process.

The Responder STA Address field is the MAC address of the responding STA that grants Multiple Locations Channel List Management process.

Current Geo-Location: refers to the geo-location of the STA represented by LCI with in accuracy of +-50 meter as specified in the resolution bits of the LCI.

WSM1: refers to the available channel list valid for the current geo-location specified. When the Multiple Locations Channel List Management information element is generated by the requesting STA, WSM1 indicates no available channel list information.  When the Multiple Locations Channel List Management information element is generated by the responding STA or television white space database, WSM1 indicates the valid available channel list.

Vicinity Geo-Location: refers to the geo-location of the STA around the current geo-location of the STA. It is represented by LCI with in accuracy of  less  than +-50 meter (for example +-200 meter) as specified in the resolution bits of the LCI. This is to indicate that the channel list requested for this location should be valid for the geographic bound defined by contents of the LCI including the LCI resolution.

WSM2: refers to the available channel list valid for the vicinity geo-location specified. When the Multiple Locations Channel List Management information element is generated by the requesting STA, WSM2 indicates no available channel list information.  When the Multiple Locations Channel List Management information element is generated by the responding STA or television white space database, WSM2 indicates the valid available channel list.

8.5 Action frame format details

8.5.8 Public Action frame details

8.5.8.1 Public Action frames

TGaf Editor: Insert the following rows in Table 8-131:

Table 8-131 – Public Action field values

| Action field value | Description |
| --- | --- |
| <ANA> | Multiple Locations Channel List Management |

8.5.11 Protected Dual of Public Action frames

8.5.8.1 Protected Dual of Public Action details

TGaf Editor: Insert the following rows in Table 8-143:

Table 8-143 – Protected Dual of Public Action field values

| Action field value | Description |
| --- | --- |
| <ANA> | Multiple Locations Channel List Management |

TGaf Editor: Before clause 9, insert text as shown:

Insert the following new clause after the last subclause of 8.5.8.af6:

8.5.8.af7 Multiple Locations Channel List Management frame format

The Multiple Locations Channel List Management frame is used by.

|  | Category | Action Value | Requestor STA address | Requestor STA address | Length | Current Geo-Location | WSM1 | Vicinity Geo-Location | WSM2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 16 | variable | 16 | variable |

Figure 8-58af7 – Multiple Locations Channel List Management frame format

The Category field is set to the value for public action defined in Table 8-36 (Category values).

The Action Value field is set to indicate a Multiple Locations Channel List Management frame, as defined in Table 8-131 (Public Action field values) in section 8.5.8.1.

The remaining fields are as described in the Multiple Locations Channel List Management information element (see 8.3.2.af4 (Multiple Locations Channel List Management information element).

Insert the following new clause after the last subclause of 8.5.11.7af4:

8.5.11.7af5 Protected Multiple Locations Channel List Management frame format

The Protected Multiple Locations Channel List Management frame format is the same as the Multiple Locations Channel List Management frame format (see 8.5.8.af7).  It is used instead of the Multiple Locations Channel List Management frame when Management Frame Protection is negotiated.

10. MLME

TGaf Editor: Insert the following new subclause as 10.af2, as shown:

10.af2 Multiple Locations Channel List Management

An enabling STA with dot11MLCLManagementActivated set true shall use Multiple Locations Channel List Management RLQP element to query the database for available channel lists valid for the current geo-location position and vicinity geo-location position. The current geo-location is specified by +-50meter accuracy at most in the LCI resolution bits. The vicinity geo-location is specified by a lower resolution in the LCI resolution bits to increase the coverage area.  The STA shall define a geographic bound based on the information of WSM1 and WSM2 in which it can operate without querying the database as long as operating confined to the defined geographic bound.

A dependent STA with dot11MLCLManagementActivated set true may use RLQP to query the enabling STA that enabled it for the Multiple locations Channel List information in the same way that enabling STA query database for Multiple Locations Channel List information. The information element contained in RLQP is defined in section 8.4.5.6.

A dependent STA with dot11MLCLManagementActivated set true may also use Multiple Locations Channel List Management Public Action frame to query the enabling STA that enabled it for Multiple Channel List information. The frame shall be formatted as described in section 8.5.8.af7.

An enabling STA with dot11MLCLManagementActivated set true may send Multiple Locations Channel List Management information element as described in 8.4.2.af4 in Beacon, Probe response frame, or Public Action frame to update the dependent STAs that are enabled the for Multiple Locations Channel List management.