IEEE P802.11
Wireless LANs

| D0.1 minor changes in PHY spec |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-12-6 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Amichai Sanderovich | Wilocity Ltd. |  |  | Amichai.Sanderovich@wilocity.com |

Abstract

This document proposes required minor changes to PHY specifications in Draft 1.0 of TGad.

TGad Editor: Change the text in P343L34-39 as follows:

Ga128(n)=A7(128-n), Gb128(n)=B7(128-n) when the procedure uses Dk = [1 8 2 4 16 32 64] (k=1,2,…,7) and Wk = [-1 -1 -1 -1 +1 -1 -1].

Ga64(n)=A6(64-n), Gb64(n)=B6(64-n) when the procedure uses Dk= [2 1 4 8 16 32] and Wk =[1 1 -1 -1 1 -1].

TGad Editor: Change the text in P344L1-2 as follows:

Ga32(n)=A5(32-n), Gb32(n)=B5(32-n) when the procedure uses Dk=[ 1  4  8 2 16] and Wk =[-1 1 -1 1 -1].

TGad Editor: Change the text in P308L4-7 as follows:

The filter  is defined by the following coefficients (from h0 to h70):

[-1,0,1,1,-2,-3,0,5,5,-3,-9,-4,10,14,-1,-20,-16,14,33,9,-35,-42,11,64,40,-50,-96,-15,120,126,-62,-256,-148,360,985,1267,985,360,-148,-256,-62,126,120,-15,-96,-50,40,64,11,-42,-35,9,33,14,-16,-20,-1,14,10,-4,-9,-3,5,5,0,-3,-2,1,1,0,-1]. Normalized to have a norm of  so that .

TGad Editor: Change the text in Table 73 as follows:

The Sequence Ga128(n), to be transmitted from left to right, up to down

+1 +1 -1 -1 -1 -1 -1 -1 -1 +1 -1 +1 +1 -1 -1 +1 +1 +1 -1 -1 +1 +1 +1 +1 -1 +1 -1 +1 -1 +1 +1 -1 -1 -1 +1 +1 +1 +1 +1 +1 +1 -1 +1 -1 -1 +1 +1 -1 +1 +1 -1 -1 +1 +1 +1 +1 -1 +1 -1 +1 -1 +1 +1 -1 +1 +1 -1 -1 -1 -1 -1 -1 -1 +1 -1 +1 +1 -1 -1 +1 +1 +1 -1 -1 +1 +1 +1 +1 -1 +1 -1 +1 -1 +1 +1 -1 +1 +1 -1 -1 -1 -1 -1 -1 -1 +1 -1 +1 +1 -1 -1 +1 -1 -1 +1 +1 -1 -1 -1 -1 +1 -1 +1 -1 +1 -1 -1 +1

TGad Editor: Change the text in Table 74 as follows:

The Sequence Gb128(n), to be transmitted from left to right, up to down

-1 -1 +1 +1 +1 +1 +1 +1 +1 -1 +1 -1 -1 +1 +1 -1 -1 -1 +1 +1 -1 -1 -1 -1 +1 -1 +1 -1 +1 -1 -1 +1 +1 +1 -1 -1 -1 -1 -1 -1 -1 +1 -1 +1 +1 -1 -1 +1 -1 -1 +1 +1 -1 -1 -1 -1 +1 -1 +1 -1 +1 -1 -1 +1 +1 +1 -1 -1 -1 -1 -1 -1 -1 +1 -1 +1 +1 -1 -1 +1 +1 +1 -1 -1 +1 +1 +1 +1 -1 +1 -1 +1 -1 +1 +1 -1 +1 +1 -1 -1 -1 -1 -1 -1 -1 +1 -1 +1 +1 -1 -1 +1 -1 -1 +1 +1 -1 -1 -1 -1 +1 -1 +1 -1 +1 -1 -1 +1

TGad Editor: Change the text in Table 75 as follows:

The Sequence Ga64(n), to be transmitted from left to right, up to down

-1 -1 +1 -1 +1 -1 -1 -1 +1 +1 -1 +1 +1 -1 -1 -1 -1 -1 +1 -1 +1 -1 -1 -1 -1 -1 +1 -1 -1 +1 +1 +1 -1 -1 +1 -1 +1 -1 -1 -1 +1 +1 -1 +1 +1 -1 -1 -1 +1 +1 -1 +1 -1 +1 +1 +1 +1 +1 -1 +1 +1 -1 -1 -1

TGad Editor: Change the text in Table 76 as follows:

The sequence Gb64(n), to be transmitted from left to right, up to down

+1 +1 -1 +1 -1 +1 +1 +1 -1 -1 +1 -1 -1 +1 +1 +1 +1 +1 -1 +1 -1 +1 +1 +1 +1 +1 -1 +1 +1 -1 -1 -1 -1 -1 +1 -1 +1 -1 -1 -1 +1 +1 -1 +1 +1 -1 -1 -1 +1 +1 -1 +1 -1 +1 +1 +1 +1 +1 -1 +1 +1 -1 -1 -1

TGad Editor: Change the text in Table 77 as follows:

The Sequence Ga32(n), to be transmitted from left to right

+1 +1 +1 +1 +1 -1 +1 -1 -1 -1 +1 +1 +1 -1 -1 +1 +1 +1 -1 -1 +1 -1 -1 +1 -1 -1 -1 -1 +1 -1 +1 -1

TGad Editor: Change the text in Table 78 as follows:

The Sequence Gb32(n), to be transmitted from left to right

-1 -1 -1 -1 -1 +1 -1 +1 +1 +1 -1 -1 -1 +1 +1 -1 +1 +1 -1 -1 +1 -1 -1 +1 -1 -1 -1 -1 +1 -1 +1 -1