IEEE P802.11
Wireless LANs

| Security Comment Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 04 December 2010 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Yong Liu | Marvell |  | 1-408-222-8412 | yongliu@marvell.com |

Abstract

This document provides solutions of CIDs 941, 1215, 1163, 641, 972, 974, 975, 1217, 977, 533, 557, 534, 978, 979, 535, 536 in realtion to the Draft P802.11ad_D1.0 and the data base 11-10-1220-08-00ad-lb168-comment-database

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 941 | 34.17 | 5.8.3a | "lower MAC address" - how is the "lower" comparison performed on MAC addresses? (hint there are at least two incompatible ways of doing this) | Specify or reference how to do this comparison. | The same usage can be found in P802.11REVmb_D6.02 10.22.4 TDLS direct-link establishment(11z). Any reference in REVmb for this? |
| 1215 | 65.01 | 7.3.2. 25.1 | RSNA includes a pre-shared key as well as an IEEE 802.1X AKM suite. Tgad refers to both modes in Clause 8. There is currently only one defined for Tgad in Clause 7. | Add a second AKM suite for pre-shared key with GCM. | AKM suite (e.g. PSK) is independent from cipher suite. Why another PSK suite is needed particularly for GCM? Need some clarification from the commenter. |
| 1163 | 136.18 | 8.3.5.3 | In Annex H of 802.11mb Draft 4.0 and in Annex K of 802.11mb Draft 5.0, RSNA reference and test vectors are given. The 802.11ad amendment should include reference code and at least one test vector for GCM with the GMAC protocol (GCMP). NIST has publishe | Add instructions to Annex K of 802.11mb Draft 5.0, to add a complete example of the GCMP encryption method. | Open. Need some help to resolve this comment. |
| 641 | 140.22 | 8.3.5.4.3 | "shall maintain a separate replay counter for each possible IEEE 802.11 MSDU or A-MSDU priority" - what does priority mean here? It is ambiguous. Is it "AC", "UP" or "TID"? | Replace "priority" with something more explicit. | The same wording was copied from the CCMP text. |
| 972 | 144.08 | 8.4.5 | "Communication of all non-IEEE-802.1X MSDUs sent or received via the Controlled Port is not authorized." - what the heck does this mean? What is the normative effect of this statement? | If it has none, remove it or turn into a NOTE. | The same text was used for the IBSS case. |
| 974 | 145.15 | 8.4.11 | Please note that rekeying has been introduced into REVmb. | Align the description given here with REVmb. | This subcluase provides some general description on RSNA rekeying due to PN exhaustion, which is likely for very high rate cases, like 11ad. REVmb provides general supports for rekeying. Adjust some text in 8.5.3 4-Way Handshake to be consistent with REVmb. |
| 975 | 145.32 | 8.4.11 | "In addition, if both the Authenticator and the Supplicant 32 support multiple keys for unicast traffic, a smooth switchover to the new key is possible using the 33 following procedure." - no it isn't possible as described. The para at line 36 says that | Resolve this inconsistency. Perhaps modify the primitive so (which has no definition - oops :0) so that the SME can tell the difference between true exhaustion and warning threshold. | AGREE. Suggest defining two primitives: MLME-PN-Exhaustion.indication and MLME-PN-Warning.indication, and two thresholds: dot11PNExhaustionThreshold, and dot11PNWarningThreshold |
| 1217 | 146.41 | 8.4.12.1 | The PMK is bound to the par of STA MAC addresses that establish the PMKSA. In order to do non-transparent multi-band RSNA, the MAC addresses for each BAND should be bound into the PMKSA. | If non-transparent multi-band RSNA is to be used, change the derivation of the PMKSA to include MAC addresses for both bands. | PMK can be PSK or part of MSK. Why is it bounded to MAC address pair? The idea here is that once a PMK is created for one band, it is cached for other bands also. So that no additional authentications is needed for other bands. |
| 977 | 147.37 | 8.4.12.2 | "PTKSA for both the current operating band and other supported band(s)." - Is there is only one shared PN space? Is there any interface to support switching over bands at a specific PN? | Add description of operation of the PN and replay logic for transparent RSNA. Add PN to an appropriate MLME interface to support this. | Open. For transparent case, there is a single PTKSA for all bands; therefore, only one shared PN space for all bands. The PN assignment and MPDU encapsulation should be transparent to band usage. Any new MLME interface is added for multi-band support? |
| 533 | 148.15 | 8.5.2 | 802.11ad extends EAPOL-Key KDEs by adding a new subfield to GTK and MAC address KDEs and by adding a new Key ID KDE. However, there are no matching changes for FTIE. Is this done on purpose? Shouldn't it be possible to specify the Key ID also when using F | Extended FTIE (7.3.2.48 in 802.11r-2008) to allow the new Key ID (and maybe Band ID?) mechanism to be used with FT Protocol. | OPEN. The KeyID is related to rekeying, and should be discussed in REVmb. For Band ID, not sure whether FT protocol is supported by DBand and multi-band operations? |
| 557 | 148.24 | 8.5.2 | Table 8-4 wasn't defined with an extensible column (like the information element table was), so currently defined KDEs are not extensible by adding new fields at the end. | delete the change to the MAC address KDE | AGREE. Delete Band ID from MAC address KDE. |
| 534 | 148.24 | 8.5.2 | The unconditional addion of the Band ID subfield into the MAC address KDE format (Figure 8-27) would make all deployed implementations non-compliant with the IEEE 802.11 standard. This is not acceptable; the standard needs to continue to allow the previou | Move Band ID into its own KDE or make it optional part of the MAC address KDE to maintain backwards compatibility with deployed systems. | AGREE. Delete Band ID from MAC address KDE. |
| 978 | 149.06 | 8.5.3.2 | Bravo. This sentence is completely impenetrable. | Reword with a list structure that reflects the logic of the condition. | Reword |
| 979 | 149.21 | 8.5.3.3 | TGad has really excelled itself this time. Unless I missed a dot somewhere, we have a 23-line sentence - about ten times the parsable limit of most mortals. | Reword with a (probably nested) list structure that reflects the logic of the condition. | Reword |
| 535 | 150.15 | 8.5.3.3 | Use of “Class 3 unicast MPDUs” is somewhat problematic here since this clause applies to all uses of 4-way handshake, i.e., including IBSS where data frames are actually Class 1 frames. | Replace “to receive Class 3 unicast MPDUs protected by the PTK” with “to receive unicast MPDUs protected by the PTK”. | OPEN for more discussion. For Dband, the Class 3 unicast MPDUs also apply to mmWave IBSS. For more general IBSS, it may not be true. |
| 536 | 150.24 | 8.5.3.3 | Is moving of the MLME-SETKEYS.request from after sending of Message 4 to before that trying to indicate that Message 4 would be protected with the new key? If so, it would make existing deployments non-compliant and as such, would not be an appropriate ch | Clarify the intent of this change and if it is indeed trying to change how Message 4 gets protected, make that change conditional on use of DBand to maintain backwards compatibility with deployed systems. | Don't see a backward compatibility issue here. The MLME-SETKEYS.request is used before Message 4 only when the Extended Key ID for Unicast subfield of the RSN Capabilities field is set to 1. |

### RSNA rekeying

The 802.11 MAC shall issue an MLME-PN-Warning.indication when the Packet Number assignment for a particular PTKSA, GTKSA or STKSA reaches or exceeds dot11PNWarningThreshold for the first time. The indication shall only be issued once for a given PTKSA, GTKSA or STKSA. The SME may use the indication as a trigger to establish a new PTKSA, GTKSA or STKSA before the Packet Number space is exhausted. (CID975)

### EAPOL-Key frames

.11 Editor Note: remove Band ID from the MAC address KDE in Figure 8-27 (CID 534, 557)

### 4-Way Handshake

#### 4-Way Handshake Message 2

.11 Editor Note: Modify the description of the Key Data field as follows:

Key Data =

- the sending STA’s RSNIE for PTK generation or peer RSNIE for the current operating band, or;

- the sending STA’s Multi-band element for PTK generation for a supported band other than the current operating band, if dot11MultibandSupportEnabled is true, or;

- the sending STA’s RSN element and Multi-band element(s) for generating a single PTK for all involved bands, if dot11MultibandSupportEnabled is true and both the Authenticator and the Supplicant use the same MAC address in the current operating band and the other supported band(s), or;

- the sending STA’s RSN element and Multi-band element(s) for generating a different PTK for each involved band, if dot11MultibandSupportEnabled is true and the Joint Multi-band RSNA subfield of the RSN capabilities field is set to 1 for both the Authenticator and the Supplicant, and either the Authenticator or the Supplicant uses different MAC addresses for different bands. (CID 978)

- Lifetime of SMK and SMKID for STK generation.

#### 4-Way Handshake Message 3

.11 Editor Note: Modify the description of the Key Data field as follows:

Key Data =

- for PTK generation for the current operating band, the AP’s Beacon/Probe Response frame’s RSN element for the current operating band, and, optionally, a second RSN element that is the Authenticator’s pairwise cipher suite assignment for the current operating band, and, if a group cipher has been negotiated, the encapsulated GTK and the GTK’s key identifier (see 8.5.2) for the current operating band, or;

- for PTK generation for a supported band other than the current operating band, the Authenticator’s Beacon/mmWave Beacon/Announce/Probe Response/Information Response frame’s Multi-band element associated with the supported band, and optionally, a second Multi-band element that indicates the Authenticator’s pairwise cipher suite assignment for the supported band, and, if group cipher for the supported band is negotiated, the encapsulated GTK and the GTK’s key identifier for the supported band, if dot11MultibandSupportEnabled is true, or;

- for generating a single PTK for all involved bands, the Authenticator’s Beacon/mmWave Beacon/Announce/Probe Response/Information Response frame’s RSN element and Multi-band element(s), and optionally, additional RSN element and Multi-band element(s) that indicate the Authenticator’s assignment of one pairwise cipher suite for all involved bands; if a group cipher for all involved bands is negotiated, the encapsulated GTK and the GTK’s key identifier for all involved bands, if dot11MultibandSupportEnabled is true and both the Authenticator and the Supplicant use the same MAC address in the current operating band and the other supported band(s), or;

- for generating different PTKs for the current operating band and other supported band(s), the Authenticator’s Beacon/mmWave Beacon/Announce/Probe Response/Information Response frame’s RSN element and Multi-band element(s), and optionally, additional RSN element and Multi-band elements that indicate the Authenticator’s pairwise cipher suite assignments for one or more involved bands; if group ciphers for the involved bands are negotiated, the encapsulated GTKs and the GTK’s key identifiers for the involved bands, if dot11MultibandSupportEnabled is true and the Joint Multi-band RSNA subfield is set to 1 for both the Authenticator and Supplicant, and either the Authenticator or the Supplicant uses different MAC addresses for different bands.

- For STK generation Initiator RSNIE, Lifetime of SMK is used.

- If the Extended KeyID for Unicast subfield of the RSN Capabilities field is set to 1 for both the Authenticator/STA_I and Supplicant/STA_P, then the Authenticator/STA includes the Key ID KDE with the assigned key identifier for the current operating band; or the Authenticator includes the Key ID KDE(s) with the assigned key identifier(s) for one or more supported bands, if dot11MultibandSupportEnabled is true. (CID979)

.11 Editor Note: Replace the text after the sentence “Processing for PTK generation is as follows:” and before the sentence ” The Authenticator sends Message 3 to the Supplicant.” with the following REVmb text:

If the Extended Key ID for Unicast subfield of the RSN Capabilities field is 1 for both the Authenticator and the Supplicant, then the Authenticator assigns a new Key ID for the PTKSA in the range 0 through 1 that is different from the Key ID assigned in the previous handshake and uses the MLME-SETKEYS.request primitive to install the new key in the IEEE 802.11 MAC to receive Class 3 individually addressed MPDUs protected by the PTK with the assigned Key ID. Otherwise the Key ID 0 is used and installation of the key is deferred until after Message 4 has been received. (CID 974)

.11 Editor Note: in the paragraph that starts with “On reception of Message 3, the Supplicant silently…” replace the changed text with the following REVmb text:

d) If the Extended Key ID for Unicast subfield of the RSN Capabilities field is 1 for both the

Authenticator and Supplicant: Uses the MLME-SETKEYS.request primitive to configure the IEEE

802.11 MAC to receive Class 3 individually addressed MPDUs protected by the PTK with the assigned Key ID. (CID 974)

g) Uses the MLME-SETKEYS.request primitive to configure the IEEE 802.11 MAC to send and, if the receive key has not yet been installed, to receive Class 3 individually addressed MPDUs protected by the PTK. The GTK is also configured by MLME-SETKEYS primitive. (CID 974)

.11 Editor Note: in the paragraph that starts with “The STA_I sends Message 3 to the STA_P. On reception of Message 3, the STA_P silently…” replace bullet d) and e) as:

d) If the Extended Key ID for Unicast subfield of the RSN Capabilities field is 1 for both the

Authenticator and Supplicant then prior to sending Message 4, STA_P uses the MLMESETKEYS.

request primitive to configure the IEEE 802.11 MAC to receive Class 3 individually

addressed MPDUs protected by the STK with the assigned Key ID.

e) After sending Message 4, STA_P uses the MLME-SETKEYS.request primitive to configure the

IEEE 802.11 MAC to send and, if the receive key has not yet been installed, to receive Class 3

individually addressed MPDUs protected by the STK with the assigned Key ID.(CID 974)

#### 4-Way Handshake Message 4

.11 Editor Note: in the paragraph that starts with “On reception of Message 4, the Authenticator verifies that the Key Replay Counter…” modify as follows:

b) If the MIC is valid, the Authenticator uses the MLME-SETKEYS.request primitive to configure the

IEEE 802.11 MAC to send and, if the receive key has not yet been installed, to receive Class 3

individually addressed MPDUs using for the new PTK.

.11 Editor Note: in the paragraph that starts with “The STA_P sends Message 4 to the STA_I. On reception …” modify as follows:

b) If the MIC is valid, the STA_I uses the MLME-SETKEYS.request primitive to configure the 802.11

MAC to send and, if the receive key has not yet been installed, to receive Class 3 individually

addressed MPDUs using for the new STK. (CID 974)