IEEE P802.11
Wireless LANs

| Comment Resolution for Neighbor Report |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-03-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Daewon Lee | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea | +82-31-450-7897 | daewon.lee@lge.com |
| Yongho Seok | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea | +82-31-450-1497 | yongho.seok@lge.com |
| Illsoo Sohn | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea | +82-31-450-1882 | illsoo.sohn@lge.com |
| Eunsun Kim | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang-Shi, Kyungki-Do, 431-749, Korea | +82-31-450-1860 | esun.kim@lge.com |
| Youhan Kim | Atheros | 1700 Technology Drive, San Jose, CA 95110 | +1-408-830-5835 | youhan.kim@atheros.com |
| Matthew Fischer | Broadcom | 190 Mathilda Place, Sunnyvale, CA 94086 | +1-408-543-3370 | mfischer@broadcom.com |
| Jae Seung Lee | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82 42 860 1326 | jasonlee@etri.re.kr |
| Robert Stacey | Intel | 2111 NE 25th Ave, Hillsboro OR, 97124, USA | +1 503 724 0893 | robert.j.stacey@intel.com |
| Brian Hart | Cisco Systems | 170 W Tasman Dr, San Jose, CA, 95134, USA | 1-408-5253346 | brianh@cisco.com |
| Simone Merlin | Qualcomm | 5665 Morehouse Dr., San Diego, CA 92121 |  | smerlin@qualcomm.com |
| Raja Banerjea | Marvell | 5488 Marvell Lane, Santa Clara CA, 95054 | 1-408-222-3713 | rajab@marvell.com |
| Chunhui (Allan) Zhu | Samsung Electronics | 75 W. Plumeria Dr., San Jose, CA 95131 | +1-408-544-5667 | c.zhu@samsung.com |
| Bo Sun | ZTE Corporation | ZTE Building, #10 South Tangyan Rd., Xi'an, China | 86 29 88724130 | sun.bo1@zte.com.cn |
| Liwen Chu | ST-Ericsson | 2525 Augustine Drive, Santa Clara CA 95054 | +1.408.467.8436 | Liwen.Chu@st.com |
| Edward Au | Huawei Technologies | Huawei Industrial Base, Bantian Longgang, Shenzhen 518129, China | +1 773 782 6875 | edwardau@huawei.com |
| Vish Ponnampalam | Mediatek | 2860 Junction Ave, San Jose, CA 95134, USA | +1-408-526-1899 | vish.ponnampalam@mediatek.com |

Abstract

This document is a normative text proposal for supporting a neighbour report of VHT STA.

Introduction

- Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGac Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGac Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGaf Editor:  Editing instructions preceded by “TGac Editor” are instructions to the TGac editor to modify existing material in the TGac draft.   As a result of adopting the changes, the TGac editor will execute the instructions rather than copy them to the TGac Draft.

Submission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

Discussion:

In this document, we finalize the section for neighbor report element for VHT.

One note we should make is that the VHT Operation subelement byte size according to D0.1 is 3 Bytes + TBD Bytes. Since the TBD Bytes may change depending on other comment resolution we took the value 3 byte.

| 1049 | Seok, Yongho | 8.4.2.39 | 389 | 54 | TR | (Refer to Draft 802.11mb-D.6.0) To support VHT operations, need to indicate VHT in Neighbor Report element. | Add VHT bit in Neighbor Report element. |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Agree in principle to support the neighbor report of VHT STA as described in Draft P802.11ac_D0.1 comments 1049. Also agree to following editing instructions below.

## Editing instructions:

- 7.3.2.37 Nighbor Report element

TGac editor: Replace Figure 7-95c with the following:

|  | B0 B1 |  | B2 | B3 | B4 B9 | B10 | B11 | B12 | B13 B31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AP Reachability | Security |  | Key Scope | Capabilities | Mobility Domain | High Throughput | Very High Throughput | Reserved |
| Bits: | 2 |  | 1 | 1 | 6 | 1 | 1 | 1 | 19 |

Figure 7-95c—BSSID Information field

TGac editor: Insert the following paragraph after the paragraph in7.3.2.37 that starts “High Throughput”:

The Very High Throughput bit, when set to one, indicates that the AP represented by this BSSID is an VHT AP including the VHT Capabilities element in its Beacons, and that the contents of that VHT Capabilities element are identical to the VHT Capabilities element advertised by the AP sending the report.

TGac editor: Changes the paragraph that starts “Bits 12-31 are reserved.” as follows:

Bits 1213-31 are reserved.

TGac editor:Insert the following entries in Table 7-43b as follows:

Table 7-43b– Optional Subelement IDs for Neighbor Report

| Subelement ID | Name | Length field (Octets) | Extensible |
| --- | --- | --- | --- |
| 46 | VHT Capabilities subelement | 13 | Yes |
| 4647-60 | Reserved |  |  |
| 63 | VHT Operation subelement | 3 | Yes |
| 6364-65 | Reserved |  |  |

TGac editor: Insert the following after the paragraph starting “The Secondary Channel Offset subelement”:

The VHT Capabilities subelement is the same as the VHT Capabilities element as defined in 7.3.2.61 (VHT Capabilities element).

The VHT Operation subelement is the same as the VHT Operation element as defined in 7.3.2.62 (VHT Operation element).