IEEE P802.11
Wireless LANs

| 802.11 TGmb Proposed Resolution for CID 11001 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-03-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |

Abstract

This submission contains a proposed resolution to CID 11001,  which is assigned to the author for resolution.

R1:  updated following 2011-03-14 TGmb meeting.    Changes made during that meeting are highlighted thus.

R2:  Added comment 11002

R3:  Minor adjustments to exclude .11u unanswered questions from changes and tidy up for motioning,   shown thus.

# The Comments

| CID | Page | Clause | Comment | Proposed Change | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- |
| 11001 | 111.21 | 6.3.7.5.2 | Editor's Note: There is no clear 1:1mapping between these values andthe Status Code values. This field issupposed to determine the contentsof the Status Code field of the AssociateResponse frame. There arecodes above that appear to have nocounterpart in Table 8-35, and thereare Status Codes there related toassociation that have no counterparthere. | Add a "name" column to the Status Code definitions, review all Clause 6 parameters that are intended to map into a Status Code field value and populate name column in Status Code definition where there is a clear match with an existing Status Code description.Reserve new Status Code entries for any such unmatched Clause 6 parameter, inventing new names or descriptions as appropriate. | GEN |
| 11002 | 120.23 | 6.3.8.5.2 | Editor's Note: List does not match"refused" list in confirm | Copy ResultCode enumation list from .confirm primitive. | GEN |

# Proposed resolution

Agree in principle.  Make changes as indicated in document 11-11-0316r<latest>,  which achieves the intent of the proposed change.

# Discussion

The status code and reason code definitions could be improved:

- Some of them are defined using an explanatory paragraph, so an alternative form of representation is needed in Clause 6,  where short names rather than essays are required for enumeration lists.

- Sometimes the mapping between clause 6 names and clause 8 assignments is unclear.

- This lack of clarity resulted in mapping tables, such as in 10.4.4.

  - This mapping table essentially provides a short name for a subset of Status Code values.

  - The danger (realized multiple times) is that people don’t realize this is the same namespace as (in this case) the Status Code values,  and groups allocate themselves values independently from the Status Code table.

  - This happened recently,  and even the ANA misunderstood what was happening and allowed independent allocations from this subset namespace.

I believe the following fixups are necessary:

- Add a “name” column to status code and result code tables.

- Remove any (Clause 10) mapping tables of short name to Status Code or Reason Code by merging their name columns into the status code or result code tables.

- Ensure that each enumeration value used in Clause 6 maps exactly onto the name field of a Clause 8 Status/Reason code table entry,  adding new codes as necessary,  or adding names to existing entries where there is a clear correspondence.

- Replace any recent additions in Clause 6 that use the “essay form” to refer to one of these codes with a shorter name,  and add names to status/reason codes to support this as necessary.

- Ensure consistency between .confirm and .response by making the enumerated list of the .confirm contain all the values in the .response.

# The Changes

Changes are specified relative to D7.03.

Change Table 8-35 as follows:

|  |  |  | - Reason codesinput.doc |
| --- | --- | --- | --- |
| Reason code | Name | Meaning |  |
| 0 |  | Reserved |  |
| 1 |  | Unspecified reason |  |
| 2 |  | Previous authentication no longer valid |  |
| 3 |  | Deauthenticated because sending STA is leaving (or has left) IBSS or ESS |  |
| 4 |  | Disassociated due to inactivity |  |
| 5 |  | Disassociated because AP is unable to handle all currently associated STAs |  |
| 6 |  | Class 2 frame received from nonauthenticated STA |  |
| 7 |  | Class 3 frame received from nonassociated STA |  |
| 8 |  | Disassociated because sending STA is leaving (or has left) BSS |  |
| 9 |  | STA requesting (re)association is not authenticated with responding STA |  |
| 10 |  | Disassociated because the information in the Power Capability element is -unacceptable |  |
| 11 |  | Disassociated because the information in the Supported Channels element is -unacceptable |  |
| 12 |  | Disassociated due to BSS Transition Management(11v) |  |
| 13 |  | Invalid (#1684)element, i.e., an (#1684)element defined in this standard for which the content does not meet the specifications in Error: Reference source not found |  |
| 14 |  | Message integrity code (MIC) failure |  |
| 15 |  | 4-Way Handshake timeout |  |
| 16 |  | Group Key Handshake timeout |  |
| 17 |  | (#1684)element in 4-Way Handshake different from (Re)Association Request/Probe Response/Beacon frame |  |
| 18 |  | Invalid group cipher |  |
| 19 |  | Invalid pairwise cipher |  |
| 20 |  | Invalid AKMP |  |
| 21 |  | Unsupported RSN (#1684)element version |  |
| 22 |  | Invalid RSN (#1684)element capabilities |  |
| 23 |  | IEEE 802.1X authentication failed |  |
| 24 |  | Cipher suite rejected because of the security policy |  |
| 25(11z) |  | TDLS direct-link teardown due to TDLS peer STA unreachable via the TDLS direct link |  |
| 26(11z) |  | TDLS direct-link teardown for unspecified reason |  |
| 27(11u) |  | Disassociated because session terminated by SSP request |  |
| 28(11u) |  | Disassociated because of lack of SSP roaming agreement |  |
| 29(11u) |  | Requested service rejected because of SSP cipher suite or AKM requirement |  |
| 30(11u) |  | Requested service not authorized in this location |  |
| 31(11n) | SERVICE_CHANGE_PRECLUDES_TS | TS deleted because QoS AP lacks sufficient bandwidth for this QoS STA due to a change in BSS service characteristics or operational mode (e.g., an HT BSS change from 40 MHz channel to 20 MHz channel) |  |
| 32 |  | Disassociated for unspecified, QoS-related reason |  |
| 33 |  | Disassociated because QoS AP lacks sufficient bandwidth for this QoS STA |  |
| 34 |  | Disassociated because excessive number of frames need to be acknowledged, but are not acknowledged due to AP transmissions and/or poor channel conditions |  |
| 35 |  | Disassociated because STA is transmitting outside the limits of its TXOPs |  |
| 36 | STA_LEAVING | Requested from peer STA as the STA is leaving the BSS (or resetting) |  |
| 37 | END_TS END_BA END_DLS | Requested from peer STA as it does not want to use the mechanism |  |
| 38 | UNKNOWN_TS UNKNOWN_BA | Requested from peer STA as the STA received frames using the mechanism for which a setup is required |  |
| 39 | TIMEOUT | Requested from peer STA due to timeout |  |
| 45 | PEERKEY_MISMATCH | Peer STA does not support the requested cipher suite |  |
| 46(11u) | PEER_INITIATED | In a DLS Teardown frame: The teardown was initiated by the DLS peer. In a Disassociation frame: Disassociated because authorized access limit reached |  |
| 47(11u) | AP_INITIATED | In a DLS Teardown frame: The teardown was initiated by the AP. In a Disassociation frame: Disassociated due to external service requirements |  |
| 48(Ed) |  | Invalid FT Action frame count(#1252) |  |
| 49(Ed) |  | Invalid pairwise master key identifier (PMKI)(#1252) |  |
| 50(Ed) |  | Invalid MDE(#1252) |  |
| 51(Ed) |  | Invalid FTE(#1252) |  |
| 52(Ed)–65 535 |  | Reserved |  |

Change Table 8-36 as follows:

|  |  |  | - Status codesinput.doc |
| --- | --- | --- | --- |
| Status code | Name | Meaning |  |
| 0 | SUCCESS | Successful |  |
| 1 | REFUSED, REFUSED_REASON_UNSPECIFIED | Unspecified failure |  |
| 2(11z) |  | TDLS wakeup schedule rejected but alternative schedule provided |  |
| 3(11z) |  | TDLS wakeup schedule rejected |  |
| 4 |  | Reserved |  |
| 5(11z) |  | Security disabled |  |
| 6(11z) |  | Unacceptable lifetime |  |
| 7(11z) |  | Not in same BSS |  |
| 8–9(11z) |  | Reserved |  |
| 10 | REFUSED_CAPABILITIES_MISMATCH | Cannot support all requested capabilities in the Capability Information field |  |
| 11 |  | Reassociation denied due to inability to confirm that association exists |  |
| 12 |  | Association denied due to reason outside the scope of this standard |  |
| 13 |  | Responding STA does not support the specified authentication algorithm |  |
| 14 |  | Received an Authentication frame with authentication transaction sequence number out of expected sequence |  |
| 15 |  | Authentication rejected because of challenge failure |  |
| 16 |  | Authentication rejected due to timeout waiting for next frame in sequence |  |
| 17 |  | Association denied because AP is unable to handle additional associated STAs |  |
| 18 | REFUSED_BASIC_RATES_MISMATCH | Association denied due to requesting STA not supporting all of the data rates in the BSSBasicRateSet parameter |  |
| 19 |  | Association denied due to requesting STA not supporting the short preamble option |  |
| 20 |  | Association denied due to requesting STA not supporting the PBCC modulation option |  |
| 21 |  | Association denied due to requesting STA not supporting the Channel Agility option |  |
| 22 |  | Association request rejected because Spectrum Management capability is required |  |
| 23 |  | Association request rejected because the information in the Power Capability -element is unacceptable |  |
| 24 |  | Association request rejected because the information in the Supported Channels -element is unacceptable |  |
| 25 |  | Association denied due to requesting STA not supporting the Short Slot Time option |  |
| 26 |  | Association denied due to requesting STA not supporting the DSSS-OFDM option |  |
| 27(11r) |  | Association denied because the requesting STA does not support HT features(11n) |  |
| 28(11r) |  | R0KH unreachable |  |
| 29(11r) |  | Association denied because the requesting STA does not support the phased coexistence operation (PCO) transition time required by the AP(11n) |  |
| 30(11w) | REFUSED_TEMPORARILY | Association request rejected temporarily; try again later |  |
| 31(11w) |  | Robust Management frame policy violation |  |
| 32 |  | Unspecified, QoS-related failure |  |
| 33 |  | Association denied because QoS AP has insufficient bandwidth to handle another QoS STA |  |
| 34 |  | Association denied due to excessive frame loss rates and/or poor conditions on current operating channel |  |
| 35 |  | Association (with QoS BSS) denied because the requesting STA does not support the QoS -facility |  |
| 36 |  | Reserved |  |
| 37 | REFUSED | The request has been declined |  |
| 38 | INVALID_PARAMETERS | The request has not been successful as one or more parameters have invalid values |  |
| 39 | REJECTED_WITH_SUGGESTED_CHANGES | The TS has not been created because the request cannot be honored; however, a suggested TSPEC is provided so that the initiating STA may attempt to set another TS with the suggested changes to the TSPEC |  |
| 40 |  | Invalid (#1684)element, i.e., an (#1684)element defined in this standard for which the content does not meet the specifications in Error: Reference source not found |  |
| 41 |  | Invalid group cipher |  |
| 42 |  | Invalid pairwise cipher |  |
| 43 |  | Invalid AKMP |  |
| 44 |  | Unsupported RSN (#1684)element version |  |
| 45 |  | Invalid RSN (#1684)element capabilities |  |
| 46 |  | Cipher suite rejected because of security policy |  |
| 47 | REJECTED_FOR_DELAY_PERIOD | The TS has not been created; however, the HC may be capable of creating a TS, in response to a request, after the time indicated in the TS Delay element |  |
| 48 | NOT_ALLOWED | Direct link is not allowed in the BSS by policy |  |
| 49 | NOT_PRESENT | The Destination STA is not present within this BSS |  |
| 50 | NOT_QOS_STA | The Destination STA is not a QoS STA |  |
| 51 |  | Association denied because the ListenInterval is too large |  |
| 52(11r) |  | Invalid FT Action frame count |  |
| 53(11r) |  | Invalid pairwise master key identifier (PMKID) |  |
| 54(11r) |  | Invalid MDE(#1684) |  |
| 55(11r) |  | Invalid FTE(#1684) |  |
| 56(11v) |  | Requested TCLAS processing is not supported by the AP. |  |
| 57(11v) |  | The AP has insufficient TCLAS processing resources to satisfy the request. |  |
| 58(11v) |  | The TS has not been created because the request cannot be honored; however, the HC suggests the STA transitions to other BSSs to setup the TS. |  |
| 59(11u) | ADVERTISEMENT_PROTOCOL_NOT_SUPPORTED | GAS Advertisement Protocol not supported |  |
| 60(11u) | NO_OUTSTANDING_REQUEST | No outstanding GAS request |  |
| 61(11u) | RESPONSE_NOT_RECEIVED_FROM _SERVER | GAS Response not received from the Advertisement Server |  |
| 62(11u) | GAS_QUERY_TIMEOUT | STA timed out waiting for GAS Query Response |  |
| 63(11u) | QUERY_RESPONSE_TOO_LARGE | GAS Response is larger than query response length limit |  |
| 64(11u) | REJECTED_HOME_WITH_SUGGESTED_CHANGES | Request refused because home network does not support request |  |
| 65(11u) | SERVER_UNREACHABLE | Advertisement Server in the network is not currently reachable |  |
| 66(Ed) |  | Reserved |  |
| 67(11u) | REJECTED_FOR_SSP_PERMISSIONS | Request refused due to permissions received via SSPN interface |  |
| 68(11u) |  | Request refused because AP does not support unauthenticated access |  |
| 69-71(Ed) |  | Reserved |  |
| 72(11z) |  | Invalid contents of RSNE |  |
| 73(11v) |  | U-APSD Coexistence is not supported. |  |
| 74(11v) |  | Requested U-APSD Coexistence mode is not supported. |  |
| 75(11v) |  | Requested Interval/Duration value cannot be supported with U-APSD Coexistence. |  |
| 76–78 (11v)(11u) |  | Reserved |  |
| 79(11u) | TRANSMISSION_FAILURE | Transmission failure |  |
| 80(11v) | REQUESTED_TCLAS_NOT_SUPPORTED | Requested TCLAS Not Supported. |  |
| 81(11v) | TCLAS_RESOURCES_EXHAUSTED | TCLAS Resources Exhausted. |  |
| 82(11v) | REJECTED_WITH_SUGGESTED_BSS_TRANSITION | Rejected with Suggested BSS Transition. |  |
| 83–65 535 (11z) |  | Reserved |  |

Editor: Request the following new status codes from the ANA and add to above table with specified description.

| Name | Used in primitive | Description |
| --- | --- | --- |
| REFUSED_EXTERNAL_REASON | ASSOCIATE | (Re)association refused for some external reason |
| REFUSED_AP_OUT_OF_MEMORY | ASSOCIATE | (Re)association refused because of memory limits at the AP |
| REJECTED_EMERGENCY_SERVICES_NOT_SUPPORTED | ASSOCIATE | (Re)association refused because emergency services are not supported at the AP. |
| QUERY_RESPONSE_OUTSTANDING | GAS & PDGAS | (no description proposed at present, this may be updated later) |

- TS setup

Change para 14 of 10.4.4 as follows and delete table 10-2.

The (#2243)HC’s MLME(#4050) transmits an ADDTS Response frame containing this TSPEC and status. The encoding of the ResultCode values to Status Code field values is defined inEncoding of ResultCode to Status Code field valueTable 8-36.

- TS deletion

Change 10.4.9 as follows:

There are two types of TS deletion: non-AP STA-initiated and HC-initiated. In both cases, the SME entity above the MAC generates an MLME-DELTS.request primitive specifying the TSID and direction of the TS to be deleted and the reason for deleting the TS. This causes the MAC to send a DELTS Action frame. The encoding of ReasonCode values to Reason Code field (see 8.4.1.7 (Reason Code field)) values is defined in Table 8-35.Encoding of ReasonCode to Reason Code field value for DELTS.

Also merge entries from list above into enumerations in .request/.confirm in MLME-DELTS primitives.

- Procedure at the recipient

Change 10.5.2.3 as follows:

A recipient shall operate as follows in order to support Block Ack initialization and modification:

- When an ADDBA Request frame is received from another STA, the MLME shall issue an MLME-ADDBA.indication primitive.

- Upon receipt of the MLME-ADDBA.response primitive, the STA shall respond by an ADDBA Response frame with a result code as defined in 8.5.5.3 (ADDBA Response frame format).(Ed)

- If the result code is SUCCESS, the Block Ack is considered to be established with the originator. Contained in the frame are the type of Block Ack and the number of buffers that have been allocated for the support of this block.

- If the result code is REFUSED, the Block Ack is not considered to have been established.

The encoding of ResultCode values to Status Code field values is defined inEncoding of ResultCode to Status Code field valueTable 8-36.

Also remove TIMEOUT from ADDBA.response enumeration.

- Procedure at the initiator of the Block Ack teardown

Change 10.5.3.2 as follows:

Upon receipt of an MLME-DELBA.request primitive, the (#2243)(#3098)(#3121)MLME shall tear down the Block Ack using the following procedure:

- The MLME(#3121) shall transmit a DELBA frame.

- Upon the receipt of an acknowledgment to the DELBA frame, the MLME issues an MLME-DELBA.confirm primitive.

Encoding of ReasonCode to Reason Code field value for DELBA.

Add TIMEOUT to enumeration in DELBA.request

- Setup procedure at the AP

Change 10.7.2.3 as follows:

Upon receipt of the DLS Request frame (step 1a in Error: Reference source not found), the AP shall

- Send DLS Response frame to the STA that sent the DLS Request frame with a result code of Not Allowed in the BSS, if direct links are not allowed in the BSS (step 2b in Error: Reference source not found), or for the AP with dot11SSPNInterfaceActivated set to true(Ed) with a result code of Not Allowed by SSP if the dot11NonAPStationAuthDls MIB variable in either of the non-AP STA’s dot11InterworkingTable is false.(11u)

- Send DLS Response frame to the STA that sent the DLS Request frame with a result code of Not Present, if the destination STA is not present in the BSS (step 2b in Error: Reference source not found).

- Send DLS Response frame to the STA that sent the DLS Request frame with a result code of Not a QoS(#1519) STA, if the destination STA does not have QoS facility (step 2b in Error: Reference source not found).

- Send the DLS Request frame, with all fields having the same value as the DLS Request frame received by the AP, to the destination STA (step 1b in Error: Reference source not found), independently of whether the AP is capable of decoding all of the fields in the body of the frame.(11n)

Upon receipt of the DLS Response frame from a STA (step 2a in Error: Reference source not found), the AP shall send DLS Response frame to the source STA (step 2b in Error: Reference source not found).

.

Delete TIMEOUT from DLS.confirm enumeration.

- STA-initiated DLS teardown procedure

Change 10.7.4.2 as follows:

Upon receipt of MLME-DLSTeardown.request primitive from the SME, the STA shall initiate the -teardown of the direct link by sending the DLS Teardown frame to the AP. The applicable values of ReasonCode defined in ReasonCode values for DLS -teardown.  The encoding of the Reason Code field is defined in Table 8-35.

| - ReasonCode values for DLS -teardown |  |  |
| --- | --- | --- |
| ReasonCode |  | Applicable at |
| STA_LEAVING |  | Non-AP STA |
| END_DLS |  | Non-AP STA |
| UNKNOWN_DLS |  | Non-AP STA |
| TIMEOUT |  | Non-AP STA |
| PEERKEY_MISMATCH |  | AP |
| PEER_INITIATED |  | AP and Non-AP STA |
| AP_INITIATED |  | Non-AP STA |

Merge entries from list above into DLSTeardown.request/.indication primitives.

- STA procedures to post a GAS Query to an Advertisement Server(11u)

Change 10.24.3.1.3 as follows:

Upon receipt of a GAS Initial Request frame, an MLME-GAS.indication primitive shall be issued to the STA’s SME. Upon receipt of an MLME-GAS.response primitive, the STA shall transmit a GAS Initial Response frame to the requesting STA according to the following procedures. If the requesting STA is in the associated state and in the power-save mode, the responding STA shall buffer the frame for transmission according to the procedures in Error: Reference source not found; otherwise the STA shall queue the frame for transmission.

- If the Advertisement Protocol ID in the Advertisement Protocol element does not equal the value contained in any dot11GASAdvertisementID MIB object, then the STA shall not post the query to an Advertisement Server. The STA shall transmit a directed GAS Initial Response frame to the requesting STA containing a dialog token whose value is identical to the dialog token in the GAS Initial Request frame, a Status Code equal to “GAS Advertisement Protocol not supported” (see) Table 8-36), an Advertisement Protocol element containing the Advertisement Protocol ID used in the GAS Initial Request frame and a Comeback Delay and Query Response Length both set to 0.

|  | Action: Mike to check with .11u |
| --- | --- |

Merge entries from table above into GAS and PDGAS .response/.confirm primitives.

Change any now dangling references in primitives to refer to appropriate (Status Code / Reason Code) table.

## Changes to Clause 6 interfaces

Note,  document 11-11-0284r2 proposes removal of the following .confirms (ignore the **).  These primitives were not checked for the purpose of the current work:

- DEAUTHENTICATE**

- DEASSOCIATE **

- RESET

- START **

- MREQUEST **

- MREPORT **

- SETKEYS

- DELETEKEYS

- SETPROTECTION

- DELTS **

- HL-SYNC

- DELBA  **

- SCHEDULE

- NEIGHBORREPREQ

- MLME-REMOTE-REQUEST

- TIMING_ADVERTISEMENT

- TDLSSETUP-REQUEST

- TDLSSETUP-RESPONSE

- TDLSSETUP-CONFIRM

- TDLSTEARDOWN

- EVLREQUEST

- EVVLREPORT

- DIAGREQUEST

- DIAGREPORT

- CLINTERFERENCE REQUEST

- CLINTERFERENCE RESPONSE

- CLINTERFERENCE REPORT

- TIMINGMSMTRQ

- WNMNOTIFICATIONREQUEST

- WNMNOTIFICATIONRESPONSE

Note also that the same document proposes removal of some locally-generated ResultCodes.  This is not shown in the changes below.

Note that the following primitives follow a “three primitives” pattern, and are not included in the list above -  i.e. they have .request/.confirm/.indication,  but no response.   It is unclear from these primitives which of the ResultCodes should be locally generated (i.e. not requiring a supporting Status Code) and which are

remotely generated.   I have taken no action on these primitives.

- DLS

- DLSTeardown

- SYNC

- TIMING_ADVERTISEMENT

- TIMINGMSMT

6.3.7.5 MLME-ASSOCIATE.response

6.3.7.5.2 Semantics of the service primitive

Change ResultCode enumeration as follows:

SUCCESS,

REFUSED_REASON_UNSPECIFIED,

REFUSED_CAPABILITIES_MISMATCH,

REFUSED_EXTERNAL_REASON,

REFUSED_AP_OUT_OF_MEMORY,

REFUSED_BASIC_RATES_MISMATCH,

REJECTED_EMERGENCY_SERVICES_-

NOT_SUPPORTED,(11u)

REFUSED_TEMPORARILY

And make matching change to the .confirm

And make matching changes to the REASSOCIATE primitives.

6.3.29.5 MLME-ADDBA.response

6.3.29.5.2 Semantics of the service primitive

Change the ResultCode enumeration as follows:

SUCCESS, REFUSED, INVALID_PARAMETERS

6.3.38.4 MLME-DSETPC.response(11y)

6.3.38.4.2 Semantics of the service primitive(11y)

Modify the .confirm parameter as follows: SUCCESS, INVALID_PARAMETERS, TIMEOUT, REFUSED

- DSE Power Constraint frame format(11y)

Change para6 and Table 8-206 of 8.5.8.10 as follows:

The Reason Result Code field is used to indicate the reason that a DSE Power Constraint frame was generated. The length of the Reason Result Code field is 1 octet. The reason result codes that have been allocated are shown in Reason Result Code field values.

|  |  | - Reason Result Code field values |  |
| --- | --- | --- | --- |
| Reason Result Code field value | Name |  | Description |
| 0 |  |  | Reserved |
| 1 |  |  | Reserved |
| 2 |  |  | Power constraint requested |
| 3 | SUCCESS |  | Success |
| 4 | REFUSED |  | Refused – no reason specified |
| 5 | INVALID PARAMETERS |  | Request not successful as one or more parameters have invalid values |
| 6 |  |  | Reserved |
| 7 |  |  | Reserved |
| 8-255 |  |  | Reserved |

This is as far as the 2011-03-14 meeting got.  We did start on 8.5.8.4,  but then time ran out.

6.3.39.4 MLME-ENABLEMENT.response(11y)

6.3.39.4.2 Semantics of the service primitive(11y)

Modify the ResultCode enumeration as follows:

SUCCESS, REFUSED, INVALID_PARAMETERS, TOO_MANY_SIMULTANEOUS_REQUESTS

- DSE Enablement frame format(11y)

Modify Table 8-204 as follows:

The Reason Result Code field is used to indicate the reason that a DSE Enablement frame was generated. The length of the Reason Result Code field is 1 octet. The reason result codes that have been allocated are shown in Reason Result Code field values.

|  |  | - Reason Result Code field values |  |
| --- | --- | --- | --- |
| Reason Result Code field value | Name |  | Description |
| 0 |  |  | Reserved |
| 1 |  |  | Reserved |
| 2 |  |  | Enablement requested |
| 3 | SUCCESS |  | Success |
| 4 | REFUSED |  | Request declined |
| 5 | INVALID_PARAMETERS |  | Request not successful as one or more parameters have invalid values |
| 6 | TOO_MANY_SIMULTANEOUS_REQUESTS |  | Enablement denied because the enabling STA is unable to handle additional dependent STAs |
| 7 |  |  | Reserved |
| 8–255 |  |  | Reserved |

6.3.64.3 MLME-TIMBROADCAST.confirm(11v)

6.3.64.3.2 Semantics of the service primitive(11v)

Discussion:   The TimBroadcast Response element contains a status value,  so the only valid values for the ResultCode parameter should be locally-generated statuses.   There are no references in the text to the deleted ResultCodes,  (i.e. there’s no obvious local generation of these codes)  and there’s nowhere for them OTA.

Change the ResultCode enumeration as follows:

SUCCESS, TRANSMISSION_FAILURE

The same logic applies also to the following:

Make matching changes to the DMS.confirm primitive.

6.3.71.2 MLME-GAS.confirm(11u)

6.3.71.2.2 Semantics of the service primitive(11u)

Discussion:  802.11u allocated a status code (TIMEOUT).

| 62(11u) | TIMEOUT | STA timed out waiting for GAS Query Response |
| --- | --- | --- |

If this is a timeout between the AP and some external entity providing a GAS response,  then having this as a ResultCode is perfectly reasonable.   However,  the name is overly general in this case,  as it confuses with the locally-generated TIMEOUT on many .confirms.  In R1,  TIMEOUT was renamed GAS_QUERY_TIMEOUT.

However,  if it is intended to reflect a locally generated status,  then TIMOUT is appropriate,  and the status code 62 should be released as this doesn’t appear on air.

Change the ResultCode enumeration as follows:

SUCCESS, GAS_QUERY_TIMEOUT,

UNSPECIFIED_FAILURE,

ADVERTISEMENT_PROTOCOL_NOT_SUPPORTED,

QUERY_RESPONSE_OUTSTANDING,

QUERY_RESPONSE_TOO_LARGE,

SERVER_UNREACHABLE,

TRANSMISSION_FAILURE

Change TIMEOUT to GAS_QUERY_TIMEOUT in the .response primitive.

Make same changes to PDGAS.confirm.