IEEE P802.11
Wireless LANs

| D0.1 Comment Resolution – CID 806 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: March 14, 2011 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Youhan Kim | Atheros |  |  | youhan.kim@atheros.com |

Abstract

This document provides resolution for CID 806 on TGac D0.1.

| CommentID | Subclause | Page | Line | Comment Type | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- |
| 806 | 7.3.2.61.2 | 30 | 17 | TR | Supported channel width set does not allow for 80+80 MHz non-contiguous setting only. | Suggested change: Set to 0 if the STA supports 160 MHz Set to 1 if the STA supports 80+80 MHz Set to 2 if the STA supports both 160 MHz and 80+80 MHz Set to 3 the STA does not support either 80+80 or 160MHz |

Response:

DISAGREE.  Requirement R3.1.D in the ‘Specification Framework for TGac’ (992r15) specifies that “Contiguous and non-contiguous 160 MHz devices shall be capable of transmitting and receiving frames between each other when the two segments of the non-contiguous 160 MHz device are placed in frequency to match the tone allocation of the contiguous 160 MHz device.”  Hence, an STA supporting 80+80 MHz must be capable of supporting 160 MHz as well.