IEEE P802.11
Wireless LANs

| D0.1 Comment Resolution – VHT Compressed Beamforming Report field and MU Exclusive Report field |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: March 16, 2011 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Youhan Kim | Atheros |  |  | youhan.kim@atheros.com |

Abstract

This document provides resolution for CID related the VHT Compressed Beamforming Report field the MU Exclusive Report field in TGac D0.1.

CID: 36, 106, 560, 884, 891, 1019, 1020, 1021, 1129, 1130, 1131, 1132, 1134, 1323, 1324, 1346, 1406, 1407, 1408

| CID | Subclause | Page | Line | Comment Type | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- |
| 106 | 7.3.1.32 | 22 | 38 | TR | The structure of the VHT compressed beamforming report field for 160M/80+80M is not clearly defined. | Clarify the spec text and clearly define the structure of the VHT compressed beamforming report field for 160M/80+80M. Details of the proposed text change are presented in our contribution. |
| 560 | 7.3.1.32 | 22 | 38 | TR | Need better wording to clarify that the two instances of 80 Hz Compressed Beamforming report field are used to convey the compressed beamforming report for 160 and 80+80 MHz. | Update paragraph starting on line 38 for better clarification. |
| 1324 | 7.3.1.32 | 22 | 41 | TR | From "For contiguous 160MHz, the VHT Compressed Beamforming Report field has the structure defined in Table 7-11 (VHT Compressed Beamforming Report field (80 MHz)) with tone shifting ±256." it is not clear if first the upper or lower subband should be included in the feedback report | Clarify |
| 1346 | 7.3.1.32 | 22 | 37-43 | TR | Text in this paragraph is unclear. Need to describe in detail on the CV FB field for 80+80 and 160. | Make new tables similar to 7-11 for 80+80 non-contiguous and 160Contiguous modes. Specify that there are two sets of SNR reports in 80+80 non-contiguous mode. |
| 1134 | 7.3.1.32 | 23 | 43 | TR | A note is insufficient here. This needs to be normative, i.e. For 160 MHz, the subcarrier indicies included are for each segment are given by the 80 MHz row. Also, it should be stated which segment appears first (I assume the low frequency segment) | Convert note to normative text and also define the segment order |
| 1129 | 7.3.1.32 | 16 | 5 | ER | It is unecessary to have separate tables for Nr = 5, 6, 7. | Combine into a single table. |
| 1130 | 7.3.1.32 | 20 | 4 | ER | It is unecessary to have separate tables for 20 MHz, 40 MHz and 80 MHz. | Combine tables into one using parameters similar to Table 7-13. |
| 1131 | 7.3.1.32 | 20 | 22 | ER | The name of the field and the meaning seem inconsistent | Change meaning to "Beamforming feedback marix V for subcarrier k=-28. Similar throughout. |
| 1019 | 7.3.1.32 | 20 | 38 | ER | Beamforming Feedback Matrix V for carrier 2 is not in case of Grouping=4 (Refer Table 7-12 in page 23). | Erase the row of 'Beamforming Feedback Matrix V for carrier 2' |
| 1323 | 7.3.1.32 | 20 | 38 | TR | The entry "Beamforming Feedback Matrix V for carrier 2" isn't true for Ng = 4 (see Table 7-12 for correct indices). Also the entry "Beamforming Feedback Matrix V for carrier 2+Ng" isn't correct for Ng = 4. | Remove the entry "Beamforming Feedback Matrix V for carrier 2" and change the entries "Beamforming Feedback Matrix V for carrier 2+Ng" and "Beamforming Feedback Matrix V for carrier 2+2Ng" to "Beamforming Feedback Matrix V for carrier 0+Ng" and "Beamforming Feedback Matrix V for carrier 0+2Ng", resp. And change the entry "Beamforming Feedback Matrix V for carrier 1" to "If Ng > 1, Beamforming Feedback Matrix V for carrier 1" (otherwise we'll get a double count for carrier 1 with above changes for Ng = 1) |
| 1406 | 7.3.1.32 | 20 | 38 | ER | Beamforming Feedback Matrix V for carrier 2 is not in case of Grouping=4 (Refer Table 7-12 in page 23). | Erase the row of 'Beamforming Feedback Matrix V for carrier 2' |
| 1020 | 7.3.1.32 | 20 | 41, 43 | ER | 'Beamforming Feedback Matrix V for carrier 2+Ng' and 'Beamforming Feedback Matrix V for carrier 2+2Ng' don't match to in case of Crouping=4 in Table 7-12 (Page 23) | Change 'Beamforming Feedback Matrix V for carrier 2+Ng' and 'Beamforming Feedback Matrix V for carrier 2+2Ng' to ''Beamforming Feedback Matrix V for carrier Ng' and 'Beamforming Feedback Matrix V for carrier 2Ng,' respectively. |
| 1407 | 7.3.1.32 | 20 | 41, 43 | ER | 'Beamforming Feedback Matrix V for carrier 2+Ng' and 'Beamforming Feedback Matrix V for carrier 2+2Ng' don't match to in case of Crouping=4 in Table 7-12 (Page 23) | Change 'Beamforming Feedback Matrix V for carrier 2+Ng' and 'Beamforming Feedback Matrix V for carrier 2+2Ng' to ''Beamforming Feedback Matrix V for carrier Ng' and 'Beamforming Feedback Matrix V for carrier 2Ng,' respectively. |

Response:

AGREE IN PRINCIPLE.  It would be cleaner to have a single format for all BW modes – average SNR per stream followed by per-tone V matrix, from the lowest subcarrier to the highest subcarrier in frequency.  This would also eliminate any ambiguity for 160 MHz and 80+80 MHz.

| CID | Subclause | Page | Line | Comment Type | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- |
| 36 | 7.3.1.32 | 22 | 41 | TR | "±256" should be replaced with "±128". | As in comment. |
| 1021 | 7.3.1.32 | 22 | 41 | TR | with tone shifring ±256' is written. However, in case of 160MHz, the tone index is -250~+250. | Change 'with tone shifring ±256' to 'with tone shifring ±250' |
| 1408 | 7.3.1.32 | 22 | 41 | TR | the line says 'with tone shifring ±256'. However, in case of 160MHz, the tone index is -250~+250. | Change 'with tone shifring ±256' to 'with tone shifring ±250' |

Response:

AGREE IN PRINCIPLE.  The rewritten 7.3.1.32 now explicitly defines subcarrier indices, thus ‘tone shifting’ is no longer required.

| CID | Subclause | Page | Line | Comment Type | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- |
| 884 | 7.3.1.32 | 22 | 48 | TR | Did not understand the term -22 in the text "The SNR values in Table 7-9 (VHT Compressed Beamforming Report field (20 MHz)), Table 7-10 (VHT Compressed Beamforming Report field (40 MHz)) and Table 7-11 (VHT Compressed Beamforming Report field (80 MHz)) are encoded as an 8-bit twos complement value of 4 ×(SNR_average – 22), where SNR_average is the sum of the values of SNR per tone (in decibels) divided by the number of tones represented. This encoding covers the SNR range from –10 dB to 53.75 dB in 0.25 dB steps" | Should it have been -10? (Assuming 8 bits represents from [0 to 63.75] in quarter dB precision and the factor -10 shifts it to [-10 to 53.75])? |

Response:

DISAGREE.  -10 dB – 22 = -32.  Then, -32 * 4 = -128, which is the lower limit of an 8-bit twos’ complement integer.  Similarly, 53.75 dB – 22 = 31.75.  Then, 31.75 * 4 = 127, which is the upper limit of an 8-bit twos’ complement integer.  Thus, the current text is correct.  However, to avoid similar confusions in the future, suggest converting the equation to a table explicitly mapping the value of the field to the SNR value.

| CID | Subclause | Page | Line | Comment Type | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- |
| 1132 | 7.3.1.32 | 22 | 56 | TR | It is not clear from the description, what exactly grouping is. It is described as a method for reporting a single value for a group Ng of adjacent subcarriers. But what exactly is this single value? The table seems to imply that it is for a specific subcarrier (the subcarrier identified in the table). | Sufficient to say for "group Ng of contiguous subcarriers" (remove adjacent - they can't all be adjacent). The text needs to be specific as to whether this is a single subcarrier that represents the group or whether it is some average value. |

Response:

DISAGREE.  How to come up with the single value for a group is implementation specific.

| CID | Subclause | Page | Line | Comment Type | Comment | SuggestedRemedy |
| --- | --- | --- | --- | --- | --- | --- |
| 891 | 7.3.1.33 | 24 | 23 | TR | The definition of N should be more tightly related to the average SNR per stream | Change line to: "is the average noise plus interference power measured at the beamformee that was used to calculate SNR bar |

Response:

AGREE IN PRINCIPLE.  However, definition of N has now been moved to subclause 7.3.1.32, which makes this comment no longer relevant.

Proposed Resolution Text:

TGac editor: Edit subclauses 7.3.1.32 and 7.3.1.33 as follows:

#### 7.3.1.32 VHT Compressed Beamforming Report field

The VHT Compressed Beamforming Report field is used by the VHT Compressed Beamforming frame (see Error: Reference source not found) to carry explicit feedback information in the form of angles representing compressed beamforming feedback matrices V for use by a transmit beamformer to determine steering matrices Q, as described in 9.19.3 (Explicit feedback beamforming) and 20.3.12.2 (Explicit feedback beamforming).

The size of the VHT Compressed Beamforming Report field depends on the values in the VHT MIMO Control field.

The VHT Compressed Beamforming Report field contains the channel matrix elements indexed, first, by matrix angles in the order shown in Table 7-1 to Table 7-5Table 7-X1 (Order of angles in the Compressed Beamforming Report field Compressed Beamforming Feedback Matrix subfield) and, second, by data subcarrier index from lowest frequency to highest frequency. The explanation on how these angles are generated from the beamforming feedback matrix V is given in 20.3.12.2.5 (Compressed beamforming feedback matrix).  In Tables 7-X1 (Order of angles in the Compressed Beamforming Feedback Matrix subfield),

Nc	is the number of columns in a compressed beamforming feedback matrix determined by the Nc Index field of the VHT MIMO Control field,

Nr	is the number of rows in a compressed beamforming feedback matrix determined by the Nr Index field of the VHT MIMO Control field.

Table 7-17-X1—Order of angles in the MIMO Compressed Steering Matrices Report field compressed beamforming feedback matrix subfieldfor Nr <= 4

| Size of V(Nr x Nc) |  |  |  | Number of angles (Na) |  |  |  |  | The order of angles in the Quantized Steering Matrices Feedback Information Compressed Beamforming Feedback Matrix subfield |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2x1 |  |  |  | 2 |  |  |  |  | 11, 21 |
| 2x2 |  |  |  | 2 |  |  |  |  | 11, 21 |
| 3x1 |  |  |  | 4 |  |  |  |  | 11, 21, 21, 31, |
| 3x2 |  |  |  | 6 |  |  |  |  | 11, 21, 21, 31, 22, 32 |
| 3x3 |  |  |  | 6 |  |  |  |  | 11, 21, 21, 31, 22, 32 |
| 4x1 |  |  |  | 6 |  |  |  |  | 11, 21, 31, 21, 31, 41 |
| 4x2 |  |  |  | 10 |  |  |  |  | 11, 21, 31, 21, 31, 41, 22, 32, 32, 42 |
| 4x3 |  |  |  | 12 |  |  |  |  | 11, 21, 31, 21, 31, 41, 22, 32, 32, 42, 33, 43 |
| 4x4 |  |  |  | 12 |  |  |  |  | 11, 21, 31, 21, 31, 41, 22, 32, 32, 42, 33, 43 |
| 5×1 | 8 |  |  |  |  | 11, 21, 31, 41, ψ21, ψ31, ψ41, ψ51 |  |  |  |
| 5×2 | 14 |  |  |  |  | 11, 21, 31, 41, ψ21, ψ31, ψ41, ψ51, 22, 32, 42, ψ32, ψ42, ψ52 |  |  |  |
| 5×3 | 18 |  |  |  |  | 11, 21, 31, 41, ψ21, ψ31, ψ41, ψ51, 22, 32, 42, ψ32, ψ42, ψ52, 33, 43, ψ43, ψ53 |  |  |  |
| 5×4 | 20 |  |  |  |  | 11, 21, 31, 41, ψ21, ψ31, ψ41, ψ51, 22, 32, 42, ψ32, ψ42, ψ52, 33, 43, ψ43, ψ53, 44, ψ54 |  |  |  |
| 5×5 | 20 |  |  |  |  | 11, 21, 31, 41, ψ21, ψ31, ψ41, ψ51, 22, 32, 42, ψ32, ψ42, ψ52, 33, 43, ψ43, ψ53, 44, ψ54 |  |  |  |
| 6×1 |  | 10 |  |  |  |  | 11, 21, 31, 41, 51, ψ21, ψ31, ψ41, ψ51, ψ61 |  |  |
| 6×2 |  | 18 |  |  |  |  | 11, 21, 31, 41, 51, ψ21, ψ31, ψ41, ψ51, ψ61, 22, 32, 42, 52, ψ32, ψ42, ψ52, ψ62 |  |  |
| 6×3 |  | 24 |  |  |  |  | 11, 21, 31, 41, 51, ψ21, ψ31, ψ41, ψ51, ψ61, 22, 32, 42, 52, ψ32, ψ42, ψ52, ψ62, 33, 43, 53, ψ43, ψ53, ψ63 |  |  |
| 6×4 |  | 28 |  |  |  |  | 11, 21, 31, 41, 51, ψ21, ψ31, ψ41, ψ51, ψ61, 22, 32, 42, 52, ψ32, ψ42, ψ52, ψ62, 33, 43, 53, ψ43, ψ53, ψ63, 44, 54, ψ54, ψ64 |  |  |
| 6×5 |  | 30 |  |  |  |  | 11, 21, 31, 41, 51, ψ21, ψ31, ψ41, ψ51, ψ61, 22, 32, 42, 52, ψ32, ψ42, ψ52, ψ62, 33, 43, 53, ψ43, ψ53, ψ63, 44, 54, ψ54, ψ64, 55, ψ65 |  |  |
| 6×6 |  | 30 |  |  |  |  | 11, 21, 31, 41, 51, ψ21, ψ31, ψ41, ψ51, ψ61, 22, 32, 42, 52, ψ32, ψ42, ψ52, ψ62, 33, 43, 53, ψ43, ψ53, ψ63, 44, 54, ψ54, ψ64, 55, ψ65 |  |  |
| 7×1 | 12 |  |  |  |  |  |  | 11, 21, 31, 41, 51, 61, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71 |  |
| 7×2 | 22 |  |  |  |  |  |  | 11, 21, 31, 41, 51, 61, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, 22, 32, 42, 52, 62, ψ32, ψ42, ψ52, ψ62, ψ72 |  |
| 7×3 | 30 |  |  |  |  |  |  | 11, 21, 31, 41, 51, 61, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, 22, 32, 42, 52, 62, ψ32, ψ42, ψ52, ψ62, ψ72, 33, 43, 53, 63, ψ43, ψ53, ψ63, ψ73 |  |
| 7×4 | 36 |  |  |  |  |  |  | 11, 21, 31, 41, 51, 61, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, 22, 32, 42, 52, 62, ψ32, ψ42, ψ52, ψ62, ψ72, 33, 43, 53, 63, ψ43, ψ53, ψ63, ψ73, 44, 54, 64, ψ54, ψ64, ψ74 |  |
| 7×5 | 40 |  |  |  |  |  |  | 11, 21, 31, 41, 51, 61, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, 22, 32, 42, 52, 62, ψ32, ψ42, ψ52, ψ62, ψ72, 33, 43, 53, 63, ψ43, ψ53, ψ63, ψ73, 44, 54, 64, ψ54, ψ64, ψ74, 55, 65, ψ65, ψ75 |  |
| 7×6 | 42 |  |  |  |  |  |  | 11, 21, 31, 41, 51, 61, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, 22, 32, 42, 52, 62, ψ32, ψ42, ψ52, ψ62, ψ72, 33, 43, 53, 63, ψ43, ψ53, ψ63, ψ73, 44, 54, 64, ψ54, ψ64, ψ74, 55, 65, ψ65, ψ75, 66, ψ76 |  |
| 7×7 | 42 |  |  |  |  |  |  | 11, 21, 31, 41, 51, 61, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, 22, 32, 42, 52, 62, ψ32, ψ42, ψ52, ψ62, ψ72, 33, 43, 53, 63, ψ43, ψ53, ψ63, ψ73, 44, 54, 64, ψ54, ψ64, ψ74, 55, 65, ψ65, ψ75, 66, ψ76 |  |
| 8×1 |  |  | 14 |  | 11, 21, 31, 41, 51, 61, 71, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, ψ81 |  |  |  |  |
| 8×2 |  |  | 26 |  | 11, 21, 31, 41, 51, 61, 71, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, ψ81, 22, 32, 42, 52, 62, 72, ψ32, ψ42, ψ52, ψ62, ψ72, ψ82 |  |  |  |  |
| 8×3 |  |  | 36 |  | 11, 21, 31, 41, 51, 61, 71, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, ψ81, 22, 32, 42, 52, 62, 72, ψ32, ψ42, ψ52, ψ62, ψ72, ψ82, 33, 43, 53, φ63, 73, ψ43, ψ53, ψ63, ψ73, ψ83 |  |  |  |  |
| 8×4 |  |  | 44 |  | 11, 21, 31, 41, 51, 61, 71, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, ψ81, 22, 32, 42, 52, 62, 72, ψ32, ψ42, ψ52, ψ62, ψ72, ψ82, 33, 43, 53, 63, 73, ψ43, ψ53, ψ63, ψ73, ψ83,44, 54, 64, 74, ψ54, ψ64, ψ74, ψ84 |  |  |  |  |
| 8×5 |  |  | 50 |  | 11, 21, 31, 41, 51, 61, 71, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, ψ81, 22, 32, 42, 52, 62, 72, ψ32, ψ42, ψ52, ψ62, ψ72, ψ82, 33, 43, 53, 63, 73, ψ43, ψ53, ψ63, ψ73, ψ83,44, 54, 64, 74, ψ54, ψ64, ψ74, ψ84, 55, 65, 75, ψ65, ψ75, ψ85 |  |  |  |  |
| 8×6 |  |  | 54 |  | 11, 21, 31, 41, 51, 61, 71, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, ψ81, 22, 32, 42, 52, 62, 72, ψ32, ψ42, ψ52, ψ62, ψ72, ψ82, 33, 43, 53, 63, 73, ψ43, ψ53, ψ63, ψ73, ψ83,44, 54, 64, 74, ψ54, ψ64, ψ74, ψ84, 55, 65, 75, ψ65, ψ75, ψ85, 66, 76, ψ76, ψ86 |  |  |  |  |
| 8×7 |  |  | 56 |  | 11, 21, 31, 41, 51, 61, 71, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, ψ81, 22, 32, 42, 52, 62, 72, ψ32, ψ42, ψ52, ψ62, ψ72, ψ82, 33, 43, 53, 63, 73, ψ43, ψ53, ψ63, ψ73, ψ83,44, 54, 64, 74, ψ54, ψ64, ψ74, ψ84, 55, 65, 75, ψ65, ψ75, ψ85, 66, 76, ψ76, ψ86, 77, ψ87 |  |  |  |  |
| 8×8 |  |  | 56 |  | 11, 21, 31, 41, 51, 61, 71, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, ψ81, 22, 32, 42, 52, 62, 72, ψ32, ψ42, ψ52, ψ62, ψ72, ψ82, 33, 43, 53, 63, 73, ψ43, ψ53, ψ63, ψ73, ψ83,44, 54, 64, 74, ψ54, ψ64, ψ74, ψ84, 55, 65, 75, ψ65, ψ75, ψ85, 66, 76, ψ76, ψ86, 77, ψ87 |  |  |  |  |

Table 7-2— Order of angles in the MIMO Compressed Steering Matrices Report field compressed beamforming matrix V for Nr = 5Nr > 4

| Size of V (Nr × Nc) | Number of angles (Na) | The order of angles in the Quantized Beamforming Feedback Matrices Information field |
| --- | --- | --- |
| 5×1 | 8 | 11, 21, 31, 41, ψ21, ψ31, ψ41, ψ51 |
| 5×2 | 14 | 11, 21, 31, 41, ψ21, ψ31, ψ41, ψ51, 22, 32, 42, ψ32, ψ42, ψ52 |
| 5×3 | 18 | 11, 21, 31, 41, ψ21, ψ31, ψ41, ψ51, 22, 32, 42, ψ32, ψ42, ψ52, 33, 43, ψ43, ψ53 |
| 5×4 | 20 | 11, 21, 31, 41, ψ21, ψ31, ψ41, ψ51, 22, 32, 42, ψ32, ψ42, ψ52, 33, 43, ψ43, ψ53, 44, ψ54 |
| 5×5 | 20 | 11, 21, 31, 41, ψ21, ψ31, ψ41, ψ51, 22, 32, 42, ψ32, ψ42, ψ52, 33, 43, ψ43, ψ53, 44, ψ54 |

Table 7-3— Order of angles in the MIMO Compressed Steering Matrices Report field compressed beamforming matrix V for NrNr = 6

| Size of V (Nr × Nc) | Number of angles (Na) | The order of angles in the Quantized Beamforming Feedback Matrices Information field |
| --- | --- | --- |
| 6×1 | 10 | 11, 21, 31, 41, 51, ψ21, ψ31, ψ41, ψ51, ψ61 |
| 6×2 | 18 | 11, 21, 31, 41, 51, ψ21, ψ31, ψ41, ψ51, ψ61, 22, 32, 42, 52, ψ32, ψ42, ψ52, ψ62 |
| 6×3 | 24 | 11, 21, 31, 41, 51, ψ21, ψ31, ψ41, ψ51, ψ61, 22, 32, 42, 52, ψ32, ψ42, ψ52, ψ62, 33, 43, 53, ψ43, ψ53, ψ63 |
| 6×4 | 28 | 11, 21, 31, 41, 51, ψ21, ψ31, ψ41, ψ51, ψ61, 22, 32, 42, 52, ψ32, ψ42, ψ52, ψ62, 33, 43, 53, ψ43, ψ53, ψ63, 44, 54, ψ54, ψ64 |
| 6×5 | 30 | 11, 21, 31, 41, 51, ψ21, ψ31, ψ41, ψ51, ψ61, 22, 32, 42, 52, ψ32, ψ42, ψ52, ψ62, 33, 43, 53, ψ43, ψ53, ψ63, 44, 54, ψ54, ψ64, 55, ψ65 |
| 6×6 | 30 | 11, 21, 31, 41, 51, ψ21, ψ31, ψ41, ψ51, ψ61, 22, 32, 42, 52, ψ32, ψ42, ψ52, ψ62, 33, 43, 53, ψ43, ψ53, ψ63, 44, 54, ψ54, ψ64, 55, ψ65 |

Table 7-4— Order of angles in the MIMO Compressed Steering Matrices Report field compressed beamforming matrix V for NrNr = 7

| Size of V (Nr × Nc) | Number of angles (Na) | The order of angles in the Quantized Beamforming Feedback Matrices Information field |
| --- | --- | --- |
| 7×1 | 12 | 11, 21, 31, 41, 51, 61, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71 |
| 7×2 | 22 | 11, 21, 31, 41, 51, 61, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, 22, 32, 42, 52, 62, ψ32, ψ42, ψ52, ψ62, ψ72 |
| 7×3 | 30 | 11, 21, 31, 41, 51, 61, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, 22, 32, 42, 52, 62, ψ32, ψ42, ψ52, ψ62, ψ72, 33, 43, 53, 63, ψ43, ψ53, ψ63, ψ73 |
| 7×4 | 36 | 11, 21, 31, 41, 51, 61, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, 22, 32, 42, 52, 62, ψ32, ψ42, ψ52, ψ62, ψ72, 33, 43, 53, 63, ψ43, ψ53, ψ63, ψ73, 44, 54, 64, ψ54, ψ64, ψ74 |
| 7×5 | 40 | 11, 21, 31, 41, 51, 61, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, 22, 32, 42, 52, 62, ψ32, ψ42, ψ52, ψ62, ψ72, 33, 43, 53, 63, ψ43, ψ53, ψ63, ψ73, 44, 54, 64, ψ54, ψ64, ψ74, 55, 65, ψ65, ψ75 |
| 7×6 | 42 | 11, 21, 31, 41, 51, 61, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, 22, 32, 42, 52, 62, ψ32, ψ42, ψ52, ψ62, ψ72, 33, 43, 53, 63, ψ43, ψ53, ψ63, ψ73, 44, 54, 64, ψ54, ψ64, ψ74, 55, 65, ψ65, ψ75, 66, ψ76 |
| 7×7 | 42 | 11, 21, 31, 41, 51, 61, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, 22, 32, 42, 52, 62, ψ32, ψ42, ψ52, ψ62, ψ72, 33, 43, 53, 63, ψ43, ψ53, ψ63, ψ73, 44, 54, 64, ψ54, ψ64, ψ74, 55, 65, ψ65, ψ75, 66, ψ76 |

Table 7-5— Order of angles in the MIMO Compressed Steering Matrices Report field compressed beamforming matrix V for NrNr = 8

| Size of V (Nr × Nc) | Number of angles (Na) | The order of angles in the Quantized Beamforming Feedback Matrices Information field |
| --- | --- | --- |
| 8×1 | 14 | 11, 21, 31, 41, 51, 61, 71, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, ψ81 |
| 8×2 | 26 | 11, 21, 31, 41, 51, 61, 71, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, ψ81, 22, 32, 42, 52, 62, 72, ψ32, ψ42, ψ52, ψ62, ψ72, ψ82 |
| 8×3 | 36 | 11, 21, 31, 41, 51, 61, 71, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, ψ81, 22, 32, 42, 52, 62, 72, ψ32, ψ42, ψ52, ψ62, ψ72, ψ82, 33, 43, 53, φ63, 73, ψ43, ψ53, ψ63, ψ73, ψ83 |
| 8×4 | 44 | 11, 21, 31, 41, 51, 61, 71, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, ψ81, 22, 32, 42, 52, 62, 72, ψ32, ψ42, ψ52, ψ62, ψ72, ψ82, 33, 43, 53, 63, 73, ψ43, ψ53, ψ63, ψ73, ψ83,44, 54, 64, 74, ψ54, ψ64, ψ74, ψ84 |
| 8×5 | 50 | 11, 21, 31, 41, 51, 61, 71, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, ψ81, 22, 32, 42, 52, 62, 72, ψ32, ψ42, ψ52, ψ62, ψ72, ψ82, 33, 43, 53, 63, 73, ψ43, ψ53, ψ63, ψ73, ψ83,44, 54, 64, 74, ψ54, ψ64, ψ74, ψ84, 55, 65, 75, ψ65, ψ75, ψ85 |
| 8×6 | 54 | 11, 21, 31, 41, 51, 61, 71, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, ψ81, 22, 32, 42, 52, 62, 72, ψ32, ψ42, ψ52, ψ62, ψ72, ψ82, 33, 43, 53, 63, 73, ψ43, ψ53, ψ63, ψ73, ψ83,44, 54, 64, 74, ψ54, ψ64, ψ74, ψ84, 55, 65, 75, ψ65, ψ75, ψ85, 66, 76, ψ76, ψ86 |
| 8×7 | 56 | 11, 21, 31, 41, 51, 61, 71, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, ψ81, 22, 32, 42, 52, 62, 72, ψ32, ψ42, ψ52, ψ62, ψ72, ψ82, 33, 43, 53, 63, 73, ψ43, ψ53, ψ63, ψ73, ψ83,44, 54, 64, 74, ψ54, ψ64, ψ74, ψ84, 55, 65, 75, ψ65, ψ75, ψ85, 66, 76, ψ76, ψ86, 77, ψ87 |
| 8×8 | 56 | 11, 21, 31, 41, 51, 61, 71, ψ21, ψ31, ψ41, ψ51, ψ61, ψ71, ψ81, 22, 32, 42, 52, 62, 72, ψ32, ψ42, ψ52, ψ62, ψ72, ψ82, 33, 43, 53, 63, 73, ψ43, ψ53, ψ63, ψ73, ψ83,44, 54, 64, 74, ψ54, ψ64, ψ74, ψ84, 55, 65, 75, ψ65, ψ75, ψ85, 66, 76, ψ76, ψ86, 77, ψ87 |

The angles are quantized as defined in Table 7-6Table 7-X2 (Quantization of angles). All angles are transmitted LSB to MSB.

Table 7-67-X2--Quantization of angles

| Quantized | Quantized |
| --- | --- |
| radians where b is the number of bits used to quantize (defined by the Codebook Information field of the VHT MIMO Control field (see Error: Reference source not found) |   radians where b is the number of bits used to quantize (defined by the Codebook Information field of the VHT MIMO Control field (see Error: Reference source not found) |

The VHT Compressed Beamforming Report field for 20 MHz has the structure defined in Table 7-7 (VHT Compressed Beamforming Report field (20 MHz)), where Na is the number of angles used for beamforming feedback matrix V (see Table 7-1 to Table 7-5).

Table 7-7—VHT Compressed Beamforming Report field (20 MHz)

| Field | Size (bits) | Meaning |
| --- | --- | --- |
| SNR in space-time stream 1 | 8 | Average signal-to-noise ratio in the STA sending the report for space-time stream 1 |
| ... |  |  |
| SNR in space-time stream Nc | 8 | Average signal-to-noise ratio in the STA sending the report for space-time stream Nc |
| Beamforming Feedback Matrix V for carrier –28 | Na×(b +b)/2 | Beamforming feedback matrix V |
| Beamforming Feedback Matrix V for carrier –28+Ng | Na×(b +b)/2 | Beamforming feedback matrix V |
| Beamforming Feedback Matrix V for carrier –28+2Ng | Na×(b +b)/2 | Beamforming feedback matrix V |
| ... |  |  |
| Beamforming Feedback Matrix V for carrier –1 | Na×( b +b)/2 | Beamforming feedback matrix V |
| Beamforming Feedback Matrix V for carrier 1 | Na×( b +b)/2 | Beamforming feedback matrix V |
| Beamforming Feedback Matrix V for carrier 2 | Na×( b +b)/2 | Beamforming feedback matrix V |
| Beamforming Feedback Matrix V for carrier 2+Ng | Na×( b +b)/2 | Beamforming feedback matrix V |
| Beamforming Feedback Matrix V for carrier 2+2Ng | Na×( b +b)/2 | Beamforming feedback matrix V |
| ... |  |  |
| Beamforming Feedback Matrix V for carrier 28 | Na×( b +b)/2 | Beamforming feedback matrix V |

The VHT Compressed Beamforming Report field for 40 MHz has the structure defined in Table 7-8

(VHT Compressed Beamforming Report field (40 MHz)), where Na is the number of angles used for the beamforming feedback matrix V (see Table 7-1 to Table 7-5).

Table 7-8—VHT Compressed Beamforming Report field (40 MHz)

| Field | Size (bits) | Meaning |
| --- | --- | --- |
| SNR in space-time stream 1 | 8 | Average signal-to-noise ratio in the STA sending the report for space-time stream 1 |
| ... |  |  |
| SNR in space-time stream Nc | 8 | Average signal-to-noise ratio in the STA sending the report for space-time stream Nc |
| Beamforming Feedback Matrix V for carrier –58 | Na×(b +b)/2 | Beamforming feedback matrix V |
| Beamforming Feedback Matrix V for carrier –58+Ng | Na×(b +b)/2 | Beamforming feedback matrix V |
| Beamforming Feedback Matrix V for carrier –58+2Ng | Na×(b +b)/2 | Beamforming feedback matrix V |
| ... |  |  |
| Beamforming Feedback Matrix V for carrier –2 | Na×( b +b)/2 | Beamforming feedback matrix V |
| Beamforming Feedback Matrix V for carrier 2 | Na×( b +b)/2 | Beamforming feedback matrix V |
| Beamforming Feedback Matrix V for carrier 2+Ng | Na×(b +b)/2 | Beamforming feedback matrix V |
| Beamforming Feedback Matrix V for carrier 2+2Ng | Na×(b +b)/2 | Beamforming feedback matrix V |
| ... |  |  |
| Beamforming Feedback Matrix V for carrier 58 | Na×( b +b)/2 | Beamforming feedback matrix V |

The VHT Compressed Beamforming Report field for 80 MHz has the structure defined in Table 7-9 (VHT Compressed Beamforming Report field (80 MHz)), where Na is the number of angles used for the beamforming feedback matrix V (see Table 7-1 to Table 7-5).

Table 7-9—VHT Compressed Beamforming Report field (80 MHz)

| Field | Size (bits) | Meaning |
| --- | --- | --- |
| SNR in space-time stream 1 | 8 | Average signal-to-noise ratio in the STA sending the report for space-time stream 1 |
| ... |  |  |
| SNR in space-time stream Nc | 8 | Average signal-to-noise ratio in the STA sending the report for space-time stream Nc |
| Beamforming Feedback Matrix V for carrier –122 | Na×(b +b)/2 | Beamforming feedback matrix V |
| Beamforming Feedback Matrix V for carrier –122+Ng | Na×(b +b)/2 | Beamforming feedback matrix V |
| Beamforming Feedback Matrix V for carrier –122+2Ng | Na×(b +b)/2 | Beamforming feedback matrix V |
| ... |  |  |
| Beamforming Feedback Matrix V for carrier –2 | Na×( b +b)/2 | Beamforming feedback matrix V |
| Beamforming Feedback Matrix V for carrier 2 | Na×( b +b)/2 | Beamforming feedback matrix V |
| Beamforming Feedback Matrix V for carrier 2+Ng | Na×(b +b)/2 | Beamforming feedback matrix V |
| Beamforming Feedback Matrix V for carrier 2+2Ng | Na×(b +b)/2 | Beamforming feedback matrix V |
| ... |  |  |
| Beamforming Feedback Matrix V for carrier 122 | Na×( b +b)/2 | Beamforming feedback matrix V |

The VHT Compressed Beamforming Report field for non-contiguous 80+80 MHz has the structure defined in Table 7-9 (VHT Compressed Beamforming Report field (80 MHz)) for each frequency segment. For contiguous 160MHz, the VHT Compressed Beamforming Report field has the structure defined in Table 7-9 (VHT Compressed Beamforming Report field (80 MHz)) with tone shifting ±256.  Na is the number of angles used for beamforming feedback matrix V (see Table 7-1 to Table 7-5).

The SNR values in Table 7-7 (VHT Compressed Beamforming Report field (20 MHz)), Table 7-8

(VHT Compressed Beamforming Report field (40 MHz)) and Table 7-9 (VHT Compressed Beamforming Report field (80 MHz)) are encoded as an 8-bit twos complement value of 4 ×(SNR_average – 22), where SNR_average is the sum of the values of SNR per tone (in decibels) divided  by the number of tones represented. This encoding covers the SNR range from –10 dB to 53.75 dB in 0.25 dB steps. Each SNR value per tone in stream i (before being averaged) corresponds to the SNR associated with the column i of the beamforming feedback matrix V determined at the beamformee. Each SNR corresponds to the predicted SNR at the beamformee when the beamformer applies the matrix V.

Grouping is a method that reduces the size of the VHT Compressed Beamforming Report field by reporting a single value for each group of Ng adjacent subcarriers. With grouping, the size of the VHT Compressed Beamforming Report field is Nc×8+Ns×(Na×( b +b)/2) bits, where the number of subcarriers sent, Ns, is a function of Ng and the channel width defined by the Channel Width field in VHT MIMO Control field (see Error: Reference source not found). The value of Ns and the specific carriers for which matrices are sent is defined in Error: Reference source not found (Number of subcarriers and tone mapping). If the size of the VHT Compressed Beamforming Report field is not an integral multiple of 8 bits, up to 7 zeros are appended to the end of the report to make its size an integral multiple of 8 bits.

Table 7-10--Number of subcarriers and tone mapping

| BW | Grouping (Ng) | Ns | Carriers for which matrices are sent |
| --- | --- | --- | --- |
| 20 MHz | 1 | 52 | All subcarriers -28 to 28 except ±21, ±7 and 0. |
|  | 2 | 30 | ±28, ±26, ±24, ±22, ±20, ±18, ±16, ±14, ±12, ±10, ±8, ±6, ±4, ±2, ±1 |
|  | 4 | 16 | ±28, ±24, ±20, ±16, ±12, ±8, ±4, ±1 |
| 40 MHz | 1 | 108 | All subcarriers -58 to 58 except ±53, ±25, ±11, ±1 and 0 |
|  | 2 | 58 | ±58, ±56, ±54, ±52, ±50, ±48, ±46, ±44, ±42, ±40, ±38, ±36, ±34, ±32, ±30, ±28, ±26, ±24, ±22, ±20, ±18, ±16, ±14, ±12, ±10, ±8, ±6, ±4, ±2 |
|  | 4 | 30 | ±58, ±54, ±50, ±46, ±42, ±38, ±34, ±30, ±26, ±22, ±18, ±14, ±10, ±6, ±2 |
| 80 MHz | 1 | 234 | All subcarriers -122 to 122 except ±103, ±75, ±39, ±11, ±1 and 0. |
|  | 2 | 122 | ±122, ±120, ±118, ±116, ±114, ±112, ±110, ±108, ±106, ±104, ±102, ±100, ±98, ±96, ±94, ±92, ±90, ±88, ±86, ±84, ±82, ±80, ±78, ±76, ±74, ±72, ±70, ±68, ±66, ±64, ±62, ±60, ±58, ±56, ±54, ±52, ±50, ±48, ±46, ±44, ±42, ±40, ±38, ±36, ±34, ±32, ±30, ±28, ±26, ±24, ±22, ±20, ±18, ±16, ±14, ±12, ±10, ±8, ±6, ±4, ±2 |
|  | 4 | 62 | ±122, ±118, ±114, ±110, ±106, ±102, ±98, ±94, ±90, ±86, ±82, ±78, ±74, ±70, ±66, ±62, ±58, ±54, ±50, ±46, ±42, ±38, ±34, ±30, ±26, ±22, ±18, ±14, ±10, ±6, ±2 |

Note: When BW=160MHz, the above BW=80MHz table indicates the subcarrier indices to be fed back for each 80MHz frequency segment.

The VHT Compressed Beamforming Report field has the structure defined in Table 7-X3, where Na is the number of angles used for the compressed beamforming feedback matrix subfield (see Table 7-X1).

Table 7-X3 – VHT Compressed Beamforming Report field

| Subfield | Size (bits) | Meaning |
| --- | --- | --- |
| Average SNR of Space-Time Stream 1 | 8 | Signal-to-noise ratio at the beamformee for space-time stream 1 averaged over subcarriers. See Equation (7-X1). |
| … | … | … |
| Average SNR of Space-Time Stream Nc | 8 | Signal-to-noise ratio at the beamformee for space-time stream Nc averaged over subcarriers. See Equation (7-X1). |
| Compressed Beamforming Feedback Matrix for subcarrier k = scidx(0) | Na×(b +b)/2 | Compressed beamforming feedback matrix as defined in Table 7-X1 |
| Compressed Beamforming Feedback Matrix for subcarrier k = scidx(1) | Na×(b +b)/2 | Compressed beamforming feedback matrix as defined in Table 7-X1 |
| Compressed Beamforming Feedback Matrix for subcarrier k = scidx(2) | Na×(b +b)/2 | Compressed beamforming feedback matrix as defined in Table 7-X1 |
| … | … | … |
| Compressed Beamforming Feedback Matrix for subcarrier k = scidx(Ns-1) | Na×( b +b)/2 | Compressed beamforming feedback matrix as defined in Table 7-X1 |

Ns is the number of subcarriers for which the Compressed Beamforming Feedback Matrix subfield is sent back to the beamformer.  Beamformee may choose to reduce Ns by using a method referred to as grouping, in which only a single Compressed Beamforming Feedback Matrix is reported for each group of Ng adjacent subcarriers.  Ns is a function of the Channel Width and Grouping subfields in VHT MIMO Control Field (see 7.3.1.31).  Table 7-X4 lists Ns, the exact subcarrier indices and their order for which the Compressed Beamforming Matrix subfield is sent back.

Table 7-X4 – Subcarrier for which Compressed Beamforming Feedback Matrix subfield is sent back

| Channel width | Ng | Ns | Subcarriers for which Compressed Feedback Beamforming Matrix subfield is sent: scidx(0), scidx(1), … scidx(Ns-1) |
| --- | --- | --- | --- |
| 20 MHz | 1 | 52 | -28, -27, -26, -25, -24, -23, -22, -20, -19, -18, -17, -16, -15, -14, -13, -12, -11, -10, -9, -8, -6, -5, -4, -3, -2, -1, 1, 2, 3, 4, 5, 6, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 22, 23, 24, 25, 26, 27, 28 NOTE – Pilot subcarriers (±21, ±7) and DC subcarrier (0) are skipped |
|  | 2 | 30 | -28, -26, -24, -22, -20, -18, -16, -14, -12, -10, -8, -6, -4, -2, -1, 1, 2, 4, 6, 8, 10, 12, 14, 16, 18, 20, 22, 24, 26, 28 |
|  | 4 | 16 | -28, -24, -20, -16, -12, -8, -4, -1, 1, 4, 8, 12, 16, 20, 24, 28 |
| 40 MHz | 1 | 108 | -58, -57, -56, -55, -54, -52, -51, -50, -49, -48, -47, -46, -45, -44, -43, -42, -41, -40, -39, -38, -37, -36, -35, -34, -33, -32, -31, -30, -29, -28, -27, -26, -24, -23, -22, -21, -20, -19, -18, -17, -16, -15, -14, -13, -12, -10, -9, -8, -7, -6, -5, -4, -3, -2, 2, 3, 4, 5, 6, 7, 8, 9, 10, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 46, 47, 48, 49, 50, 51, 52, 54, 55, 56, 57, 58 NOTE – Pilot subcarriers (±53, ±25, ±11) and DC subcarriers (0, ±1) are skipped. |
|  | 2 | 58 | -58, -56, -54, -52, -50, -48, -46, -44, -42, -40, -38, -36, -34, -32, -30, -28, -26, -24, -22, -20, -18, -16, -14, -12, -10, -8, -6, -4,-2, 2, 4, 6, 8, 10, 12, 14, 16, 18, 20, 22, 24, 26, 28, 30, 32, 34, 36, 38, 40, 42, 44, 46, 48, 50, 52, 54, 56, 58 |
|  | 4 | 30 | -58, -54, -50, -46, -42, -38, -34, -30, -26, -22, -18, -14, -10, -6,-2, 2, 6, 10, 14, 18, 22, 26, 30, 34, 38, 42, 46, 50, 54, 58 |
| 80 MHz | 1 | 234 | -122, -121, -120, -119, -118, -117, -116, -115, -114, -113, -112, -111, -110, -109, -108, -107, -106, -105, -104, -102, -101, -100, -99, -98, -97, -96, -95, -94, -93, -92, -91, -90, -89, -88, -87, -86, -85, -84, -83, -82, -81, -80, -79, -78, -77, -76, -74, -73, -72, -71, -70, -69, -68, -67, -66, -65, -64, -63, -62, -61, -60, -59, -58, -57, -56, -55, -54, -53, -52, -51, -50, -49, -48, -47, -46, -45, -44, -43, -42, -41, -40, -38, -37, -36, -35, -34, -33, -32, -31, -30, -29, -28, -27, -26, -25, -24, -23, -22, -21, -20, -19, -18, -17, -16, -15, -14, -13, -12, -10, -9, -8, -7, -6, -5, -4, -3, -2, 2, 3, 4, 5, 6, 7, 8, 9, 10, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 40, 41, 42, 43, 44, 45, 46, 47, 48, 49, 50, 51, 52, 53, 54, 55, 56, 57, 58, 59, 60, 61, 62, 63, 64, 65, 66, 67, 68, 69, 70, 71, 72, 73, 74, 76, 77, 78, 79, 80, 81, 82, 83, 84, 85, 86, 87, 88, 89, 90, 91, 92, 93, 94, 95, 96, 97, 98, 99, 100, 101, 102, 104, 105, 106, 107, 108, 109, 110, 111, 112, 113, 114, 115, 116, 117, 118, 119, 120, 121, 122 NOTE – Pilot subcarriers (±103, ±75, ±39, ±11) and DC subcarriers (0, ±1) are skipped. |
|  | 2 | 122 | -122, -120, -118, -116, -114, -112, -110, -108, -106, -104, -102, -100, -98, -96, -94, -92, -90, -88, -86, -84, -82, -80, -78, -76, -74, -72, -70, -68, -66, -64, -62, -60, -58, -56, -54, -52, -50, -48, -46, -44, -42, -40, -38, -36, -34, -32, -30, -28, -26, -24, -22, -20, -18, -16, -14, -12, -10, -8, -6, -4, -2, 2, 4, 6, 8, 10, 12, 14, 16, 18, 20, 22, 24, 26, 28, 30, 32, 34, 36, 38, 40, 42, 44, 46, 48, 50, 52, 54, 56, 58, 60, 62, 64, 66, 68, 70, 72, 74, 76, 78, 80, 82, 84, 86, 88, 90, 92, 94, 96, 98, 100, 102, 104, 106, 108, 110, 112, 114, 116, 118, 120, 122 |
|  | 4 | 62 | -122, -118, -114, -110, -106, -102, -98, -94, -90, -86, -82, -78, -74, -70, -66, -62, -58, -54, -50, -46, -42, -38, -34, -30, -26, -22, -18, -14, -10, -6, -2, 2, 6, 10, 14, 18, 22, 26, 30, 34, 38, 42, 46, 50, 54, 58, 62, 66, 70, 74, 78, 82, 86, 90, 94, 98, 102, 106, 110, 114, 118, 122 |
| 160 MHz | 1 | 468 | -250, -249, -248, -247, -246, -245, -244, -243, -242, -241, -240, -239, -238, -237, -236, -235, -234, -233, -232, -230, -229, -228, -227, -226, -225, -224, -223, -222, -221, -220, -219, -218, -217, -216, -215, -214, -213, -212, -211, -210, -209, -208, -207, -206, -205, -204, -202, -201, -200, -199, -198, -197, -196, -195, -194, -193, -192, -191, -190, -189, -188, -187, -186, -185, -184, -183, -182, -181, -180, -179, -178, -177, -176, -175, -174, -173, -172, -171, -170, -169, -168, -166, -165, -164, -163, -162, -161, -160, -159, -158, -157, -156, -155, -154, -153, -152, -151, -150, -149, -148, -147, -146, -145, -144, -143, -142, -141, -140, -138, -137, -136, -135, -134, -133, -132, -131, -130, -126, -125, -124, -123, -122, -121, -120, -119, -118, -117, -116, -115, -114, -113, -112, -111, -110, -109, -108, -107, -106, -105, -104, -103, -102, -101, -100, -99, -98, -97, -96, -95, -94, -93, -92, -91, -90, -89, -88, -87, -86, -85, -84, -83, -82, -81, -80, -79, -78, -77, -76, -75, -74, -73, -72, -71, -70, -69, -68, -67, -66, -65, -64, -63, -62, -61, -60, -59, -58, -57, -56, -55, -54, -53, -52, -51, -50, -49, -48, -47, -46, -45, -44, -43, -42, -41, -40, -39, -38, -37, -36, -35, -34, -33, -32, -31, -30, -29, -28, -27, -26, -25, -24, -23, -22, -21, -20, -19, -18, -17, -16, -15, -14, -13, -12, -11, -10, -9, -8, -7, -6, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 46, 47, 48, 49, 50, 51, 52, 53, 54, 55, 56, 57, 58, 59, 60, 61, 62, 63, 64, 65, 66, 67, 68, 69, 70, 71, 72, 73, 74, 75, 76, 77, 78, 79, 80, 81, 82, 83, 84, 85, 86, 87, 88, 89, 90, 91, 92, 93, 94, 95, 96, 97, 98, 99, 100, 101, 102, 103, 104, 105, 106, 107, 108, 109, 110, 111, 112, 113, 114, 115, 116, 117, 118, 119, 120, 121, 122, 123, 124, 125, 126, 130, 131, 132, 133, 134, 135, 136, 137, 138, 140, 141, 142, 143, 144, 145, 146, 147, 148, 149, 150, 151, 152, 153, 154, 155, 156, 157, 158, 159, 160, 161, 162, 163, 164, 165, 166, 168, 169, 170, 171, 172, 173, 174, 175, 176, 177, 178, 179, 180, 181, 182, 183, 184, 185, 186, 187, 188, 189, 190, 191, 192, 193, 194, 195, 196, 197, 198, 199, 200, 201, 202, 204, 205, 206, 207, 208, 209, 210, 211, 212, 213, 214, 215, 216, 217, 218, 219, 220, 221, 222, 223, 224, 225, 226, 227, 228, 229, 230, 232, 233, 234, 235, 236, 237, 238, 239, 240, 241, 242, 243, 244, 245, 246, 247, 248, 249, 250 NOTE – Pilot subcarriers (±231, ±203, ±167, ±139), DC subcarriers (0, ±1, ±2, ±3, ±4, ±5) and subcarriers ±127, ±128, ±129 are skipped. |
|  | 2 | 244 | -250, -248, -246, -244, -242, -240, -238, -236, -234, -232, -230, -228, -226, -224, -222, -220, -218, -216, -214, -212, -210, -208, -206, -204, -202, -200, -198, -196, -194, -192, -190, -188, -186, -184, -182, -180, -178, -176, -174, -172, -170, -168, -166, -164, -162, -160, -158, -156, -154, -152, -150, -148, -146, -144, -142, -140, -138, -136, -134, -132, -130, -126, -124, -122, -120, -118, -116, -114, -112, -110, -108, -106, -104, -102, -100, -98, -96, -94, -92, -90, -88, -86, -84, -82, -80, -78, -76, -74, -72, -70, -68, -66, -64, -62, -60, -58, -56, -54, -52, -50, -48, -46, -44, -42, -40, -38, -36, -34, -32, -30, -28, -26, -24, -22, -20, -18, -16, -14, -12, -10, -8, -6, 6, 8, 10, 12, 14, 16, 18, 20, 22, 24, 26, 28, 30, 32, 34, 36, 38, 40, 42, 44, 46, 48, 50, 52, 54, 56, 58, 60, 62, 64, 66, 68, 70, 72, 74, 76, 78, 80, 82, 84, 86, 88, 90, 92, 94, 96, 98, 100, 102, 104, 106, 108, 110, 112, 114, 116, 118, 120, 122, 124, 126, 130, 132, 134, 136, 138, 140, 142, 144, 146, 148, 150, 152, 154, 156, 158, 160, 162, 164, 166, 168, 170, 172, 174, 176, 178, 180, 182, 184, 186, 188, 190, 192, 194, 196, 198, 200, 202, 204, 206, 208, 210, 212, 214, 216, 218, 220, 222, 224, 226, 228, 230, 232, 234, 236, 238, 240, 242, 244, 246, 248, 250 NOTE – Subcarriers 0, ±2, ±4 and ±128 are skipped. |
|  | 4 | 124 | -250, -246, -242, -238, -234, -230, -226, -222, -218, -214, -210, -206, -202, -198, -194, -190, -186, -182, -178, -174, -170, -166, -162, -158, -154, -150, -146, -142, -138, -134, -130, -126, -122, -118, -114, -110, -106, -102, -98, -94, -90, -86, -82, -78, -74, -70, -66, -62, -58, -54, -50, -46, -42, -38, -34, -30, -26, -22, -18, -14, -10, -6, 6, 10, 14, 18, 22, 26, 30, 34, 38, 42, 46, 50, 54, 58, 62, 66, 70, 74, 78, 82, 86, 90, 94, 98, 102, 106, 110, 114, 118, 122, 126, 130, 134, 138, 142, 146, 150, 154, 158, 162, 166, 170, 174, 178, 182, 186, 190, 194, 198, 202, 206, 210, 214, 218, 222, 226, 230, 234, 238, 242, 246, 250 NOTE – Subcarriers ±2 are skipped. |
| 80+80 MHz | 1 | 468 | -122(L), -121(L), -120(L), -119(L), -118(L), -117(L), -116(L), -115(L), -114(L), -113(L), -112(L), -111(L), -110(L), -109(L), -108(L), -107(L), -106(L), -105(L), -104(L), -102(L), -101(L), -100(L), -99(L), -98(L), -97(L), -96(L), -95(L), -94(L), -93(L), -92(L), -91(L), -90(L), -89(L), -88(L), -87(L), -86(L), -85(L), -84(L), -83(L), -82(L), -81(L), -80(L), -79(L), -78(L), -77(L), -76(L), -74(L), -73(L), -72(L), -71(L), -70(L), -69(L), -68(L), -67(L), -66(L), -65(L), -64(L), -63(L), -62(L), -61(L), -60(L), -59(L), -58(L), -57(L), -56(L), -55(L), -54(L), -53(L), -52(L), -51(L), -50(L), -49(L), -48(L), -47(L), -46(L), -45(L), -44(L), -43(L), -42(L), -41(L), -40(L), -38(L), -37(L), -36(L), -35(L), -34(L), -33(L), -32(L), -31(L), -30(L), -29(L), -28(L), -27(L), -26(L), -25(L), -24(L), -23(L), -22(L), -21(L), -20(L), -19(L), -18(L), -17(L), -16(L), -15(L), -14(L), -13(L), -12(L), -10(L), -9(L), -8(L), -7(L), -6(L), -5(L), -4(L), -3(L), -2(L), 2(L), 3(L), 4(L), 5(L), 6(L), 7(L), 8(L), 9(L), 10(L), 12(L), 13(L), 14(L), 15(L), 16(L), 17(L), 18(L), 19(L), 20(L), 21(L), 22(L), 23(L), 24(L), 25(L), 26(L), 27(L), 28(L), 29(L), 30(L), 31(L), 32(L), 33(L), 34(L), 35(L), 36(L), 37(L), 38(L), 40(L), 41(L), 42(L), 43(L), 44(L), 45(L), 46(L), 47(L), 48(L), 49(L), 50(L), 51(L), 52(L), 53(L), 54(L), 55(L), 56(L), 57(L), 58(L), 59(L), 60(L), 61(L), 62(L), 63(L), 64(L), 65(L), 66(L), 67(L), 68(L), 69(L), 70(L), 71(L), 72(L), 73(L), 74(L), 76(L), 77(L), 78(L), 79(L), 80(L), 81(L), 82(L), 83(L), 84(L), 85(L), 86(L), 87(L), 88(L), 89(L), 90(L), 91(L), 92(L), 93(L), 94(L), 95(L), 96(L), 97(L), 98(L), 99(L), 100(L), 101(L), 102(L), 104(L), 105(L), 106(L), 107(L), 108(L), 109(L), 110(L), 111(L), 112(L), 113(L), 114(L), 115(L), 116(L), 117(L), 118(L), 119(L), 120(L), 121(L), 122(L), -122(H), -121(H), -120(H), -119(H), -118(H), -117(H), -116(H), -115(H), -114(H), -113(H), -112(H), -111(H), -110(H), -109(H), -108(H), -107(H), -106(H), -105(H), -104(H), -102(H), -101(H), -100(H), -99(H), -98(H), -97(H), -96(H), -95(H), -94(H), -93(H), -92(H), -91(H), -90(H), -89(H), -88(H), -87(H), -86(H), -85(H), -84(H), -83(H), -82(H), -81(H), -80(H), -79(H), -78(H), -77(H), -76(H), -74(H), -73(H), -72(H), -71(H), -70(H), -69(H), -68(H), -67(H), -66(H), -65(H), -64(H), -63(H), -62(H), -61(H), -60(H), -59(H), -58(H), -57(H), -56(H), -55(H), -54(H), -53(H), -52(H), -51(H), -50(H), -49(H), -48(H), -47(H), -46(H), -45(H), -44(H), -43(H), -42(H), -41(H), -40(H), -38(H), -37(H), -36(H), -35(H), -34(H), -33(H), -32(H), -31(H), -30(H), -29(H), -28(H), -27(H), -26(H), -25(H), -24(H), -23(H), -22(H), -21(H), -20(H), -19(H), -18(H), -17(H), -16(H), -15(H), -14(H), -13(H), -12(H), -10(H), -9(H), -8(H), -7(H), -6(H), -5(H), -4(H), -3(H), -2(H), 2(H), 3(H), 4(H), 5(H), 6(H), 7(H), 8(H), 9(H), 10(H), 12(H), 13(H), 14(H), 15(H), 16(H), 17(H), 18(H), 19(H), 20(H), 21(H), 22(H), 23(H), 24(H), 25(H), 26(H), 27(H), 28(H), 29(H), 30(H), 31(H), 32(H), 33(H), 34(H), 35(H), 36(H), 37(H), 38(H), 40(H), 41(H), 42(H), 43(H), 44(H), 45(H), 46(H), 47(H), 48(H), 49(H), 50(H), 51(H), 52(H), 53(H), 54(H), 55(H), 56(H), 57(H), 58(H), 59(H), 60(H), 61(H), 62(H), 63(H), 64(H), 65(H), 66(H), 67(H), 68(H), 69(H), 70(H), 71(H), 72(H), 73(H), 74(H), 76(H), 77(H), 78(H), 79(H), 80(H), 81(H), 82(H), 83(H), 84(H), 85(H), 86(H), 87(H), 88(H), 89(H), 90(H), 91(H), 92(H), 93(H), 94(H), 95(H), 96(H), 97(H), 98(H), 99(H), 100(H), 101(H), 102(H), 104(H), 105(H), 106(H), 107(H), 108(H), 109(H), 110(H), 111(H), 112(H), 113(H), 114(H), 115(H), 116(H), 117(H), 118(H), 119(H), 120(H), 121(H), 122(H) NOTE – Subcarrier x(L) denotes subcarrier index x in the frequency segment lower in frequency, and subcarrier x(H) denotes subcarrier index x in the frequency segment higer in frequency. NOTE – Pilot subcarriers (±103, ±75, ±39, ±11) and DC subcarriers (0, ±1) are skipped in each frequency segment. |
|  | 2 | 244 | -122(L), -120(L), -118(L), -116(L), -114(L), -112(L), -110(L), -108(L), -106(L), -104(L), -102(L), -100(L), -98(L), -96(L), -94(L), -92(L), -90(L), -88(L), -86(L), -84(L), -82(L), -80(L), -78(L), -76(L), -74(L), -72(L), -70(L), -68(L), -66(L), -64(L), -62(L), -60(L), -58(L), -56(L), -54(L), -52(L), -50(L), -48(L), -46(L), -44(L), -42(L), -40(L), -38(L), -36(L), -34(L), -32(L), -30(L), -28(L), -26(L), -24(L), -22(L), -20(L), -18(L), -16(L), -14(L), -12(L), -10(L), -8(L), -6(L), -4(L), -2(L), 2(L), 4(L), 6(L), 8(L), 10(L), 12(L), 14(L), 16(L), 18(L), 20(L), 22(L), 24(L), 26(L), 28(L), 30(L), 32(L), 34(L), 36(L), 38(L), 40(L), 42(L), 44(L), 46(L), 48(L), 50(L), 52(L), 54(L), 56(L), 58(L), 60(L), 62(L), 64(L), 66(L), 68(L), 70(L), 72(L), 74(L), 76(L), 78(L), 80(L), 82(L), 84(L), 86(L), 88(L), 90(L), 92(L), 94(L), 96(L), 98(L), 100(L), 102(L), 104(L), 106(L), 108(L), 110(L), 112(L), 114(L), 116(L), 118(L), 120(L), 122(L), -122(H), -120(H), -118(H), -116(H), -114(H), -112(H), -110(H), -108(H), -106(H), -104(H), -102(H), -100(H), -98(H), -96(H), -94(H), -92(H), -90(H), -88(H), -86(H), -84(H), -82(H), -80(H), -78(H), -76(H), -74(H), -72(H), -70(H), -68(H), -66(H), -64(H), -62(H), -60(H), -58(H), -56(H), -54(H), -52(H), -50(H), -48(H), -46(H), -44(H), -42(H), -40(H), -38(H), -36(H), -34(H), -32(H), -30(H), -28(H), -26(H), -24(H), -22(H), -20(H), -18(H), -16(H), -14(H), -12(H), -10(H), -8(H), -6(H), -4(H), -2(H), 2(H), 4(H), 6(H), 8(H), 10(H), 12(H), 14(H), 16(H), 18(H), 20(H), 22(H), 24(H), 26(H), 28(H), 30(H), 32(H), 34(H), 36(H), 38(H), 40(H), 42(H), 44(H), 46(H), 48(H), 50(H), 52(H), 54(H), 56(H), 58(H), 60(H), 62(H), 64(H), 66(H), 68(H), 70(H), 72(H), 74(H), 76(H), 78(H), 80(H), 82(H), 84(H), 86(H), 88(H), 90(H), 92(H), 94(H), 96(H), 98(H), 100(H), 102(H), 104(H), 106(H), 108(H), 110(H), 112(H), 114(H), 116(H), 118(H), 120(H), 122(H) |
|  | 4 | 124 | -122(L), -118(L), -114(L), -110(L), -106(L), -102(L), -98(L), -94(L), -90(L), -86(L), -82(L), -78(L), -74(L), -70(L), -66(L), -62(L), -58(L), -54(L), -50(L), -46(L), -42(L), -38(L), -34(L), -30(L), -26(L), -22(L), -18(L), -14(L), -10(L), -6(L), -2(L), 2(L), 6(L), 10(L), 14(L), 18(L), 22(L), 26(L), 30(L), 34(L), 38(L), 42(L), 46(L), 50(L), 54(L), 58(L), 62(L), 66(L), 70(L), 74(L), 78(L), 82(L), 86(L), 90(L), 94(L), 98(L), 102(L), 106(L), 110(L), 114(L), 118(L), 122(L), -122(H), -118(H), -114(H), -110(H), -106(H), -102(H), -98(H), -94(H), -90(H), -86(H), -82(H), -78(H), -74(H), -70(H), -66(H), -62(H), -58(H), -54(H), -50(H), -46(H), -42(H), -38(H), -34(H), -30(H), -26(H), -22(H), -18(H), -14(H), -10(H), -6(H), -2(H), 2(H), 6(H), 10(H), 14(H), 18(H), 22(H), 26(H), 30(H), 34(H), 38(H), 42(H), 46(H), 50(H), 54(H), 58(H), 62(H), 66(H), 70(H), 74(H), 78(H), 82(H), 86(H), 90(H), 94(H), 98(H), 102(H), 106(H), 110(H), 114(H), 118(H), 122(H) |

The Average SNR of Space-Time Stream i subfield in the Table 7-X3 (VHT Compressed Beamforming Report Field) is an 8-bit two’s complement integer whose definition is shown in Table 7-X5.

Table 7-X5 – Average SNR of Space-Time Stream s subfield

| Average SNR of space-time stream i subfield |  |
| --- | --- |
| -128 | -10 dB |
| -127 | -9.75 dB |
| -126 | -9.5 dB |
| … | … |
| +126 | 53.5 dB |
| +127 | 53.75 dB |

The  in Table 7-X5 is given by Equation (7-X1).

(7-X1)

Here,

(7-X2)

is the predicted SNR for the space-time stream i at subcarrier k assuming that the beamformer applies the compressed beamforming feedback matrix V, where

is the estimated MIMO channel for subcarrier k

is column i of the compressed beamforming feedback matrix V for subcarrier k

is the noise plus interference power measured at the beamformee

If the size of the VHT Compressed Beamforming Report field shown in Table 7-X3 is not an integral multiple of 8 bits, up to 7 zeros are appended to the end of the field to make its size an integral multiple of 8 bits.

When operating with a 40MHz, 80MHz, and 160MHz channel width, feedback with BW=20MHz corresponds to the tones in the primary 20 MHz channel.

When operating with an 80MHz and 160MHz channel width, feedback with BW=40MHz corresponds to the tones in the primary 40 MHz channel.

When operating with a an 80+80 MHz or 160MHz channel width, feedback with BW=80MHz corresponds to the tones in the primary 80 MHz channel.

#### 7.3.1.33 MU Exclusive Beamforming Report field

The MU Exclusive Beamforming Report field is included in the VHT Compressed Beamforming frame if the Feedback Type subfield in the VHT MIMO Control field is set to MU (see Error: Reference source not found7.3.1.31).

The format of the MU Exclusive Beamforming Report field is shown in Table 7-11 – MU Exclusive Beamforming Report fieldTable 7-X6 and consists of Delta SNR subfields for each space-time stream (1 through Nc) ofover a subset of the subcarriers spaced Ng’ = 2Ng apart, where Ng is given by the Grouping subfield in the VHT MIMO Control field (See 7.3.1.31) starting from the lowest frequency subcarrier and continuing to the highest frequency subcarrier. The subset of subcarriers included is determined by the values of the Channel Width and Grouping subfields of the VHT MIMO Control field as listed in Table 7-12. For each subcarrier included, the deviation in dB of the SNR of that subcarrier for each column of V relative to the average SNR of the corresponding space-time stream is computed using Equation (7-X3).

where

is the estimated MIMO channel for subcarrier k

is column i of the beamforming matrix  for subcarrier k

is the noise plus interference power measured at the beamformee

is the average SNR of space-time stream i reported in the VHT Compressed Beamforming Report field (SNR in space-time stream i field)

where

is given by Equation (7-X2)

is given by Equation (7-X1)

Each Delta SNR subfield contains the  computed using Equation (7-X3) and quantized to 4 bits in the range -8 dB to 7 dB and with 1 dB granularity.  Table 7-11 shows the structure of the MU Exclusive Beamforming Report field.

Table 7-11 – MU Exclusive Beamforming Report field

| Subfield | Size (Bits) | Meaning |
| --- | --- | --- |
| Delta SNR for space-time stream 1 for subcarrier k = scidx(0) | 4 | as defined in Equation (7-X3) |
| … | … | … |
| Delta SNR for space-time stream Nc for subcarrier k = scidx(0) | 4 | as defined in Equation (7- X3) |
| Delta SNR for space-time stream 1 for subcarrier k = scidx(1) | 4 | as defined in Equation (7- X3) |
| … | … | … |
| Delta SNR for space-time stream Nc for subcarrier k = scidx(1) | 4 | as defined in Equation (7- X3) |
| … | … | … |
| Delta SNR for space-time stream 1 for subcarrier k = scidx(Ns’-1) | 4 | as defined in Equation (7- X3) |
| … | … | … |
| Delta SNR for space-time stream Nc for subcarrier k = scidx(Ns’-1) | 4 | as defined in Equation (7- X3) |

In Table 7-11, Ns’ is the number of subcarriers for which the Delta SNR subfield is sent back to the beamformer.  Table 7-12 shows Ns’, the exact subcarrier indices and their order for which the Delta SNR is sent back.

Table 7-12 – Subcarrier indices for which the Delta SNR subfield is sent

| Channel width | Ng’ | Ns’ | Subcarriers for which the Delta SNR subfield is sent: scidx(0), scidx(1), … scidx(Ns’-1) |
| --- | --- | --- | --- |
| 20 MHz | 2 | 30 | -28, -26, -24, -22, -20, -18, -16, -14, -12, -10, -8, -6, -4, -2, -1, 1, 2, 4, 6, 8, 10, 12, 14, 16, 18, 20, 22, 24, 26, 28 |
|  | 4 | 16 | -28, -24, -20, -16, -12, -8, -4, -1, 1, 4, 8, 12, 16, 20, 24, 28 |
|  | 8 | 10 | -28, -20, -12, -4, -1, 1, 4, 12, 20, 28 |
| 40 MHz | 2 | 58 | -58, -56, -54, -52, -50, -48, -46, -44, -42, -40, -38, -36, -34, -32, -30, -28, -26, -24, -22, -20, -18, -16, -14, -12, -10, -8, -6, -4,-2, 2, 4, 6, 8, 10, 12, 14, 16, 18, 20, 22, 24, 26, 28, 30, 32, 34, 36, 38, 40, 42, 44, 46, 48, 50, 52, 54, 56, 58 |
|  | 4 | 30 | -58, -54, -50, -46, -42, -38, -34, -30, -26, -22, -18, -14, -10, -6,-2, 2, 6, 10, 14, 18, 22, 26, 30, 34, 38, 42, 46, 50, 54, 58 |
|  | 8 | 16 | -58, -50, -42, -34, -26, -18, -10, -2, 2, 10, 18, 26, 34, 42, 50, 58 |
| 80 MHz | 2 | 122 | -122, -120, -118, -116, -114, -112, -110, -108, -106, -104, -102, -100, -98, -96, -94, -92, -90, -88, -86, -84, -82, -80, -78, -76, -74, -72, -70, -68, -66, -64, -62, -60, -58, -56, -54, -52, -50, -48, -46, -44, -42, -40, -38, -36, -34, -32, -30, -28, -26, -24, -22, -20, -18, -16, -14, -12, -10, -8, -6, -4, -2, 2, 4, 6, 8, 10, 12, 14, 16, 18, 20, 22, 24, 26, 28, 30, 32, 34, 36, 38, 40, 42, 44, 46, 48, 50, 52, 54, 56, 58, 60, 62, 64, 66, 68, 70, 72, 74, 76, 78, 80, 82, 84, 86, 88, 90, 92, 94, 96, 98, 100, 102, 104, 106, 108, 110, 112, 114, 116, 118, 120, 122 |
|  | 4 | 62 | -122, -118, -114, -110, -106, -102, -98, -94, -90, -86, -82, -78, -74, -70, -66, -62, -58, -54, -50, -46, -42, -38, -34, -30, -26, -22, -18, -14, -10, -6, -2, 2, 6, 10, 14, 18, 22, 26, 30, 34, 38, 42, 46, 50, 54, 58, 62, 66, 70, 74, 78, 82, 86, 90, 94, 98, 102, 106, 110, 114, 118, 122 |
|  | 8 | 32 | -122, -114, -106, -98, -90, -82, -74, -66, -58, -50, -42, -34, -26, -18, -10, -2, 2, 10, 18, 26, 34, 42, 50, 58, 66, 74, 82, 90, 98, 106, 114, 122 |
| 160 MHz | 2 | 244 | -250, -248, -246, -244, -242, -240, -238, -236, -234, -232, -230, -228, -226, -224, -222, -220, -218, -216, -214, -212, -210, -208, -206, -204, -202, -200, -198, -196, -194, -192, -190, -188, -186, -184, -182, -180, -178, -176, -174, -172, -170, -168, -166, -164, -162, -160, -158, -156, -154, -152, -150, -148, -146, -144, -142, -140, -138, -136, -134, -132, -130, -126, -124, -122, -120, -118, -116, -114, -112, -110, -108, -106, -104, -102, -100, -98, -96, -94, -92, -90, -88, -86, -84, -82, -80, -78, -76, -74, -72, -70, -68, -66, -64, -62, -60, -58, -56, -54, -52, -50, -48, -46, -44, -42, -40, -38, -36, -34, -32, -30, -28, -26, -24, -22, -20, -18, -16, -14, -12, -10, -8, -6, 6, 8, 10, 12, 14, 16, 18, 20, 22, 24, 26, 28, 30, 32, 34, 36, 38, 40, 42, 44, 46, 48, 50, 52, 54, 56, 58, 60, 62, 64, 66, 68, 70, 72, 74, 76, 78, 80, 82, 84, 86, 88, 90, 92, 94, 96, 98, 100, 102, 104, 106, 108, 110, 112, 114, 116, 118, 120, 122, 124, 126, 130, 132, 134, 136, 138, 140, 142, 144, 146, 148, 150, 152, 154, 156, 158, 160, 162, 164, 166, 168, 170, 172, 174, 176, 178, 180, 182, 184, 186, 188, 190, 192, 194, 196, 198, 200, 202, 204, 206, 208, 210, 212, 214, 216, 218, 220, 222, 224, 226, 228, 230, 232, 234, 236, 238, 240, 242, 244, 246, 248, 250 NOTE – Subcarriers 0, ±2, ±4 and ±128 are skipped. |
|  | 4 | 124 | -250, -246, -242, -238, -234, -230, -226, -222, -218, -214, -210, -206, -202, -198, -194, -190, -186, -182, -178, -174, -170, -166, -162, -158, -154, -150, -146, -142, -138, -134, -130, -126, -122, -118, -114, -110, -106, -102, -98, -94, -90, -86, -82, -78, -74, -70, -66, -62, -58, -54, -50, -46, -42, -38, -34, -30, -26, -22, -18, -14, -10, -6, 6, 10, 14, 18, 22, 26, 30, 34, 38, 42, 46, 50, 54, 58, 62, 66, 70, 74, 78, 82, 86, 90, 94, 98, 102, 106, 110, 114, 118, 122, 126, 130, 134, 138, 142, 146, 150, 154, 158, 162, 166, 170, 174, 178, 182, 186, 190, 194, 198, 202, 206, 210, 214, 218, 222, 226, 230, 234, 238, 242, 246, 250 NOTE – Subcarriers ±2 are skipped. |
|  | 8 | 64 | -250, -242, -234, -226, -218, -210, -202, -194, -186, -178, -170, -162, -154, -146, -138, -130, -126, -118, -110, -102, -94, -86, -78, -70, -62, -54, -46, -38, -30, -22, -14, -6, 6, 14, 22, 30, 38, 46, 54, 62, 70, 78, 86, 94, 102, 110, 118, 126, 130, 138, 146, 154, 162, 170, 178, 186, 194, 202, 210, 218, 226, 234, 242, 250 |
| 80+80 MHz | 2 | 244 | -122(L), -120(L), -118(L), -116(L), -114(L), -112(L), -110(L), -108(L), -106(L), -104(L), -102(L), -100(L), -98(L), -96(L), -94(L), -92(L), -90(L), -88(L), -86(L), -84(L), -82(L), -80(L), -78(L), -76(L), -74(L), -72(L), -70(L), -68(L), -66(L), -64(L), -62(L), -60(L), -58(L), -56(L), -54(L), -52(L), -50(L), -48(L), -46(L), -44(L), -42(L), -40(L), -38(L), -36(L), -34(L), -32(L), -30(L), -28(L), -26(L), -24(L), -22(L), -20(L), -18(L), -16(L), -14(L), -12(L), -10(L), -8(L), -6(L), -4(L), -2(L), 2(L), 4(L), 6(L), 8(L), 10(L), 12(L), 14(L), 16(L), 18(L), 20(L), 22(L), 24(L), 26(L), 28(L), 30(L), 32(L), 34(L), 36(L), 38(L), 40(L), 42(L), 44(L), 46(L), 48(L), 50(L), 52(L), 54(L), 56(L), 58(L), 60(L), 62(L), 64(L), 66(L), 68(L), 70(L), 72(L), 74(L), 76(L), 78(L), 80(L), 82(L), 84(L), 86(L), 88(L), 90(L), 92(L), 94(L), 96(L), 98(L), 100(L), 102(L), 104(L), 106(L), 108(L), 110(L), 112(L), 114(L), 116(L), 118(L), 120(L), 122(L), -122(H), -120(H), -118(H), -116(H), -114(H), -112(H), -110(H), -108(H), -106(H), -104(H), -102(H), -100(H), -98(H), -96(H), -94(H), -92(H), -90(H), -88(H), -86(H), -84(H), -82(H), -80(H), -78(H), -76(H), -74(H), -72(H), -70(H), -68(H), -66(H), -64(H), -62(H), -60(H), -58(H), -56(H), -54(H), -52(H), -50(H), -48(H), -46(H), -44(H), -42(H), -40(H), -38(H), -36(H), -34(H), -32(H), -30(H), -28(H), -26(H), -24(H), -22(H), -20(H), -18(H), -16(H), -14(H), -12(H), -10(H), -8(H), -6(H), -4(H), -2(H), 2(H), 4(H), 6(H), 8(H), 10(H), 12(H), 14(H), 16(H), 18(H), 20(H), 22(H), 24(H), 26(H), 28(H), 30(H), 32(H), 34(H), 36(H), 38(H), 40(H), 42(H), 44(H), 46(H), 48(H), 50(H), 52(H), 54(H), 56(H), 58(H), 60(H), 62(H), 64(H), 66(H), 68(H), 70(H), 72(H), 74(H), 76(H), 78(H), 80(H), 82(H), 84(H), 86(H), 88(H), 90(H), 92(H), 94(H), 96(H), 98(H), 100(H), 102(H), 104(H), 106(H), 108(H), 110(H), 112(H), 114(H), 116(H), 118(H), 120(H), 122(H) |
|  | 4 | 124 | -122(L), -118(L), -114(L), -110(L), -106(L), -102(L), -98(L), -94(L), -90(L), -86(L), -82(L), -78(L), -74(L), -70(L), -66(L), -62(L), -58(L), -54(L), -50(L), -46(L), -42(L), -38(L), -34(L), -30(L), -26(L), -22(L), -18(L), -14(L), -10(L), -6(L), -2(L), 2(L), 6(L), 10(L), 14(L), 18(L), 22(L), 26(L), 30(L), 34(L), 38(L), 42(L), 46(L), 50(L), 54(L), 58(L), 62(L), 66(L), 70(L), 74(L), 78(L), 82(L), 86(L), 90(L), 94(L), 98(L), 102(L), 106(L), 110(L), 114(L), 118(L), 122(L), -122(H), -118(H), -114(H), -110(H), -106(H), -102(H), -98(H), -94(H), -90(H), -86(H), -82(H), -78(H), -74(H), -70(H), -66(H), -62(H), -58(H), -54(H), -50(H), -46(H), -42(H), -38(H), -34(H), -30(H), -26(H), -22(H), -18(H), -14(H), -10(H), -6(H), -2(H), 2(H), 6(H), 10(H), 14(H), 18(H), 22(H), 26(H), 30(H), 34(H), 38(H), 42(H), 46(H), 50(H), 54(H), 58(H), 62(H), 66(H), 70(H), 74(H), 78(H), 82(H), 86(H), 90(H), 94(H), 98(H), 102(H), 106(H), 110(H), 114(H), 118(H), 122(H) |
|  | 8 | 64 | -122(L), -114(L), -106(L), -98(L), -90(L), -82(L), -74(L), -66(L), -58(L), -50(L), -42(L), -34(L), -26(L), -18(L), -10(L), -2(L), 2(L), 10(L), 18(L), 26(L), 34(L), 42(L), 50(L), 58(L), 66(L), 74(L), 82(L), 90(L), 98(L), 106(L), 114(L), 122(L), -122(H), -114(H), -106(H), -98(H), -90(H), -82(H), -74(H), -66(H), -58(H), -50(H), -42(H), -34(H), -26(H), -18(H), -10(H), -2(H), 2(H), 10(H), 18(H), 26(H), 34(H), 42(H), 50(H), 58(H), 66(H), 74(H), 82(H), 90(H), 98(H), 106(H), 114(H), 122(H) |

Table 7-11—MU Exclusive Beamforming Report field

| Field | Size (Bits) | Meaning |
| --- | --- | --- |
| Delta SNR for space-time stream 1 for carrier at negative band edge | 4 | in dB with range -8 dB to 7 dB with 1 dB granularity for and |
| … |  |  |
| Delta SNR for space-time stream Nc for carrier at negative band edge | 4 | in dB with range -8 dB to 7 dB with 1 dB granularity for and |
| Delta SNR for space-time stream 1 for carrier at negative band edge + Ng’ | 4 | in dB from -8 dB to 7 dB with 1 dB granularity for and |
| … |  |  |
| Delta SNR for space-time stream Nc for carrier at negative band edge + Ng’ | 4 | in dB from -8 dB to 7 dB with 1 dB granularity for and |
| … |  |  |
| Delta SNR for space-time stream 1 for carrier at negative band edge + (m-1)Ng’ | 4 | in dB from -8 dB to 7 dB with 1 dB granularity for and |
| … |  |  |
| Delta SNR for space-time stream Nc for carrier at negative band edge + (m-1)Ng’ | 4 | in dB from -8 dB to 7 dB with 1 dB granularity for and |
| Delta SNR for space-time stream 1 for carrier at negative DC edge | 4 | in dB from -8 dB to 7 dB with 1 dB granularity for and |
| … |  |  |
| Delta SNR for space-time stream Nc for carrier at negative DC edge | 4 | in dB from -8 dB to 7 dB with 1 dB granularity for and |
| Delta-SNR for space-time stream 1 for carrier at positive DC edge | 4 | in dB from -8 dB to 7 dB with 1 dB granularity for and |
| … |  |  |
| Delta-SNR for space-time stream Nc for carrier at positive DC edge | 4 | in dB from -8 dB to 7 dB with 1 dB granularity for and |
| Delta-SNR for space-time stream 1 for carrier at positive band edge – (m-1)Ng’ | 4 | in dB from -8 dB to 7 dB with 1 dB granularity for and |
| … |  |  |
| Delta-SNR for space-time stream Nc for carrier at positive band edge – (m-1)Ng’ | 4 | in dB from -8 dB to 7 dB with 1 dB granularity for and |
| … |  |  |
| Delta SNR for space-time stream 1 for carrier at positive band edge - Ng’ | 4 | in dB from -8 dB to 7 dB with 1 dB granularity for and |
| … |  |  |
| Delta SNR for space-time stream Nc for carrier at positive band edge - Ng’ | 4 | in dB from -8 dB to 7 dB with 1 dB granularity for and |
| Delta SNR for space-time stream 1 for carrier at positive band edge | 4 | in dB from -8 dB to 7 dB with 1 dB granularity for and |
| … |  |  |
| Delta SNR for space-time stream Nc for carrier at positive band edge | 4 | in dB from -8 dB to 7 dB with 1 dB granularity for and |

In Table 7-11 – MU Exclusive Beamforming Report field,

where  is the smallest integer greater than or equal to

is the absolute value of the band edge subcarrier index; 28 for 20 MHz, 58 for 40 MHz and 122 for 80 MHz,

is the absolute value of the DC edge subcarrier index; 1 for 20 MHz, 2 for 40 MHz and 2 for 80 MHz,

Ng’=2Ng	where Ng is the value in the Grouping subfield of the VHT MIMO Control field.

Table 7-12--Number of subcarriers and subcarrier mapping

| Channel Width | Grouping (Ng) | Ns | Subcarriers (k) for which is sent |
| --- | --- | --- | --- |
| 20 MHz | 1 | 30 | ±28, ±26, ±24, ±22, ±20, ±18, ±16, ±14, ±12, ±10, ±8, ±6, ±4, ±2, ±1 |
|  | 2 | 16 | ±28, ±24, ±20, ±16, ±12, ±8, ±4, ±1 |
|  | 4 | 10 | ±28, ±20, ±12, ±4, ±1 |
| 40 MHz | 1 | 58 | ±58, ±56, ±54, ±52, ±50, ±48, ±46, ±44, ±42, ±40, ±38, ±36, ±34, ±32, ±30, ±28, ±26, ±24, ±22, ±20, ±18, ±16, ±14, ±12, ±10, ±8, ±6, ±4, ±2 |
|  | 2 | 30 | ±58, ±54, ±50, ±46, ±42, ±38, ±34, ±30, ±26, ±22, ±18, ±14, ±10, ±6, ±2 |
|  | 4 | 16 | ±58, ±50, ±42, ±34, ±26, ±18, ±10, ±2 |
| 80 MHz | 1 | 122 | ±122, ±120, ±118, ±116, ±114, ±112, ±110, ±108, ±106, ±104, ±102, ±100, ±98, ±96, ±94, ±92, ±90, ±88, ±86, ±84, ±82, ±80, ±78, ±76, ±74, ±72, ±70, ±68, ±66, ±64, ±62, ±60, ±58, ±56, ±54, ±52, ±50, ±48, ±46, ±44, ±42, ±40, ±38, ±36, ±34, ±32, ±30, ±28, ±26, ±24, ±22, ±20, ±18, ±16, ±14, ±12, ±10, ±8, ±6, ±4, ±2 |
|  | 2 | 62 | ±122, ±118, ±114, ±110, ±106, ±102, ±98, ±94, ±90, ±86, ±82, ±78, ±74, ±70, ±66, ±62, ±58, ±54, ±50, ±46, ±42, ±38, ±34, ±30, ±26, ±22, ±18, ±14, ±10, ±6, ±2 |
|  | 4 | 32 | ±122, ±114, ±106, ±98, ±90, ±82, ±74, ±66, ±58, ±50, ±42, ±34, ±26, ±18, ±10, ±2 |

For 160 MHz and 80+80 MHz, the subcarrier indices for 80MHz from Table 7-12 are used for each 80MHz frequency segment, with the low frequency segment appearing first, followed by the high frequency segment.