IEEE P802.11
Wireless LANs

| User guide for 802.11ac waveform generator |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-05-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Fei Tong | CSR |  |  | Fei.Tong@csr.com |

Abstract

This document contains the user guide for the command line version of the 802.11ac waveform generator running in MATLAB.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Waveform Generator Description

## Introduction

The proposed IEEE 802.11ac physical layer standard has an enourmous number of different modes and options. In order to reduce the amount of interoperability issues and to assist readers of the specificiation, it has been proposed to add to the standard an implementation of a waveform generator that will be able to generate all the different modes described in the specification.

The proposal and discussions have been posted on the server as document IEEE802.11-11-0295.

# Interface description

## Input to the waveform generation function

We use command line options to configure transmission; this can give a user flexibility to define various specific transmission configurations or use default values. With the exception of PDU input, all input options are strings.

The supported configuration options are listed together in Table 1-3 with valid values. These options correspond to the fields defined in TXVECTOR.

Note: in the following table, x denotes the place-holder for variable value.

PPDU format and transmission bandwidth

Table 1 command options for formats and bandwidth configuration

| Command option name | String Values |
| --- | --- |
| FORMAT | 'Legacy' (NON_HT) ‘NONHT-DUP’ ‘HT_MM’ ‘HT_GF’ ‘VHT’ Default value is VHT |
| BW | ‘BW:x’ x takes value of 20 40 80 (valid only for VHT) 160 (valid only for VHT) 80+80 (valid only for VHT) Default value is 80 |
| MU | ‘MU:x’ x takes value from 1:4 Valid only for VHT Default value is 1 |
| ShortGI | ‘ShortGI:x’, x takes value from 0 normal guard interval 1 short guard interval |
| CH_OFFSET | ‘CH_OFFSET:x’, x takes value from -4, -3, -2, -1, 0, 1, 2, 3, 4 denotes frequency shift with 20MHz step with respect to the carrier centre |

### PDU

PDU input is an array of integer numbers. For MU-MIMO mode, each PDU input is sequentially allocated to user from lowest user index (see detailed examples in later section).

MIMO configuration

Table 2 command options for MIMO configuration

| Command option name | String Values |
| --- | --- |
| N_tx | ‘N_tx:x’, x takes value from 1,8 |
| STBC | ‘STBC:x’, x takes value from 0,1 for VHT |
| N_ss | For SU, ‘N_ss:x’, x takes value from 1..8; For MU, ‘N_ss:x,x,x,x’, x takes value from 1-4, then number of x equals to the number of user |

FEC configuration

Table 3 command options for FEC configuration

| Command option name | String Values |
| --- | --- |
| MCS | For SU, ‘MCS:x’, x takes value from 0-7 for NON_HT 0-76 for HT_MM and HT_GF 0-9 for VHT For MU, ‘MCS:x,x,x,x’, number of x equals to number of users |
| LDPC_CODING | For SU, ‘LDPC_CODING:x’, x takes value from 0, BCC encoding 1, LDPC encoding For MU, ‘LDPC_CODING:x,x,x,x’, number of x equals to number of users |
| Scram_seed | For SU, ‘Scram_seed:x’, x takes value between 1 – 127 For MU, ‘Scram_seed: :x,x,x,x’, number of x equals to number of users |
| AddCRC | ‘AddCRC:x’, x takes value from 0: no CRC added 1: add CRC in PDU payload |

## Output of the waveform generation function

### Baseband waveform

The first output of the waveform generation function is a cell array containing baseband waveforms. Each element in the array contains the baseband waveform for one band segment. The maximum number of elements should be 2 for 80+80 bandwidth option.

Each cell element contains a baseband waveform of a particular segment bandwidth. It has a dimension of Ntx x M, where Ntx denotes the number of Tx antennas, M denotes the number of time-domain samples of the baseband waveform.

### Test-vector structure

The second output of the waveform generation function is a test vector structure, which contains the bits or symbols logged at intermediatory observation points along the transmitter processing chain. In the following, all the fields and sub-fields contained in the test vector structure are explained.

L_STF field

| Sub-fields name | Data and format |
| --- | --- |
| l_stf_t | Nx2 cell array, each cell element (n,k) contains the 4 us time domain waveform of L-STF fields in nth segment and kth L-STF field |

L_LTF field

| Sub-fields name | Data and format |
| --- | --- |
| l_ltf_t | Nx2 cell array, each cell element (n,k) contains the 4 us time domain waveform of L-LTF fields in nth segment and kth L-LTF field |

L_SIG field

| Sub-fields name | Data and format |
| --- | --- |
| l_sig_bits | Bit sequence of L-SIG field |
| l_sig_conv | Encoded bit sequence of L-SIG field |
| l_sig_ilv | Interleaved bit sequence of L-SIG field |
| l_sig_fd | Symbols carried on 64 sub-carriers |
| l_sig_fbfdtx | Symbols carried on sub-carriers of the whole bandwidth |

V_SIGA field

| Sub-fields name | Data and format |
| --- | --- |
| v_siga_bits | 24x2, each column contains the bit sequence of kth VHT-SIGA field |
| v_siga_conv | Encoded bit sequence of VHT-SIGA field |
| v_siga_ilv | Interleaved bit sequence of kth VHT-SIGA field |
| v_siga_fd | Symbols carried on all sub-carriers of kth VHT-SIGA field |

V_STF field

| Sub-fields name | Data and format |
| --- | --- |
| v_stf_t | Nx1 cell array, each cell element contains the 4 us time domain waveform of VHT-STF fields in nth segment and kth VHT-STF field |

L_LTF field

| Sub-fields name | Data and format |
| --- | --- |
| v_ltf_t | NxM cell array, each cell element (n,m) contains the 4 us time domain waveform of VHT-LTF fields in nth segment and mth L-LTF field |
| v_ltf_fd | NxM cell array, each cell element (n,m) contains the frequency domain signal over the whole band of nth segment and mth VHT-LTF fields |

V_SIGB field

| Sub-fields name | Data and format |
| --- | --- |
| v_sigb_bits | Bit sequence of VHT-SIGB field |
| v_sigb_crc | CRC of SIGB field |
| v_sigb_conv | Encoded bit sequence of VHT-SIGB field |
| v_sig_ilv | Interleaved bit sequence of VHT-SIGB field |
| v_sig_fd | Frequency domain signal over the whole band |
| v_sigb_fdsts | Nsts x N array, each row contains the frequency domain signal over the whole band of a space-time stream |

## Running the generator

This code has been tested with MATLAB v7.4. If you have any problems running it with different versions of MATLAB please let us know.

Before running

- You need to unzip the package ieee_tx11ac.zip attached in this document onto a folder X.

- With folder X as current folder, you need to run Matlab script build_mex_11ac to compile the mex files

- To run waveform generator in any folder, you need to include folder X and folder X/parity_ck in the path variable

Generate waveform

The function tx_11ac  generates the baseband waveform. The following example generates the baseband waveform for VHT mode single TX and single stream MCS 120MHz bandwidth baseband waveform.

Examples:

%Single user 20MHz VHT transmission

pdu_dat = 1:1000;

[sbb_str, tv_str] = tx_11ac('VHT', 'BW:VHT20M', 'MCS:1', pdu_dat, 'N_ss:1');

%Single user 80+80MHz VHT transmission

pdu_dat = 1:1000;

[sbb_str, tv_str] = tx_11ac('VHT', 'BW:VHT20M', 'MCS:1', pdu_dat, 'N_ss:1');

%Two users 20MHz VHT transmission

pdu_dat_user1 = 1:1000;

pdu_dat_user2 = 1:1000;

[sbb_str, tv_str] = tx_11ac('MU:2', 'VHT', 'BW:VHT20M', 'MCS:1,2', pdu_dat_user1, pdu_dat_user2, 'N_ss:1,2', 'N_tx:3');

## Codes